| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0096-v1 |
| --- | --- |

| Title: | [AHG9] On miscellaneous aspects of NNPFGC and NNPFGA |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Junghak Nam Seung hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes several modifications that are asserted to provide improvement to some aspects of the NNPFGC and NNPFGA SEI messages design.

# On complexity information signalling

## Problem statement

The current NNPFGC design includes signalling of complexity information in which the semantics are shown below:

nnpfgc_complexity_info_present_flag, nnpfgc_parameter_type_idc, nnpfgc_log2_parameter_bit_length_minus3, nnpfgc_num_parameters_idc, nnpfgc_num_kmac_operations_idc, and nnpfgc_total_kilobyte_size have the semantics of nnpfc_complexity_info_present_flag, nnpfc_parameter_type_idc, nnpfc_log2_parameter_bit_length_minus3, nnpfc_num_parameters_idc, nnpfc_num_kmac_operations_idc, and nnpfc_total_kilobyte_size, respectively, but with the exception that the semantics are specified for the NNPF group defined by this SEI message rather than the NNPF defined by an NNPFC SEI message. When nnpfgc_grouping_type is equal to 1, nnpfgc_complexity_info_present_flag shall be equal to 0.

It is asserted that the above semantics have the following problems:

- It is constrained that the value of nnpfgc_complexity_info_present_flag shall be equal to 0 when nnpfgc_grouping_type is equal to 1 (i.e., alternative grouping). This constraint is unnecessary since the signalling can still be present in such situation as long as the signalled complexity is the max complexity of any NNPF in the NNPFGC.

- It should be clear that the complexity information signalling, if present, in an NNPFGC shall represent the maximum / highest complexity for invoking the NNPFGC for any combinations of NNPFs in the NNPFGC. This is important considering that in complex case where an NNPFGC nnpfgcA has member that is another NNPFGC nnpfgcB, there can be more than one possible set of NNPFs that are executed when the nnpfgcA is activated. For example, consider that nnpfgcA {cascade: nnpf1, nnpf2, nnpfgcB {alternative: nnpf3, nnpf4}}. When nnpfgcA is activated, decoder may actually executed the following {nnpf1 then nnpf2 then nnpf3} or {nnpf1 then nnpf2 then nnpf4}. When complexity signalling is present in nnpfgcA, it shall be the max complexity of these two sets of filters.

## Proposal

We propose the following modification to overcome the problems described above:

- Remove the constraint such that when nnpfgc_grouping_type is equal to 1, nnpfgc_complexity_info_present_flag shall be equal to 0.

- Specify that when nnpfgc_grouping_type is equal to 1 and nnpfgc_complexity_info_present_flag is equal to 1, the signalled complexity represents the maximum complexity invoking any of the members in the NNPFGC.

- Specify that when nnpfgc_complexity_info_present_flag is equal to 1, the signalled complexity information represent the maximum complexity for invoking any valid combination from the members of the NNPFGC.

## Text implementation

…

nnpfgc_complexity_info_present_flag, nnpfgc_parameter_type_idc, nnpfgc_log2_parameter_bit_length_minus3, nnpfgc_num_parameters_idc, nnpfgc_num_kmac_operations_idc, and nnpfgc_total_kilobyte_size have the semantics of nnpfc_complexity_info_present_flag, nnpfc_parameter_type_idc, nnpfc_log2_parameter_bit_length_minus3, nnpfc_num_parameters_idc, nnpfc_num_kmac_operations_idc, and nnpfc_total_kilobyte_size, respectively, but with the exception that the semantics are specified for the NNPF group defined by this SEI message rather than the NNPF defined by an NNPFC SEI message. When nnpfgc_grouping_type is equal to 1, nnpfgc_complexity_info_present_flag shall be equal to 0. When nnpfgc_grouping_type is equal to 1 and nnpfgc_complexity_info_present_flag is equal to 1, the signalled complexity information represents the maximum complexity for invoking any of the members in the NNPFGC. When nnpfgc_complexity_info_present_flag is equal to 1, the signalled complexity information represent the maximum complexity for invoking any valid combination from the members when the NNPFGC is activated.

…

# On input pictures signaling for first NNPF in target NNFPGC

## Problem statement

For each NNPF in the target NNPFGC, the signaling information for input pictures needed for the NNPFs are present. Such signalling of input pictures is not needed at all for the first NNPF since it will always exactly be the same as input pictures signaled NNPFC SEI message that contains that NNPF. Such redundancy should be removed.

## Proposal

We propose to omit signalling of input pictures for the first NNPF in the target NNPFGC in the NNPFGA SEI message.

## Text implementation

…

| nn_post_filter_group_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfga_target_id | ue(v) |
| nnpfga_cancel_flag | u(1) |
| if( !nnpfga_cancel_flag ) { |  |
| nnpfga_persistence_flag | u(1) |
| nnpfga_num_filters_minus2 | ue(v) |
| for( i = 0; i <= nnpfga_num_filters_minus2 + 1; i++ ) { |  |
| nnpfga_target_base_flag[ i ] | u(1) |
| if( i > 0 ) { |  |
| nnpfga_input_all_pics_flag[ i ] | u(1) |
| if( !nnpfga_input_all_pics_flag[ i ] ) { |  |
| nnpfga_num_input_pics_minus1[ i ] | ue(v) |
| for( j = 0; j <= nnpfga_num_input_pics_minus1[ i ]; j++ ) |  |
| nnpfga_input_pic_skip_count[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| nnpfga_num_output_entries[ i ] | ue(v) |
| for( j = 0; j < nnpfga_num_output_entries[ i ]; j++ ) |  |
| nnpfga_output_flag[ i ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |

…

nnpfga_input_all_pics_flag[ i ] equal to 1 specifies that the input pictures to the i-th NNPF are selected from the list of candidate input pictures candInputPicList[ i ] without skipping. nnpfga_input_all_pics_flag[ i ] equal to 0 specifies that the input pictures to the i-th NNPF are selected from the list of candidate input pictures candInputPicList[ i ] in a manner that some candidate input pictures are skipped. When not present, the value of nnpfga_input_all_pics_flag[ i ] is inferred to be equal to 1.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).