| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0098-v1 |
| --- | --- |

| Title: | [AHG9] On temporal reversal feature in source picture timing information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Junghak Nam Seung hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

It is asserted that the current methods for assigning the value of SourcePictureTime[ 0 ] in source picture timing information (SPTI) SEI message does not work. While the method of assigning it with externally provided value may work, the other method by setting the source picture timing for that first output picture to be equal to 0 does not work since it is simply wrong.

This contribution proposes two options to overcome the asserted problem:

Option 1: when type includes temporal reversal, allow possibility to explicitly signal the elemental distance between the first picture and the last picture for the temporal reversal and the following applies:

- When the elemental distance between the first picture and the last picture is provided, the SourcePictureTime[ 0 ] is set to this value.

- When the elemental distance between the first picture and the last picture is not provided, it is constrained that the value is provided externally.

- The distance information shall not be present when the SPTI SEI message does not include temporal reversal type.

Option 2: specify a constraint such that when the SPTI SEI message includes temporal reversal picture timing type (i.e., the value of temporalReversalFlag is equal to 1), the value of SourcePictureTime[ 0 ] shall be provided by external means.

# Problem Statement

In the 31st JVET meeting, the first design of source picture timing information (SPTI) SEI message was agreed to be included in the Technologies under Consideration for future extensions of VSEI (JVET-AE2032).

The semantics of SPTI SEI message includes description of picture n and its source picture timing computation as described / shown by excerpt below:

…

When picture n is a picture that is output having TemporalId less than or equal to i and is not the first picture of the bitstream that is output, the value of the variable SourcePictureTime[ n ] is derived as follows:

–	If temporalReversalFlag is equal to 0, SourcePictureTime[ n ] = SourcePictureTime[ previousPicInOutputOrder ] + SourcePictureInterval[ i

–	Otherwise, temporalReversalFlag is equal to 1, SourcePictureTime[ n ] = SourcePictureTime[ previousPicInOutputOrder ] − SourcePictureInterval[ i ]

where previousPicInOutputOrder is the last picture that is output having TemporalId less than or equal to i that precedes picture n in output order (if any). When the value of SourcePictureTime[ 0 ] is not provided by external means not specified in this document, the value of SourcePictureTime[ 0 ] is inferred to be 0.

…

In authors’ understanding of the above text, when SPTI SEI message has timing type of temporal reversal, the source picture timing of source picture associated with the first output picture where the SEI applies currently either is provided by external means or be inferred to be equal to 0. While the externally provided value may work, setting the source picture timing for that first output picture to be equal to 0 does not work since it is simply wrong.

# Proposal

To overcome the asserted problem above, we propose the following two options:

Option 1: when type includes temporal reversal, allow possibility to explicitly signal the elemental distance between the first picture and the last picture for the temporal reversal and the following applies:

- When the elemental distance between the first picture and the last picture is provided, the SourcePictureTime[ 0 ] is set to this value.

- When the elemental distance between the first picture and the last picture is not provided, it is constrained that the value is provided externally.

- The distance information shall not be present when the SPTI SEI message does not include temporal reversal type.

Option 2: specify a constraint such that when the SPTI SEI message includes temporal reversal picture timing type (i.e., the value of temporalReversalFlag is equal to 1), the value of SourcePictureTime[ 0 ] shall be provided by external means.

# Text implementation

## Option 1

…

| source_picture_timing_info( payloadSize ) { | Descriptor |
| --- | --- |
| spti_cancel_flag | u(1) |
| if( !spti_cancel_flag ) { |  |
| spti_persistence_flag | u(1) |
| spti_source_picture_timing_type | u(8) |
| spti_source_timing_equals_output_timing_flag | u(1) |
| if( !spti_source_timing_equals_output_timing_flag ) { |  |
| spti_time_scale | u(32) |
| spti_num_units_in_elemental_source_picture_interval | u(32) |
| if( spti_source_picture_timing_type & 0x10 ) { |  |
| spti_temporal_reversal_dist_present_flag | u(1) |
| if( spti_num_temporal_reversed_pics_present_flag ) |  |
| spti_temporal_reversal_dist_minus1 | ue(v) |
| } |  |
| spti_max_sublayers_minus_1 | u(3) |
| for( i = 0; i <= spti_max_sublayers_minus1; i++ ) { |  |
| spti_sublayer_source_picture_interval_scale_factor[ i ] | ue(v) |
| spti_sublayer_synthesized_picture_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |

…

spti_time_scale specifies the number of time units that pass in one second. The value of spti_time_scale shall not be equal to 0. For example, a time coordinate system that measures time using a 27 MHz clock has an spti_time_scale of 27,000,000.

spti_num_units_in_elemental_source_picture_interval specifies the number of time units of a clock operating at the frequency spti_time_scale Hz that corresponds to the indicated elemental source picture interval of consecutive pictures in output order in the CLVS.

The indicated elemental source picture interval, also to be denoted by the variable ElementalSourcePictureInterval, in units of seconds, is equal to the quotient of spti_num_units_in_elemental_source_picture_interval divided by spti_time_scale. For example, to represent an elemental source picture interval equal to 0.04 seconds, spti_time_scale may be equal to 27,000,000 and spti_num_units_in_elemental_source_picture_interval may be equal to 1,080,000.

spti_temporal_reversal_dist_present_flag equal to 1 specifies that spti_temporal_reversal_dist_minus1 is present. spti_temporal_reversal_dist_present_flag equal 0 specifies that spti_temporal_reversal_dist_minus1 is not present. When not present, the value of spti_temporal_reversal_dist_present_flag is inferred to be equal to 0.

spti_temporal_reversal_dist_minus1 plus 1 specifies the elemental source picture distance between the first picture and the last pictures in this temporal reversal.

spti_max_sublayers_minus1 plus 1 specifies the maximum number of temporal sublayers that may be present in the CLVS.

…

Let picture 0 be picture that is the same access unit as the SPTI SEI message and picture n, for n is greater than 0, be the n-th picture from picture 0. When picture n is a picture that is output having TemporalId less than or equal to i and is not the first picture of the bitstream that is output, the value of the variable SourcePictureTime[ n ] is derived as follows:

–	If temporalReversalFlag is equal to 0, SourcePictureTime[ n ] = SourcePictureTime[ n − 1 previousPicInOutputOrder ] + SourcePictureInterval[ i ]

–	Otherwise, temporalReversalFlag is equal to 1, SourcePictureTime[ n ] = SourcePictureTime[ n − 1 previousPicInOutputOrder ] − SourcePictureInterval[ i ]

where previousPicInOutputOrder is the last picture that is output having TemporalId less than or equal to i that precedes picture n in output order (if any). When the value of SourcePictureTime[ 0 ] is not provided by external means not specified in this document, the value of SourcePictureTime[ 0 ] is inferred to be 0.

The value of SourcePictureTime[ 0 ] is determined as follows:

–	If temporalReversalFlag is equal to 1 and spti_temporal_reversal_dist_present_flag is equal to 1, SourcePictureTime[ 0 ] = ElementalSourcePictureInterval * (spti_temporal_reversal_dist_minu1 + 1).

–	Else if provided by external means not specified in this document, the value of SourcePictureTime[ 0 ] is set according to the value provided by the external means.

–	Else, the value of SourcePictureTime[ 0 ] is set to be equal to 0.

When temporalReversalFlag is equal to 1 and spti_temporal_reversal_dist_present_flag is equal to 0, it is constrained that the value of SourcePictureTime[ 0 ] is provided by external means.

spti_sublayer_synthesized_picture_flag[ i ], when present, equal to 1 indicates that decoded output pictures belonging to the ith temporal sublayer are synthesized and do not correspond to unmodified original source pictures. spti_sublayer_synthesized_picture_flag[ i ] equal to 0 provides no such indication. When not present, the value of spti_sublayer_synthesized_picture_flag[ i ] is inferred to be equal to 0.

…

## Option 2

…

When picture n is a picture that is output having TemporalId less than or equal to i and is not the first picture of the bitstream that is output, the value of the variable SourcePictureTime[ n ] is derived as follows:

–	If temporalReversalFlag is equal to 0, SourcePictureTime[ n ] = SourcePictureTime[ previousPicInOutputOrder ] + SourcePictureInterval[ i ]

–	Otherwise, temporalReversalFlag is equal to 1, SourcePictureTime[ n ] = SourcePictureTime[ previousPicInOutputOrder ] - SourcePictureInterval[ i ]

where previousPicInOutputOrder is the last picture that is output having TemporalId less than or equal to i that precedes picture n in output order (if any). When the value of SourcePictureTime[ 0 ] is not provided by external means not specified in this document, the value of SourcePictureTime[ 0 ] is inferred to be 0. It is constrained that when temporalReversalFlag is equal to 1, the value of SourcePictureTime[ 0 ] shall be provided by external means.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).