| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0107-v1 |
| --- | --- |

| Title: | AHG9: On miscellaneous aspects for encoder optimization info SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Hendry Jaehyun Lim Seung hwan Kim Dong-Gyu Gwak | Email: | chulkeun.kim@lge.com |
| Source: | LG electronics |  |  |

_____________________________

- Abstract

This contribution proposes the following modifications to the encoder optimization information SEI message:

- Updates on the optimization_type.

- Simplification of optimization_type table(editorial)

- Changing temporal/spatial subsampling to temporal/spatial resampling

- Add a constraint on optimization_for_human_viewing_flag and optimization_for_machine_analysis_flag such that they cannot be equal to 0 at the same time.

Introduction

The current Technologies under Consideration for future extensions of VSEI (JVET-AE2032) includes encoder optimization information SEI message. This contribution proposes several modification to improve some aspects of the SEI message.

The parts of the syntax and semantics that are relevant for this contribution is shown below.

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| optimization_cancel_flag | u(1) |
| if( !optimization_cancel_flag ){ |  |
| optimization_persistence_flag | u(1) |
| optimization_for_human_viewing_flag | u(1) |
| optimization_for_machine_analysis_flag | u(1) |
| optimization_type | u(16) |
| } |  |
| } |  |

optimization_for_human_viewing_flag equal to 1 specifies that purposes for the applied optimization include human viewing. optimization_for_human_viewing_flag equal to 0 specifies that purposes for the applied optimization may or may not include human viewing.

optimization_for_machine_analysis_flag equal to 1 specifies that purposes for the applied optimization include machine analsysis. optimization_for_machine_analysis_flag equal to 0 specifies that purposes for the applied optimization may or may not include machine analysis.

optimization_type indicates the types of optimization method as specified in Table 2.

Table x – Definition of optimization_type

| Value | Interpretation |
| --- | --- |
| optimization_type = = 0 | May be used as determined by the application |
| optimization_type > 0 && ( optimization_type & 0x01 ) = = 0 | No object-based optimization |
| ( optimization_type & 0x01 ) != 0 | With object-based optimization; the pictures for which this SEI message persists have been pre-processed or encoded so that detected objects in the pictures are optimized with respect to other parts of the pictures for the indicated optimization purposes |
| optimization_type > 0 && ( optimization_type & 0x02 ) = = 0 | No temporal subsampling optimization |
| ( optimization_type & 0x02 ) != 0 | With temporal subsampling optimization |
| optimization_type > 0 && ( optimization_type & 0x04 ) = = 0 | No spatial subsampling optimization |
| ( optimization_type & 0x04 ) != 0 | With spatial subsampling optimization |
| optimization_type > 0 && ( optimization_type & 0x08 ) = = 0 | No temporal quality optimization |
| ( optimization_type & 0x08 ) != 0 | With temporal quality optimization |
| optimization_type > 0 && ( optimization_type & 0x10 ) = = 0 | No spatial quality optimization |
| ( optimization_type & 0x10 ) != 0 | With spatial quality optimization; the pictures for which this SEI message persists have been pre-processed or encoded to reduce unnecessary information or improve the quality of necessary information.(e.g to reduce the amount of noise and remove speckles at the picture-level) |

Proposals

Proposed changes to syntax and semantics on top of JVET-AE2032-v1 are marked with yellow for added text and with red strike-through for removed text.

Proposal 1: Updates on optimization_type

We propose the following:

- Simplification of optimization_type table. This is an editorial change with intent to improve the readability of the optimization_type table.

- Changing subsampling to resampling. Currently, temporal/spatial subsampling is described in the optimization_type table, which means that it includes only down sampling capabilities. However, it is asserted that optimization done by encoder can also include upsampling. Therefore, we propose to change subsampling to resampling to also include upsampling capabilities.

The text implementation for the proposed changes are as shown below:

Table x – Definition of optimization_type

| Value | Interpretation |
| --- | --- |
| optimization_type = = 0 | May be used as determined by the application |
| optimization_type > 0 && ( optimization_type & 0x01 ) = = 0 | No object-based optimization |
| ( optimization_type & 0x01 ) != 0 | With object-based optimization; the pictures for which this SEI message persists have been pre-processed or encoded so that detected objects in the pictures are optimized with respect to other parts of the pictures for the indicated optimization purposes |
| optimization_type > 0 && ( optimization_type & 0x02 ) = = 0 | No temporal subsampling optimization |
| ( optimization_type & 0x02 ) != 0 | With temporal subsampling optimization |
| optimization_type > 0 && ( optimization_type & 0x04 ) = = 0 | No spatial subsampling optimization |
| ( optimization_type & 0x04 ) != 0 | With spatial subsampling optimization |
| optimization_type > 0 && ( optimization_type & 0x08 ) = = 0 | No temporal quality optimization |
| ( optimization_type & 0x08 ) != 0 | With temporal quality optimization |
| optimization_type > 0 && ( optimization_type & 0x10 ) = = 0 | No spatial quality optimization |
| ( optimization_type & 0x10 ) != 0 | With spatial quality optimization; the pictures for which this SEI message persists have been pre-processed or encoded to reduce unnecessary information or improve the quality of necessary information.(e.g to reduce the amount of noise and remove speckles at the picture-level) |

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Object-based optimization; the pictures for which this SEI message persists have been pre-processed or encoded so that detected objects in the pictures are optimized with respect to other parts of the pictures for the indicated optimization purposes |
| 0x02 | Temporal resampling optimization |
| 0x04 | Spatial resampling optimization |
| 0x08 | Temporal quality optimization |
| 0x10 | Spatial quality optimization; the pictures for which this SEI message persists have been pre-processed or encoded to reduce unnecessary information or improve the quality of necessary information.(e.g to reduce the amount of noise and remove speckles at the picture-level) |

Proposal 2: Add a constraint on the value of optimization_for_human_viewing_flag and optimization_for_machine_analysis_flag

Currently, the text allows the values of both optimization_for_human_viewing_flag and optimization_for_machine_analysis_flag to be equal to 0. In such case, it may mean purpose of optimization that is not for human viewing nor machine analysis. However, it is asserted that currently there is no need other than human viewing and machine analysis. Thus, to prevent confusion, it is proposed adding a constraint that the values of these two flags cannot be equal to 0 at the same time.

The text implementation of the proposed constraint is as shown below:

optimization_for_human_viewing_flag equal to 1 specifies that purposes for the applied optimization include human viewing. optimization_for_human_viewing_flag equal to 0 specifies that purposes for the applied optimization may or may not include human viewing.

optimization_for_machine_analysis_flag equal to 1 specifies that purposes for the applied optimization include machine analsysis. optimization_for_machine_analysis_flag equal to 0 specifies that purposes for the applied optimization may or may not include machine analysis.

optimization_for_human_viewing_flag and optimization_for_machine_analysis_flag cannot be equal to 0 at the same time.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).