| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0107-v2 |
| --- | --- |

| Title: | AHG9: On miscellaneous aspects for encoder optimization info SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Hendry Jaehyun Lim Seung hwan Kim Dong-Gyu Gwak | Email: | chulkeun.kim@lge.com |
| Source: | LG electronics |  |  |

_____________________________

# Abstract

This contribution proposes the following modifications to the encoder optimization information SEI message:

- Updates on the optimization_type.

- Simplification of optimization_type table(editorial)

- Changing temporal/spatial subsampling to temporal/spatial resampling

- Add a constraint on optimization_for_human_viewing_flag and optimization_for_machine_analysis_flag such that they cannot be equal to 0 at the same time.

Version 2 of this contribution provides more complete text implementation for the proposed editorial changes and include the agreed items from the BoG discussion.

# Introduction

The current Technologies under Consideration for future extensions of VSEI (JVET-AE2032) includes encoder optimization information SEI message. This contribution proposes several modification to improve some aspects of the SEI message.

The parts of the syntax and semantics that are relevant for this contribution is shown below.

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| optimization_cancel_flag | u(1) |
| if( !optimization_cancel_flag ){ |  |
| optimization_persistence_flag | u(1) |
| optimization_for_human_viewing_flag | u(1) |
| optimization_for_machine_analysis_flag | u(1) |
| optimization_type | u(16) |
| } |  |
| } |  |

optimization_for_human_viewing_flag equal to 1 specifies that purposes for the applied optimization include human viewing. optimization_for_human_viewing_flag equal to 0 specifies that purposes for the applied optimization may or may not include human viewing.

optimization_for_machine_analysis_flag equal to 1 specifies that purposes for the applied optimization include machine analsysis. optimization_for_machine_analysis_flag equal to 0 specifies that purposes for the applied optimization may or may not include machine analysis.

optimization_type indicates the types of optimization method as specified in Table 2.

Table x – Definition of optimization_type

| Value | Interpretation |
| --- | --- |
| optimization_type = = 0 | May be used as determined by the application |
| optimization_type > 0 && ( optimization_type & 0x01 ) = = 0 | No object-based optimization |
| ( optimization_type & 0x01 ) != 0 | With object-based optimization; the pictures for which this SEI message persists have been pre-processed or encoded so that detected objects in the pictures are optimized with respect to other parts of the pictures for the indicated optimization purposes |
| optimization_type > 0 && ( optimization_type & 0x02 ) = = 0 | No temporal subsampling optimization |
| ( optimization_type & 0x02 ) != 0 | With temporal subsampling optimization |
| optimization_type > 0 && ( optimization_type & 0x04 ) = = 0 | No spatial subsampling optimization |
| ( optimization_type & 0x04 ) != 0 | With spatial subsampling optimization |
| optimization_type > 0 && ( optimization_type & 0x08 ) = = 0 | No temporal quality optimization |
| ( optimization_type & 0x08 ) != 0 | With temporal quality optimization |
| optimization_type > 0 && ( optimization_type & 0x10 ) = = 0 | No spatial quality optimization |
| ( optimization_type & 0x10 ) != 0 | With spatial quality optimization; the pictures for which this SEI message persists have been pre-processed or encoded to reduce unnecessary information or improve the quality of necessary information.(e.g to reduce the amount of noise and remove speckles at the picture-level) |

# Proposals

Proposed changes to syntax and semantics on top of JVET-AE2032-v1 are marked with yellow for added text and with red strike-through for removed text.

## Proposal 1: Updates on optimization_type

We propose the following:

- Simplification of optimization_type table. This is an editorial change with intent to improve the readability of the optimization_type table.

- Changing subsampling to resampling. Currently, temporal/spatial subsampling is described in the optimization_type table, which means that it includes only down sampling capabilities. However, it is asserted that optimization done by encoder can also include upsampling. Therefore, we propose to change subsampling to resampling to also include upsampling capabilities.

During the discussion of, the following notes were taken:

For the change suggested on item 1, semantics would need to be added how the bitmask is to be combined with the optimization_type syntax element. It was commented that in terms of software implementation, an explicit expression via flags is better understandable.

Revisit: Provide formulation of semantics for the interpretation of optimization_type.

Agreed on re-formulation spatial/temporal resampling (which would also apply to upsampling).

Based on the notes above, we update the text implementation as shown below:

Updates to the syntax:

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| eoi_cancel_flag | u(1) |
| if( !optimization_cancel_flag ){ |  |
| eoi_persistence_flag | u(1) |
| eoi_for_human_viewing_flag | u(1) |
| eoi_for_machine_analysis_flag | u(1) |
| eoi_type | u(16) |
| if( ObjectBasedFlag ) |  |
| eoi_object_based_idc | ue(v) |
| if( TemporalResamplingFlag ) { |  |
| eoi_temporal_resampling_type_flag | u(1) |
| eoi_num_int_pics | ue(v) |
| } |  |
| } |  |
| } |  |

Updates to the semantics:

eoi_cancel_flag equal to 1 specifies that the persistence of the previous applied optimization is cancelled. eoi_cancel_flag equal to 0 indicates that eoi_persistence_flag, eoi_for_human_viewing_flag, eoi_for_machine_analysis_flag and eoi_type follow.

eoi_persistence_flag specifies the persistence of the optimization the current layer. eoi_persistence_flag equal to 0 specifies that the optimization applies for the current picture only. eoi_persistence_flag equal to 1 specifies that the optimization applies for the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer associated with an encoder_optimization_info SEI message is output that follows the current picture in output order.

eoi_for_human_viewing_flag equal to 1 specifies that purposes for the applied optimization include human viewing. eoi_for_human_viewing_flag equal to 0 specifies that purposes for the applied optimization may or may not include human viewing.

eoi_for_machine_analysis_flag equal to 1 specifies that purposes for the applied optimization include machine analsysis. eoi_for_machine_analysis_flag equal to 0 specifies that purposes for the applied optimization may or may not include machine analysis.

eoi_type indicates the types of optimization method as specified in Table 2 where ( eoi_type & bitMask ) not equal to 0 indicates that the optimization type with the bitMask value in Table 2 has been applied. When eoi_type is greater than 0 and ( eoi_type & bitMask ) is equal to 0, the optimization type with the bitMask value has not been applied. When eoi_type is equal to 0, optimization as determined by the application has been used.

Table x – Definition of optimization_type

| Value | Interpretation |
| --- | --- |
| optimization_type = = 0 | May be used as determined by the application |
| optimization_type > 0 && ( optimization_type & 0x01 ) = = 0 | No object-based optimization |
| ( optimization_type & 0x01 ) != 0 | With object-based optimization; the pictures for which this SEI message persists have been pre-processed or encoded so that detected objects in the pictures are optimized with respect to other parts of the pictures for the indicated optimization purposes |
| optimization_type > 0 && ( optimization_type & 0x02 ) = = 0 | No temporal subsampling optimization |
| ( optimization_type & 0x02 ) != 0 | With temporal subsampling optimization |
| optimization_type > 0 && ( optimization_type & 0x04 ) = = 0 | No spatial subsampling optimization |
| ( optimization_type & 0x04 ) != 0 | With spatial subsampling optimization |
| optimization_type > 0 && ( optimization_type & 0x08 ) = = 0 | No temporal quality optimization |
| ( optimization_type & 0x08 ) != 0 | With temporal quality optimization |
| optimization_type > 0 && ( optimization_type & 0x10 ) = = 0 | No spatial quality optimization |
| ( optimization_type & 0x10 ) != 0 | With spatial quality optimization; the pictures for which this SEI message persists have been pre-processed or encoded to reduce unnecessary information or improve the quality of necessary information.(e.g to reduce the amount of noise and remove speckles at the picture-level) |

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Object-based optimization; the pictures for which this SEI message persists have been pre-processed or encoded so that detected objects in the pictures are optimized with respect to other parts of the pictures for the indicated optimization purposes |
| 0x02 | Temporal resampling optimization |
| 0x04 | Spatial resampling optimization |
| 0x08 | Temporal quality optimization |
| 0x10 | Spatial quality optimization; the pictures for which this SEI message persists have been pre-processed or encoded to reduce unnecessary information or improve the quality of necessary information.(e.g to reduce the amount of noise and remove speckles at the picture-level) |

The variables ObjectBasedFlag, TemporalResamplingFlag, SpatialResamplingFlag, TemporalQualityFlag, and SpatialQualityFlag, specifying whether optimization_type indicates the type of the optimization to include object-based optimization, temporal resampling optimization, spatial resampling optimization, temporal quality optimization and spatial quality optimization, respectively, are derived as follows:

ObjectBasedFlag = ( ( optimization_type & 0x01 ) > 0 ) ? 1 : 0
TemporalResamplingFlag = ( ( optimization_type & 0x02 ) > 0 ) ? 1 : 0
SpatialResamplingFlag = ( ( optimization_type & 0x04 ) > 0 ) ? 1 : 0	(xx)
TemporalQualityFlag = ( ( optimization_type & 0x08 ) > 0 ) ? 1 : 0
SpatialQualityFlag = ( ( optimization_type & 0x10 ) > 0 ) ? 1 : 0

eoi_object_based_idc, when present, indicates the type of object-based optimization as specified in Table x2, where ( eoi_object_based_idc & bitMask ) not equal to 0 indicates that the object-based optimization type associated with the bitMask value in Table x2 has been applied. When eoi_object_based_idc is greater than 0 and ( eoi_object_based_idc & bitMask ) is equal to 0, the object-based optimization type associated with the bitMask value has not been applied. When eoi_object_based_idc is equal to 0, an application-defined type of object-based optimization has been applied. The value of eoi_object_based_idc shall be in the range of 0 to 7, inclusive, in bitstreams conforming to this version of this Specification. Values of 8 to 65 535, inclusive, for eoi_object_based_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. When the value of eoi_object_based_idc is in the range of 8 to 65 535, inclusive, decoders conforming to this version of this Specification shall ignore eoi_object_based_idc.

Table x2 – Definition of eoi_object_based_idc

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Areas outside the detected objects have been blurred prior to encoding. |
| 0x02 | Areas outside the detected objects have been encoded with coarser transform-domain quantization than the quantization used for the detected objects. |
| 0x04 | Areas outside the detected objects have been overwritten. For example, an encoding system can overwrite areas outside the detected objects with a constant sample value. |

eoi_temporal_resampling_type_flag equal to 0 specifies that the temporal resampling optimization is subsampling operation. eoi_temporal_resampling_type_flag equal to 1 specifies that the temporal resampling optimization is upsampling operation.

eoi_num_int_pics greater than 0 indicates that the count of pictures that the encoding system excluded or added between each pair of coded pictures in output order within the persistence of this SEI message is constant. eoi_num_int_pics equal to 0 indicates that the count of pictures that the encoding system excluded or added between each pair of coded pictures in output order within the persistence of this SEI message is unknown or varying. When eoi_temporal_resampling_type_flag is equal to 0 and eoi_num_int_pics is greater than 0, eoi_num_int_pics specifies the count of pictures that the encoding system excluded between each pair of coded pictures. When eoi_temporal_resampling_type_flag is equal to 1 and eoi_num_int_pics is greater than 0, eoi_num_int_pics specifies the count of pictures that the encoding system added between each pair of source pictures. The value of eoi_num_int_pics shall be in the range of 0 to 63, inclusive.

## Proposal 2: Add a constraint on the value of optimization_for_human_viewing_flag and optimization_for_machine_analysis_flag

Currently, the text allows the values of both optimization_for_human_viewing_flag and optimization_for_machine_analysis_flag to be equal to 0. In such case, it may mean purpose of optimization that is not for human viewing nor machine analysis. However, it is asserted that currently there is no need other than human viewing and machine analysis. Thus, to prevent confusion, it is proposed adding a constraint that the values of these two flags cannot be equal to 0 at the same time.

The text implementation of the proposed constraint is as shown below:

optimization_for_human_viewing_flag equal to 1 specifies that purposes for the applied optimization include human viewing. optimization_for_human_viewing_flag equal to 0 specifies that purposes for the applied optimization may or may not include human viewing.

optimization_for_machine_analysis_flag equal to 1 specifies that purposes for the applied optimization include machine analsysis. optimization_for_machine_analysis_flag equal to 0 specifies that purposes for the applied optimization may or may not include machine analysis.

optimization_for_human_viewing_flag and optimization_for_machine_analysis_flag cannot be equal to 0 at the same time.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).