| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0109-v1 |
| --- | --- |

| Title: | EE2-6.1: Spatial CABAC tuning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Alireza Aminlou Pekka Astola Döne Bugdayci Sansli | Tel: Email: | - first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports results of EE2-6.1a and EE2-6.1b exploration experiments investigating spatial CABAC tuning approach originally proposed in JVET-AE0058. The tests were implemented on top of ECM-10.0 software. The only difference between tests 6.1a and 6.1b is the maximum number of bins stored per CTU. In Test 6.1a it is limited to 512 bins, while Test 6.1b limits it to 256 bins. The summarized results are reported as follows.

For test EE2-6.1a the impact on coding efficiency and runtimes over ECM-10.0 are reportedly {for Y, U, V, EncT, DecT, VmPeak}:

AI {-0.04%, -0.03%, -0.06%, 99.2%, 99.7%, 100.0%}
RA {-0.07%, -0.04%, -0.10%, 100.2%, 99.9%, 100.1%}
LB {-0.11%, -0.13%, -0.37%, 99.1%, 98.2%, 100.0%}

For test EE2-6.1b the impact on coding efficiency and runtimes over ECM-10.0 are reportedly {for Y, U, V, EncT, DecT, VmPeak }:

AI {-0.04%, 0.00%, -0.06%, 99.6%, 99.6%, 100.0%}
RA {-0.06%, -0.07%, -0.13%, 99.5%, 100.4%, 100.2%}
LB {-0.11%, 0.16%, -0.08%, 99.3%, 99.8%, 100.1%}

It is asserted that when QP gets lower the number of bins tuned increases, but the percentage of bins tuned compared to the total number of decoded bins decreases. For the lowest QP used in CTC the number of tuned corresponds to 1.9 % of the decoded bins for Test 6.1a and 1.1 % for Test 6.1b.

It is further reported that the worst-case memory requirement for the proposed method under CTC conditions is 10986 bytes for Test 6.1a and 5706 bytes for Test 6.1b.

# Proposed method

## Introduction

It is asserted that the current CABAC context update mechanism may lead to “overfitting” the probability estimates to the statistical properties of the CTU or couple of CTUs that are preceding the current CTU in scan order.

Figure 1 illustrates the processing order of the blocks and related syntax elements in ECM. CTUs are scanned in raster scan order, while CUs inside CTUs are scanned in nested tree order. When starting to process the current CTU (CTUC) the processing “jumps” from the bottom-right CU (yellow CU 8) of CTUL to the top-left CU of the current CTU (blue CU 0 of CTUC). At that stage, the context parameters of the CABAC engine can be expected to be finetuned for the bottom-right area of the previous CTU instead of the ideal case where the context parameters would be finetuned for the actual next CU to be processed.

It is proposed to use the bins of the syntax elements of the bottom CUs of the above CTU (the green CUs of CTUA in the example of Fig. 1) to tune the CABAC contexts when starting to process a new CTU. It was found that tuning the state parameters of the shorter window probability estimator is typically enough and only minor additional impact is achieved by tuning both probability estimators.

Figure 1: Example of processing order of the CUs in the left and above CTUs

## Proposed changes to ECM

It is proposed to buffer bins relating to syntax elements of bottom CUs of CTUs in one CTU line. The number of stored bins per CTU is limited to a given number (maximum of 512 and 256 stored binds were used in Test 6.1a and Test 6.1b, respectively). This is done to keep the buffering and updating cost at low level. It is also proposed to limit the number of stored bins related to a single context to a relative low number to avoid situation where a dominant syntax element may alone reserve a significant portion of the total storage. In the experiments reported here, the limit for the number of stored bins for a single context was set to 8.

When starting to encode or decode a new CTU, the stored bins from the above CTU are retrieved and fed into the context update mechanism of the shorter window probability estimator (with “window 0”). The bins are fed to context update process in the reverse order with respect to their coding order to simulate encoding or decoding that approaches the top-left corner of the new CTU from the direction of bottom-right corner of the CTU above.

# Memory analysis

Memory required for enabling the proposed operation consists of two components.

Firstly, we need to buffer a certain number of decoded bins for each CTU on one CTU line in the image. An element of such buffer should hold a 1-bit bin value and a 10-bit context ID (for the current ECM where the number of contexts is below 1024). The number of elements needed for each CTU is 512 for Test 6.1a and 256 for Test 6.2b. Following table summarizes the memory requirement per CTU for the bin buffers.

| Test | Bits / element | Number of elements | Total bits / CTU | Total bytes / CTU |
| --- | --- | --- | --- | --- |
| EE2-6.1a | 11 | 512 | 5632 | 704 |
| EE2-6.1b | 11 | 256 | 2816 | 352 |

Secondly, to keep track on how many bins for each context is currently stored in the active CTU, there is a need to have a 4-bit counter for each context (to hold a value between 0 and 8, inclusive – making sure no more than 8 bins are stored for any individual context). The number of contexts in ECM 10.0 is 852 making the needed size of these counters 4 * 852 bits = 3408 bits = 426 bytes.

The total worst-case memory requirement then depends on the number of CTUs in a CTU row. Considering the CTC settings for different classes of content and the memory elements outlined above, the total amount of memory needed for Tests 6.1a and 6.1b are as follows:

| Class | CTUs / row | EE2-6.1a memory [Bytes] | EE2-6.1b memory [Bytes] |
| --- | --- | --- | --- |
| A1 | 15 | 10986 | 5706 |
| A2 | 15 | 10986 | 5706 |
| B | 15 | 10986 | 5706 |
| C | 7 | 5354 | 2890 |
| D | 4 | 3242 | 1834 |
| E | 10 | 7466 | 3946 |
| F | 15 | 10986 | 5706 |
| TGM | 15 | 10986 | 5706 |

It should be noted that there are naturally alternative ways to implement the buffering. For example, each context can have its own 8-bit bin buffer with an indication of how many of those are used for a specific CTU (using e.g. a 4-bit counter). This kind of an implementation may have some benefits, such as, all the bins associated with a single context are grouped together for input to the context update.

# Usage analysis

Attached JVET-AF0109-stats.xlsx Excel sheet reports usage data of the tool for all sequences and QPs individually. A summary of the statistics is generated for the tables below.

Generally, when QP gets lower the number of bins tuned increases, but the percentage of bins tuned compared to the total number of decoded bins decreases. For the lowest QP used in CTC the number of tuned bins corresponds to 1.9 % of the decoded bins for Test 6.1a and 1.1 % for Test 6.2b.

## Test 6.1a with maximum stored bins/CTU limited to 512

| AI | Test 6.1a |  |  | bins tuned / | bins total / | bins tuned / |
| --- | --- | --- | --- | --- | --- | --- |
| QP | CTUs tuned | bins tuned | bins total | CTUs tuned | CTUs tuned | bins total |
| 22 | 133518 | 52745851 | 2801937861 | 395 | 20985 | 1.9 % |
| 27 | 133518 | 45433339 | 1223213954 | 340 | 9161 | 3.7 % |
| 32 | 133518 | 37962920 | 707811082 | 284 | 5301 | 5.4 % |
| 37 | 133518 | 30754664 | 441278111 | 230 | 3305 | 7.0 % |

| RA | Test 6.1a |  |  | bins tuned / | bins total / | bins tuned / |
| --- | --- | --- | --- | --- | --- | --- |
| QP | CTUs tuned | bins tuned | bins total | CTUs tuned | CTUs tuned | bins total |
| 19 | 17488 | 6789291 | 350541950 | 388 | 20045 | 1.9 % |
| 21 | 13873 | 4557954 | 157173642 | 329 | 11329 | 2.9 % |
| 22 | 30446 | 7774223 | 240254247 | 255 | 7891 | 3.2 % |
| 23 | 60461 | 13193633 | 389601813 | 218 | 6444 | 3.4 % |
| 24 | 17488 | 6127159 | 199741971 | 350 | 11422 | 3.1 % |
| 26 | 134985 | 24741917 | 655854306 | 183 | 4859 | 3.8 % |
| 28 | 273706 | 33434345 | 670262321 | 122 | 2449 | 5.0 % |
| 29 | 564589 | 52080793 | 933512137 | 92 | 1653 | 5.6 % |
| 31 | 13873 | 3070697 | 53470676 | 221 | 3854 | 5.7 % |
| 32 | 121112 | 11743375 | 167481457 | 97 | 1383 | 7.0 % |
| 34 | 291194 | 23950211 | 315097993 | 82 | 1082 | 7.6 % |
| 35 | 60461 | 5837873 | 71255998 | 97 | 1179 | 8.2 % |
| 36 | 500513 | 20654549 | 222730957 | 41 | 445 | 9.3 % |
| 38 | 121112 | 7541863 | 77252530 | 62 | 638 | 9.8 % |
| 40 | 273706 | 12431116 | 109501026 | 45 | 400 | 11.4 % |
| 41 | 547101 | 16292395 | 134553390 | 30 | 246 | 12.1 % |
| 43 | 121112 | 5118756 | 39778366 | 42 | 328 | 12.9 % |
| 45 | 243260 | 6701030 | 42478569 | 28 | 175 | 15.8 % |
| 46 | 486640 | 8242690 | 46032117 | 17 | 95 | 17.9 % |

| LB | Test 6.1a |  |  | bins tuned / | bins total / | bins tuned / |
| --- | --- | --- | --- | --- | --- | --- |
| QP | CTUs tuned | bins tuned | bins total | CTUs tuned | CTUs tuned | bins total |
| 21 | 1571 | 579798 | 21209070 | 369 | 13500 | 2.7 % |
| 23 | 53586 | 11042461 | 238767646 | 206 | 4456 | 4.6 % |
| 26 | 163664 | 23279221 | 384254462 | 142 | 2348 | 6.1 % |
| 28 | 270836 | 28478792 | 373553966 | 105 | 1379 | 7.6 % |
| 31 | 1571 | 422543 | 8117523 | 269 | 5167 | 5.2 % |
| 33 | 215679 | 15975039 | 160940196 | 74 | 746 | 9.9 % |
| 35 | 217250 | 10036829 | 89466502 | 46 | 412 | 11.2 % |
| 36 | 1571 | 340516 | 5333078 | 217 | 3395 | 6.4 % |
| 38 | 53586 | 3983811 | 33018526 | 74 | 616 | 12.1 % |
| 39 | 162093 | 6583850 | 48817900 | 41 | 301 | 13.5 % |
| 41 | 217250 | 6321179 | 44132946 | 29 | 203 | 14.3 % |
| 44 | 162093 | 4572220 | 27174397 | 28 | 168 | 16.8 % |
| 46 | 217250 | 4342454 | 24141028 | 20 | 111 | 18.0 % |

## Test 6.1b with maximum stored bins/CTU limited to 256

| AI | Test 6.1b |  |  | bins tuned / | bins total / | bins tuned / |
| --- | --- | --- | --- | --- | --- | --- |
| QP | CTUs tuned | bins tuned | bins total | CTUs tuned | CTUs tuned | bins total |
| 22 | 133518 | 29801846 | 2802128949 | 223 | 20987 | 1.1 % |
| 27 | 133518 | 28301484 | 1223092565 | 212 | 9161 | 2.3 % |
| 32 | 133518 | 26433401 | 707923921 | 198 | 5302 | 3.7 % |
| 37 | 133518 | 23958712 | 441211889 | 179 | 3305 | 5.4 % |

| RA | Test 6.1b |  |  | bins tuned / | bins total / | bins tuned / |
| --- | --- | --- | --- | --- | --- | --- |
| QP | CTUs tuned | bins tuned | bins total | CTUs tuned | CTUs tuned | bins total |
| 19 | 17488 | 3834721 | 350542522 | 219 | 20045 | 1.1 % |
| 21 | 13873 | 2698197 | 157107708 | 194 | 11325 | 1.7 % |
| 22 | 30446 | 4967859 | 240385381 | 163 | 7895 | 2.1 % |
| 23 | 60461 | 8874341 | 389645555 | 147 | 6445 | 2.3 % |
| 24 | 17488 | 3696870 | 199622991 | 211 | 11415 | 1.9 % |
| 26 | 134985 | 17544192 | 655764527 | 130 | 4858 | 2.7 % |
| 28 | 273706 | 26361278 | 669687480 | 96 | 2447 | 3.9 % |
| 29 | 564589 | 42288222 | 933536316 | 75 | 1653 | 4.5 % |
| 31 | 13873 | 2174939 | 53495050 | 157 | 3856 | 4.1 % |
| 32 | 121112 | 9846478 | 167261919 | 81 | 1381 | 5.9 % |
| 34 | 291194 | 20262486 | 315160243 | 70 | 1082 | 6.4 % |
| 35 | 60461 | 4961328 | 71285883 | 82 | 1179 | 7.0 % |
| 36 | 500513 | 18329051 | 222322439 | 37 | 444 | 8.2 % |
| 38 | 121112 | 6791382 | 76795974 | 56 | 634 | 8.8 % |
| 40 | 273706 | 11465355 | 109682087 | 42 | 401 | 10.5 % |
| 41 | 547101 | 15040901 | 134498137 | 27 | 246 | 11.2 % |
| 43 | 121112 | 4819048 | 39797289 | 40 | 329 | 12.1 % |
| 45 | 243260 | 6464470 | 42446366 | 27 | 174 | 15.2 % |
| 46 | 486640 | 7951123 | 46012599 | 16 | 95 | 17.3 % |

| LB | Test 6.1b |  |  | bins tuned / | bins total / | bins tuned / |
| --- | --- | --- | --- | --- | --- | --- |
| QP | CTUs tuned | bins tuned | bins total | CTUs tuned | CTUs tuned | bins total |
| 21 | 1571 | 341716 | 21229409 | 218 | 13513 | 1.6 % |
| 23 | 53586 | 7763420 | 238665833 | 145 | 4454 | 3.3 % |
| 26 | 163664 | 18348798 | 384707337 | 112 | 2351 | 4.8 % |
| 28 | 270836 | 24220490 | 373915497 | 89 | 1381 | 6.5 % |
| 31 | 1571 | 301887 | 8123809 | 192 | 5171 | 3.7 % |
| 33 | 215679 | 14773123 | 160907628 | 68 | 746 | 9.2 % |
| 35 | 217250 | 9723598 | 89441785 | 45 | 412 | 10.9 % |
| 36 | 1571 | 270415 | 5344836 | 172 | 3402 | 5.1 % |
| 38 | 53586 | 3797150 | 33003947 | 71 | 616 | 11.5 % |
| 39 | 162093 | 6474633 | 48873191 | 40 | 302 | 13.2 % |
| 41 | 217250 | 6262443 | 44122185 | 29 | 203 | 14.2 % |
| 44 | 162093 | 4549567 | 27123131 | 28 | 167 | 16.8 % |
| 46 | 217250 | 4306640 | 23992398 | 20 | 110 | 18.0 % |

# Coding efficiency analysis

## Test 6.1a with maximum stored bins/CTU limited to 512

Following tables summarize the coding efficiency results for Test 6.1a. Full results are available in the accompanied Excel sheets.

|  | All Intra Main 10 – EE2-6.1a |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.03 % | -0.12 % | -0.12 % | 101.8% | 98.7% | 99.9% |
| Class A2 | -0.04 % | -0.07 % | -0.06 % | 98.7% | 98.6% | 100.0% |
| Class B | -0.05 % | -0.03 % | -0.06 % | 98.4% | 99.0% | 100.0% |
| Class C | -0.03 % | -0.04 % | 0.02 % | 98.4% | 100.3% | 100.0% |
| Class E | -0.05 % | 0.10 % | -0.12 % | 99.5% | 102.1% | 100.0% |
| Overall | -0.04 % | -0.03 % | -0.06 % | 99.2% | 99.7% | 100.0% |
| Class D | -0.01 % | 0.09 % | -0.09 % | 98.7% | 95.4% | 100.0% |
| Class F | -0.09 % | 0.04 % | 0.00 % | 99.3% | 98.8% | 100.0% |
| TGM | -0.05 % | -0.07 % | -0.05 % | 98.3% | 105.5% | 100.0% |
|  | Random Access Main 10 – EE2-6.1a |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.04 % | -0.21 % | -0.09 % | 101.0% | 97.4% | 100.2% |
| Class A2 | -0.02 % | -0.16 % | -0.09 % | 99.0% | 95.4% | 99.9% |
| Class B | -0.10 % | 0.13 % | -0.06 % | 101.5% | 98.5% | 100.3% |
| Class C | -0.08 % | -0.06 % | -0.15 % | 99.0% | 107.4% | 100.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.07 % | -0.04 % | -0.10 % | 100.2% | 99.9% | 100.1% |
| Class D | -0.03 % | -0.27 % | 0.04 % | 99.8% | 96.3% | 100.0% |
| Class F | -0.06 % | 0.13 % | 0.00 % | 92.5% | 102.6% | 100.0% |
| TGM | -0.05 % | -0.03 % | 0.00 % | 97.7% | 104.7% | 100.0% |
|  | Low delay B Main 10 – EE2-6.1a |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.01 % | -0.29 % | -0.62 % | 99.3% | 96.3% | 100.1% |
| Class C | -0.08 % | -0.15 % | -0.38 % | 99.4% | 97.6% | 100.0% |
| Class E | -0.34 % | 0.14 % | 0.07 % | 98.6% | 102.4% | 100.0% |
| Overall | -0.11 % | -0.13 % | -0.37 % | 99.1% | 98.2% | 100.0% |
| Class D | -0.04 % | -0.05 % | -0.11 % | 99.0% | 97.8% | 100.0% |
| Class F | -0.01 % | -0.23 % | 0.36 % | 99.6% | 100.8% | 99.8% |
| TGM | 0.07 % | 0.28 % | 0.11 % | 99.2% | 101.2% | 95.8% |

## Test 6.1b with maximum stored bins/CTU limited to 256

Following tables summarize the coding efficiency results for Test 6.1b. Full results are available in the accompanied Excel sheets.

|  | All Intra Main 10 – EE2-6.1b |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.03 % | 0.02 % | -0.08 % | 102.1% | 96.5% | 99.9% |
| Class A2 | -0.03 % | -0.02 % | -0.04 % | 100.4% | 96.7% | 100.0% |
| Class B | -0.04 % | 0.04 % | -0.02 % | 99.3% | 100.2% | 100.0% |
| Class C | -0.03 % | 0.05 % | 0.03 % | 100.4% | 101.3% | 100.0% |
| Class E | -0.05 % | -0.05 % | -0.26 % | 95.9% | 102.6% | 100.0% |
| Overall | -0.04 % | 0.01 % | -0.06 % | 99.6% | 99.6% | 100.0% |
| Class D | 0.01 % | -0.05 % | -0.07 % | 97.1% | 103.8% | 100.0% |
| Class F | -0.06 % | -0.05 % | 0.04 % | 99.4% | 106.5% | 100.0% |
| TGM | -0.07 % | -0.11 % | -0.06 % | 100.0% | 111.7% | 100.0% |
|  | Random Access Main 10 – EE2-6.1b |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.07 % | -0.16 % | -0.08 % | 101.9% | 96.2% | 100.6% |
| Class A2 | -0.02 % | -0.14 % | -0.07 % | 99.3% | 99.3% | 100.2% |
| Class B | -0.05 % | 0.02 % | -0.18 % | 97.5% | 98.7% | 100.1% |
| Class C | -0.08 % | -0.04 % | -0.14 % | 100.4% | 106.6% | 100.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.06 % | -0.07 % | -0.13 % | 99.5% | 100.4% | 100.2% |
| Class D | 0.00 % | -0.08 % | -0.16 % | 100.4% | 100.2% | 100.0% |
| Class F | -0.06 % | 0.08 % | 0.18 % | 98.4% | 106.2% | 100.2% |
| TGM | 0.02 % | 0.01 % | 0.05 % | 96.0% | 98.9% | 100.0% |
|  | Low delay B Main 10 – EE2-6.1b |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.09 % | 0.03 % | -0.37 % | 99.0% | 98.0% | 100.3% |
| Class C | -0.12 % | -0.40 % | -0.14 % | 99.8% | 99.5% | 100.0% |
| Class E | -0.13 % | 1.11 % | 0.48 % | 98.9% | 103.3% | 100.0% |
| Overall | -0.11 % | 0.16 % | -0.08 % | 99.3% | 99.8% | 100.1% |
| Class D | 0.05 % | 0.43 % | -0.08 % | 98.4% | 98.4% | 100.0% |
| Class F | -0.01 % | -0.16 % | 1.00 % | 98.7% | 102.7% | 99.9% |
| TGM | 0.06 % | 0.06 % | 0.11 % | 98.7% | 103.3% | 96.0% |

# Additional analysis on LD-B configuration of joint test EE2-6.3b

Joint test EE2-6.3b between EE2-6.1b (spatial CABAC tuning) and EE2-6.2 (retraining of Intra slice initialization) appear to show no additional benefit in LD-B from spatial CABAC tuning when applied on top of retrained context initialization. However, results in general for LD-B configuration appear very noisy. To analyse the performance of the proposed EE2-6.1b variant more thoroughly, it was further tested with slight modifications and applied together with the context initialization of EE2-6.2. More specifically, the limit for the bins that can be stored for each context was altered from 8 (as proposed in EE2-6.1b) to 4 to illustrate the noisy behaviour of LD-B encodings. The following table summarizes the coding efficiency results with different values of that parameter.

| bins |  |  | Average |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| /ctx | Test | Y | U | V | B | C | E | D | F | TGM |
| N/A | 6.2 | -0.03 % | -0.04 % | 0.00 % | 0.01 % | -0.01 % | -0.12 % | 0.11 % | 0.26 % | 0.05 % |
| 8 | 6.3b | -0.03 % | 0.49 % | -0.04 % | -0.05 % | -0.02 % | -0.01 % | 0.01 % | 0.13 % | 0.08 % |
| 7 | 6.3b** | -0.08 % | 0.24 % | -0.16 % | -0.07 % | -0.05 % | -0.14 % | 0.05 % | 0.23 % | 0.04 % |
| 6 | 6.3b** | -0.12 % | -0.02 % | -0.10 % | -0.07 % | -0.11 % | -0.19 % | 0.04 % | 0.05 % | 0.08 % |
| 5 | 6.3b** | -0.07 % | 0.12 % | -0.51 % | -0.03 % | -0.14 % | -0.06 % | 0.08 % | 0.11 % | -0.09 % |
| 4 | 6.3b** | -0.09 % | 0.05 % | -0.41 % | -0.09 % | -0.15 % | 0.00 % | 0.14 % | 0.15 % | -0.04 % |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).