| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0115 |
| --- | --- |

| Title: | Non-EE2: IBC with a further upward-extended reference area for screen content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Haruhisa Kato Kei Kawamura (KDDI) 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 70 3508 5829 yo-kidani@kddi.com |
| Source: | KDDI Corporation (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes an intra block copy (IBC) with a further upward-extended reference area for screen content to enhance the coding performance of IBC. In the current ECM design, the reference area for IBC is extended to two CTU rows above the current CTU, but there is room to achieve additional coding gain for screen content by extending the reference area beyond the referenceable two CTU rows. Therefore, this contribution proposes two further upward extension methods for the IBC reference area, qualitatively comparing their advantages and disadvantages in terms of complexity. Specifically, method 1 extends the reference area to the picture's upper side boundary, while method 2 does four CTU rows above the current CTU.

The experimental results reportedly show the proposed method brings some coding gains over ECM-10.0. The results of method 2 are shown as follows:

AI: Class F {-0.14%, -0.22%, -0.10 %, 100.1%, 99.9%}, Class TGM {-1.21%, -1.21%, -1.20 %, 99.4%, 99.3%};

RA: Class F {-x.xx%, -x.xx%, -x.xx %, xxx%, xxx%}, Class TGM {-0.66%, -0.61%, -0.64%, 99.9%, 100.9%}.

Introduction

In the current ECM design, the reference area for IBC is extended to two CTU rows above current CTU, as shown in Figure 1. This extension was proposed by JVET-Z0153 [1], considering the extra memory required new intra coding tools in ECM such as IntraTMP.

Figure 1 Reference area for IBC when CTU (m, n) is coded. The blue block denotes the current CTU; green blocks denote the reference area; and the white blocks denote invalid reference area.

However, there is room to achieve an additional coding gain for screen content by extending the reference area beyond the referenceable two CTU rows. Figure 2 shows an example of IBC with a further-upward extended reference area in Desktop.

Figure 2 An example of IBC with further-upward extended reference area in Desktop. The blue arrows denote the block vectors of IBC; and the gray grid indicates the CTU divisions of the picture.

Proposed method

This contribution proposes two further upward extension methods for the IBC reference area. Specifically, method 1 extends the reference area to the picture's upper side boundary, while method 2 extends two CTU rows, i.e., IBC can refer to four CTU rows above the current CTU.

The proposed extensions are applied only for screen content because this extension does not bring coding gains for camera-captured content. There are less likely blocks that perfectly match in reference areas far from the current CTU. As an extra memory, method 1 and method 2 require a buffer for one picture and a buffer for four CTU rows., respectively.

The buffer for one picture in method 1 can be acceptable considering the current ECM with enabling wavefront parallel processing (WPP) operated on multi-threads on a hardware decoder. The current ECM allows IBC to utilize the reference samples up to the picture left side boundary. Hence, when WPP is operated on multi-threads, the current ECM requires a buffer for one picture. On the other hand, the current ECM also allows IBC to utilize the reference samples up to the picture's right-side boundary, which prohibits enabling WPP. Therefore, in the next section, we first show the test results of ECM-10, where WPP is enabled without a right-side reference area from the current CTU that affects the coding performance of intra coding tools. Then, we show the results where the IBC reference area is extended to the picture's upper side boundary on top of the prior test.

The buffer for four CTU rows above the current CTU can also be acceptable when applying this extension only to HD resolution or less. The resolution-dependent control is realized by level information, for example.

Experimental results

The proposed methods are implemented on top of ECM-10.0. The following four tests were conducted only for screen content (class F and class TGM) based on the common test condition and evaluation procedures for enhanced compression tool testing in AI and RA configuration [2]. More detailed results can be found in the attached spreadsheets.

- Test 1a: Method 1

- Test 1b: ECM-10 with WPP and without right-side reference area

- Test 1c: Test 1a + Test 1b

- Test 2: Method 2

Table 1 Test 1a results over ECM-10.0

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class F | -0.18% | -0.31% | -0.13% | 100.5% | 99.9% | 100.0% |
| Class TGM | -1.28% | -1.31% | -1.28% | 100.0% | 99.7% | 100.1% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class F | -0.09% | -0.01% | -0.07% | 100.4% | 99.8% | 100.0% |
| Class TGM | -0.73% | -0.66% | -0.71% | 99.9% | 100.4% | 100.1% |

Table 2 Test 1b results over ECM-10.0

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |

Table 3 Test 1c results over ECM-10 with WPP without right-side reference of IBC

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10 with WPP without right-side reference area |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over ECM-10 with WPP without right-side reference area |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |

Table 4 Test 4 over ECM-10.0

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class F | -0.14% | -0.22% | -0.10% | 100.1% | 99.9% | 100.0% |
| Class TGM | -1.21% | -1.21% | -1.20% | 99.4% | 99.3% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class TGM | -0.66% | -0.61% | -0.64% | 99.9% | 100.9% | 100.0% |

Conclusion

This contribution proposes two further upward extension methods for IBC reference area. The proposed methods bring some coding gains without increasing encoding runtime. It is recommended to adopt the proposed methods into the next EE2.

Reference

- J. Xu and N. Zhang, “EE2-3.2: IBC Reference Area Extension,” JVET-Z0153, by teleconference, April 2022.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AE2017, Geneva, July. 2023.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).