| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0128-v2 |
| --- | --- |

| Title: | EE2-3.2: LIC flag derivation for merge candidates with template costs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang1, Kai Zhang2, Hongbin Liu1, Yang Wang1, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhangna.cynthia@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In ECM, the LIC flag is inherited for a merge candidate. It is proposed to derive the LIC flag of a merge candidate based on template costs in EE2-3.2. A SAD-based template cost, denoted as C0, and a Mean Removal SAD (MRSAD)-based template cost, denoted as C1, are calculated. The LIC flag is set to be false, if C0 <= C1 and is set to be true, if C0 > C1.

On top of the ECM10.0, simulation results of EE2-3.2 are reported as below:

RA: { -0.06%, -0.03%, 0.12%, 99.9%, 100.4%, 100.1%};

LB: {-0.12%, -0.18%, -0.34%, 100.3%, 100.8%, 100.0%}.

# Introduction

In ECM, the LIC flag is inherited for a merge candidate. It may not be accurate in a region with illuminance changes.

# Proposed methods

In EE2-3.2, it is proposed to derive the LIC flag of a merge candidate based on template costs.

The LIC flag of a merge candidate is derived by comparing two template costs: a SAD-based template cost, denoted as C0, and a Mean Removal SAD (MRSAD)-based template cost, denoted as C1. The LIC flag is set to be false, if C0 <= C1 and is set to be true, if C0 > C1.

To favor the inherited LIC flag, C0 is multiplied by α if the inherited LIC flag is false while C1 is multiplied by α if the inherited LIC flag is true, where α < 1.

# Experimental Results

The proposed method has been implemented on top of the ECM-10.0 [1]. Testing results of EE2-3.2 under RA/LB configurations are summarized in Table 1. The simulations were performed following the common test conditions [2]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of EE2-3.2.

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.06% | -0.02% | 0.12% | 100.0% | 98.9% | 100.6% |
| Class A2 | -0.02% | -0.13% | 0.00% | 99.7% | 100.1% | 99.9% |
| Class B | -0.05% | 0.15% | 0.23% | 99.9% | 101.2% | 100.1% |
| Class C | -0.08% | -0.17% | 0.07% | 100.0% | 100.9% | 100.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.06% | -0.03% | 0.12% | 99.9% | 100.4% | 100.1% |
| Class D | 0.00% | 0.04% | 0.02% | 100.6% | 100.7% | 100.0% |
| Class F | -0.08% | -0.03% | -0.07% | 100.3% | 100.3% | 100.1% |
|  | Low delay B Main 10 |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.07% | -0.28% | -0.62% | 100.4% | 100.5% | 100.0% |
| Class C | -0.17% | -0.10% | -0.09% | 100.2% | 101.2% | 100.0% |
| Class E | -0.12% | -0.13% | -0.22% | 100.1% | 100.8% | 100.0% |
| Overall | -0.12% | -0.18% | -0.34% | 100.3% | 100.8% | 100.0% |
| Class D | 0.03% | 0.04% | -0.13% | 101.1% | 99.9% | 100.0% |
| Class F | 0.01% | 0.34% | -0.35% | 98.6% | 98.5% | 100.0% |

- Conclusions

In this contribution, to further improve the efficiency of inter prediction, it is proposed to derive the LIC flag of a merge candidate based on template costs. The proposed method can bring some coding gains with almost no coding time change. It is recommended to adopt EE2-3.2 into the next version of ECM.

# References

[1] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-10.0

[2] M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” JVET-AE2017, Jul. 2023.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).