| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0130 |
| --- | --- |

| Title: | EE2-2.4: IntraCIIP as additional mode of IntraTMP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser, Philippe Bordes, Franck Galpin, Kevin Reuze | Email: | karam.naser@interdigital.com, |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution presents the experimental results of EE2-2.4 test on IntraCIIP mode. It is a new intra prediction mode where the regular prediction is blended with IntraTMP prediction using the existing CIIP blending of ECM-10.0. The simulation results are reported as follows:

AI: -0.03%	-0.03%	-0.08% 	EncT 100.2% DecT 100.4%

RA:

# Introduction

IntraCIIP was first presented in JVET-AC0201 and was further modified in JVET-AE0214. Its basic idea is to employ the existing CIIP process for intra-coded CU’s. Specifically, CIIP is an inter prediction mode, where the motion compensated prediction is blended with intra prediction. The TIMD derivation method is used to derive the intra prediction mode.

CIIP Blending

Regular Intra Prediction

IntraTMP prediction

TIMD process

IntraTMP process

Intra mode

Block vectors

Figure 1 Illustration of IntraCIIP prediction

In IntraCIIP, the inter part is replaced by IntraTMP prediction and TIMD mode is used for the intra part. Figure 1 depicts the IntraCIIP prediction. The blending method of CIIP is not changed. For near-horizontal modes (2 <= angular mode index < 34), the current block is vertically divided as shown in Figure 2(a); for near-vertical modes (34 <= angular mode index <= 66), the current block is horizontally divided as shown in Figure 2(b).

Figure 2 The division method for angular modes.

The blending weights (wIntra, wInter) for different sub-blocks are shown in Table 1.

Table 1. The modified weights used for angular modes.

| The sub-block index | (wIntra, wInter) |
| --- | --- |
| 0 | (6, 2) |
| 1 | (5, 3) |
| 2 | (3, 5) |
| 3 | (2, 6) |

# Proposal

In this EE test, it is proposed to add IntraCIIP as an additional mode of IntraTMP. That is, a syntax element, named IntraCiipFlag, is signaled as a submode of IntraTMP. IntraCIIP is only allowed when IntraTMP fusion, IntraTMP filtering and IntraTMP subpel are not applied.

# Experimental results

The proposal was implemented in ECM-10.0 and tested with CTC-AI and RA conditions. The following results are obtained:

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.01% | 0.08% | -0.06% | 100.6% | 100.1% | 99.1% |
| Class A2 | -0.05% | -0.05% | -0.04% | 100.3% | 98.0% | 99.5% |
| Class B | -0.01% | -0.08% | -0.08% | 100.5% | 99.6% | 99.9% |
| Class C | -0.03% | 0.01% | 0.01% | 99.8% | 101.0% | 100.0% |
| Class E | -0.07% | -0.09% | -0.25% | 100.0% | 103.5% | 100.0% |
| Overall | -0.03% | -0.03% | -0.08% | 100.2% | 100.4% | 99.7% |
| Class D | 0.04% | 0.08% | -0.01% | 99.1% | 100.2% | 100.0% |
| Class F | -0.02% | 0.03% | -0.05% | 99.9% | 102.1% | 99.9% |
| Class TGM | 0.05% | 0.05% | 0.01% | 99.8% | 102.7% | 99.7% |

# Conclusion

This contribution presents the results of EE2-2.4 test on IntraCIIP. It is reported that this new prediction mode provides 0.03% AI gain without increasing the coding time.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).