| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0131 |
| --- | --- |

| Title: | EE2-2.2: DIMD with filtered template |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Chuan Zhou Zhuoyi Lv (vivo) | Email: | chuan.zhou@vivo.com zhuoyi.lv@vivo.com |
| Source: | vivo Mobile Communication Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the simulation results of EE2-2.2. In this test, the template is filtered using a 3x3 filter operator before obtaining the gradient histogram. In EE2-2.2a, the gradient histogram derivation uses 3x2 and 2x3 gradient operators, respectively. In EE2-2.2b, the same 3x3 Sobel gradient operators as in ECM are used.

On top of the ECM10.0, simulation results of EE2-2.2a are reported as below:

AI: -0.08% (Y), 0.04% (U), -0.02% (V),  102.8% (EncT),  99.2% (DecT)

RA : -0.04% (Y), 0.03% (U), 0.03% (V),  100.8% (EncT),  99.8% (DecT)

On top of the ECM10.0, simulation results of EE2-2.2b are reported as below:

AI: -0.07% (Y), 0.01% (U), -0.04% (V),  102.7% (EncT),  99.1% (DecT)

RA : -0.07% (Y), -0.04% (U), -0.07% (V),  100.8% (EncT),  100.2% (DecT)

Introduction

When DIMD is applied, up to five intra modes are derived from the reconstructed neighbor samples, and those five predictors are combined with the planar mode predictor with the weights derived from the histogram of gradients [1]. The following 3x3 Sobel gradient operators are convoluted with the template:

Mx=-101-202-101    and    My=-1-2-1000121

Proposed methods

A 3x3 filter operator is used to filter the template as follows.

F=-1-1-1-19-1-1-1-1

In EE2-2.2a, the following 3x2 and 2x3 gradient operators are used for the gradient histogram derivation of left and top templates respectively.

My=1-12-21-1    and    Mx=-1-30013

My=10-130-3    and    Mx=-1-2-1121

In EE2-2.2b, the following 3x3 Sobel gradient operators are used, which is the same as in ECM, while using unfiltered samples for gradient computation at poisitions where no filtered samples are available.

Mx=-101-202-101    and    My=-1-2-1000121

Simulation results

The proposed method is tested on top of ECM10.0 under the common test conditions [1].

Table 1. Simulation results of EE2-2.2a

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.09% | -0.01% | -0.11% | 102.9% | 100.0% | 100.0% |
| Class A2 | -0.07% | 0.11% | 0.01% | 102.6% | 99.3% | 100.0% |
| Class B | -0.08% | 0.05% | 0.02% | 103.2% | 99.2% | 100.0% |
| Class C | -0.06% | 0.06% | 0.15% | 102.3% | 98.2% | 100.0% |
| Class E | -0.09% | -0.02% | -0.25% | 103.2% | 99.3% | 100.0% |
| Overall | -0.08% | 0.04% | -0.02% | 102.8% | 99.2% | 100.0% |
| Class D | -0.04% | 0.07% | -0.09% | 102.3% | 99.3% | 100.0% |
| Class F | -0.06% | -0.05% | 0.30% | 101.7% | 100.1% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.05% | -0.05% | -0.04% | 100.9% | 99.2% | 99.1% |
| Class A2 | 0.01% | -0.01% | 0.12% | 100.6% | 99.7% | 101.1% |
| Class B | -0.08% | 0.08% | 0.08% | 100.7% | 99.7% | 100.4% |
| Class C | -0.02% | 0.07% | -0.06% | 100.8% | 100.4% | 100.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.04% | 0.03% | 0.03% | 100.8% | 99.8% | 100.2% |
| Class D | -0.02% | 0.00% | 0.19% | 100.9% | 100.2% | 100.0% |
| Class F | -0.06% | 0.06% | -0.09% | 101.8% | 101.0% | 100.1% |

Table 2. Simulation results of EE2-2.2b

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.11% | -0.13% | 0.01% | 102.6% | 99.4% | 100.0% |
| Class A2 | -0.09% | -0.04% | -0.04% | 102.3% | 98.8% | 100.0% |
| Class B | -0.07% | 0.00% | -0.06% | 102.9% | 99.0% | 100.0% |
| Class C | -0.06% | 0.02% | 0.03% | 102.3% | 98.4% | 100.0% |
| Class E | -0.06% | 0.21% | -0.16% | 103.4% | 100.0% | 100.0% |
| Overall | -0.07% | 0.01% | -0.04% | 102.7% | 99.1% | 100.0% |
| Class D | -0.03% | -0.06% | -0.02% | 102.5% | 99.7% | 100.0% |
| Class F | -0.08% | 0.15% | 0.12% | 101.2% | 99.8% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.10% | -0.19% | -0.03% | 100.9% | 99.9% | 99.2% |
| Class A2 | -0.04% | -0.26% | 0.02% | 100.5% | 99.8% | 100.5% |
| Class B | -0.08% | 0.18% | -0.11% | 101.4% | 100.9% | 100.3% |
| Class C | -0.04% | -0.04% | -0.11% | 100.1% | 99.7% | 100.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.07% | -0.04% | -0.07% | 100.8% | 100.2% | 100.1% |
| Class D | -0.06% | -0.22% | -0.03% | 100.2% | 99.8% | 100.0% |
| Class F | 0.06% | 0.30% | 0.40% | 101.2% | 100.6% | 99.9% |

References

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AE2017, Jul. 2023.

Patent rights declaration(s)

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).