| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0133-v2 |
| --- | --- |

| Title: | EE2-6.2: updating I-slice context model parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao Yan Ye Jie Chen Xinwei Li | Tel: Email: | {ruling.lrl, yan.ye}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

# Abstract

The contribution reports the results of EE2 test 6.2 which studies I-slice context model parameters. The initial probabilities, window sizes and adaptive weights for I-slice context models are trained and updated using the bitstream generated by ECM-10.0 and CTC sequences with QP 17 to QP 42 in AI configuration. It is reported that on top of ECM-10.0, the overall coding performance and runtime for {Y, U, V, EncT, DecT, VmPeak} is {-0.15%, -0.08%, -0.04%, 100.0%, 99.6%, 100.0%} for AI, {-0.04%, 0.00%, 0.00%, 99.9%, 99.9%, 100.2%} for RA and {-0.03%, -0.04%, 0.00%, 100.0%, 99.4%, 100.0%} for LB.

# Introduction

The context model parameters have not been retrained since ECM-5.0. In this proposal, the context model parameters for I-slice are retrained. The training scripts are modified from the one used in VVC’s CE on CABAC initialization. The training bitstream are generated using ECM-10.0 and CTC sequences (including class A1, class A2, class B, class C, class D, class E and class F) with QP 17 to QP 42 in AI configuration.

# Simulation results

The trained I-slice context model parameters are integrated into ECM-10.0 and are tested under CTC for enhanced compression tool testing [2] with all intra, random access and low delay B configurations. Table 1 shows the simulation results of the trained parameters.

Table 1. Simulation results of EE2-6.2

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.09% | 0.12% | 0.06% | 99.7% | 99.8% | 100.0% |
| Class A2 | -0.08% | 0.04% | 0.04% | 100.7% | 100.4% | 100.1% |
| Class B | -0.14% | -0.04% | -0.06% | 100.3% | 99.8% | 100.0% |
| Class C | -0.21% | -0.13% | 0.00% | 99.5% | 99.3% | 100.0% |
| Class E | -0.24% | -0.37% | -0.21% | 99.9% | 98.6% | 100.0% |
| Overall | -0.15% | -0.08% | -0.04% | 100.0% | 99.6% | 100.0% |
| Class D | -0.19% | -0.45% | -0.37% | 98.9% | 99.0% | 100.0% |
| Class F | -0.31% | -0.20% | 0.01% | 99.5% | 98.6% | 100.0% |
| Class TGM | -0.21% | -0.19% | -0.14% | 100.0% | 99.8% | 100.0% |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 0.01% | -0.16% | 0.12% | 100.1% | 100.3% | 100.1% |
| Class A2 | -0.03% | -0.06% | 0.16% | 100.3% | 100.2% | 100.2% |
| Class B | -0.06% | 0.26% | -0.02% | 99.9% | 99.7% | 100.2% |
| Class C | -0.04% | -0.16% | -0.21% | 99.6% | 99.8% | 100.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.04% | 0.00% | 0.00% | 99.9% | 99.9% | 100.2% |
| Class D | -0.04% | -0.09% | -0.21% | 99.4% | 99.8% | 100.0% |
| Class F | -0.10% | -0.18% | 0.06% | 99.7% | 99.1% | 99.8% |
| Class TGM | -0.12% | -0.07% | -0.11% | 100.0% | 99.6% | 100.0% |

|  | Low delay B Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.01% | -0.07% | -0.33% | 100.3% | 99.5% | 100.1% |
| Class C | -0.01% | -0.21% | 0.12% | 99.9% | 99.2% | 100.0% |
| Class E | -0.12% | 0.23% | 0.39% | 99.5% | 99.6% | 100.0% |
| Overall | -0.03% | -0.04% | 0.00% | 100.0% | 99.4% | 100.0% |
| Class D | 0.11% | 0.39% | -0.74% | 99.3% | 99.4% | 100.0% |
| Class F | 0.26% | 0.33% | 0.51% | 100.6% | 99.0% | 99.9% |
| Class TGM | 0.05% | -0.02% | -0.01% | 100.7% | 100.7% | 99.8% |

# References

- R.-L. Liao, Y. Ye, J. Chen and X. Li, “AHG12: Updating context model parameters for ECM,” JVET-AE0147, Jul. 2023.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AE2017, Jul. 2023.

# Patent rights declaration(s)

Alibaba group does not have any current or pending patent rights relating to the technology described in this contribution.