| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0136 |
| --- | --- |

| Title: | EE2-2.5: TIMD with IntraTMP and IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser, Fabrice Le Léannec, Philippe Bordes, Franck Galpin, Antoine Robert | Email: | karam.naser@interdigital.com, |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution presents the experimental results of EE2-2.5 test on TIMD improvement with IntraTMP and IBC prediction. It is proposed to extend TIMD template-based mode selection by predictions from IntraTMP and IBC. The template derived modes, being regular or IntraTMP/IBC based, are blended using the same weight derivation of TIMD. In EE2-2.5a, this method is studied, while EE2-2.5b test studies the combination of this method with IntraCIIP (Test EE2-2.4)

The following results are obtained for EE2-2.5a:

AI: -0.04%	0.03%	-0.02%	EncT 100.2% DecT 99.9%

RA:

The following results are obtained for EE2-2.5b:

AI: -0.05%	-0.04%	-0.08%	EncT 100.5% DecT 101.7%

RA:

# Introduction

The extension of TIMD prediction with block vector-based prediction, using IntraTMP/IBC, was proposed in JVET-AE0185. It was proposed to include, alongside with other regular prediction modes, prediction obtained by IntraTMP and/or IBC. That is, the template cost is used to evaluate both regular prediction and block-vector ones. The best 2 predictors are blended using their SATD cost as follows (same as in ECM-10):

weight1 = costMode2 / (costMode1+ costMode2)

weight2 = 1 - weight1

# Proposal

In EE2-2.5a test, it is proposed to consider block vectors from neighboring blocks, which are coded by IntraTMP or IBC, and use them to compute predictor candidates for TIMD cost evaluation. Specifically, the TIMD mode selection process, based on template SATD cost, includes additionally prediction obtained using block vectors from both direct neighbors and non-adjacent ones. The modified TIMD process can therefore result in combining regular and IntraTMP/IBC predictions or combining two different IntraTMP/IBC predictions. This enriches the prediction quality of the current TIMD mode which is limited to regular mode.

In EE2-2.5b test, the combination of EE2-2.5a and EE2-2.4 tests is studied. EE2-2.4 test is about IntraCIIP, where the intra prediction mode is blended with IntraTMP using CIIP blending method.

# Experimental results

Both EE2-2.5a and EE2-2.5b are implemented on ECM-10.0 and tested with CTC-AI and RA conditions. The following results are obtained:

EE2-2.5a:

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 0.02% | 0.11% | 0.06% | 99.8% | 98.2% | 99.9% |
| Class A2 | -0.03% | -0.04% | -0.05% | 99.1% | 98.0% | 99.7% |
| Class B | -0.04% | 0.00% | -0.09% | 100.7% | 100.4% | 100.0% |
| Class C | -0.04% | 0.06% | 0.03% | 99.8% | 100.5% | 100.0% |
| Class E | -0.08% | 0.01% | -0.04% | 101.5% | 102.0% | 100.0% |
| Overall | -0.04% | 0.03% | -0.02% | 100.2% | 99.9% | 100.0% |
| Class D | -0.02% | 0.07% | -0.08% | 100.5% | 100.7% | 100.0% |
| Class F | -0.09% | -0.05% | -0.01% | 100.6% | 103.9% | 100.0% |
| Class TGM | -0.06% | 0.00% | -0.02% | 100.0% | 103.4% | 100.0% |

EE2-2.5b:

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 0.00% | 0.00% | -0.08% | 100.5% | 100.3% | 99.1% |
| Class A2 | -0.09% | -0.01% | -0.03% | 99.5% | 100.6% | 99.5% |
| Class B | -0.04% | 0.02% | -0.04% | 101.3% | 101.7% | 100.0% |
| Class C | -0.04% | -0.08% | 0.01% | 100.4% | 101.8% | 100.0% |
| Class E | -0.09% | -0.14% | -0.36% | 100.5% | 104.3% | 100.1% |
| Overall | -0.05% | -0.04% | -0.08% | 100.5% | 101.7% | 99.8% |
| Class D | 0.02% | 0.12% | 0.07% | 100.4% | 100.3% | 100.0% |
| Class F | -0.08% | -0.06% | 0.10% | 100.9% | 106.3% | 100.0% |
| Class TGM | -0.01% | 0.01% | -0.05% | 100.2% | 104.0% | 100.0% |

# Conclusion

This contribution presents the results of EE2-2.5a and EE2-2.5b tests on TIMD with IntraTMP/IBC and its combination with IntraCIIP. It is reported that this new prediction mode provides 0.05% AI gain without increasing the coding time.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).