| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0137 |
| --- | --- |

| Title: | AHG12: IntraTMP with Merge Candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser, Fabrice Le Léannec, Antoine Robert, Franck Galpin | Email: | karam.naser@interdigital.com, |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In the current IntraTMP design, the template matching is performed on a predefined search area to obtain a candidate list of predictors specified by their block vectors. Compared to IBC mode, the readily available block vectors from neighboring merge candidates are not explored in IntraTMP mode despite being more correlated with the properties of the current block. This contribution proposes extending the IntraTMP candidate list with all the merge candidates block vectors. The following results are obtained:

AI:

RA:

# Introduction

In IntraTMP mode, 6 search areas (Figure 1) are used to find a candidate list of 19 candidates, each specified by its block-vector (BV). The search range of all search regions is subsampled by a factor of 3. After finding the best match, a refinement process is performed. The refinement is done via a second template matching search around the best match with a reduced range.

Figure 1 Intra template matching search area used.

# Proposal

It is proposed to augment the IntraTMP candidate list with predictors specified by BV from the neighboring PU’s. Specifically, the same candidate list as for IBC is considered for IntraTMP. Whenever a block vector is found in the merge list, it is added to the current IntraTMP candidate list. The candidate list is refined with a second template matching step, which is already used in IntraTMP.

The proposed method enriches the IntraTMP candidate list with new BV’s that are not previously checked by IntraTMP process. This is because it includes IBC block vectors, whose search region is larger than IntraTMP one, and also because it possibly includes some candidates that are not selected in the first pass of template matching (the coarse search with subsampled regions). It is also noted that the added BV’s are correlated with the current block statistics as they were selected previously for predicting nearby blocks.

# Experimental results

The proposal was implemented in ECM-10.0 and tested with CTC-AI and RA conditions. The following results are obtained:

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.01% | 0.13% | 0.04% | 100.0% | 99.6% | 99.1% |
| Class A2 | -0.05% | -0.04% | -0.07% | 100.0% | 100.3% | 99.5% |
| Class B | -0.06% | 0.01% | -0.14% | 100.7% | 102.1% | 99.9% |
| Class C | -0.01% | 0.05% | 0.13% | 100.9% | 101.5% | 100.0% |
| Class E | -0.09% | -0.11% | -0.02% | 101.2% | 102.3% | 100.0% |
| Overall | -0.05% | 0.01% | -0.02% | 100.6% | 101.3% | 99.7% |
| Class D | -0.01% | 0.05% | -0.01% | 101.8% | 100.5% | 100.0% |
| Class F | -0.16% | -0.15% | 0.10% | 99.9% | 102.7% | 99.9% |
| Class TGM | -0.15% | -0.11% | -0.12% | 98.9% | 103.5% | 99.7% |

# Conclusion

This contribution proposes using the IBC-like merge candidates block vectors for the IntraTMP candidate list. The method shows 0.05% coding gain in AI. It is proposed to study this method in the next round of EE.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).