| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0140-v1 |
| --- | --- |

| Title: | [AHG9] Inter Picture Dependency SEI message |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Gilles Teniou Stephan Wenger Shan Liu | Tel: Email: (teniou,swenger,shanl)[at]global.tencent.com |  |
| Source: | Tencent |  |  |  |

# Abstract

Nowadays, video applications rely more and more on mixed hardware/software-based implementations, particularly for decoders/transcoders located in the Cloud. Such cloud media processing benefits from massive parallel computing resources such as GPUs. In many scenarios, the decoding process needs to happen as fast as possible. As an example, live TV programs that are later proposed as on-demand contents, need to be made available as soon as possible. The TV program from the live stream is decoded and re-encoded into multiple formats and resolutions suitable for on-demand streaming. This contribution proposes to address the fast-decoding mode by signaling pictures that can be decoded independently.

# Problem statement

When decoding a video bitstream, a single decoder instance is usually set up. All the picture units are decoded sequentially with parallelized decoding opportunities only at the lower levels under some constraints.

This is explained by the fact that pictures within the same CVS have some dependencies such as decoded pictures used as a reference for later ones (in coding order).

Within a Picture Unit:

- Sub-pictures may be decoded in parallel within the same Picture Unit.

- Tiles could also be parallelized if cross-tiles prediction and entropy dependencies are prevented.

- Slices contained in a same Picture Unit, could be parallelized after entropy decoding, if no inter slice dependency is guaranteed.

- Wavefront Parallel Processing (WPP) also allows creation of picture partitions that can be processed in parallel.

However, inter-picture dependency is not signaled at the high level whereas some pictures may be decoded without waiting all the previous ones to be decoded.

Nowadays, encoders are much more complex than decoders and some of them already use per-picture parallelization for encoding speed efficiency. To do so, some inter-picture restrictions are necessary, thus impacting the prediction structure of the CVS. Those restrictions in terms of inter-picture dependency are not signaled today and visible from a decoder viewpoint.

The figure below illustrates a conventional example of coding configuration.

In this configuration, the pictures B0, B1, B2, B3, B4 and B5 need to be decoded sequentially as they potentially depend on each other. The picture B6 can be decoded in parallel of picture B5. To enable such a process, the picture B6 needs to signal that the previous one is independent from it. The pictures B6 and B7 are also independent from each other and only rely on the availability of the picture B3. It is then interesting to signal the latest picture required to be available prior to the decoding of the current picture.

| Current picture decoding order | Dependent of latest picture | Parallel decoding with the previous picture | Number of parallel instances of picture decoder |
| --- | --- | --- | --- |
| 0 | - | no | 1 |
| 1 | 0 | no | 1 |
| 2 | 1 | no | 1 |
| 3 | 2 | no | 1 |
| 4 | 3 | no | 1 |
| 5 | 4 | no | 1 |
| 6 | 4 | yes | 2 |
| 7 | 3 | yes | 3 |
| 8 | 7 | no | 1 |
| 9 | 7 | yes | 2 |
| 10 | 2 | yes | 3 |
| 11 | 10 | no | 1 |
| 12 | 11 | no | 1 |
| 13 | 11 | yes | 2 |
| 14 | 10 | yes | 3 |
| 15 | 14 | no | 1 |
| 16 | 14 | yes | 2 |

In this scenario, the decoding process of the GOP can be optimized with 3 decoder instances acting like the following:

|  | t0 | t1 | t2 | t3 | t4 | t5 | t6 | t7 | t8 | t9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Decoder1: | 0 | 1 | 2 | 3 | 4 | 5 | 8 | 11 | 12 | 15 |
| Decoder2: |  |  |  |  |  | 6 | 9 |  | 13 | 16 |
| Decoder3: |  |  |  |  |  | 7 | 10 |  | 14 |  |

With the assumption that the decoding time of a frame being constant [1], the theoretical decoding speed may be significantly faster. This approach is expected to be even more efficient with larger GOP sizes.

Picture Unit parallelization can be further optimized each time the following picture in decoding order does not use the current one, even if the current one is flagged as a reference picture for the following one. This could be a choice of an encoder implementation to favor inter-picture parallel processing at both the encoder and decoder sides.

To prepare the decoding configuration, it is necessary to indicate the number of picture decoder instances to be instantiated. A possible decoder architecture is described in the following figure:

To be useful, the indication of picture dependency needs to be known as early as possible in the decoder process. For this reason, the definition of an associated SEI message implies the need to be a Prefix SEI (NAL Unit type: PREFIX_SEI_NUT).

# Proposal

## VSEI

### 8.X.1	Inter picture dependency SEI message syntax

[Implementation proposal #1

Signal picture dependency and recommended decoder instance]

| ### inter_picture_dependency(payloadSize) { | ### C | ### Descriptor |
| --- | --- | --- |
| ### ipd_update_flag | ### 5 | ### u(1) |
| ### if(ipd_update_flag) { |  |  |
| ### simultaneous_decoded_picture_minus1 | ### 5 | ### ue(v) |
| ### } |  |  |
| ### current_picture_dependency_id | ### 5 | ### ue(v) |
| ### } |  |  |

[Implementation proposal #2

Signal picture dependency using POC]

| ### inter_picture_dependency(payloadSize) { | ### C | ### Descriptor |
| --- | --- | --- |
| ### ipd_update_flag | ### 5 | ### u(1) |
| ### if(ipd_update_flag) { |  |  |
| ### simultaneous_decoded_picture_minus1 | ### 5 | ### ue(v) |
| ### } |  |  |
| ### current_picture_dependency_id | ### 5 | u(v) |
| ### } |  |  |

[Implementation proposal #3

Signal depdendency using serial number]

| ### inter_picture_dependency(payloadSize) { | ### C | ### Descriptor |
| --- | --- | --- |
| ### ipd_update_flag | ### 5 | ### u(1) |
| ### if(ipd_update_flag) { |  |  |
| ### simultaneous_decoded_picture_minus1 | ### 5 | ### ue(v) |
| ### } |  |  |
| ### current_picture_serial_number | ### 5 | u(8) |
| ### picture_dependency_serial_number | ### 5 | u(8) |
| ### } |  |  |

### 8.X.2	Inter Picture Dependency SEI message semantics

The Inter Picture Dependency (IPD) SEI message carries parameters allowing to decode more than one picture unit simultaneously.

Use of this SEI message requires the definition of the following variables:

- Let NbPictureDecoders be the number of picture decoder instances used to decode the bitstream.

  - At initialization, NbPictureDecoders is equal to 1.

- The picture decoder instances initialization that signals the number of parallel picture decoder instances expected to run within the CVS.

  - ipd_update_flag equal to 0 indicates that the number of simultaneous picture decoders information is not present and the simulataneous_decoded_picture_minus1 is assumed to be equal to zero.

  - idp_update_flag equal to 1 indicates that the number of simultaneous picture decoders information is present and signaled in simulataneous_decoded_picture_minus1.

    - simulataneous_decoded_picture_minus1 signals the maximum number of picture decoder instances minus one, that are expected to run simultaneously.

    - When simulataneous_decoded_picture_minus1 is present:

NbPictureDecoders = Max(simulataneous_decoded_picture_minus1+1, NbPictureDecoders)

- If unchanged, NbPictureDecoders remains valid until the end of the current CVS.

[Implementation proposal #1

Signal picture dependency and recommended decoder instance]

- current_picture_dependency_id indicates if the current picture is independent from the previous one in decoding order.  current_picture_dependency_id also carries the identifier of the picture decoder instance that may be used to decode the corresponding picture unit.

  - current_picture_dependency_id equal to 0 indicates that the current picture unit requires the availability the decoded previous picture in decoding order.

  - current_picture_dependency_id different from 0 indicates that the current picture unit can be decoded without waiting the availability of the decoded previous picture in decoding order. In this case, current_picture_independent carries the identifier of the picture unit decoder instance that may be used.

[Implementation proposal #2

Signal picture dependency using POC]

- current_picture_dependency_id indicates the picture order count of the latest picture in decoding order required to be availble for decoding the current picture unit. current_picture_dependency_id is equal to ph_pic_order_cnt_lsb from the picture unit header of the latest picture required to be decoded prior to decoding the current picture unit.

[Implementation proposal #3

Signal depdendency using serial number]

- Let SerialNumber be a one-byte integer identifier of the picture. At the start of a CVS, the first picture unit is assigned SerialNumber equal to 0. For each following picture in the decoding order in the CVS SerialNumber is incremented by one. A wraparound is applied to SerialNumber so that the following value after 255 is 0.

  - current_picture_serial_number indicates the serial number of the current picture unit.

  - picture_dependency_serial_number indicates the serial number of the latest picture in decoding order required to be decoded before being able to decode the current picture unit.

## HEVC and VVC

D.2.1	General SEI payload syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| SeiExtensionBitsPresentFlag = 0 |  |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if […] |  |
| else if( payloadType = = XXX ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| inter_picture_dependency ( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| […] |  |
| if( SeiExtensionBitsPresentFlag ¦ ¦ more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| sei_reserved_payload_extension_data | u(v) |
| sei_payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| sei_payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

# References

[1]	A.-M. Alt and D. Simon, “Control strategies for h.264 video decodingunder resources constraints,” ACM SIGOPS Operating Systems Review,vol. 44, no. 3, pp. 53–58, 2010. https://www.researchgate.net/publication/47750105_Control_strategies_for_H264_video_decoding_under_resources_constraints

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).