| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0143-v1 |
| --- | --- |

| Title: | AHG11: Unified CNN super resolution for resampling-based video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chaoyi Lin1, Yue Li2, Junru Li2, Kai Zhang2, Li Zhang2 1Bafangcheng 9th building F14 Yuhang District, 311100, Hangzhou City, Zhejiang Province, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | linchaoyi.cy@bytedance.com yue.li@bytedance.com lijunru@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

The unified filter architecture for the in-loop filter has been adopted into NNVC. However, for super resolution (SR) technology, there is no corresponding unified SR model. This contribution proposes a unified SR architecture, which is designed to be aligned to the unified in-loop filter architecture and complexity. The experimental results are reported as:

For LOP SR (20.8 kMAC/pixel), the simulation results reportedly show {-3.86%, -1.19%, 0.56%} and {-3.20%, -6.15%, -0.11%} BD rate savings for {Y, Cb, Cr} under RA and AI configurations, respectively.

For HOP SR (469 kMAC/pixel), the simulation results reportedly show {-5.70%, -2.25%, -0.75%} and {-6.16%, -8.7%, -4.78%} BD rate savings for {Y, Cb, Cr} under RA and AI configurations, respectively.

# Introduction

In JVET-AD meeting, unified filter architectures for the High Operation Point (HOP) and Low Operation Point (LOP) NN-based in-loop filters were defined in JVET-AD0380 [1], respectively. On the other hand, there is no unified model in NNVC for NN-based super resolution (SR) in resampling-based video coding.

This contribution proposes a solution for NN-based SR, in which components are unified with the in-loop filter presented in JVET-AE2023 [2]. The complexity is also designed to be aligned with current unified in-loop filter model in NNVC 6.0:

The complexity of proposed LOP SR and HOP SR are designed to be 20.8 and 469 kMAC/pixel, respectively, compared to 469 kMAC/pixel of the current SR model in NNVC 6.0.

# Proposed Unified LOP SR Architecture

The proposed unified LOP SR architecture and hyperparameters are illustrated in Figure 1.. The input interface of the filter is aligned to the unified in-loop filter. The chroma components are upsampled to the same size of luma components before feeding into the network. Then a 1×1 convolution is used to fuse the concatenated features. Then, split of luma and chroma with different complexity is designed in a same way to the LOP in-loop filter. The chroma branch includes a smaller number of channels and basic backbone blocks compared to luma branch.

Figure 1 proposed LOP SR

## Inference

The network information in the inference stage is provided in Table 1.

Table 1 Network information for the proposed LOP SR model in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | Intel(R) Xeon(R) Platinum 8260 CPU @ 2.40GHz |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 0.1 M |
|  | Total Parameter Number | 0.1 M |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 0.4 MB |
|  | Multiply Accumulate (MAC) | 20.8 kMAC/pixel |
| Optional | Total Conv. Layers |  |
|  | Total FC Layers | 0 |
|  | Batch size: | 1 |
|  | Patch size | Whole frame |

## Training details

DIV2K, BVI_DVC, and TVD datasets are used for training. The NNVC 6.0 is utilized to compress these images with QPs {22, 27, 32, 37, 42}.

Table 2 Network information for the proposed LOP SR model in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla-V100-SXM2-32GB |
|  | Framework: | PyTorch v1.11 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 120 |
|  | Batch size: | 16 |
|  | Training time: | 120h/model |
|  | Training data information: | DIV2K, BVI_DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | Turn off all the NN tools |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 128 |
|  | Learning rate: | 4e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

# Proposed Unified HOP SR Architecture

The proposed unified HOP SR architecture and hyperparameters are illustrated in Figure 2. The input interface of the filter is aligned to the unified in-loop filter. The chroma components are upsampled to the same size of luma components before feeding into the network. Then a 1×1 convolution is used to fuse the concatenated features. The back bone blocks are the same as HOP In-loop filter. The RPR upsampled reconstruction will be added with output of network to generate the final upsampled reconstruction.

Figure 2 proposed HOP SR

## Inference

The network information in the inference stage is provided in Table 3.

Table 3 Network information for the proposed HOP SR model in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | Intel(R) Xeon(R) Platinum 8260 CPU @ 2.40GHz |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 1.9 M |
|  | Total Parameter Number | 1.9 M |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 7.6 MB |
|  | Multiply Accumulate (MAC) | 469kMAC/pixel |
| Optional | Total Conv. Layers |  |
|  | Total FC Layers | 0 |
|  | Batch size: | 1 |
|  | Patch size | Whole frame |

## Training details

DIV2K, BVI_DVC, and TVD datasets are used for training. The NNVC 6.0 is utilized to compress these images with QPs {22, 27, 32, 37, 42}.

Table 4 Network information for the proposed HOP SR model in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla-V100-SXM2-32GB |
|  | Framework: | PyTorch v1.11 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 120 |
|  | Batch size: | 32 |
|  | Training time: | 240h/model |
|  | Training data information: | DIV2K, BVI_DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | Turn off all the NN tools |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

# Experimental results

The proposed CNN-based up-sampling filter is tested on top of NNVC 6.0, according to the common test conditions [3]. The tested QP values are {22, 27, 32, 37, 42}.

## LOP SR results

Under RA configurations, the proposed method can bring {-3.86%, -1.19%, 0.56%} BD-rate changes on average for {Y, U, V} components, respectively.

Under AI configurations, the proposed method can bring {-3.20%, -6.15%, -0.11%} BD-rate changes on average for {Y, U, V} components, respectively.

Table 5 Results of the proposed method for RA configurations

|  | Random Access Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM Anchor |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -5.36% | -6.37% | -4.14% |
| Class A2 | -2.36% | 3.99% | 5.26% |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  |
| avg. A1&A2 | -3.86% | -1.19% | 0.56% |

Table 6 Results of the proposed method for AI configurations

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM Anchor |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -3.50% | -6.19% | -4.20% |
| Class A2 | -2.90% | -6.12% | 3.98% |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% |
| avg. A1&A2 | -3.20% | -6.15% | -0.11% |

## HOP SR results

Under RA configurations, the proposed method can bring {-5.70%, -2.25%, -0.75%} BD-rate changes on average for {Y, U, V} components, respectively.

Under AI configurations, the proposed method can bring {-6.16%, -8.7%, -4.78%}BD-rate changes on average for {Y, U, V} components, respectively.

Table 7 Results of the proposed method for RA configurations

|  | Random Access Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM Anchor |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -7.79% | -7.62% | -5.83% |
| Class A2 | -3.60% | 3.11% | 4.33% |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  |
| avg. A1&A2 | -5.70% | -2.25% | -0.75% |

Table 8 Results of the proposed method for AI configurations

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM Anchor |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -5.67% | -7.49% | -7.04% |
| Class A2 | -6.65% | -9.91% | -2.53% |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% |
| avg. A1&A2 | -6.16% | -8.70% | -4.78% |

## NNVC-6.0 SR results

This section provides the results of official NNVC-6.0 SR model for comparison.

Under AI configurations, the official NNVC-6.0 SR can bring {-5.80%, -7.38%, -4.61%} BD-rate changes on average for {Y, U, V} components, respectively.

Under RA configurations, the official NNVC-6.0 SR can bring {-5.34%, -2.58%, -2.17%} BD-rate changes on average for {Y, U, V} components, respectively.

Table 9 Results of the official NNVC-6.0 SR model for RA configurations

|  | Random Access Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM Anchor |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -7.45% | -8.36% | -8.06% |
| Class A2 | -3.22% | 3.20% | 3.71% |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  |
| avg. A1&A2 | -5.34% | -2.58% | -2.17% |

Table 10 Results of the official NNVC-6.0 SR model for AI configurations

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM Anchor |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -5.54% | -7.19% | -7.10% |
| Class A2 | -6.06% | -7.56% | -2.12% |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% |
| avg. A1&A2 | -5.80% | -7.38% | -4.61% |

# Conclusion

This contribution proposes a unified structure for NN-based SR. The proposed architecture is designed to be closely aligned to the unified in-loop filter architecture and results show a promising coding performance. It is recommended that the proposed unified structure to be adopted into NNVC.

# Reference

[1] E. Alshina, F. Galpin, “BoG report on NN-filter design unification”, JVET-AD0380

[2] E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, L. Wang, Z. Xie, “Exploration Experiments on Neural Network-based Video Coding (EE1)” JVET-AE2023

[3] E. Alshina, R.-L. Liao, S. Liu, A. Segall, “Common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-AE2016

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).