| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0146-v1 |
| --- | --- |

| Title: | AHG9: On Face Motion Information for Generative Face Video |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Boon Teo Jing Yuan Thong Kiyofumi Abe Chong Soon Lim Karlekar Jayashree | Tel: Email: | +65-6550-5417 hanboon.teo@sg.panasonic.com jingyuan.thong@sg.panasonic.com abe.kiyo@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

At the 29th JVET meeting, JVET-AC0088 presented a generative face video SEI message to allow for compression of talking-face videos at ultra-low bitrate. An expansion of its scope was presented in JVET-AD0051 at the 30th JVET meeting to encompass multiple facial representations. During the 31st JVET meeting, further enhancements to the generative face video SEI message syntax were propositioned in JVET-AE0280 which combined ideas proposed in JVET-AE0080 and JVET-AE0083. Also, the informative proposal JVET-AE0088 demonstrated the effectiveness of 3-D facial landmarks for generating talking-face videos, and the framework’s independence across number of landmarks for generation.

This contribution aims to provide further details presented in JVET-AE0088 and enhanced refinements to the generative face video SEI message from JVET-AE0280.

# Introduction

Generative face video involves the mapping of expressions and pose of source persons to target persons, while simultaneously ensuring that the identity and attributes of the targets are being preserved. Such techniques are currently adopted in many applications, like video conferencing, social media and games.

With the rise in popularity of generative networks, numerous works [1][2][3] have utilized models such as generative adversarial networks (GAN) to create talking-face videos. To enhance the photo-realistic aspect of the generated outputs, various methods such as projection of image features into latent spaces or extraction of motion representations have been introduced in recent works.

JVET-AC0088 [4] leveraged on the extraction of facial parameters of a 3D Morphable Model (3DMM) [5] from video frames, from which a generative face video SEI message was proposed for the transmission of these parameters. Building on the proposed message, JVET-AD0051 [6] expanded its scope to encompass multiple facial representations: 2-D landmarks, 2-D keypoints, 3-D keypoints and facial semantics. Some improvements to JVET-AD0051’s SEI message syntax were proposed in JVET-AE0280 [7], which sought to combine the ideas presented in JVET-AE0080 [8] and JVET-AE0083 [9]. These included the incorporation of fusion pictures, keypoint residuals, specification of neural network used for generation, and addendums to possible types of matrices.

JVET-AE0088 [10] outlined a framework that taps on retrieving interpretable 3-D facial landmarks from video frames, from which face parameters were extracted and transmitted in the bitstream. The framework exhibits independence across number of landmarks used for face generation.

This contribution seeks to introduce refinements to the generative face video message from JVET-AE0280 based on the framework introduced in JVET-AE0088.

# Proposal

The methodology discussed below may efficiently capture variations in different face region movements, and reduce the bitrate required for transmission. Note that the methodology can be used across different types of face representations and is generalizable to any example in the generative face video SEI message.

Here, this contribution leverages on the framework in JVET-AE0088 that taps on 3-D facial landmarks as a base example to illustrate the methodology. Figure 2 shows some examples of landmark representations.

## Grouping Landmarks according to Face Regions

As head movements are typically larger than that of core central face region, landmarks can be partitioned into one or more major groups to effectively capture variations in movements of different face regions. Each major group can represent a key feature of the face or formed based on facial muscle movements.

An example of such grouping is illustrated via different colors in Figure 2. Here, landmarks are partitioned into 3 major groups. Points that contain eye-related and mouth-related information of the face are assigned to the ‘eye’ and ‘mouth’ major groups respectively, while the rest are assigned to ‘contour’ major group.

Figure 2: Different Number of Landmarks [12][13] extracted from a face - (a) 478, (b) 128, (c) 78

## Formation of Sub-groups

To account for asymmetrical face expressions, major groups of points may be further split into sub-groups.

For example, to allow for scenarios of raising different eyebrows, blinks and winks, the ‘eye’ major group in Figure 2 may be further divided into ‘left eye’, ‘left brow’, ‘right eye’ and ‘right brow’ subgroups. In addition, the ‘mouth’ major group may be further split to upper lip sub-group and lower lip sub-group to allow for scenarios where the face makes a crooked or slanted mouth shape.

The additional splitting into smaller sub-groups may be useful if users have a larger bandwidth or wish to capture finer details of face actions, but may be omitted under smaller bandwidths or no inherent need for finer details, as sub-grouping may lead to a higher bitrate for transmission.

## Face Parameter Extraction via Transforms

To reduce the bitrate required for transmission, affine transforms can be used to extract face parameters. Face landmarks of each frame are treated as point clouds, and compared against previous frames to derive one or more sets of transformations. These transforms will be used to reconstruct the face landmarks of each driving frame at the decoder.

In the affine transformation framework, a 3-D point with coordinates x1, y1, z1 can be transformed to another location with coordinates x2, y2, z2 via the following equation, where a11,  a12,  a13,  a14,  a21,  a22, a23, a24, a31, a32, a33 and a34 form the 12 parameters of the 3-D affine model:

x2y2z21= M∙x1y1z11=a11a12a13a14a21a22a23a24a31a32a33a340001∙x1y1z11  			     (1)

With the introduction of landmark groupings, transforms can be derived based on each group of points from landmarks across different frames. These transformations aim to effectively retrieve variations in motion information of different groups representing different face regions.

As fewer parameters can now be used to describe face movements from a larger group of landmarks, this leads to a reduction in bits and hence a lower bandwidth requirement. Also, this can be adopted for different face landmark representations, and hence is independence on the landmark representation.

For the case of sub-groups, sets of transformation parameters will be derived based on each sub-group.

## Group-level Ordering of Transforms

Additionally, in some scenarios, the overall head movements are larger than subtle eye or mouth openings, leading to ineffective transform representations when purely comparing changes in landmark positions for both ‘eye’ and ‘mouth’ groups due to changes in overall head motion outweighing that of localized motions. Hence, a hierarchical process can be used to efficiently capture such differences in motion.

An example of this process is shown in Equations (2) – (4) below. The transform parameters of the contour group are first derived from the first (A) and second (B) set of landmarks, which indicates overall global head motion. Then, the contour transform can be applied on set A to indicate the new head pose, from which local eye and mouth motions can be derived. In this manner, smaller variations in movements of core central region can be effectively captured and disentangled from the larger variations in overall head poses.

PointscontourB= Tglobal∙PointscontourA                                                     (2)

PointseyeB= Teye∙Tglobal∙PointseyeA                                                       (3)

PointsmouthB=Tmouth∙Tglobal∙PointsmouthA                                          (4)

## Decomposition of Transforms

To further boost the interpretability of each transform, matrix M in (1) can be decomposed into translation T, rotation R, scaling S, and shear H matrices. The decomposed form is represented by Equation (5), where:

M= T∙R∙S∙H= 100tx010ty001tz0001∙r11r12r130r21r22r230r31r32r3300001∙sx0000sy0000sz00001∙1hxyhxz001hyz000100001      (5)

Each rotation matrix R can be further expressed as Euler angles: pitch, yaw and roll, as per Figure 3.

Figure 3 – Illustration of Pitch, Yaw and Roll Rotation Angles of a Face

During transmission of each transform, the encoder can send all the values, or omit some values, or even entire matrices. If any translation t, rotation r, scale s, or shear h value is not signaled in the bitstream, they would adopt a default value of 0, 0, 1 and 0 respectively at the decoder. If entire translation T, rotation R, scale S or shear H matrices are not signaled, they would default to identity matrices at the decoder.

# Proposed Bitstream Design

This chapter presents 2 possible options for refinements to the proposed bitstream design which integrates the abovementioned discussion into the syntax outlined in JVET-AE0280.

Option 1 (Chapter 3.2) explicitly creates additional flags and tables to denote the use of major groups and sub-groups during the extraction of face motion information, while Option 2 (Chapter 3.3) integrate the matrices for major and sub-groups into the possible types of matrices to be signalled in the message.

Here, a scalable design is adopted to support varying number of groups and varying number of parameters per transform, thus catering for networks of various bandwidths.

## Proposed SEI message scope

Table 1 - Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| … | … |
| Generative face video | Specified by the syntax of the SEI message |

## Option 1 of Generative face video SEI message

Add a new mode for the proposed method to gfv_matrix_type_idx of JVET-AE0280, and unify the original index 2 – 6 matrices of gfv_matrix_type_idx of JVET-AE0280 as a subset of the proposed method.

In particular, the following parameters are proposed to be added:

- gfv_groups_present_flag, gfv_num_groups_minus1
To indicate the groups that landmarks are partitioned to, for effective capturing of motions.

- gfv_major_group_type_idx, gfv_sub_group_type_idx, gfv_num_group_matrices_minus1
To identify each group of landmarks, possibilities of sub-groups, and number of matrices present for that group (to indicate if transform is decomposed or is present as an entire affine matrix). Sub-groups only exist when the corresponding major group is present.

- gfv_group_order_flag
To identify if the group should be treated as a global motion matrix (default) or localised motions.

- gfv_transform_order_idx, gfv_transform_type_idx
For a particular decomposed transform, the type of transform matrices that are decoded from the bitstream. This is only applicable if the affine transform is decomposed into fundamental matrices.

- gfv_kp_group_flag, gfv_kp_group_idx, gfv_kp_sub_group_idx
To indicate the group (and possible sub-group) that a landmark is assigned to.

### Syntax of generative face video SEI message [Option 1]

Table 2 - generative face video SEI message syntax

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| gfv_base_pic_flag /*indicate if current decoded output picture is a base picture*/ | u(1) |
| if( gfv_base_pic_flag ) { /*specify Generator( )*/ |  |
| gfv_key_present_flag |  |
| if( gfv_key_present_flag ) { |  |
| gfv_key | u(32) |
| else { |  |
| gfv_nn_base_flag | u(1) |
| gfv_nn_mode_idc | ue(v) |
| if( gfv_nn_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfv_nn_reserved_zero_bit_a | u(1) |
| gfv_nn_tag_uri | st(v) |
| gfv_nn_uri | st(v) |
| } |  |
| } |  |
| } else /* current decoded output picture is a driving picture*/ |  |
| gfv_drive_pic_fusion_flag /*indicate if DrivePicture is input to Generator( )*/ | ue(v) |
| gfv_coordinate_present_flag | u(1) |
| if( gfv_coordinate_present_flag ) { |  |
| gfv_coordinate_precision_factor_minus1 | ue(v) |
| gfv_num_kp_minus1 | ue(v) |
| gfv_kp_pred_flag | u(1) |
| gfv_coordinate_z_present_flag | u(1) |
| gfv_kp_group_flag | u(1) |
| if(gfv_coordinate_z_present_flag ) |  |
| gfv_coordinate_z_max_value_minus1 | ue(v) |
| for( i = 0; i <= num_kp_minus1; i++ ) { |  |
| if(!gfv_kp_pred_flag) { |  |
| gfv_coordinate_x_abs[ i ] | u(v) |
| if( gfv_coordinate_x_abs[ i ] ) |  |
| gfv_coordinate_x_sign_flag[ i ] | u(1) |
| gfv_coordinate_y_abs[ i ] | u(v) |
| if( gfv_coordinate_y_abs[ i ] ) |  |
| gfv_coordinate_y_sign_flag[ i ] | u(1) |
| if( gfv_coordinate_z_present_flag ) { |  |
| gfv_coordinate_z_abs[ i ] | u(v) |
| if( gfv_coordinate_z_abs[ i ] ) |  |
| gfv_coordinate_z_sign_flag[ i ] | u(1) |
| } |  |
| } |  |
| else { |  |
| gfv_coordinate_dx_abs[ i ] | u(v) |
| if(gfv_coordinate_dx_abs[ i ] ) |  |
| gfv_coordinate_dx_sign_flag[ i ] | u(1) |
| gfv_coordinate_dy_abs[ i ] | u(v) |
| if( gfv_coordinate_dy_abs[ i ] ) |  |
| gfv_coordinate_dy_sign_flag[ i ] | u(1) |
| if( gfv_coordinate_z_present_flag ) { |  |
| gfv_coordinate_dz_abs[ i ] | u(v) |
| if( gfv_coordinate_dz_abs[ i ] ) |  |
| gfv_coordinate_dz_sign_flag[ i ] | u(1) |
| } |  |
| } |  |
| if (gfv_kp_group_flag) |  |
| gfv_kp_group_idx[ i ] | u(8) |
| gfv_kp_sub_group_idx[ i ] | u(8) |
| } |  |
| } |  |
| gfv_matrix_present_flag | u(1) |
| if(gfv_matrix_present_flag ) { |  |
| gfv_matrix_element_precision_factor_minus1 | ue(v) |
| gfv_num_matrix_types_minus1 | ue(v) |
| for( i = 0; i <= num_matrix_types_minus1; i++ ) { |  |
| gfv_matrix_type_idx[ i ] | u(6) |
| if( gfv_matrix_type_idx[ i ] = = 0 ¦ ¦ gfv_matrix_type_idx[ i ] = = 1 ) { |  |
| if( gfv_coordinate_present_flag ) |  |
| gfv_num_matrices_equal_to_num_kps_flag[ i ] | u(1) |
| if( !gfv_coordinate_present_flag ¦ ¦ !gfv_num_matrix_equal_to_num_kps_flag[ i ] ) |  |
| gfv_num_matrices_info[ i ] | ue(v) |
| } |  |
| else if( gfv_matrix_type_idx[ i ] = = 2 ){ |  |
| gfv_groups_present_flag[ i ] | ue(v) |
| if ( ! gfv_groups_present_flag[ i ] ) |  |
| gfv_num_matrices_info[ i ] | ue(v) |
| else { |  |
| gfv_num_groups_minus1[ i ] | ue(v) |
| for (g = 0; g <= gfv_num_groups_minus1[ i ]; g++) { |  |
| gfv_major_group_type_idx[ i ][ g ] | u(8) |
| gfv_sub_group_type_idx[ i ][ g ] | u(8) |
| gfv_group_order_flag[ i ][ g ] | u(1) |
| gfv_num_group_matrices_minus1[ i ][ g ] | ue(v) |
| if ( gfv_num_group_matrices_minus1[ i ][ g ] > 0 ) { |  |
| for ( t = 0; t <= gfv_num_group_matrices_minus1[ i ][ g ]; t++) { |  |
| gfv_transform_order_idx[ i ][ g ][ t ] | u(8) |
| gfv_transform_type_idx[ i ][ g ][ t ] | u(8) |
| gfv_matrix_for_3D_space_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| else if( gfv_matrix_type_idx[ i ] = = 2 ¦ ¦ gfv_matrix_type_idx[ i ] = = 3 ¦ ¦ gfv_matrix_type_idx[ i ] >= 7 ){ |  |
| else if( gfv_matrix_type_idx[ i ] >= 3 ) { |  |
| if( gfv_matrix_type_idx[ i ] >= 7 ) |  |
| gfv_num_matrices_minus1[ i ] | ue(v) |
| gfv_matrix_width_minus1[ i ] | ue(v) |
| gfv_matrix_height_minus1[ i ] | ue(v) |
| } |  |
| else if( gfv_matrix_type_idx[ i ] >= 4 && gfv_matrix_type_idx[ i ] <= 6 && !gfv_coordinate_present_flag ) |  |
| gfv_matrix_for_3D_space_flag[ i ] | u(1) |
| for( j = 0; j < numMatrices[ i ]; j++ ) { |  |
| for( k = 0; k < matrixHeight[ i ]; k++ ) { |  |
| for( l = 0; l <matrixWidth[ i ]; l++ ) { |  |
| gfv_matrix_element_int[ i ][ j ][ k ][ l ] | ue(v) |
| gfv_matrix_element_dec[ i ][ j ][ k ][ l ] | u (v) |
| if( gfv_matrix_element_int[ i][ j ][ k ][ l ] ¦ ¦ gfv_matrix_element_dec[ i ][ j ][ k ][ l ] ) |  |
| gfv_matrix_element_sign_flag[ i ][ j ][ k ][ l ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| if( gfv_base_pic_flag && !gfv_key_present_flag) |  |
| if( gfv_nn_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfv_nn_reserved_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| gfv_nn_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

### Semantics of generative face video SEI message [Option 1]

The generative face video (GFV) SEI message indicates facial parameters and specifies a neural network, denoted as Generator( ), that may be used to generate novel output pictures using the indicated facial parameters and previously decoded output pictures.

NOTE – Facial parameters could be determined from source pictures prior to encoding. Such source pictures may be referred to as driving pictures.

NOTE – Previously decoded output pictures input to Generator( ) may be a base picture (a decoded output picture that provides the reference texture from which novel face pictures may be generated) and, optionally, a picture that can be fused by Generator( ) to improve background texture and facial details. When the current picture is not a base picture, the GFV SEI message may be used to generate a novel face picture based on the previously decoded base picture, the facial parameters conveyed by the GFV SEI message, and, optionally, the fusion picture. Base and fusion pictures may be coded conforming to Rec. ITU-T H.264, Rec. ITU-T H.265, Rec. ITU-T H.266, etc.

NOTE – A Generator( ) may be a generative adversarial network (GAN), for example.

Use of this SEI message requires the definition of the following variables:

- Input picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

- Luma sample array baseCroppedYPic and chroma sample arrays baseCroppedCbPic and baseCroppedCrPic for a decoded output picture, denoted as BasePicture, corresponding to a source base picture.

- Luma sample array driveCroppedYPic and chroma sample arrays driveCroppedCbPic and driveCroppedCrPic for a decoded output picture, denoted as DrivePicture, corresponding to a source driving picture.

- Bit depth BitDepthY for the luma sample array of the input pictures.

- Bit depth BitDepthC for the chroma sample arrays, if any, of the input pictures.

- A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

gfv_id contains an identifying number that may be used to identify face feature information and specify a neural network that may be used as Generator( ). The value of gfv_id shall be in the range of 0 to 232–− 2, inclusive. Values of gfv_id from 256 to 511, inclusive, and from 231 to 232–− 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering a GFV SEI message with gfv_id in the range of 256 to 511, inclusive, or in the range of 231 to 232–− 2, inclusive, shall ignore the SEI message.

NOTE – Different values of gfv_id in different GFV SEI messages could be used to identify different faces when more than one face is present in an output picture, for example.

gfv_base_pic_flag equal to 1 indicates the current decoded output picture corresponds to a base picture. gfv_base_pic_flag equal to 0 indicates the current decoded output picture does not correspond to a base picture.

The following constraints apply to the value of gfv_base_pic_flag:

–	When a GFV SEI message is the first GFV SEI message, in decoding order, that has a particular gfv_id value within the current CLVS, the value of gfv_base_pic_flag shall be equal to 1.

–	When a GFV SEI message that has a particular gfv_id value has gfv_base_pic_flag being equal to 0, this SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS or up to but excluding the decoded picture that follows the current decoded picture in output order within the current CLVS and is associated with a subsequent GFV SEI message, in decoding order, having gfv_base_pic_flag equal to 0 and that particular gfv_id value within the current CLVS, whichever is earlier.

gfv_key_present_flag equal to 1 indicates that the syntax element gfv_key is present and the syntax elements gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_reserved_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_payload_byte[ i ] are not present. gfv_key_present_flag equal to 0 indicates that the syntax element gfv_key is not present and the syntax elements gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_reserved_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_payload_byte[ i ].

gfv_key indicates a key that may be used to identify the analysis network used to generate the syntax elements of the current generative face video SEI message. The value of gfv_key may be used to determine whether the analysis network at the encoder matches with the generative network at the decoder.

NOTE 1 – The generative network at the decoder may use the SEI message to generate the video pictures only if it recognizes the value of gfv_key. The value of gfv_key may be specified by the applications that rely on matching networks at the encoder and the decoder. For such applications, if the parameters in the received SEI message are extracted with an analysis network that is not matched with the generative network at the decoder, the generative network should ignore the received SEI message.

gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_reserved_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_payload_byte[ i ] specify a neural network that may be used as a Generator( ). gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_reserved_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_payload_byte[ i ] have the same syntax and semantics as nnpfc_base_flag, nnpfc_mode_idc, nnpfc_reserved_zero_bit_a, nnpfc_tag_uri, nnpfc_uri, nnpfc_payload_byte[ i ], respectively.

gfv_drive_pic_fusion_flag, when present, equal to 1 indicates the current decoded output picture, which corresponds to a driving picture that may be used for fusion, may be input to Generator( ). gfv_drive_pic_fusion_flag equal to 0 indicates the current decoded output picture should not be input to Generator( ).

NOTE – A gfv_drive_pic_fusion_flag value of 1 could be used to indicate the current decoded output picture could be used to improve face details or handle background changes, as examples.

gfv_coordinate_present_flag equal to 1 indicates that coordinate information of keypoints is present. gfv_coordinate_present_flag equal to 0 indicates that coordinate information of keypoints is not present.

It is a requirement of bitstream conformance that when gfv_matrix_type_idx[ i ] for any i from 0 to gfv_num_matrix_types_minus1 is equal to 0 or 1, the value of gfv_coordinate_present_flag shall be equal to 1.

gfv_coordinate_precision_factor_minus1 plus 1 indicates the length, in bits, of gfv_coordinate_x_abs[i], gfv_coordinate_y_abs[i] and gfv_coordinate_z_abs[i].

gfv_num_kps_minus1 plus 1 indicates the number of keypoints. The value of gfv_num_kp_minus1 shall be in the range of 0 to 210 – 1, inclusive.

gfv_kp_pred_flag equal to 1 indicates syntax elements gfv_coordinate_dx_abs[ i ] ,gfv_coordinate_dy_abs[ i ], and gfv_coordinate_dz_abs[ i ] are present and syntax elements gfv_coordinate_dx_sign_flag[ i ], gfv_coordinate_dy_sign_flag[ i ] and gfv_coordinate_dz_sign_flag[ i ] may be present. gfv_kp_pred_flag equal to 0 indicates gfv_coordinate_x_abs[ i ] ,gfv_coordinate_y_abs[ i ], and gfv_coordinate_z_abs[ i ] are present and syntax elements gfv_coordinate_x_sign_flag[ i ], gfv_coordinate_y_sign_flag[ i ] and gfv_coordinate_z_sign_flag[ i ] may be present.

gfv_coordinate_z_present_flag equal to 1 indicates z-axis coordinate information of the keypoints is present. coordinate_z_present_flag equal to 0 indicates that the z-axis coordinate information of the keypoints is not present.

gfv_coordinate_z_max_value_minus1 plus 1 indicates the maximum absolute value of z-axis coordinates of keypoints.

gfv_coordinate_x_abs[ i ] indicates the normalized absolute value of x-axis coordinate of the i-th keypoint.

gfv_coordinate_x_sign_flag[ i ] specifies the sign of x-axis coordinate of the i-th keypoint. When gfv_coordinate_x_sign_flag[ i ] is not present, it is inferred to be equal to 0.

gfv_coordinate_y_abs[ i ] specifies the normalized absolute value of y-axis coordinate of i-th keypoint.

gfv_coordinate_y_sign_flag[ i ] specifies the sign of y-axis coordinate of i-th keypoint. When gfv_coordinate_y_sign_flag[i] is not present, it is inferred to be equal to 0.

gfv_coordinate_z_abs[ i ] specifies the normalized absolute value of z-axis coordinate of i-th keypoint.

gfv_coordinate_z_sign_flag[ i ] specifies the sign of z-axis coordinate of i-th key point. When gfv_coordinate_z_sign_flag[ i ] is not present, it is inferred to be equal to 0.

gfv_coordinate_dx_abs[ i ] indicates the absolute difference value of the normalized value of the x-axis coordinate of the i-th keypoint.

gfv_coordinate_dx_sign_flag[ i ] specifies the sign of the difference value of the x-axis coordinate of the i-th keypoint. When gfv_coordinate_dx_sign_flag[ i ] is not present, it is inferred to be equal to 0.

gfv_coordinate_dy_abs[ i ] specifies the absolute difference value of the normalized y-axis coordinate of the i-th keypoint.

gfv_coordinate_dy_sign_flag[ i ] specifies the sign of the difference value of the y-axis coordinate of the i-th keypoint. When gfv_coordinate_yd_sign_flag[i] is not present, it is inferred to be equal to 0.

gfv_coordinate_dz_abs[ i ] specifies the absolute difference value of the normalized z-axis coordinate of the i-th keypoint.

gfv_coordinate_dz_sign_flag[ i ] specifies the sign of the difference value of the z-axis coordinate of the i-th key point. When gfv_coordinate_dz_sign_flag[ i ] is not present, it is inferred to be equal to 0.

The variables coordinateDeltaX[ i ], coordinateDeltaY[ i ] and coordinateDeltaZ[ i ] indicating the delta x-axis coordinate, delta y-axis coordinate and delta z-axis oordinate of the i-th keypoint, respectively, are derived as follows:

coordinateDeltaX[ i ] = (1-2 * gfv_coordinate_dx_sign_flag[ i ]) * gfv_coordinate_dx_abs[ i ] * CroppedWidth1<< (gfv_coordinate_precision_factor_minus1 +1)

coordinateDeltaY[i] = (1-2 * gfv_coordinate_dy_sign_flag[ i ]) * gfv_coordinate_dy_abs[ i ] * CroppedHeight1<< (gfv_coordinate_precision_factor_minus1 +1)

coordinateDeltaZ[i] = (1-2 * coordinate_dz_sign_flag[ i ]) * gfv_coordinate_dz_abs[ i ] *(gfv_coordinate_z_max_value_minus1+1)1<< (gfv_coordinate_precision_factor_minus1 +1)

The variables coordinateX[ i ], coordinateY[ i ] and coordinateZ[ i ] indicating the x-axis coordinate, y-axis coordinate and z-axis coordinate of the i-th keypoint, respectively, are derived as follows:

When gfv_kp_pred_flag is equal to 0,

coordinateX[ i ] = (1-2 * gfv_coordinate_x_sign_flag[ i ]) * gfv_coordinate_x_abs[ i ] * CroppedWidth1<< (gfv_coordinate_precision_factor_minus1 +1)

coordinateY[i] = (1-2 * gfv_coordinate_y_sign_flag[ i ]) * gfv_coordinate_y_abs[ i ] * CroppedHeight1<< (gfv_coordinate_precision_factor_minus1 +1)

coordinateZ[i] = (1-2 * gfv_coordinate_z_sign_flag[ i ]) * gfv_coordinate_z_abs[ i ] *(gfv_coordinate_z_max_value_minus1+1)1<< (gfv_coordinate_precision_factor_minus1 +1)

when gfv_kp_pred_flag is equal to 1,

if( gfv_kp_pic_flag ) {
	coordinateX[ i ] = (( i > 0 ) ? coordinateX[ i - 1 ] : 0 ) + coordinateDeltaX[ i ]
	coordinateY[ i ] = (( i > 0 ) ? coordinateY[ i - 1 ] : 0 ) + coordinateDeltaY[ i ]
	coordinateZ[ i ] = (( i > 0 ) ? coordinateZ[ i - 1 ] : 0 ) + coordinateDeltaZ[ i ]
}
else {
	coordinateX[ i ] = BaseKpCoordinateX[ i ] + coordinateDeltaX[ i ]
	coordinateY[ i ] = BaseKpCoordinateY[ i ] + coordinateDeltaY[ i ]
	coordinateZ[ i ] = BaseKpCoordinateZ[ i ] + coordinateDeltaZ[ i ]
}

where BaseKpCoordinateX[ i ], BaseKpCoordinateY[ i ], BaseKpCoordinateZ[ i ] indicating the x-axis, y-axis and z-axis coordinates, respectively, of the i-th keypoint for the base picture are derived as follows:

if( gfv_base_pic_flag ) {
	BaseKpCoordinateX[ i ] = coordinateX[ i ]
	BaseKpCoordinateY[ i ] = coordinateY[ i ]
	BaseKpCoordinateZ[ i ] = coordinateZ[ i ]
}

gfv_kp_group_flag specifies the presence or absence of a pre-defined mapping table to map each keypoint to its assigned group of facial feature regions. A value of 1 indicates that keypoints would be assigned to groups, while a value of 0 indicates that keypoints would not be assigned to any groups.

gfv_kp_group_idx[ i ] specifies the index of the group of facial region that the i-th keypoint is assigned to.

gfv_kp_sub_group_idx[ i ] specifies the index of the sub-group of facial region that the i-th keypoint is assigned to.

gfv_matrix_present_flag equal to 1 indicates that matrix parameters are present. gfv_matrix_present_flag equal to 0 indicates that matrix parameters are not present.

gfv_matrix_element_precision_factor_minus1 plus 1 indicates the length, in bits, of gfv_matrix_element_dec[ i ][ j ][ k ][ l ].

gfv_num_matrix_types_minus1 plus 1 indicates the number of matrix types signalled in the SEI message. The value of gfv_matrix_type_num_minus1 shall be in the range of 0 to 26 – 1, inclusive.

gfv_matrix_type_idx[ i ] indicates the index of the i-th matrix type as specified in Table 3.

Table 3 - Specification of gfv_matrix_type_idx

| Value | Specification |
| --- | --- |
| 0 | Affine translation matrix with the size of 2*2 or 3*3. |
| 1 | Covariance matrix with size of 2*2 or 3*3. |
| 2 | Face object motion matrix |
| 2 | Mouth matrix representing mouth motion. |
| 3 | Eye matrix representing the open-close status and level of eyes. |
| 4 | Head rotation paramters with the size of 2*2 or 3*3 representing the head rotation in 2D space or 3D space. |
| 5 | Head translation matrix with the size of 1*2 or 1*3 representing head translationin 2D space or 3D space. |
| 6 | Head location matrix with size of 1*2 or 1*3 representing the head location in 2D space or 3D space. |
| 73 | Compact feature matrix with the size being specified by gfv_matrix_width_minus1[i] and gfv_matrix_height_minus1[i]. |
| 4…31 | Other matrix that may be used as determined by the application with the size being specified by gfv_matrix_width_minus1[i] and gfv_matrix_height_minus1[i]. |
| 32…63 | Reserved |

NOTE 2 . The undefined matrix type is used to represent the matrxi type rather than affine translation matrix, covariance matrix, rotation matrix, translation matrix and compact feature matrix. It is may be used by the user to extend the matrix type.

gfv_num_matrices_equal_to_num_kps_flag[ i ] equal to 1 indicates that the number of matrices of the i-th matrix type is equal to gfv_num_kps_minus1 + 1. gfv_num_matrices_equal_to_num_kps_flag[ i ] equal to 0 indicates the number of matrices of the i-th matrix type is not equal to gfv_num_coordinates_minus1 + 1.

gfv_num_matrices_info[ i ] provides information to derive the number of the matrices of the i-th matrix type.

gfv_groups_present_flag[ i ] indicates if keypoints should be split into groups, from which transformation parameters would be computed on. A value of 0 indicates there are no groups present, and hence the specification would fallback to the default behavior of purely decoding gfv_num_matrices_info[ i ] for affine or covariance types of matrices.

gfv_num_groups_minus1[ i ] plus 1 indicates the number of keypoint-groups signalled in the SEI message. When present together with gfv_groups_present_flag[ i ], this indicates there is at least 1 group that keypoints are assumed to be segregated into when performing any transformation. The value of gfv_num_groups_minus1 shall be in the range of 0 to 26 – 1, inclusive.

gfv_major_group_type_idx[ i ][ g ] indicates the index of the type of g-th major group of the face for the i-th matrix type, as specified in Table 4.

Table 4 - Specification of gfv_major_group_type_idx

| Value | Specification |
| --- | --- |
| 0 | Head |
| 1 | Eyes |
| 2 | Mouth |
| 3 … 255 | Reserved |

gfv_sub_group_type_idx[ i ][ g ] indicates the index of the sub-group arising from the corresponding g-th major group for the i-th matrix type, as specified in Table 5 and Table 6. Note that the reference table to refer for the definition of sub-groups would be dependent on the index of the corresponding major group from Table 4. For example, Table 5 and Table 6 are applicable when gfv_major_group_type_idx has a value of 1 and 2 respectively.

Table 5 - Specification of gfv_sub_group_type_idx, for sub-groups formed from the "Eye" major group

| Value | Specification |
| --- | --- |
| 0 | Two eyes + Two Eyebrows |
| 1 | Two eyes |
| 2 | Two eyebrows |
| 3 | Right Eye + Right Eyebrow |
| 4 | Left Eye + Left Eyebrow |
| 5 | Right Eye |
| 6 | Left Eye |
| 7 | Right Eyebrow |
| 8 | Left Eyebrow |
| 9 … 255 | Reserved |

Table 6 - Specification of gfv_sub_group_type_idx, for sub-groups formed from the "Mouth" major group

| Value | Specification |
| --- | --- |
| 0 | Entire Mouth Region |
| 1 | Upper Lip |
| 2 | Lower Lip |
| 3 | Outer Lip Region |
| 4 | Inner Lip Region |
| 5 … 255 | Reserved |

gfv_group_order_flag[ i ][ g ] indicates the group-level ordering for the g-th group of the i-th matrix type. A value of 0 indicates the matrix associated with this group has an overall global motion, while 0 indicates a local motion.

gfv_num_group_matrices_minus1[ i ][ g ] plus 1 indicates the number of matrices for the g-th group of the i-th matrix type. A value of 0 indicates that the entire matrix would be a single affine transformation matrix, while a value of more than 0 indicates that the decomposed form of the affine transformation matrix would be present for the g-th group, and these matrices should be combined to recreate the affine transform.

gfv_transform_order_idx[ i ][ g ] indicates the intra-group ordering of the decomposed matrices for the g-th group of the i-th matrix type, and will be in the form of a running-order index with values counting up from 0. Matrices are pre-multiplied as they are received, such as the form “… x transform_order_idx_2 x transform_order_idx_1 x transform_order_idx_0”.

gfv_transform_type_idx[ i ][ g ][ t ] indicates the matrix type of the t-th decomposed matrix for the g-th group of the i-th matrix type, as specified in Table 7, and is only valid when gfv_num_matrices_minus1 is more than 0.

Table 7 - Specification of gfv_transform_type_idx

| Value | Specification |
| --- | --- |
| 0 | Affine Transformation Matrix |
| 1 | Translation Matrix |
| 2 | Full Rotation Matrix |
| 3 | Scale Matrix |
| 4 | Shear Matrix |
| 5 | Location Matrix |
| 6 | Rotation Angles |
| 7 … 255 | Reserved |

gfv_matrix_width_minus1[ i ] plus 1 indicates the width of the matrix of the i-th matrix type.

gfv_matrix_height_minus1[ i ] plus 1 indicates the height of the matrix of the i-th matrix type.

gfv_matrix_for_3D_space_flag[ i ] equal to 1 indicates the matrix of the i-th matrix type is a matrix defined in three-dimensional space. gfv_matrix_for_3D_space_flag[ i ] equal to 0 indicates the matrix of the i-th matrix type is a matrix defined in two-dimensional space.

When gfv_matrix_width_minus1[ i ] is not present, it is inferred as follows:

––	If (gfv_matrix_type_idx[ i ] is equal to 2, with gfv_transform_type_idx[ i ][ g ][ t ] is equal to 0), and one of coordinate_z_present_flag and gfv_matrix_for_3D_space_flag[ i ] is present and equal to 1, then  gfv_matrix_width_minus1[i] is inferred to be equal to 3

––	otherwise, if (gfv_matrix_type_idx[ i ] is equal to 0 or 1 or 4) or (gfv_matrix_type_idx[ i ] is equal to 2, with gfv_transform_type_idx[ i ][ g ][ t ] is equal to 2), and one of coordinate_z_present_flag and gfv_matrix_for_3D_space_flag[ i ] is present and equal to 1, then  gfv_matrix_width_minus1[i] is inferred to be equal to 2

––	otherwise, if (gfv_matrix_type_idx[ i ] is equal to 2, with gfv_transform_type_idx[ i ][ g ][ t ] is equal to 0), and one of coordinate_z_present_flag and gfv_matrix_for_3D_space_flag[ i ] is present and equal to 0, then  gfv_matrix_width_minus1[i] is inferred to be equal to 2

––	otherwise, if (gfv_matrix_type_idx[ i ] is equal to 0 or 1 or 4) or (gfv_matrix_type_idx[ i ] is equal to 2 with gfv_transform_type_idx[ i ][ g ][ t ] is equal to 2), and one of coordinate_z_present_flag and gfv_matrix_for_3D_space_flag[ i ] is present and equal to 0, then gfv_matrix_width_minus1[ i ] is inferred to be equal to 1

––	otherwise (matrix_type_idx[ i ] is equal to 5 or 6 2 and gfv_transform_type_idx[ i ][ g ][ t ] is not equal to 0 or 2), gfv_matrix_width_minus1[ i ] is inferred to be equal to 0

When gfv_matrix_height_minus1[ i ] is not present, it is inferred as follows:

––	If (gfv_matrix_type_idx[ i ] is equal to 0, 1, 4, 5 or 6 or 2), and one of gfv_coordinate_z_present_flag and gfv_matrix_for_3D_space_flag[ i ] is present and equal to 1, gfv_matrix_height_minus1[ i ] is inferred to be equal to 2.

––	otherwise (gfv_matrix_type_idx[ i ] is equal to 0, 1, 4, 5 or 6 or 2), and one of gfv_coordinate_z_present_flag and gfv_matrix_for_3D_space_flag[ i ] is 0), gfv_matrix_height _minus1[ i ] is inferred to be equal to 1.

The variables matrixWidth[ i ] and matrixHeight[ i ] indicating the width and height of the matrix of the i-th matrix type are derived as follows

matrixWidth[ i ] = gfv_matrix_width_minus1[ i ] + 1

matrixHeight[ i ] = gfv_matrix_height_minus1[ i ] + 1

gfv_num_matrices_minus1[ i ] plus 1 indicates the number of matrices of the i-th matrix type.

The variable numMatrices[ i ] indicating the number of the matrices of the i-th matrix type is derived as follows:

if( gfv_matrix_type_idx[ i ] == 0 || gfv_matrix_type_idx[ i ] == 1 ) {

if( gfv_coordinate_present_flag )

numMatrices[ i ] = gfv_num_matrices_equal_to_num_kps_flag[ i ] ? gfv_num_kps_minus1 + 1 : ( gfv_num_matrices_info[ i ] < gfv_num_kp_minus1 ? gfv_num_matrices_info [ i ] + 1 : gfv_num_matrices_info [ i ] + 2 )

else

numMatrices[ i ] = gfv_num_matrices_info[ i ] + 1

}

else if( gfv_matrix_type_idx[ i ] >= 2 &&  gfv_matrix_type_idx[ i ] < 7)

numMatrices[ i ] = 1

else if( gfv_matrix_type_idx[ i ] == 2 ) {

if ( ! gfv_groups_present_flag[ i ] )

numMatrices[ i ] = gfv_num_matrices_info[ i ] + 1

else {

numMatrices[ i ] = 0

for( g = 0; g <= gfv_num_groups_minus1[ i ]; g++ )

numMatrices[ i ] += ( gfv_num_matrices_minus1[ i ][ g ] + 1 )

}

else

numMatrices[ i ] = gfv_num_matrices_minus1[ i ] + 1

gfv_matrix_element_int[ i ][ j ][ k ][ l ] indicates the integer part of the value of the matrix element at position (k, l) of the j-th matrix of the i-th matrix type.

gfv_matrix_element_dec[ i ][ j ][ k ][ l ] indicates the decimal part of the value of the matrix element at position (k, l) of the j-th matrix of the i-th matrix type.

gfv_matrix_element_sign_flag[ i ][ j ][ k ][ l ] indicates the sign of the matrix element at position (k, l) of the j-th matrix of the i-th matrix type. When gfv_matrix_element_sign_flag[ i ][ j ][ k ][ l ]is not present, it is inferred to be equal to 0.

The variable matrixElementVal[ i ][ j][ k ][ l ] representing the value of the matrix element at position (k, l) of the j-th matrix of the i-th matrix type is derived as follows:

matrixElementVal[ i][ j ][ k ][ l ] = 1-2*gfv_matrix_element_sign_flag[ i ][ j ][ k ][ l ]*(gfv_matrix_element_int i  j  k  l  +gfv_matrix_element_dec[ i ][ j ][ k ][ l ]1<< (gfv_matrix_element_precision_factor_minus1 +1))

Generator ( ) is a process to generate the sample values of an output picture corresponding to a driving picture. It is only invoked when gfc_base_pic_flag is equal to 0.

Inputs to Generator() are:

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 0 and ChromaIDC is equal to 0: inputBaseY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 0 and ChromaIDC is not equal to 0: inputBaseY, inputBaseCb, inputBaseCr, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 1 and ChromaIDC is equal to 0: inputBaseY, inputDriveY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 1 and ChromaIDC is not equal to 0: inputBaseY, inputBaseCb, inputBaseCr, inputDriveY, inputDriveCb, inputDriveCr , inputBaseKeyPoint, inputBaseMatrix,, inputDriveKeyPoint, inputDriveMatrix

Outputs of Generator( ) are:

- When chromaIDC is equal to 0, a luma sample array genY

- When chromaIDC is not equal to 0, a luma sample array genY and chroma sample arrays genCb and genCr.

The following process is used to generate a video picture:

DeriveInputTensors( ) 
if( gfv_base_pic_flag == 0 && gfv_drive_pic_fusion_flag == 0) {
  if(chromaIDC == 0 ) 
    Generator( inputBaseY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix) 
  else 
    Generator( inputBaseY, inputBaseCb, inputBaseCr, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix) 
}
else if(gfv_base_pic_flag == 0 && gfv_drive_pic_fusion_flag == 1) {
  if(chromaIDC == 0 ) 
    Generator( inputBaseY, inputDriveY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix) 
  else 
    Generator( inputBaseY, inputBaseCb, inputBaseCr, inputDriveY, inputDriveCb, inputDriveCr , inputBaseKeyPoint, inputBaseMatrix,, inputDriveKeyPoint, inputDriveMatrix) 
}

StoreOutputTensors( )

Input values to Generator( ) are real numbers and the functions InpY( ) and InpC( ) are specified as follows:

InpY( x ) = x ÷ ( ( 1  <<  BitDepthY ) – 1 )

InpC( x ) = x ÷ ( ( 1  <<  BitDepthC ) – 1 )

Output values from Generator( ) are real numbers and the functions OutY( ) and OutC( ) are specified as follows:

OutY( x ) = Clip3( 0, ( 1  <<  BitDepthY ) – 1 , x * ( ( 1  <<  BitDepthY ) – 1 )

OutC( x ) = Clip3( 0, ( 1  <<  BitDepthC ) – 1 , x * ( ( 1  <<  BitDepthC ) – 1 )

The process DeriveInputTensors( ) for deriving the inputs of Generator ( ) is specified as follows:

When gfv_base_pic_flag is equal to 1, the BasePicture input tensor inputBaseY, inputBaseCb and inputBaseCr are derived as follows:

for( x = 0; x< CroppedWidth; x++ ) {
  for ( y = 0; y< CroppedHeight; y++ ) {
    inputBaseY[ x ][ y ] = InpY( baseCroppedYPic[ x ][ y ] ) 
  }
}
if (chromaIDC !=0) {
  for( x=0; x< CroppedWidth/ SubWidthC; x++ ) {
    for ( y=0; y< CroppedHeight/ SubHeightC; y++ ) {
      inputBaseCb[ x][ y ] = InpC( baseCroppedCbPic[ x][ y ] )
      inputBaseCr[ x][ y ] = InpC( baseCroppedCrPic[ x ][ y ] )
    }
  }
}

When gfv_drive_pic_fusion_flag is equal to 1, the DrivePicture luma sample array inputDriveY, inputDriveCb and input DriveCr are derived as follows:

for( x = 0; x< CroppedWidth; x++ ) {
  for ( y = 0; y< CroppedHeight; y++ ) {
    inputDriveY[ x ][ y ] = InpY( driveCroppedYPic[ x ][ y ] )
  }
}
if (chromaIDC !=0) {
  for( x=0; x< CroppedWidth/ SubWidthC; x++ ) {
    for ( y=0; y< CroppedHeight/ SubHeightC; y++ ) {
      InputDriveCb[ x][ y ] = InpC( driveCroppedCbPic[ x][ y ] )
      InputDriveCr[ x][ y ] = InpC( driveCroppedCrPic[ x ][ y ] ) 
    }
  }
}

When gfv_base_pic_flag is equal to 0, the keypoint coordinate array inputDriveKeyPoint and the matrix inputDriveMatrix for the current picture are derived as follows:

if( gfv_coordinate_present_flag ) {
  for ( i=0; i< = gfv_num_kps_minus1; i++ ) {
    inputDriveKeyPoint[ i ][ 0 ] = coordinateX[ i ]
    inputDriveKeyPoint[ i ][ 1 ] = coordinateY[ i ]
    if ( gfv_coordinate_z_present_flag ) 
      inputDriveKeyPoint[ i ][ 2 ] = coordinateZ[ i ]
  }
}
else {
  for ( i=0; i< = gfv_num_kps_minus1; i++ ) {
    inputDriveKeyPoint[ i ][ 0 ] = 0
    inputDriveKeyPoint[ i ][ 1 ] = 0
    if ( gfv_coordinate_z_present_flag ) 
      inputDriveKeyPoint[ i ][ 2 ] = 0
  }
}
if( gfv_matrix_present_flag ) {
  for ( i=0; i<=gfv_num_matrix_types_minus1; i++ ) {
    for ( j=0; j< numMatrices[ i ]; j++ ) {
      for( k=0; k< matrixHeight [ i ]; k++ ) {
        for ( l=0;l< matrixWidth [ i ]; l++) {
          inputDriveMatrix[ i ][ j ][ k ][ l ] = matrixElementVal[ i ][ j][ k][ l ] 
        }
      }
    }
  }
}
else {
  for ( i=0; i<= gfv_num_matrix_types_minus1; i++ ) {
    for ( j=0; j< numMatrices[ i ]; j++ ) {
      for( k=0; k< matrixHeight [ i ]; k++ ) {
        for ( l=0;l< matrixWidth [ i ]; l++) {
          inputDriveMatrix[ i ][ j ][ k ][ l ] = 0
        }
      }
    }
  }
}

When gfv_base_pic_flag is equal to 1, the keypoint coordinate array inputBaseKeyPoint and the matrix inputBaseMatrix for the base picture are derived as follows:

if( gfv_coordinate_present_flag ) {
  for ( i=0; i< = gfv_num_kps_minus1; i++ ) {
    inputBaseKeyPoint[ i ][ 0 ] = coordinateX[ i ]
    inputBaseKeyPoint[ i ][ 1 ] = coordinateY[ i ]
    if ( gfv_coordinate_z_present_flag ) 
      inputBaseKeyPoint[ i ][ 2 ] = coordinateZ[ i ]
  }
}
else {
  for ( i=0; i< =num_kps_minus1; i++ ) {
    inputBaseKeyPoint [ i ][ 0 ] = 0
    inputBaseKeyPoint [ i ][ 1 ] = 0
    if ( gfv_coordinate_z_present_flag ) 
      inputBaseKeyPoint [ i ][ 2 ] = 0
  }
}
if( gfv_matrix_present_flag ) {
  for ( i=0; i<= gfv_num_matrix_types_minus1; i++ ) {
    for ( j=0; j< numMatrices[ i ]; j++ ) {
      for( k=0; k< matrixHeight [ i ]; k++ ) {
        for ( l=0;l< matrixWidth [ i ]; l++) {
          inputBaseMatrix[ i ][ j ][ k ][ l ] = matrixElementVal[ i ][ j][ k][ l ] 
        }
      }
    }
  }
}
else {
  for ( i=0; i<=gfv_num_matrix_types_minus1; i++ ) {
    for ( j=0; j< numMatrices[ i ]; j++ ) {
      for( k=0; k< matrixHeight [ i ]; k++ ) {
        for ( l=0;l< matrixWidth [ i ]; l++) {
          inputBaseMatrix [ i ][ j ][ k ][ l ] = 0
        }
      }
    }
  }
}

The process StoreOutputTensors( ) for deriving the output is specified as follows:

when gfv_base_pic_flag is equal to 0, the output sample array outYPic[ x ][ y ], outCbPic[ x ][ y ], and outCrPic[ x ][ y ] are derived as follows:

for(x=0; x< CroppedWidth; x++){
  for(y=0; y< CroppedHeight; y++){
    outputYPic[ x ][ y ] = OutY( genY[ x ][ y ] )
  }
}

if(chromaIDC != 0) {
  for(x=0; x< CroppedWidth/ SubWidthC; x++){
    for(y=0; y< CroppedHeight/ SubHeightC; y++){
      outputCbPic[ x ][ y ] = OutC( genCb[ x ][ y ] )
      outputCrPic[ x][ y ] = OutC( genCr[ x ][ y ] )
    }
  }
}
when gfv_base_pic_flag is equal to 1, the output sample array outYPic[ x ][ y ], outCbPic[ x ][ y ], and outCrPic[ x ][ y ] are derived as follows:

for(x=0; x< CroppedWidth; x++){
  for(y=0; y< CroppedHeight; y++){
    outputYPic[ x ][ y ] = baseCroppedYPic [ x ][ y ] 
  }
}

if(chromaIDC != 0) {
  for(x=0; x< CroppedWidth/ SubWidthC; x++){
    for(y=0; y< CroppedHeight/ SubHeightC; y++){
      outputCbPic[ x ][ y ] = baseCroppedCbPic [ x ][ y ] 
      outputCrPic[ x][ y ] = baseCroppedCbPic [ x ][ y ] 
    }
  }
}

## Option 2 of Generative face video SEI message

Add each group and subgroup of the proposed method to gfv_matrix_type_idx of JVET-AE0280, and integrate the original index 2 – 6 matrices of JVET-AE0280 into the group of the proposed method.

In particular, the following parameters are proposed to be added:

- gfv_group_order_flag
To identify if the group should be treated as a global motion matrix (default) or localised motions.

- gfv_transform_order_idx, gfv_transform_type_idx
For a particular decomposed transform, the type of transform matrices that are decoded from the bitstream. This is only applicable if the affine transform is decomposed into fundamental matrices.

- gfv_matrix_for_equal_scale_flag
For scaling matrices of decomposed transforms, to indicate if scaling factors for all dimensions should be signaled only once and treated as equal values across all dimensions.

- gfv_kp_group_flag, gfv_kp_group_idx, gfv_kp_sub_group_idx
To indicate the group (and possible sub-group) that a landmark is assigned to.

In addition, the following parameters are proposed to be modified:

- gfv_matrix_width, gfv_matrix_height 
Size of each decoded matrix. In this proposal, its definition is removed from gfv_matrix_type_idx’s scope to allow for flexibility in defining matrix types without size restriction, and scalability in sending any number of parameters for each matrix. In addition, both matrix height and width can be equal to 0 to allow for scenarios where no parameters of the affine model are sent.

- gfv_matrix_type_idx
Definition expanded to include more groups or sub-groups that landmarks can be partitioned to.

### Syntax of generative face video SEI message [Option 2]

Table 2 - generative face video SEI message syntax

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| gfv_base_pic_flag /*indicate if current decoded output picture is a base picture*/ | u(1) |
| if( gfv_base_pic_flag ) { /*specify Generator( )*/ |  |
| gfv_key_present_flag |  |
| if( gfv_key_present_flag ) { |  |
| gfv_key | u(32) |
| else { |  |
| gfv_nn_base_flag | u(1) |
| gfv_nn_mode_idc | ue(v) |
| if( gfv_nn_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfv_nn_reserved_zero_bit_a | u(1) |
| gfv_nn_tag_uri | st(v) |
| gfv_nn_uri | st(v) |
| } |  |
| } |  |
| } else /* current decoded output picture is a driving picture*/ |  |
| gfv_drive_pic_fusion_flag /*indicate if DrivePicture is input to Generator( )*/ | ue(v) |
| gfv_coordinate_present_flag | u(1) |
| if( gfv_coordinate_present_flag ) { |  |
| gfv_coordinate_precision_factor_minus1 | ue(v) |
| gfv_num_kp_minus1 | ue(v) |
| gfv_kp_pred_flag | u(1) |
| gfv_coordinate_z_present_flag | u(1) |
| gfv_kp_group_flag | u(1) |
| if(gfv_coordinate_z_present_flag ) |  |
| gfv_coordinate_z_max_value_minus1 | ue(v) |
| for( i = 0; i <= num_kp_minus1; i++ ) { |  |
| if(!gfv_kp_pred_flag) { |  |
| gfv_coordinate_x_abs[ i ] | u(v) |
| if( gfv_coordinate_x_abs[ i ] ) |  |
| gfv_coordinate_x_sign_flag[ i ] | u(1) |
| gfv_coordinate_y_abs[ i ] | u(v) |
| if( gfv_coordinate_y_abs[ i ] ) |  |
| gfv_coordinate_y_sign_flag[ i ] | u(1) |
| if( gfv_coordinate_z_present_flag ) { |  |
| gfv_coordinate_z_abs[ i ] | u(v) |
| if( gfv_coordinate_z_abs[ i ] ) |  |
| gfv_coordinate_z_sign_flag[ i ] | u(1) |
| } |  |
| } |  |
| else { |  |
| gfv_coordinate_dx_abs[ i ] | u(v) |
| if(gfv_coordinate_dx_abs[ i ] ) |  |
| gfv_coordinate_dx_sign_flag[ i ] | u(1) |
| gfv_coordinate_dy_abs[ i ] | u(v) |
| if( gfv_coordinate_dy_abs[ i ] ) |  |
| gfv_coordinate_dy_sign_flag[ i ] | u(1) |
| if( gfv_coordinate_z_present_flag ) { |  |
| gfv_coordinate_dz_abs[ i ] | u(v) |
| if( gfv_coordinate_dz_abs[ i ] ) |  |
| gfv_coordinate_dz_sign_flag[ i ] | u(1) |
| } |  |
| } |  |
| if (gfv_kp_group_flag) |  |
| gfv_kp_group_idx[ i ] | u(8) |
| gfv_kp_sub_group_idx[ i ] | u(8) |
| } |  |
| } |  |
| gfv_matrix_present_flag | u(1) |
| if(gfv_matrix_present_flag ) { |  |
| gfv_matrix_element_precision_factor_minus1 | ue(v) |
| gfv_num_matrix_types_minus1 | ue(v) |
| for( i = 0; i <= num_matrix_types_minus1; i++ ) { |  |
| gfv_matrix_type_idx[ i ] | u(6) |
| if( gfv_matrix_type_idx[ i ] = = 0 ¦ ¦ gfv_matrix_type_idx[ i ] = = 1 ) { |  |
| if( gfv_coordinate_present_flag ) |  |
| gfv_num_matrices_equal_to_num_kps_flag[ i ] | u(1) |
| if( !gfv_coordinate_present_flag ¦ ¦ !gfv_num_matrix_equal_to_num_kps_flag[ i ] ) |  |
| gfv_num_matrices_info[ i ] | ue(v) |
| } |  |
| else if( gfv_matrix_type_idx[ i ] = = 2 ¦ ¦ gfv_matrix_type_idx[ i ] = = 3 ¦ ¦ gfv_matrix_type_idx[ i ] >= 7 ){ |  |
| else if( gfv_matrix_type_idx[ i ] >= 2 ){ |  |
| if( gfv_matrix_type_idx[ i ] >= 7 ) |  |
| gfv_num_matrices_minus1[ i ] | ue(v) |
| if ( gfv_matrix_type_idx[ i ] >= 3 && gfv_matrix_type_idx[ i ] <= 11 !gfv_coordinate_present_flag ) { |  |
| gfv_group_order_flag[ i ] | u(1) |
| for ( t = 0 ; t <= gfv_num_matrices_minus1[ i ] ; t++) { |  |
| gfv_transform_type_idx[ i ][ t ] | u(8) |
| if ( gfv_transform_type_idx[ i ][ t ] > 0 ) { |  |
| gfv_transform_order_idx[ i ][ t ] | u(8) |
| if ( gfv_transform_type_idx[ i ][ t ] == 3 ) |  |
| gfv_matrix_for_equal_scale_flag [ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| gfv_matrix_width_minus1[ i ] | ue(v) |
| gfv_matrix_height_minus1[ i ] | ue(v) |
| } |  |
| for( j = 0; j < numMatrices[ i ]; j++ ) { |  |
| gfv_matrix_height[ i ][ j ] | ue(v) |
| for( k = 0; k < gfv_matrix_height[ i ][ j ]; k++ ) { |  |
| gfv_matrix_width[ i ][ j ][ k ] | ue(v) |
| for( l = 0; l <gfv_matrix_width[ i ][ j ][ k ]; l++ ) { |  |
| gfv_matrix_element_int[ i ][ j ][ k ][ l ] | ue(v) |
| gfv_matrix_element_dec[ i ][ j ][ k ][ l ] | u (v) |
| if( gfv_matrix_element_int[ i][ j ][ k ][ l ] ¦ ¦ gfv_matrix_element_dec[ i ][ j ][ k ][ l ] ) |  |
| gfv_matrix_element_sign_flag[ i ][ j ][ k ][ l ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| if( gfv_base_pic_flag && !gfv_key_present_flag) |  |
| if( gfv_nn_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfv_nn_reserved_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| gfv_nn_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

### Semantics of generative face video SEI message [Option 2]

The generative face video (GFV) SEI message indicates facial parameters and specifies a neural network, denoted as Generator( ), that may be used to generate novel output pictures using the indicated facial parameters and previously decoded output pictures.

NOTE – Facial parameters could be determined from source pictures prior to encoding. Such source pictures may be referred to as driving pictures.

NOTE – Previously decoded output pictures input to Generator( ) may be a base picture (a decoded output picture that provides the reference texture from which novel face pictures may be generated) and, optionally, a picture that can be fused by Generator( ) to improve background texture and facial details. When the current picture is not a base picture, the GFV SEI message may be used to generate a novel face picture based on the previously decoded base picture, the facial parameters conveyed by the GFV SEI message, and, optionally, the fusion picture. Base and fusion pictures may be coded conforming to Rec. ITU-T H.264, Rec. ITU-T H.265, Rec. ITU-T H.266, etc.

NOTE – A Generator( ) may be a generative adversarial network (GAN), for example.

Use of this SEI message requires the definition of the following variables:

- Input picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

- Luma sample array baseCroppedYPic and chroma sample arrays baseCroppedCbPic and baseCroppedCrPic for a decoded output picture, denoted as BasePicture, corresponding to a source base picture.

- Luma sample array driveCroppedYPic and chroma sample arrays driveCroppedCbPic and driveCroppedCrPic for a decoded output picture, denoted as DrivePicture, corresponding to a source driving picture.

- Bit depth BitDepthY for the luma sample array of the input pictures.

- Bit depth BitDepthC for the chroma sample arrays, if any, of the input pictures.

- A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

gfv_id contains an identifying number that may be used to identify face feature information and specify a neural network that may be used as Generator( ). The value of gfv_id shall be in the range of 0 to 232–− 2, inclusive. Values of gfv_id from 256 to 511, inclusive, and from 231 to 232–− 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering a GFV SEI message with gfv_id in the range of 256 to 511, inclusive, or in the range of 231 to 232–− 2, inclusive, shall ignore the SEI message.

NOTE – Different values of gfv_id in different GFV SEI messages could be used to identify different faces when more than one face is present in an output picture, for example.

gfv_base_pic_flag equal to 1 indicates the current decoded output picture corresponds to a base picture. gfv_base_pic_flag equal to 0 indicates the current decoded output picture does not correspond to a base picture.

The following constraints apply to the value of gfv_base_pic_flag:

–	When a GFV SEI message is the first GFV SEI message, in decoding order, that has a particular gfv_id value within the current CLVS, the value of gfv_base_pic_flag shall be equal to 1.

–	When a GFV SEI message that has a particular gfv_id value has gfv_base_pic_flag being equal to 0, this SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS or up to but excluding the decoded picture that follows the current decoded picture in output order within the current CLVS and is associated with a subsequent GFV SEI message, in decoding order, having gfv_base_pic_flag equal to 0 and that particular gfv_id value within the current CLVS, whichever is earlier.

gfv_key_present_flag equal to 1 indicates that the syntax element gfv_key is present and the syntax elements gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_reserved_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_payload_byte[ i ] are not present. gfv_key_present_flag equal to 0 indicates that the syntax element gfv_key is not present and the syntax elements gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_reserved_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_payload_byte[ i ].

gfv_key indicates a key that may be used to identify the analysis network used to generate the syntax elements of the current generative face video SEI message. The value of gfv_key may be used to determine whether the analysis network at the encoder matches with the generative network at the decoder.

NOTE 1 – The generative network at the decoder may use the SEI message to generate the video pictures only if it recognizes the value of gfv_key. The value of gfv_key may be specified by the applications that rely on matching networks at the encoder and the decoder. For such applications, if the parameters in the received SEI message are extracted with an analysis network that is not matched with the generative network at the decoder, the generative network should ignore the received SEI message.

gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_reserved_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_payload_byte[ i ] specify a neural network that may be used as a Generator( ). gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_reserved_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_payload_byte[ i ] have the same syntax and semantics as nnpfc_base_flag, nnpfc_mode_idc, nnpfc_reserved_zero_bit_a, nnpfc_tag_uri, nnpfc_uri, nnpfc_payload_byte[ i ], respectively.

gfv_drive_pic_fusion_flag, when present, equal to 1 indicates the current decoded output picture, which corresponds to a driving picture that may be used for fusion, may be input to Generator( ). gfv_drive_pic_fusion_flag equal to 0 indicates the current decoded output picture should not be input to Generator( ).

NOTE – A gfv_drive_pic_fusion_flag value of 1 could be used to indicate the current decoded output picture could be used to improve face details or handle background changes, as examples.

gfv_coordinate_present_flag equal to 1 indicates that coordinate information of keypoints is present. gfv_coordinate_present_flag equal to 0 indicates that coordinate information of keypoints is not present.

It is a requirement of bitstream conformance that when gfv_matrix_type_idx[ i ] for any i from 0 to gfv_num_matrix_types_minus1 is equal to 0 or 1, the value of gfv_coordinate_present_flag shall be equal to 1.

gfv_coordinate_precision_factor_minus1 plus 1 indicates the length, in bits, of gfv_coordinate_x_abs[i], gfv_coordinate_y_abs[i] and gfv_coordinate_z_abs[i].

gfv_num_kps_minus1 plus 1 indicates the number of keypoints. The value of gfv_num_kp_minus1 shall be in the range of 0 to 210 – 1, inclusive.

gfv_kp_pred_flag equal to 1 indicates syntax elements gfv_coordinate_dx_abs[ i ] ,gfv_coordinate_dy_abs[ i ], and gfv_coordinate_dz_abs[ i ] are present and syntax elements gfv_coordinate_dx_sign_flag[ i ], gfv_coordinate_dy_sign_flag[ i ] and gfv_coordinate_dz_sign_flag[ i ] may be present. gfv_kp_pred_flag equal to 0 indicates gfv_coordinate_x_abs[ i ] ,gfv_coordinate_y_abs[ i ], and gfv_coordinate_z_abs[ i ] are present and syntax elements gfv_coordinate_x_sign_flag[ i ], gfv_coordinate_y_sign_flag[ i ] and gfv_coordinate_z_sign_flag[ i ] may be present.

gfv_coordinate_z_present_flag equal to 1 indicates z-axis coordinate information of the keypoints is present. coordinate_z_present_flag equal to 0 indicates that the z-axis coordinate information of the keypoints is not present.

gfv_coordinate_z_max_value_minus1 plus 1 indicates the maximum absolute value of z-axis coordinates of keypoints.

gfv_coordinate_x_abs[ i ] indicates the normalized absolute value of x-axis coordinate of the i-th keypoint.

gfv_coordinate_x_sign_flag[ i ] specifies the sign of x-axis coordinate of the i-th keypoint. When gfv_coordinate_x_sign_flag[ i ] is not present, it is inferred to be equal to 0.

gfv_coordinate_y_abs[ i ] specifies the normalized absolute value of y-axis coordinate of i-th keypoint.

gfv_coordinate_y_sign_flag[ i ] specifies the sign of y-axis coordinate of i-th keypoint. When gfv_coordinate_y_sign_flag[i] is not present, it is inferred to be equal to 0.

gfv_coordinate_z_abs[ i ] specifies the normalized absolute value of z-axis coordinate of i-th keypoint.

gfv_coordinate_z_sign_flag[ i ] specifies the sign of z-axis coordinate of i-th key point. When gfv_coordinate_z_sign_flag[ i ] is not present, it is inferred to be equal to 0.

gfv_coordinate_dx_abs[ i ] indicates the absolute difference value of the normalized value of the x-axis coordinate of the i-th keypoint.

gfv_coordinate_dx_sign_flag[ i ] specifies the sign of the difference value of the x-axis coordinate of the i-th keypoint. When gfv_coordinate_dx_sign_flag[ i ] is not present, it is inferred to be equal to 0.

gfv_coordinate_dy_abs[ i ] specifies the absolute difference value of the normalized y-axis coordinate of the i-th keypoint.

gfv_coordinate_dy_sign_flag[ i ] specifies the sign of the difference value of the y-axis coordinate of the i-th keypoint. When gfv_coordinate_yd_sign_flag[i] is not present, it is inferred to be equal to 0.

gfv_coordinate_dz_abs[ i ] specifies the absolute difference value of the normalized z-axis coordinate of the i-th keypoint.

gfv_coordinate_dz_sign_flag[ i ] specifies the sign of the difference value of the z-axis coordinate of the i-th key point. When gfv_coordinate_dz_sign_flag[ i ] is not present, it is inferred to be equal to 0.

The variables coordinateDeltaX[ i ], coordinateDeltaY[ i ] and coordinateDeltaZ[ i ] indicating the delta x-axis coordinate, delta y-axis coordinate and delta z-axis oordinate of the i-th keypoint, respectively, are derived as follows:

coordinateDeltaX[ i ] = (1-2 * gfv_coordinate_dx_sign_flag[ i ]) * gfv_coordinate_dx_abs[ i ] * CroppedWidth1<< (gfv_coordinate_precision_factor_minus1 +1)

coordinateDeltaY[i] = (1-2 * gfv_coordinate_dy_sign_flag[ i ]) * gfv_coordinate_dy_abs[ i ] * CroppedHeight1<< (gfv_coordinate_precision_factor_minus1 +1)

coordinateDeltaZ[i] = (1-2 * coordinate_dz_sign_flag[ i ]) * gfv_coordinate_dz_abs[ i ] *(gfv_coordinate_z_max_value_minus1+1)1<< (gfv_coordinate_precision_factor_minus1 +1)

The variables coordinateX[ i ], coordinateY[ i ] and coordinateZ[ i ] indicating the x-axis coordinate, y-axis coordinate and z-axis coordinate of the i-th keypoint, respectively, are derived as follows:

When gfv_kp_pred_flag is equal to 0,

coordinateX[ i ] = (1-2 * gfv_coordinate_x_sign_flag[ i ]) * gfv_coordinate_x_abs[ i ] * CroppedWidth1<< (gfv_coordinate_precision_factor_minus1 +1)

coordinateY[i] = (1-2 * gfv_coordinate_y_sign_flag[ i ]) * gfv_coordinate_y_abs[ i ] * CroppedHeight1<< (gfv_coordinate_precision_factor_minus1 +1)

coordinateZ[i] = (1-2 * gfv_coordinate_z_sign_flag[ i ]) * gfv_coordinate_z_abs[ i ] *(gfv_coordinate_z_max_value_minus1+1)1<< (gfv_coordinate_precision_factor_minus1 +1)

when gfv_kp_pred_flag is equal to 1,

if( gfv_kp_pic_flag ) {
	coordinateX[ i ] = (( i > 0 ) ? coordinateX[ i - 1 ] : 0 ) + coordinateDeltaX[ i ]
	coordinateY[ i ] = (( i > 0 ) ? coordinateY[ i - 1 ] : 0 ) + coordinateDeltaY[ i ]
	coordinateZ[ i ] = (( i > 0 ) ? coordinateZ[ i - 1 ] : 0 ) + coordinateDeltaZ[ i ]
}
else {
	coordinateX[ i ] = BaseKpCoordinateX[ i ] + coordinateDeltaX[ i ]
	coordinateY[ i ] = BaseKpCoordinateY[ i ] + coordinateDeltaY[ i ]
	coordinateZ[ i ] = BaseKpCoordinateZ[ i ] + coordinateDeltaZ[ i ]
}

where BaseKpCoordinateX[ i ], BaseKpCoordinateY[ i ], BaseKpCoordinateZ[ i ] indicating the x-axis, y-axis and z-axis coordinates, respectively, of the i-th keypoint for the base picture are derived as follows:

if( gfv_base_pic_flag ) {
	BaseKpCoordinateX[ i ] = coordinateX[ i ]
	BaseKpCoordinateY[ i ] = coordinateY[ i ]
	BaseKpCoordinateZ[ i ] = coordinateZ[ i ]
}

gfv_kp_group_flag specifies the presence or absence of a pre-defined mapping table to map each keypoint to its assigned group of facial feature regions. A value of 1 indicates that keypoints would be assigned to groups, while a value of 0 indicates that keypoints would not be assigned to any groups.

gfv_kp_group_idx[ i ] specifies the index of the group of facial region that the i-th keypoint is assigned to.

gfv_kp_sub_group_idx[ I ] specifies the index of the sub-group of facial region that the i-th keypoint is assigned to.

gfv_matrix_present_flag equal to 1 indicates that matrix parameters are present. gfv_matrix_present_flag equal to 0 indicates that matrix parameters are not present.

gfv_matrix_element_precision_factor_minus1 plus 1 indicates the length, in bits, of gfv_matrix_element_dec[ i ][ j ][ k ][ l ].

gfv_num_matrix_types_minus1 plus 1 indicates the number of matrix types signalled in the SEI message. The value of gfv_matrix_type_num_minus1 shall be in the range of 0 to 26 – 1, inclusive.

gfv_matrix_type_idx[ i ] indicates the index of the i-th matrix type as specified in Table 3.

Table 3 - Specification of gfv_matrix_type_idx

| Value | Specification |
| --- | --- |
| 0 | Affine matrix |
| 1 | Covariance matrix |
| 7 | Compact feature matrix with the size being specified by gfv_matrix_width_minus1[i] and gfv_matrix_height_minus1[i]. |
| 2 | Compact feature matrix |
| 3 | Mouth matrix representing mouth motion. |
| 4 | Eye matrix representing the open-close status and level of both eyes. |
| 4 | Head rotation paramters with the size of 2*2 or 3*3 representing the head rotation in 2D space or 3D space. |
| 5 | Head translation matrix with the size of 1*2 or 1*3 representing head translationin 2D space or 3D space. |
| 6 | Head location matrix with size of 1*2 or 1*3 representing the head location in 2D space or 3D space. |
| 5 | Head matrix representing head movements. |
| 6 | Upper Lip matrix representing movements of the upper lip. |
| 7 | Lower Lip matrix representing movements of the lower lip. |
| 8 | Right Eye matrix representing open-close status and level of the right eye. |
| 9 | Left Eye matrix representing open-close status and level of the left eye. |
| 10 | Right Eyebrow matrix representing movements of the right eyebrow. |
| 11 | Left Eyebrow matrix representing movements of the left eyebrow. |
| 12…31 | Other matrix that may be used as determined by the application. |
| 32…63 | Reserved |

NOTE 2 . The undefined matrix type is used to represent the matrxi type rather than affine translation matrix, covariance matrix, rotation matrix, translation matrix and compact feature matrix. It is may be used by the user to extend the matrix type.

gfv_num_matrices_equal_to_num_kps_flag[ i ] equal to 1 indicates that the number of matrices of the i-th matrix type is equal to gfv_num_kps_minus1 + 1. gfv_num_matrices_equal_to_num_kps_flag[ i ] equal to 0 indicates the number of matrices of the i-th matrix type is not equal to gfv_num_coordinates_minus1 + 1.

gfv_num_matrices_info[ i ] provides information to derive the number of the matrices of the i-th matrix type.

gfv_group_order_flag[ i ] indicates the group-level ordering for the i-th matrix type. A value of 0 indicates the matrix associated with this group has an overall global motion, while 0 indicates a local motion.

gfv_num_matrices_minus1[ i ] plus 1 indicates the number of matrices for the i-th matrix type. A value of 0 indicates that the entire matrix would be a single affine transformation matrix, while a value of more than 0 indicates that the decomposed form of the affine transformation matrix would be present for the i-th matrix type, and these matrices should be combined to recreate the affine transform.

gfv_transform_type_idx[ i ][ t ] indicates the matrix type of the t-th decomposed matrix for the i-th matrix type, as specified in Table 7. When gfv_num_matrices_minus1 and gfv_transform_type_idx both have values of 0, this implies the decoded matrix is sufficient as the affine transformation matrix.

Table 7 - Specification of gfv_transform_type_idx

| Value | Specification |
| --- | --- |
| 0 | Affine Transformation Matrix |
| 1 | Translation Matrix |
| 2 | Full Rotation Matrix |
| 3 | Scale Matrix |
| 4 | Shear Matrix |
| 5 | Location Matrix |
| 6 | Rotation Angles |
| 7 … 255 | Reserved |

gfv_transform_order_idx[ i ][ t ] indicates the intra-group ordering of the decomposed matrices for the t-th matrix of the i-th matrix type, and will be in the form of a running-order index with values counting up from 0. Matrices are pre-multiplied as they are received, such as the form “… x transform_order_idx_2 x transform_order_idx_1 x transform_order_idx_0”.

gfv_matrix_for_equal_scale_flag indicates if the scaling factors in all dimensions should be signaled only once and treated as equal value across all dimensions. This would only be relevant for scale matrices. For example, with this flag has a value of 1, sx = sy for 2D space and sx = sy = sz for 3D space.

gfv_matrix_width[ i ] indicates the width of the matrix of the i-th matrix type.

gfv_matrix_height[ i ] indicates the height of the matrix of the i-th matrix type.

gfv_matrix_width[ i ] and gfv_matrix_height[ i ] collectively indicate the number of elements in each matrix, and this allows for scalability in sending any number of parameters for each matrix. For example, for affine matrix, we now can have 2-parameter, 4-parameter and 6-parameter models for the 2D scenario.
If some values are not present, for example only 1 translation parameter sent for a translation matrix, default values are assumed.

The variable numMatrices[ i ] indicating the number of the matrices of the i-th matrix type is derived as follows:

if( gfv_matrix_type_idx[ i ] == 0 || gfv_matrix_type_idx[ i ] == 1 ) {

if( gfv_coordinate_present_flag )

numMatrices[ i ] = gfv_num_matrices_equal_to_num_kps_flag[ i ] ? gfv_num_kps_minus1 + 1 : ( gfv_num_matrices_info[ i ] < gfv_num_kp_minus1 ? gfv_num_matrices_info [ i ] + 1 : gfv_num_matrices_info [ i ] + 2 )

else

numMatrices[ i ] = gfv_num_matrices_info[ i ] + 1

}

else if( gfv_matrix_type_idx[ i ] >= 2 &&  gfv_matrix_type_idx[ i ] < 7)

numMatrices[ i ] = 1

else

else if( gfv_matrix_type_idx[ i ] >= 2 )

numMatrices[ i ] = gfv_num_matrices_minus1[ i ] + 1

gfv_matrix_element_int[ i ][ j ][ k ][ l ] indicates the integer part of the value of the matrix element at position (k, l) of the j-th matrix of the i-th matrix type.

gfv_matrix_element_dec[ i ][ j ][ k ][ l ] indicates the decimal part of the value of the matrix element at position (k, l) of the j-th matrix of the i-th matrix type.

gfv_matrix_element_sign_flag[ i ][ j ][ k ][ l ] indicates the sign of the matrix element at position (k, l) of the j-th matrix of the i-th matrix type. When gfv_matrix_element_sign_flag[ i ][ j ][ k ][ l ]is not present, it is inferred to be equal to 0.

The variable matrixElementVal[ i ][ j][ k ][ l ] representing the value of the matrix element at position (k, l) of the j-th matrix of the i-th matrix type is derived as follows:

matrixElementVal[ i][ j ][ k ][ l ] = 1-2*gfv_matrix_element_sign_flag[ i ][ j ][ k ][ l ]*(gfv_matrix_element_int i  j  k  l  +gfv_matrix_element_dec[ i ][ j ][ k ][ l ]1<< (gfv_matrix_element_precision_factor_minus1 +1))

Generator ( ) is a process to generate the sample values of an output picture corresponding to a driving picture. It is only invoked when gfc_base_pic_flag is equal to 0.

Inputs to Generator() are:

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 0 and ChromaIDC is equal to 0: inputBaseY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 0 and ChromaIDC is not equal to 0: inputBaseY, inputBaseCb, inputBaseCr, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 1 and ChromaIDC is equal to 0: inputBaseY, inputDriveY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 1 and ChromaIDC is not equal to 0: inputBaseY, inputBaseCb, inputBaseCr, inputDriveY, inputDriveCb, inputDriveCr , inputBaseKeyPoint, inputBaseMatrix,, inputDriveKeyPoint, inputDriveMatrix

Outputs of Generator( ) are:

- When chromaIDC is equal to 0, a luma sample array genY

- When chromaIDC is not equal to 0, a luma sample array genY and chroma sample arrays genCb and genCr.

The following process is used to generate a video picture:

DeriveInputTensors( ) 
if( gfv_base_pic_flag == 0 && gfv_drive_pic_fusion_flag == 0) {
  if(chromaIDC == 0 ) 
    Generator( inputBaseY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix) 
  else 
    Generator( inputBaseY, inputBaseCb, inputBaseCr, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix) 
}
else if(gfv_base_pic_flag == 0 && gfv_drive_pic_fusion_flag == 1) {
  if(chromaIDC == 0 ) 
    Generator( inputBaseY, inputDriveY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix) 
  else 
    Generator( inputBaseY, inputBaseCb, inputBaseCr, inputDriveY, inputDriveCb, inputDriveCr , inputBaseKeyPoint, inputBaseMatrix,, inputDriveKeyPoint, inputDriveMatrix) 
}

StoreOutputTensors( )

Input values to Generator( ) are real numbers and the functions InpY( ) and InpC( ) are specified as follows:

InpY( x ) = x ÷ ( ( 1  <<  BitDepthY ) – 1 )

InpC( x ) = x ÷ ( ( 1  <<  BitDepthC ) – 1 )

Output values from Generator( ) are real numbers and the functions OutY( ) and OutC( ) are specified as follows:

OutY( x ) = Clip3( 0, ( 1  <<  BitDepthY ) – 1 , x * ( ( 1  <<  BitDepthY ) – 1 )

OutC( x ) = Clip3( 0, ( 1  <<  BitDepthC ) – 1 , x * ( ( 1  <<  BitDepthC ) – 1 )

The process DeriveInputTensors( ) for deriving the inputs of Generator ( ) is specified as follows:

When gfv_base_pic_flag is equal to 1, the BasePicture input tensor inputBaseY, inputBaseCb and inputBaseCr are derived as follows:

for( x = 0; x< CroppedWidth; x++ ) {
  for ( y = 0; y< CroppedHeight; y++ ) {
    inputBaseY[ x ][ y ] = InpY( baseCroppedYPic[ x ][ y ] ) 
  }
}
if (chromaIDC !=0) {
  for( x=0; x< CroppedWidth/ SubWidthC; x++ ) {
    for ( y=0; y< CroppedHeight/ SubHeightC; y++ ) {
      inputBaseCb[ x][ y ] = InpC( baseCroppedCbPic[ x][ y ] )
      inputBaseCr[ x][ y ] = InpC( baseCroppedCrPic[ x ][ y ] )
    }
  }
}

When gfv_drive_pic_fusion_flag is equal to 1, the DrivePicture luma sample array inputDriveY, inputDriveCb and input DriveCr are derived as follows:

for( x = 0; x< CroppedWidth; x++ ) {
  for ( y = 0; y< CroppedHeight; y++ ) {
    inputDriveY[ x ][ y ] = InpY( driveCroppedYPic[ x ][ y ] )
  }
}
if (chromaIDC !=0) {
  for( x=0; x< CroppedWidth/ SubWidthC; x++ ) {
    for ( y=0; y< CroppedHeight/ SubHeightC; y++ ) {
      InputDriveCb[ x][ y ] = InpC( driveCroppedCbPic[ x][ y ] )
      InputDriveCr[ x][ y ] = InpC( driveCroppedCrPic[ x ][ y ] ) 
    }
  }
}

When gfv_base_pic_flag is equal to 0, the keypoint coordinate array inputDriveKeyPoint and the matrix inputDriveMatrix for the current picture are derived as follows:

if( gfv_coordinate_present_flag ) {
  for ( i=0; i< = gfv_num_kps_minus1; i++ ) {
    inputDriveKeyPoint[ i ][ 0 ] = coordinateX[ i ]
    inputDriveKeyPoint[ i ][ 1 ] = coordinateY[ i ]
    if ( gfv_coordinate_z_present_flag ) 
      inputDriveKeyPoint[ i ][ 2 ] = coordinateZ[ i ]
  }
}
else {
  for ( i=0; i< = gfv_num_kps_minus1; i++ ) {
    inputDriveKeyPoint[ i ][ 0 ] = 0
    inputDriveKeyPoint[ i ][ 1 ] = 0
    if ( gfv_coordinate_z_present_flag ) 
      inputDriveKeyPoint[ i ][ 2 ] = 0
  }
}
if( gfv_matrix_present_flag ) {
  for ( i=0; i<=gfv_num_matrix_types_minus1; i++ ) {
    for ( j=0; j< numMatrices[ i ]; j++ ) {
      for( k=0; k< matrixHeight [ i ]; k++ ) {
        for ( l=0;l< matrixWidth [ i ]; l++) {
          inputDriveMatrix[ i ][ j ][ k ][ l ] = matrixElementVal[ i ][ j][ k][ l ] 
        }
      }
    }
  }
}
else {
  for ( i=0; i<= gfv_num_matrix_types_minus1; i++ ) {
    for ( j=0; j< numMatrices[ i ]; j++ ) {
      for( k=0; k< matrixHeight [ i ]; k++ ) {
        for ( l=0;l< matrixWidth [ i ]; l++) {
          inputDriveMatrix[ i ][ j ][ k ][ l ] = 0
        }
      }
    }
  }
}

When gfv_base_pic_flag is equal to 1, the keypoint coordinate array inputBaseKeyPoint and the matrix inputBaseMatrix for the base picture are derived as follows:

if( gfv_coordinate_present_flag ) {
  for ( i=0; i< = gfv_num_kps_minus1; i++ ) {
    inputBaseKeyPoint[ i ][ 0 ] = coordinateX[ i ]
    inputBaseKeyPoint[ i ][ 1 ] = coordinateY[ i ]
    if ( gfv_coordinate_z_present_flag ) 
      inputBaseKeyPoint[ i ][ 2 ] = coordinateZ[ i ]
  }
}
else {
  for ( i=0; i< =num_kps_minus1; i++ ) {
    inputBaseKeyPoint [ i ][ 0 ] = 0
    inputBaseKeyPoint [ i ][ 1 ] = 0
    if ( gfv_coordinate_z_present_flag ) 
      inputBaseKeyPoint [ i ][ 2 ] = 0
  }
}
if( gfv_matrix_present_flag ) {
  for ( i=0; i<= gfv_num_matrix_types_minus1; i++ ) {
    for ( j=0; j< numMatrices[ i ]; j++ ) {
      for( k=0; k< matrixHeight [ i ]; k++ ) {
        for ( l=0;l< matrixWidth [ i ]; l++) {
          inputBaseMatrix[ i ][ j ][ k ][ l ] = matrixElementVal[ i ][ j][ k][ l ] 
        }
      }
    }
  }
}
else {
  for ( i=0; i<=gfv_num_matrix_types_minus1; i++ ) {
    for ( j=0; j< numMatrices[ i ]; j++ ) {
      for( k=0; k< matrixHeight [ i ]; k++ ) {
        for ( l=0;l< matrixWidth [ i ]; l++) {
          inputBaseMatrix [ i ][ j ][ k ][ l ] = 0
        }
      }
    }
  }
}

The process StoreOutputTensors( ) for deriving the output is specified as follows:

when gfv_base_pic_flag is equal to 0, the output sample array outYPic[ x ][ y ], outCbPic[ x ][ y ], and outCrPic[ x ][ y ] are derived as follows:

for(x=0; x< CroppedWidth; x++){
  for(y=0; y< CroppedHeight; y++){
    outputYPic[ x ][ y ] = OutY( genY[ x ][ y ] )
  }
}

if(chromaIDC != 0) {
  for(x=0; x< CroppedWidth/ SubWidthC; x++){
    for(y=0; y< CroppedHeight/ SubHeightC; y++){
      outputCbPic[ x ][ y ] = OutC( genCb[ x ][ y ] )
      outputCrPic[ x][ y ] = OutC( genCr[ x ][ y ] )
    }
  }
}
when gfv_base_pic_flag is equal to 1, the output sample array outYPic[ x ][ y ], outCbPic[ x ][ y ], and outCrPic[ x ][ y ] are derived as follows:

for(x=0; x< CroppedWidth; x++){
  for(y=0; y< CroppedHeight; y++){
    outputYPic[ x ][ y ] = baseCroppedYPic [ x ][ y ] 
  }
}

if(chromaIDC != 0) {
  for(x=0; x< CroppedWidth/ SubWidthC; x++){
    for(y=0; y< CroppedHeight/ SubHeightC; y++){
      outputCbPic[ x ][ y ] = baseCroppedCbPic [ x ][ y ] 
      outputCrPic[ x][ y ] = baseCroppedCbPic [ x ][ y ] 
    }
  }
}

# Conclusion

This contribution introduces the use of grouping and transforms to capture variations in different face regions movements and leverages on 3-D facial landmarks as an example for illustration. The methodology introduced in this document may also be applicable across different types of face representations.

It is suggested to adopt the proposed refinements into the generative face video SEI message.

# References

- T. C. Wang, A. Mallya, and M. Y. Liu, “One-Shot Free-View Neural Talking-Head Synthesis for Video Conferencing”, IEEE/CVF Computer Vision and Pattern Recognition (CVPR), pp. 10039 – 10049, 2021.

- A. Siarohin, S. Lathuiliere, S. Tulyakov, E. Ricci, and N. Sebe, “First Order Motion Model for Image Animation”, Conference on Neural Information Processing Systems (NeurIPS), pp. 7137 – 7147, 2019.

- E. Burkov, I. Pasechnik, A. Grigorev, and V. Lempitsky, “Neural Head Reenactment with Latent Pose Descriptors”, IEEE/CVF Computer Vision and Pattern Recognition (CVPR), pp. 13786 – 13795, 2020.

- B. Chen, J. Chen, S. R. Wang, Y. Ye, and S. Q. Wang, “AHG9: Generative Face Video SEI Message”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-AC0088, 2023.

- V. Blanz, and T. Vetter, “A Morphable Model For The Synthesis Of 3D Faces”, International Conference on Computer Graphics and Interactive Techniques (SIGGRAPH), pp. 187 – 194, 1999.

- B. Chen, J. Chen, Y. Ye, and S. Q. Wang, “AHG9: Common SEI Message of Generative Face Video”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-AD0051, 2023.

- B. Chen, J. Chen, Y. Ye, S. Q. Wang, S. McCarthy, P. Yin, G. Su, A. K. Choudhary, W. Husak, “AHG9: Common text for proposed Generative Face Video SEI Message”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-AE0280, 2023.

- S. McCarthy, P. Yin, G. Su, A. K. Choudhary, W. Husak, “AHG9: Generative Face Video SEI Message”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-AE0080, 2023.

- B. Chen, J. Chen, Y. Ye, and S. Q. Wang, “AHG9: Common SEI Message of Generative Face Video”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-AE0083, 2023.

- H. B. Teo, J. Y. Thong, K. Jayashree, C. S. Lim, and K. Abe, “AHG9: A study on Generative Face Video SEI Message”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-AE0088, 2023.

- J. Chung, A. Nagrani, A. Zisserman, “VoxCeleb2: Deep Speaker Recognition”, Interspeech, 2018.

- I. Grischenko, A. Ablavatski, Y. Kartynnik, K. Raveendran, and M. Grundmann, “Attention Mesh: High-fidelity Face Mesh Prediction in Real-time”, CVPR Workshop on Computer Vision for Augmented and Virtual Reality (CV4ARVR), 2020.

- A. Bulat, and G. Tzimiropoulos, “How far are we from solving the 2D & 3D Face Alignment problem? (and a dataset of 230,000 3D facial landmarks)”, IEEE International Conference on Computer Vision (ICCV), pp. 1021 – 1030, 2017.

# Patent rights declaration(s)

Panasonic may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).