| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0156-v3 |
| --- | --- |

| Title: | AHG6: ECM software optimizations |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Fabrice Urban Tangi Poirier Fabrice Le Léannec | Email: | fabrice.urban@interdigital.com, tangi.poirier@interdigital.com, fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

Recent developments of ECM have increased its memory consumption to a critically large amount, especially when running multiple instances on a computer. In previous meetings, several contributions proposed memory profiling and software modifications to tackle this issue. To monitor the impact on memory consumption of contributions, memory consumption reporting is now part of performance reporting.

During the development of ECM 10.0, memory reduction was achieved, with about 5 GB memory saving for the worst-case scenario (class A RA).

This contribution presents a few more optimizations that have been pushed to the ECM software after the ECM 10.0 was released. Compared to ECM 10.0, the worst-case memory consumption has been further reduced by (AI, RA, LDB): 1.0 GB, 4.0 GB, 3.3 GB, and the average encoding time has been reduced to (AI, RA, LDB): 94.2%, 95%, 95%. On the decoder side, memory is also reduced by up to (AI, RA, LDB): 0.15 GB, 1 GB, 0.15 GB and the average decoding time has been reduced to (AI, RA, LDB): 83.9%, 82.7%, 83.6%.

# Introduction

At the last meeting, several contributions raised the issue of the excessive memory consumption by the ECM software. JVET-AE0180 described a memory profiling of the ECM 9.0, showing that the most memory consuming parts of the encoder were the ALF encoder, Picture buffers, Reuse CU routines and encoding library.

During the development of ECM 10.0, memory reduction was achieved. First, the ALF encoder, which is one of the most memory-consuming tools, was optimized. ALF covariance matrices are triangle, and refactoring their storage using this property can save memory. Additionally, making their allocation dependent on the actual filter shape can further improve the memory saving. This resulted in more than a 3.5 GB memory saving for the worst-case scenario (class A RA). Additionally, as proposed in JVET-AE0180, a condition on PIC_RECON_WRAP buffer allocation was added, which can save up to 2 GB on peak memory consumption. Overall, ECM 10.0 needs around 20 GB memory per encoding in the worst-case scenario, which is 5 GB less than ECM 9.0.

Memory access is one of the main run time bottlenecks. Because of the finite processor memory resources (e.g. cache memory, memory bandwidth), reducing memory consumption of a software is quite likely to reduce the run time, because fewer memory accesses may be required and the cache hit ratio may be higher.

Several ECM optimizations are proposed in the following. Some have a significant memory reduction, whereas others target run time reduction by reducing the amount of memory that is stored, initialized or copied.

These optimizations are already integrated in the ECM master branch. Individual merge requests have been pushed for each optimization. They are described in the following with their associated MR number for reference.

# ECM optimizations description

These optimizations have been merged to the current ECM master branch, by individual merge requests. The results are bit-accurate with ECM 10.0 (i.e., they don’t change the BD-rate results).

## MAX_CTU_SIZE (MR 516)

The encoder allocates temporary working buffers for one CTU, and in particular in the Reuse CU management. Using the MAX_CTU_SIZE value (256) allocates for example 4 times the required memory when CTU size 128 is used.

Using the actual CTU size provided by encoder configuration instead of MAX_CTU_SIZE allows to reduce the required memory by 2.8 GB for CTU size 128 (all the CTC excluding class A).

## Coding Structure (CS) (MR 525)

Coding structures are used at top-level for representing coded pictures, and at substructure level for the partitioning exploration algorithm mainly. Coding structures for the top-level pictures are allocated with a lifetime equal to the Encoder Picture Buffer. However, some of the allocated data is needed only for the current picture being encoded. Some memory is already deallocated right after encoding a picture (e.g., m_coeffs, m_coeff_signs, m_pltIdx, m_runType). This optimization extends this idea and also deallocates the following after encoding a picture:

- Integer buffers of the size of a picture: m_cuIdx, m_puIdx, m_tuIdx

- Boolean buffers of the size of a picture: m_isDecomp, mcMask, mcMaskChroma

Up to 1.6 GB memory is saved in Random Access configuration for class A.

## Picture buffer (MR 523)

The picture structure contains several Pel buffers to store several versions (original, reconstructed, filtered…) of the encoded picture. During picture allocation, all these buffers are allocated, whereas they are temporary buffers needed only for the picture being encoded.

Temporary buffers are already allocated only for the current picture being encoded (PIC_RESIDUAL and PIC_PREDICTION). This optimization extends this to PIC_TRUE_ORIGINAL and PIC_FILTERED_ORIGINAL, and to PIC_TRUE_ORIGINAL_INPUT and PIC_FILTERED_ORIGINAL_INPUT for the RPR case.

To avoid allocating the temporally-filtered version at the GOP level, the temporal filter is moved from EncApp::encodePrep() to EncGOP::compressGOP() just before encoding the picture.

This saves around 1.5 GB of peak memory usage for RA class A.

## Sign prediction (MR 527)

The sign prediction process uses template matching to determine the most likely sign values for a given number of coefficients. The process has been optimized in a bit exact way by making changes including the following:

- Use static_vector instead of std::vector for the predSignsXY variable

- Use 16-bit values instead of 32-bit values to represent positions in the PositionWithLevel data structure

- Use the floorLog2 function instead of loops where appropriate

- Duplicate the corner sample in the template to have an even template size

- Remove SIMD code to avoid function calls in tight loops

- Define an inline chroma sample reshape function to avoid function calls in tight loops and additional buffers

- Use 8-bit values instead of 32-bit values to store the sign prediction type (m_coeffSigns field which is part of every TU). This is the main contributor to memory reduction.

- Reduce size of m_signPredTemplate by storing only the vectors that are used for the current block (reduction from 512KB to 4KB)

## Coding Unit structure (More than 0.5 GB memory reduction together with run time reduction) (MR 517)

The CU structure contains a lot of data stored for every CU in the encoder, meaning temporary buffers kept for the partitioning exploration and the final decision. However, some of the data stored in this structure is needed only for the current CU being encoded. Data in Table 1 have been moved from the CU structure to the IntraPrediction class.

Table 1. per-CU data type and size for temporary buffers moved to IntraPrediction class.

| Buffer type and name | size |
| --- | --- |
| int tmpXdisp[MTMP_NUM]; | 19*4 = 76 Bytes |
| int tmpYdisp[MTMP_NUM]; | 19*4 = 76 Bytes |
| IntraTMPFusionInfo tmpFusionInfo[TMP_GROUP_IDX << 1]; | 88*6 = 528 Bytes |
| int64_t tmpFlmParams[TMP_FLM_PARAMS][MTMP_NUM]; | 6*19*8 = 912 Bytes |
| int tmpNumCand; | 4 Bytes |
| TmrlMode tmrlList[MRL_LIST_SIZE]; | 2*20 = 40 Bytes |
| TOTAL | 1636 Bytes / CU |

Reducing the memory footprint of the CU structure obviously reduces the ECM memory consumption but also accelerates the encoding and decoding because of the reduction of initialisation and copy. More than 0.2 GB memory reduction is achieved together with run time reduction.

## PU (MR 518)

The PU structure contains prediction information for every PU (IntraPredictionData and InterPredictionData).

Same as previously, some of the data stored in this structure are needed only for the current PU being encoded. Data in Table 2 have been moved from the PU structure to the IntraPrediction class.

Table 2. per-PU data type and size for temporary buffers moved to IntraPrediction class.

| Buffer type and name | size |
| --- | --- |
| uint8_t intraMPM[NUM_MOST_PROBABLE_MODES]; | 22 Bytes |
| uint8_t intraNonMPM[NUM_NON_MPM_MODES]; | 45 Bytes |
| TOTAL | 67 Bytes / CU |

The memory reduction is small, but associated copies are removed and MPM list computation and sorting are removed from CABAC functions.

## MCTF (MR 520)

The MCTF allocates new pictures for motion compensated frames, but original pictures can be overwritten. The modification avoids temporary memory allocation for motion compensated frames, instead the input frame is overwritten. The memory reduction depends on the number of frames used in the temporal filter. Up to 100 MB memory is saved.

## DBF (MR 526)

The deblocking filter stores information related to filter length and transform edge status on a per-sample basis. However, such information is needed only on a 4x4 luma basis. Therefore, the data structures are shrunk, and memory usage is slightly reduced. Run time is also improved because clearing this data occurs for every tested CU (CU-based DBF optimization). Note that this change is also present in VTM.

## Upper triangular part only of covariance matrix (MR 521)

Only the upper triangular part of covariance matrix is computed since it is symmetric and stored in a triangular format. Run time is improved as it avoids unnecessary memory access and redundant computations. The ALF encoder run time is reduced.

## CABAC context initialization (MR 528)

This MR combines several optimizations:

- Repacking data of BinProbModel_Std class (grouping by size) reduces the BinProbModel_Std class size from 14 to 12 Bytes, reducing a CtxStore class from 11914 to 10212 Bytes,

- Split Ctx are grouped into one ContextSet for better readability,

- using initialization with SubCtx for bit estimation related to the Geo mode.

Run time is improved by reducing the initialization and copy of CABAC contexts.

## Use transposed buffer (MR 529)

During Template Matching cost estimation, the left side template is considered as a column of 1xN samples. This leads to do some operations such as SAD, weightedAverage and linearTranform on a sample-by-sample basis. Considering the left side template as a row enables the usage of SIMD function for these operations. Note that it changes only how the block is considered not the memory itself. This technique was already used in other parts of the ECM, and this MR extends its usage.

Run time is reduced by enabling more SIMD operations.

# Results summary

## Individual optimization report on memory

Table 3 summarizes the impact of individual optimizations on the memory consumption of ECM for the CTC configurations. Each row reports the memory saving of each optimization described above, in absolute peak memory saving compared to the ECM 10.0, and relatively compared to the peak memory usage of the ECM 10.0

For example, the MAX_CTU optimization reduces peak memory by 2.8 GBytes for classes B, C, D, E, F, in both RA and AI configurations, where CTU size 128 is used instead of CTU size 256. This represents more than 60% memory reduction for the class D. No change is expected for class A, where the CTU size is 256.

The Coding structure (CS) and Picture buffer optimizations are efficient in the RA configuration and depend on the picture size. In class A, respectively 1.6 GB and 1.5 GB are saved (resp. 9% and 8% of peak memory) compared to the ECM 10.0.

Table 3. individual memory saving reported for each optimization (absolute and relative memory saving in MBytes compared to ECM 10.0). Optimizations not reported in this table don't show significant memory reduction and target run time reduction.

| Optimization | RA peak memory saving (MB and %) |  |  |  |  | AI peak memory saving (MB and %) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Class A | Class B | Class C | Class D | Class F | Class A | Class B | Class C | Class D | Class F |
| MAX_CTU | -61 | -2857 | -2850 | -2856 | -3049 | -4 | -2856 | -2855 | -2856 | -2897 |
|  | 0% | -30% | -54% | -65% | -43% | 0% | -40% | -60% | -68% | -56% |
| CS | -1636 | -395 | -97 | -20 | -220 | -130 | -35 | -9 | -2 | -12 |
|  | -9% | -4% | -2% | -0% | -3% | -1% | 0% | 0% | 0% | 0% |
| PIC_Buf | -1523 | -397 | -84 | -21 | -118 |  |  |  |  |  |
|  | -8% | -4% | -1% | 0% | -1% |  |  |  |  |  |
| sign_pred | -695 | -579 | -551 | -544 | -592 | -596 | -560 | -545 | -544 | -546 |
|  | -4% | -6% | -10% | -12% | -8% | -7% | -8% | -12% | -13% | -10% |
| CU | -246 | -192 | -187 | -175 | -198 | -246 | -200 | -182 | -174 | -179 |
|  | -1% | -2% | -4% | -4% | -3% | -3% | -3% | -4% | -4% | -3% |
| PU | -74 | -8 | -7 | -7 | -14 | -9 | -10 | -7 | -7 | -6 |
|  | 0% | 0% | 0% | 0% | 0% | 0% | 0% | 0% | 0% | 0% |
| MCTF | -93 | -9 | -1 | 0 | -27 |  |  |  |  |  |
|  | -1% | 0% | 0% | 0% | -0% |  |  |  |  |  |
| DBF | -84 | -8 | 0 | -1 | -28 | -3 | -3 | 0 | -1 | -1 |
|  | 0% | 0% | 0% | 0% | 0% | 0% | 0% | 0% | 0% | 0% |

## Results of these optimizations compared to ECM 10.0

Because the memory reduction implies initialization and copy reduction, and better cache management, the encoding time is also reduced.

Table 4 to Table 6 show (partial) simulation results of all the proposed optimizations on top of ECM 10.0. BD-rate results are unchanged. Peak memory consumption is significantly reduced and encoding and decoding time are reduced. Memory reduction is also reported for decoder. Absolute values of memory saving in MBytes is also reported in addition to relative values in %.

Table 4. proposed optimizations compared to ECM 10.0 in All Intra configuration.

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |  |  |  |
|  |  |  |  | Encoder |  |  | Decoder |  |  |
|  | Y | U | V | run time | VMPeak | VMPeak Diff | run time | VMPeak | VMPeak Diff |
| Class A1 | 0.00% | 0.00% | 0.00% | 94.4% | 88.2% | -1066 | 82.2% | 91.2% | -155 |
| Class A2 | 0.00% | 0.00% | 0.00% | 93.0% | 88.0% | -1086 | 79.6% | 91.0% | -161 |
| Class B | 0.00% | 0.00% | 0.00% | 94.3% | 57.4% | -3094 | 81.4% | 94.9% | -48 |
| Class C | 0.00% | 0.00% | 0.00% | 94.5% | 36.4% | -3020 | 87.9% | 97.5% | -18 |
| Class E | 0.00% | 0.00% | 0.00% | 94.7% | 44.5% | -3032 | 88.9% | 97.3% | -21 |
| Overall | 0.00% | 0.00% | 0.00% | 94.2% | 57.4% | -2394 | 83.9% | 94.6% | -75 |
| Class D | 0.00% | 0.00% | 0.00% | 96.0% | 29.8% | -3002 | 83.2% | 99.0% | -7 |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | -3215 | #NUM! | 96.9% | -25 |
| Class TGM | 0.00% | 0.00% | 0.00% | 95.1% | 56.8% | -3269 | 85.3% | 94.2% | -57 |

Table 5. proposed optimizations compared to ECM 10.0 in Random Access configuration.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |  |  |  |
|  |  |  |  | Encoder |  |  | Decoder |  |  |
|  | Y | U | V | run time | VMPeak | VMPeak Diff | run time | VMPeak | VMPeak Diff |
| Class A1 | 0.00% | 0.00% | 0.00% | 93.3% | 78.7% | -4078 | 78.3% | 77.0% | -835 |
| Class A2 | 0.00% | 0.00% | 0.00% | 94.1% | 77.8% | -4368 | 82.4% | 71.9% | -959 |
| Class B | 0.00% | 0.00% | 0.00% | 94.9% | 60.2% | -3903 | 78.9% | 81.8% | -275 |
| Class C | 0.00% | 0.00% | 0.00% | 97.1% | 40.3% | -3167 | 91.7% | 92.5% | -66 |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 95.0% | 60.1% | -3835 | 82.7% | 81.5% | -460 |
| Class D | 0.00% | 0.00% | 0.00% | 94.3% | 32.0% | -3043 | 86.5% | 97.4% | -19 |
| Class F | 0.00% | 0.00% | 0.00% | 97.3% | 51.5% | -3568 | 84.4% | 88.9% | -137 |
| Class TGM | 0.00% | 0.00% | 0.00% | 94.9% | 67.5% | -3890 | 77.7% | 87.5% | -192 |

Table 6. proposed optimizations compared to ECM 10.0 in Low Delay B configuration.

|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |  |  |  |
|  |  |  |  | Encoder |  |  | Decoder |  |  |
|  | Y | U | V | run time | VMPeak | VMPeak Diff | run time | VMPeak | VMPeak Diff |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 94.6% | 58.7% | -3360 | 88.5% | 90.1% | -146 |
| Class C | 0.00% | 0.00% | 0.00% | 94.9% | 37.7% | -3080 | 80.4% | 95.5% | -38 |
| Class E | 0.00% | 0.00% | 0.00% | 95.9% | 45.8% | -3179 | 80.0% | 93.6% | -66 |
| Overall | 0.00% | 0.00% | 0.00% | 95.0% | 47.6% | -3222 | 83.6% | 92.8% | -89 |
| Class D | 0.00% | 0.00% | 0.00% | 96.1% | 30.5% | -3018 | 84.2% | 98.3% | -12 |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | -3338 | #NUM! | 93.2% | -80 |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | -3586 | #NUM! | 89.3% | -161 |

## Results of the ECM master branch (@525ca76d) (Oct 03, 2023)

The results of the ECM master branch are slightly different because some bug fixes have been implemented, that slightly change the BD-rate performances.

Table 7 to Table 9 show (partial) simulation results of ECM master branch at commit 525ca76d.

Table 7. ECM master branch (@525ca76d) compared to ECM 10.0 in All Intra configuration.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |  |
|  |  |  |  | Encoder |  |  | Decoder |
|  | Y | U | V | run time | VMPeak | VMPeak Diff | run time |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | -1057 | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | -1050 | #VALUE! |
| Class B | -0.01% | -0.15% | -0.16% | 97.9% | 57.4% | -3091 | 83.5% |
| Class C | 0.00% | 0.03% | 0.03% | 98.1% | 36.4% | -3020 | 88.9% |
| Class E | 0.01% | 0.06% | -0.15% | 99.4% | 44.5% | -3031 | 92.2% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | -2507 | #VALUE! |
| Class D | 0.00% | -0.09% | -0.06% | 97.2% | 29.8% | -3001 | 85.7% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | -3219 | #VALUE! |
| Class TGM | -0.21% | -0.24% | -0.27% | 97.7% | 56.8% | -3271 | 88.0% |

Table 8. ECM master branch (@525ca76d) compared to ECM 10.0 in Random Access configuration.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |  |
|  |  |  |  | Encoder |  |  | Decoder |
|  | Y | U | V | run time | VMPeak | VMPeak Diff | run time |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | -4015 | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | -4236 | #VALUE! |
| Class B | -0.05% | -0.01% | -0.20% | 96.2% | 60.2% | -3902 | 80.8% |
| Class C | 0.01% | 0.06% | -0.07% | 98.0% | 40.4% | -3166 | 94.3% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | -3708 | #VALUE! |
| Class D | -0.01% | -0.23% | -0.16% | 95.4% | 32.0% | -3043 | 87.8% |
| Class F | 0.03% | 0.10% | 0.17% | 98.9% | 51.4% | -3569 | 87.6% |
| Class TGM | -0.10% | -0.16% | -0.06% | 96.1% | 67.5% | -3891 | 80.5% |

Table 9. ECM master branch (@525ca76d) compared to ECM 10.0 in Low Delay B configuration.

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |  |
|  |  |  |  | Encoder |  |  | Decoder |
|  | Y | U | V | run time | VMPeak | VMPeak Diff | run time |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | -3360 | #VALUE! |
| Class C | -0.04% | -0.60% | -0.32% | 96.3% | 37.7% | -3080 | 82.1% |
| Class E | -0.03% | 1.00% | -1.48% | 98.7% | 45.8% | -3178 | 82.8% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | -3185 | #VALUE! |
| Class D | 0.09% | -0.25% | -0.55% | 98.3% | 30.5% | -3018 | 86.1% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | -3332 | #VALUE! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | -3681 | #VALUE! |

## Memory reduction since ECM-9.1

Since ECM-9.1, a significant effort has been made to reduce the memory consumption of ECM software. Table 10 summarizes the memory reduction made for each sequence class and configuration. Almost 40% memory reduction (9 GBytes) has been reached for the worst-case scenario (class A RA).

Table 10. ECM master branch (@525ca76d) compared to ECM 9.1

|  | Over ECM-9.1 encoder |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AI |  | RA |  | LDB |  |
|  | VMPeak | VMPeak Diff | VMPeak | VMPeak Diff | VMPeak | VMPeak Diff |
| Class A1 | 65.3% | -4199 | 62.4% | -8950 |  |  |
| Class A2 | 65.3% | -4189 | 62.0% | -9342 |  |  |
| Class B | 40.5% | -6131 | 43.5% | -7677 | 40.8% | -6885 |
| Class C | 31.9% | -3686 | 34.5% | -4070 | 32.5% | -3872 |
| Class E | 35.6% | -4412 |  |  | 35.9% | -4785 |
| Overall | 42.5% | -4697 | 44.0% | -6833 | 35.6% | -4953 |
| Class D | 28.3% | -3221 | 29.7% | -3391 | 28.6% | -3296 |
| Class F | 35.5% | -4741 | 41.3% | -5605 | 38.8% | -4974 |
| Class TGM | 40.5% | -6310 | #DIV/0! | 8089 | #DIV/0! | 6527 |

# Conclusion

In this contribution, we presented some ECM software optimizations targeting mainly memory usage reduction. It was noted that reducing memory usage tends to reduce the run time because of less memory accesses. We encourage further work on memory and run time reduction.