| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32st Meeting, Hanover, DE, 13–20 October 2023 | Document: JVET-AF0157-V1 |
| --- | --- |

| Title: | AHG8: A temporal resampling algorithm |  |  |
| --- | --- | --- | --- |
| Status: | Input document of JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ding Ding Xin Zhao Zizheng Liu Shan Liu | Email: | ddding@global.tencent.com xinzzhao@global.tencent.com zizhengliu@tencent.com shanl@global.tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In the V2 verison, we update the results of RA, as well as the comparison with VTM 12 anchor. In addition, we also discuss the configuration of RA to fair compare with anchor. We show our solution as an encoder decision coding method.

In JVET-AE2030, a pre-processing method based on temporal resampling is proposed, which can be used to optimize video encoders and receiving systems for machine consumption. By skipping certain frames before encoding, the bit rate can be significantly reduced without a strong negative impact on the machine consumption performance. In this contribution, a temporal resampling algorithm is detailed and its software implementation is provided. Based on the common test conditions described in JVET-AE2031, it is reported that the following coding performance can be achieved:

For VTM 20:

For object tracking

2x temporal resampling rate: -51.02% (AI), -15.88% (LD), -8.32% (RA)

4x temporal resampling rate: -75.93% (AI), -30.53% (LD), -3.84% (RA)

For object detection

2x temporal resampling rate: -49.91% (AI), -13.18% (LD), -6.72% (RA)

4x temporal resampling rate: -69.53% (AI), 5.53% (LD), 31.91% (RA)

For VTM 12:

For object tracking

2x temporal resampling rate: -50.69% (AI), -9.11% (LD), -4.04% (RA)

4x temporal resampling rate: -75.99% (AI), -24.00% (LD), -3.56%(RA)

For object detection

2x temporal resampling rate: -50.50% (AI), -7.76% (LD), 4.94% (RA)

4x temporal resampling rate: -69.61% (AI), 10.69% (LD), 29.1% (RA)

# Temporal resampling algorithm

## Framework

Fig. 1 shows a typical temporal resampling-based data processing pipeline as described in JVET-AE2030 draft technical report [1]. The input video is temporally downsampled before encoding as pre-processing and the downsampled video sequence is then fed into the video encoder and compressed into bitstream. After the bitstream is decoded, the reconstructed video sequence is received and further temporally upsampled to the original framerate. The temporal upsampling process can be achieved by a neural network-driven model.

Input video

Temporal downsample

Encoding

Decoding

Temporal upsample

Machine consumption

Figure 1: Typical temporal resampling-based compression framework.

## Encoding Decision on Random Access

To achieve random access, a certain number of I-frames must be inserted during the encoding process. Currently, the intraperiod stipulated by CTC is approximately one per second. If temporal resampling is applied without adjusting the intraperiod, the number of I-frames within the same duration will decrease. For instance, with 2x sampling, an I-frame would only appear approximately every two seconds. For a fair comparison with the anchor, approximately one Intra frame per second should be kept for the experiments on temporal resampling. it is necessary to concurrently reduce the intra period. Considering that in VTM, the Intraperiod needs to be an integral multiple of the GOP size, and under the default configuration, the VTM GOP size is 32, modifications to the GOP size and its encoding configuration are required to achieve an Intraperiod less than 32.

In 4x temporal resample:

For the frame rate of original sequence less than 50, we adopt GOP size of 8. The new GOP8 config file are used for this setup.

For the frame rate of original sequence equals and larger than 50, we adopt GOP size of 16. The default GOP16 configuration files are used for this setup.

In 2x temporal resample:

For the frame rate of original sequence less than 50, we adopt GOP size of 16. The default GOP16 configuration files config file are used for this setup.

## For the frame rate of original sequence equals and larger than 50, we adopt GOP size of 32. The default GOP32 configuration files are used for this setup.

## Temporal downsampling

To perform temporal downsampling, the original video is down-sampled in temporal domain by resampling the video frames with equal interval. Two typical temporal downsampling rates, including 2x and 4x down-sampling ratios are supported. Fig. 2 shows an example of temporal downsampling in different down-sampling ratios. Considering the original video with POC of {0, 1, 2, 3, 4, 5, 6, 7, 8, …}, with 2x downsampling ratio, the framerate is reduced to the half size of original framerate, which means the frame with POC {1, 3, 5, 7,…} are dropped, with 4x downsampling ratio , the framerate is reduced to the quarter size of original framerate, which means the frame with POC {1, 2, 3, 5, 6, 7,…} are dropped.

Figure 2: Illustration of temporal downsampling.

The video bitstream contains the information about the temporally down-sampled video. After the bitstream is decoded by the decoded, the reconstructed video needs the temporal upsampled to the original frame rate. The essential information of temporal down-sampling is also signalled into the VCM bitstreams.

## Temporal upsampling module

If the temporal downsampling index obtained from the bitstream is larger than 0 which indicates that the decoded video has been down-sampled in temporal domain, the decoder is advised to perform the temporal up-sampling after the video is reconstructed to recover the original framerate.

In this contribution, an optical-flow-based video frame interpolation network is used for the temporal domain up-sampling. Fig. 3 shows the framework of the temporal upsampling for different temporal resampling ratios. For 2x resampling ratio case, the dropped frames are interpolated by the previous and the following frames. For 4x resampling ratio, the dropped frames are interpolated in a hierarchical way. For example, at the first step the POC 2 frame is generated by POC 0 and POC 4. Then the frames of POC 1 and POC 3 are interpolated by the generated POC 2 and POC 0 and POC 4 subsequently. If the frame number of the interpolated video is smaller than the original frame number obtained from the bitstream, this temporal upsampling module duplicates the last frame to match the original frame rate.

Figure 3: Illustration of temporal up-sampling.

Fig. 4 shows the architecture of the frame interpolation network. The reconstructed frames and their corresponding QP maps are fed into a three-level hierarchical structure to extract features in different granularities. An additional module OVSR [3] is employed to further explore the relationship of the features in the lowest level. Besides, the IFRNet [4] is employed to estimate the optical flows among the intermediate frame and the two input frames. The estimated optical flows will be also downsampled via bilinear method into the similar pyramid structure as the extracted features. The three levels of feature and optical-flows are synthetized by backward warping and optical-flow-based calculation. Finally, the GridNet and ResBlock will output the generated reference frame with the synthetized features as input.

Figure 4: Architecture of the optical flow based frame interpolation network.

# Experimental results

The performance of the proposed spatial resampling network is evaluated following the common test conditions specified in JVET-AE2031 [2]. Results for resample rate 2x and 4x results are shown below respectively.

Table 1: The object detection performance of the proposed technology on SFU-HW dataset.

|  | 2x rate |  | 4x rate |  |
| --- | --- | --- | --- | --- |
| config | AI (Pareto mAP) | LD (Pareto mAP) | AI (Pareto mAP) | LD (Pareto mAP) |
| Class A | -46.80% | -5.44% | -72.13%. | -0.83% |
| Class B | -50.09% | -11.30% | -66.14% | 61.28% |
| Class C | -51.89% | -17.63% | -71.34% | -18.85% |
| Class D | -48.52% | -12.54% | -70.46% | -24.26% |
| AVG | -49.91% | -13.18% | -69.53% | 5.53% |

Table 2: The object tracking performance of the proposed technology on TVD dataset.

|  | 2x rate |  | 4x rate |  |
| --- | --- | --- | --- | --- |
| config | AI (Pareto mAP) | LD (Pareto mAP) | AI (Pareto mAP) | LD (Pareto mAP) |
| AVG | -51.02% | -15.88% | -75.93% | -30.53% |

# Conclusion

This contribution describes a temporal resampling algorithm for enhancing coding efficiency in machine-oriented video coding. Simulation results show improved coding performance by applying the proposed scheme across diverse coding configurations.

It is recommended to incorporate details regarding the temporal resampling algorithm and its software implementation in the next version of the technical report on a description of optimization of encoders and receiving systems for machine analysis of coded video content (JVET-AF2030).

# References

- J. Chen, C. Hollmann, S. Liu, “Optimization of encoders and receiving systems for machine analysis of coded video content (draft 3)”, JVET-AE2030, 31th JVET meeting, Geneva, 13-20, July, 2023

- S. Liu, C. Hollmann, “Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content”, JVET-AE2031, 31th JVET meeting, Geneva, 13-20, July, 2023

- P. Yi, Z. Wang, K. Jiang, et al. “Omniscient Video Super-Resolution”, 2021 IEEE/CVF International Conference on Computer Vision (ICCV), 2021, pp. 4409-4418.

- L. Kong, B. Jiang, D. Luo, et al. “IFRNet: Intermediate Feature Refine Network for Efficient Frame Interpolation”, 2022 IEEE/CVF Conference on Computer Vision and Pattern Recognition (CVPR), 2022, pp. 1959-1968.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).