| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0158-v3 |
| --- | --- |

| Title: | HOP In-loop filter with transformer blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yun Li Dmytro Rusanovskyy Thomas Ryder Samuel Eadie Marta Karczewicz | Tel: Email: | yli30@qti.qualcomm.com dmytror@qti.qualcomm.com tryder@qti.qualcomm.com seadie@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

A Unified Filter Architecture for the High-performance Operation Point (HOP) was defined at JVET-AD0380. Training strategy and intermediate results were presented in JVET-AE0191 and complete inference results for Stage3 HOP training are reported in JVET-AF0041.

In this proposal, transformer blocks are integrated into the Residual Block (RB) of the HOP backbone. Proposed filter architecture was constrained to the complexity and memory requirements of the HOP architecture. Training was conducted by the HOP training guidelines.

Complexity of the model in kMAC/pixel: 493 (block-wise) with 1.45M parameters.

With a short training cycle (13 epochs) and nnIntra OFF, the proposed filter (floating-point model) provides: {-8.9%, -26.4%, -27.5%} for AI test configuration, and {-11.1%, -31.2%, -31.5%} for RA test configuration, versus the VTM. Compared to the HOP anchor (nnIntra OFF), the proposed filter (floating model) provides: {-1.2%, -9.4%, -9.0%} for AI configuration, and {-0.9%, -8.8%, -8.6%} for RA configuration.

With the short training cycle and nnIntra ONN, the proposed filter provides: {-11.8%, -29.3%, -30.3%} for AI, and {-12.5%, -32.2%, -32.5%} for RA, versus the VTM. Compared to the HOP anchor (nnIntra ON), it provides: {-1.2%, -9.7%, -9.3%} for AI, and {-0.9%, -9.1%, -8.9%} for RA.

With a full training cycle and nnIntra OFF, the proposed filter provides: {-9.1%, -26.7%, -27.8%} for AI, and {-11.3%, -31.8%, -31.7%} for RA, versus the VTM. Compared to the HOP anchor (nnIntra OFF), it provides: {-1.4%, -9.8%, -9.4%} for AI, and {-1.2%, -9.7%, -8.9%} for RA.  Compared to the short training, the gain is AI: {-0.2%, -0.5%, -0.5%} and RA: {-0.2%, -0.9%, -0.3%}

It is proposed to study the proposed method in the next round of EE.

# Introduction

A Unified Filter Architecture for the High-performance Operation Point (HOP) was defined at JVET-AD0380. Training strategy and intermediate results were presented in JVET-AE0191 and complete inference results for Stage3 HOP training are reported in JVET-AF0041. In addition, JVET-AE0164 proposes to reduce HOP complexity by using a smaller rank of the decomposition for the convolution operations, and the complexity is reduced by approximately 10 percent. This leaves additional complexity budget for the further improvement of HOP.

Current HOP architecture is based on Convolutional Neural Network (CNN) with activation layers for the filtering operation. This network effectively captures the local correlations within an image block. However, the filtering process could be further improved by using non-local attention mechanisms that effectively take into account the relevant features in a larger region of the input and features, and can generate response adaptive to the inputs.

- Proposed method

In this contribution, we rely on the HOP filter architecture and the training dataset, and we further apply a smaller rank to the decomposition, i.e., C31 =R=48 as in [1], which was tested in EE1-1.1.2.

In order to perform the attention mechanism to capture the long-range dependency for the filtering process, A transformer encoder block is introduced to the end of each backbone block. The filter configuration is further adjusted to meet the HOP constrains in complexity and memory requirements, for which the number of residue block is set to N=13. Fig. 1 shows the architecture of the proposed method with changes to the HOP architecture marked in red. Resulting complexity of the filter is 493 kMAC/pixel (block-wise), and the number of parameters is 1.45M. The complexity was primarily estimated with the “ptflops” Python API, and additionally complexity ignored by the API was also estimated.

Figure 1 HOP with transformer blocks.

The information for the inference and the training stage is presented in Table 1 and 2, respectively.

Table1. Network information for the inference

| Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
|  | HW environment: |  |  |
|  | GPU Type |  | N/A |
|  | Framework: |  | Pytorch |
|  | Number of GPUs per Task |  | 0 |
|  | Total Parameter Number |  | 1.45M /model |
|  | Parameter Precision (Bits) |  | 32 or 16 |
|  | Memory Parameter (MB) |  |  |
|  | Multiply Accumulate (kMAC/pixel) block (frame) based |  | 493 (390 frame-wise) |
|  | Total Conv. Layers |  |  |
|  | Total FC Layers |  | 0 |
|  | Total Memory (MB) | Float | 6.364/model |
|  |  | Int16 |  |
|  | Batch size: |  | 1 |
|  | Patch size |  | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage |  |  |
|  | Other information: |  |  |

Table 2. Network information for the training

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA Tesla A100 |
|  | Framework: | PyTorch v3.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 20 for Training 1 and 13 for Training 2 |
|  | Batch size: | 64 |
|  | Training time: | ~280h |
|  | Training data information: | DIV2K BVI TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

- Simulation results

Simulations are performed based on the common test conditions for neural network-based video coding technology [2]. The proposed method was trained following the Stage 3 HOP training guidance specified in JVET-AE0191. The training is being conducted with a target of 20 epochs (Training 1) as specified by the guidance. Additionally, a shorter training of 13 epochs (Training 2) was completed, with modified L1/L2 scheduler, "milestones": {11,12}.

Validation loss functions for the proposed method, comparing to the HOP anchor validation loss L1 (as most informational) for Y and CbCr components are shown in Fig. 2.

Figure 2. Validation loss comparison for HOP anchor and the proposed method.

The proposed method was tested with modified NNVC6.0 reference software. Pytorch floating point inference interface based on JVET-X0066 implementation was introduced to NNVC6.0 instead of the SADL interface. From Table 3, it can be observed that a good match is obtained when comparing the implemented NNVC torch to the NNVC SADL. For RA, only class C and class D are tested.

Table 3 NNVC Torch vs. NNVC SADL, nnIntra OFF

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | 0.00% | 0.00% | -0.04% |  |  |  |
| Class A2 | 0.00% | 0.04% | 0.05% |  |  |  |
| Class B | 0.00% | 0.01% | 0.00% |  |  |  |
| Class C | 0.00% | -0.02% | -0.01% | 0.02% | 0.00% | 0.01% |
| Class E | 0.00% | 0.00% | 0.01% |  |  |  |
| Overall | 0.00% | 0.01% | 0.00% |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | -0.09% | 0.00% |

Summary of the inference results for models produced with Training 1 and 2 are reported below, complete results in CTC template are provided along this document.

Inference results with Training 2 (short training)

Comparative results for the model produced with Training 2, compared to the HOP filter are shown in Table 4. NN Intra was disabled for both the test and the anchor. Comparison against VTM is shown in Table 5.

As observed in Tables 4, the proposed filter outperforms the HOP anchor by {-1.2%, -9.4%, -9.0%} for the AI configuration and {-0.9%, -8.8%, -8.6%} for the RA configuration at a similar complexity.

Compared to VTM, shown in Table 5, the proposed method provides {-8.9%, -26.4%, -27.5%} for the AI configuration, and {-11.1%, -31.2%, -31.5%} for the RA configuration, respectively.

Table 4 Proposed method with Training 2 (float mode) vs HOP, nnIntra OFF

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -1.26% | -7.15% | -7.52% | -1.37% | -7.07% | -6.49% |
| Class A2 | -1.31% | -11.93% | -15.84% | -0.81% | -7.94% | -17.39% |
| Class B | -1.30% | -12.97% | -9.49% | -0.97% | -16.42% | -9.55% |
| Class C | -0.81% | -4.81% | -5.05% | -0.66% | -1.30% | -2.47% |
| Class E | -1.20% | -9.21% | -8.02% |  |  |  |
| Overall | -1.17% | -9.39% | -8.99% | -0.94% | -8.82% | -8.62% |
| Class D | -0.43% | -5.85% | -4.86% | -0.41% | -0.82% | -2.87% |
| Class F | -0.99% | -4.15% | -3.85% | -0.27% | -2.00% | -2.93% |

Table 5 Proposed method with Training 2 (float mode) vs VTM, nnIntra OFF

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -8.35% | -23.70% | -27.90% | -11.75% | -25.83% | -30.35% |
| Class A2 | -8.28% | -29.22% | -31.29% | -11.52% | -29.89% | -36.05% |
| Class B | -8.17% | -28.95% | -27.02% | -10.51% | -37.41% | -31.29% |
| Class C | -8.81% | -22.05% | -24.81% | -11.09% | -28.38% | -29.13% |
| Class E | -11.52% | -27.56% | -27.56% |  |  |  |
| Overall | -8.92% | -26.36% | -27.48% | -11.11% | -31.18% | -31.48% |
| Class D | -8.31% | -22.46% | -24.92% | -12.22% | -27.40% | -29.39% |
| Class F | -6.18% | -19.02% | -19.04% | -6.05% | -19.97% | -19.54% |

With the NN Intra enabled, results compared to the HOP filter are shown in Table 6, and comparison against VTM is shown in Table 7.

As observed in Tables 6, the proposed filter outperforms the HOP anchor by {-1.2%, -9.7%, -9.3%} for the AI configuration and {-0.9%, -9.1%, -8.9%} for the RA configuration, respectively.

Compared to VTM, shown in Table 7, the proposed method provides {-11.8%, -29.3%, -30.3%} for AI, and {-12.5%, -32.2%, -32.5%} for RA, respectively.

Table 6 Proposed method with Training 2 (float mode) vs HOP, nnIntra ON

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -1.22% | -8.52% | -8.21% | -1.38% | -7.14% | -6.91% |
| Class A2 | -1.30% | -12.06% | -16.47% | -0.81% | -8.33% | -17.48% |
| Class B | -1.28% | -13.14% | -9.63% | -0.97% | -16.72% | -9.92% |
| Class C | -0.84% | -4.59% | -4.81% | -0.68% | -1.58% | -2.72% |
| Class E | -1.16% | -9.65% | -8.49% |  |  |  |
| Overall | -1.15% | -9.71% | -9.27% | -0.94% | -9.09% | -8.91% |
| Class D | -0.45% | -5.61% | -4.51% | -0.39% | -1.37% | -2.94% |
| Class F | -1.06% | -4.23% | -3.87% | -0.39% | -2.12% | -3.21% |

Table 7 Proposed method with Training 2 (float mode) vs VTM, nnIntra ON

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -11.87% | -27.73% | -31.23% | -13.46% | -27.10% | -31.74% |
| Class A2 | -10.65% | -31.26% | -33.44% | -12.59% | -30.55% | -36.82% |
| Class B | -10.81% | -31.51% | -29.48% | -11.87% | -38.33% | -32.29% |
| Class C | -11.40% | -24.66% | -27.36% | -12.38% | -29.55% | -30.24% |
| Class E | -15.30% | -31.54% | -31.45% |  |  |  |
| Overall | -11.84% | -29.32% | -30.29% | -12.47% | -32.19% | -32.54% |
| Class D | -10.72% | -25.25% | -27.51% | -13.32% | -28.70% | -30.21% |
| Class F | -8.13% | -20.64% | -20.64% | -6.98% | -20.53% | -20.30% |

Inference results with Training 1(full training)

Comparative results for the model produced with Training 1, compared to the HOP filter are shown in Table 8. NN Intra was disabled for both the test and the anchor. Comparison against VTM is shown in Table 9, and Table 10 illustrates the results compared to that of the short training.

As observed in Tables 8, the proposed filter outperforms the HOP anchor by {-1.4%, -9.8%, -9.4%} for the AI configuration and {-1.2%, -9.7%, -8.9%} for the RA configuration, respectively.

Compared to VTM, shown in Table 9, the proposed method provides {-9.1%, -26.7%, -27.8%} for the AI configuration, and {-11.3%, -31.8%, -31.7%} for the RA configuration, respectively.

Table 10 illustrates the coding gain of AI: {-0.2%, -0.5%, -0.5%} and of RA: {-0.2%, -0.9%, -0.3%}, compared to that of the short training.

Table 8 Proposed method with Training 1 (float mode) vs HOP, nnIntra OFF

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -1.45% | -7.39% | -7.41% | -1.58% | -8.26% | -7.24% |
| Class A2 | -1.49% | -12.64% | -16.41% | -1.07% | -9.17% | -17.60% |
| Class B | -1.46% | -12.99% | -9.55% | -1.14% | -16.84% | -9.65% |
| Class C | -1.05% | -5.42% | -5.68% | -0.92% | -2.13% | -2.73% |
| Class E | -1.43% | -9.93% | -9.13% |  |  |  |
| Overall | -1.37% | -9.81% | -9.41% | -1.15% | -9.67% | -8.91% |
| Class D | -0.59% | -6.85% | -5.85% | -0.56% | -2.45% | -4.05% |
| Class F | -1.12% | -4.41% | -4.16% |  |  |  |

Table 9 Proposed method with Training 1 (float mode) vs VTM, nnIntra OFF

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -8.52% | -23.86% | -27.81% | -11.94% | -26.66% | -30.93% |
| Class A2 | -8.44% | -29.72% | -31.72% | -11.74% | -30.73% | -36.22% |
| Class B | -8.31% | -28.97% | -27.02% | -10.66% | -37.71% | -31.35% |
| Class C | -9.03% | -22.52% | -25.26% | -11.32% | -28.96% | -29.25% |
| Class E | -11.72% | -28.13% | -28.43% |  |  |  |
| Overall | -9.10% | -26.67% | -27.78% | -11.31% | -31.77% | -31.68% |
| Class D | -8.46% | -23.26% | -25.63% | -12.36% | -28.64% | -30.22% |
| Class F | -6.30% | -19.29% | -19.28% |  |  |  |

Table 10 Full training vs short training, nnIntra OFF

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0.19% | -0.29% | 0.11% | -0.21% | -1.28% | -0.77% |
| Class A2 | -0.18% | -0.86% | -0.74% | -0.26% | -1.31% | -0.37% |
| Class B | -0.16% | -0.07% | -0.10% | -0.17% | -0.49% | -0.10% |
| Class C | -0.24% | -0.66% | -0.68% | -0.26% | -0.85% | -0.27% |
| Class E | -0.23% | -0.80% | -1.19% |  |  |  |
| Overall | -0.20% | -0.49% | -0.48% | -0.22% | -0.91% | -0.33% |
| Class D | -0.17% | -1.05% | -1.04% | -0.15% | -1.67% | -1.23% |
| Class F | -0.13% | -0.25% | -0.32% |  |  |  |

- Conclusion

In this contribution, transformer blocks are introduced into the HOP architecture combined with the lower rank decomposition to improve the performance of HOP.

Compared to NNVC-5.0 HOP anchor (nnIntra OFF) with the short training, the proposed filter (floating-point model) provides: AI: {-1.17%, -9.39%, -8.99%} and RA: {-0.94%, -8.82%, -8.62%}. Compared to VTM, the proposed filter (float) provides: AI: {-8.92%, -26.36%, -27.48%}, and RA: {-11.11%, -31.18%, 31.48%} for the test.

Compared to NNVC-5.0 HOP anchor (nnIntra ON) with the short training, the proposed filter (float) provides: AI: {-1.15%, -9.71%, -9.27%} and RA: {-0.94%, -9.09%, -8.91%}. Compared to VTM, it provides: AI: {-11.84%, -29.32%, -30.29%}, and RA: {-12.47%, -32.19%, 31.54%}.

Compared to NNVC-5.0 HOP anchor (nnIntra OFF) with the full training, the proposed filter (float) provides: AI: {-1.37%, -9.81%, -9.41%} and RA: {-1.15%, -9.67%, -8.91%}. Compared to VTM, it provides: AI: {-9.10%, -26.67%, -27.78%}, and RA: {-11.31%, -31.77%, 31.68%}. Compared to short training, a further improvement is achieved: AI: {-0.20%, -0.49%, -0.48%}, and RA: {-0.22%, -0.91%, 0.33%}.

It is proposed to study the proposed method in the next round of EE.

- References

- EE1-1.2 Complexity-performance tradeoff of decomposition, JVET-AE164, D. Rusanovskyy, Y. Li, M. Karczewicz.

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-Z2016, E. Alshina, R.-L. Liao, S. Liu, A. Segall.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).