| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0159-v2 |
| --- | --- |

| Title: | EE2-3.6: Affine subblock BDOF refinement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi Zhang, Han Huang, Jian-Liang Lin, Vadim Seregin, Marta Karczewicz (Qualcomm) | Tel: Email: | zhizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the experiment results of EE2-3.6. It proposes to apply BDOF to refine subblock MV for an affine coded block. When an affine coded block meets the BDOF enabling condition, subblock MC is applied with BDOF refinement per subblock. The refined MVs are stored for future block MV prediction. The proposed method is applied to affine regular merge mode, affine BM merge mode, SbTMVP merge mode and affine AMVP mode. It also proposes the AMVP-Merge mode for affine block. When a block is decided to be AMVP-Merge mode, a flag is signalled to further indicate the block is an affine block or not. The proposed method was implemented on top of ECM-10.0 and the test results are as follows.

Test 3.6a, affine subblock BDOF refinement:

RA : -0.11% (Y), -0.07% (U), -0.02% (V), 100.2% (EncT), 102.5% (DecT), 100.1% (VmPeak)

Test 3.6b, AMVP-merge mode for affine:

RA : -0.02% (Y), 0.02% (U), 0.04% (V), 100.8% (EncT), 99.7% (DecT), 100.0% (VmPeak)

Test 3.6c, combination of Test 3.6a and Test 3.6b

RA : -0.13% (Y), -0.09% (U), -0.05% (V), 102.4% (EncT), 102.7% (DecT), 100.1% (VmPeak)

# Introduction

In ECM, BDOF refinement is not applied to subblocks in affine and SbTMVP modes, and AMVP-Merge mode is also not used with subblocks.

It is proposed to apply BDOF refinement to subblocks and utilize AMVP-Merge mode.

# Proposal

This contribution proposes to apply subblock MC and apply BDOF subblock MV refinement and sample adjustment to an affine or SbTMVP coded block when the block meets the BDOF condition.

An affine coded block, e.g. affine regular merge mode, affine BM merge mode, affine AMVP mode, derives MVs for each 4×4 subblock from the affine model. The BDOF process starts with the 4×4 subblocks grouping with identical MVs. The first iteration of BDOF MV refinement is processed in 8x8 subblock grid as in EMC-10.0. When the grouped subblock size is less than 256, the second iteration of BDOF MV refinement is processed in 4×4 subblock grid, and otherwise in 8×8 subblock grid. When the grouped subblock size is 4xN or Nx4, the first iteration of BDOF MV refinement is bypassed.

The BDOF enabling condition is the same as ECM-10.0, e.g., two reference pictures have equal POC distance to the current picture, and equal weight prediction.

It also proposes to apply AMVP-Merge mode to affine blocks. Like the AMVP-Merge mode in ECM-10.0, it signals reference picture list index, reference index and MVP index to indicate the AMVP predictor. A flag is signalled to indicate AMVP-Merge mode followed by affine mode flag if block size is equal or greater than 8×8 and the current picture is not a low-delay picture.

# Simulation results

## Test 3.6a: Affine subblock BDOF refinement

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 0.01% | 0.06% | 0.03% | 101.3% | 101.0% | 100.2% |
| Class A2 | -0.25% | -0.16% | -0.15% | 101.3% | 107.1% | 100.4% |
| Class B | -0.07% | -0.01% | -0.04% | 99.5% | 101.7% | 99.9% |
| Class C | -0.13% | -0.15% | 0.05% | 99.1% | 101.2% | 100.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.11% | -0.07% | -0.02% | 100.1% | 102.5% | 100.1% |
| Class D | -0.12% | 0.12% | -0.16% | 100.1% | 102.8% | 100.0% |
| Class F | -0.04% | -0.17% | -0.10% | 101.0% | 101.3% | 99.9% |
| Class TGM | 0.01% | 0.05% | 0.06% | 100.6% | 101.5% | 100.0% |

## Test 3.6b: AMVP-merge mode for affine

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.02% | -0.04% | 0.02% | 101.2% | 99.5% | 100.0% |
| Class A2 | -0.01% | 0.03% | -0.01% | 101.9% | 100.2% | 100.3% |
| Class B | -0.02% | 0.03% | 0.01% | 100.0% | 99.5% | 99.7% |
| Class C | -0.02% | 0.03% | 0.14% | 100.5% | 99.8% | 100.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.02% | 0.02% | 0.04% | 100.8% | 99.7% | 100.0% |
| Class D | -0.04% | 0.01% | -0.09% | 100.1% | 99.7% | 100.0% |
| Class F | -0.04% | -0.09% | -0.17% | 101.3% | 101.9% | 100.0% |
| Class TGM | -0.02% | 0.00% | -0.03% | 101.5% | 102.4% | 100.0% |

## Test 3.6c: Combination of Test 3.6a and Test 3.6b

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.01% | -0.02% | -0.02% | 103.6% | 100.8% | 100.2% |
| Class A2 | -0.28% | -0.16% | -0.10% | 103.6% | 107.5% | 100.5% |
| Class B | -0.08% | -0.03% | -0.05% | 102.0% | 102.1% | 99.8% |
| Class C | -0.15% | -0.17% | -0.02% | 101.1% | 101.5% | 100.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.13% | -0.09% | -0.05% | 102.4% | 102.7% | 100.1% |
| Class D | -0.16% | -0.02% | -0.10% | 101.4% | 103.0% | 100.0% |
| Class F | -0.07% | -0.08% | -0.02% | 102.1% | 102.2% | 100.1% |
| Class TGM | 0.00% | -0.04% | -0.01% | 100.9% | 100.9% | 100.0% |

# References

[1] ECM-10.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AE2017, April. 2023.

[3] Z. Zhang, H. Huang, J.-L Lin, V. Seregin, M. Karczewicz, “Non-EE2: Affine subblock BDOF refinement”, JVET-AE0148, July. 2023.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).