| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 32nd Meeting: Hannover, DE, 13–20 October 2023 | Document: JVET-AF0163-v1 |
| --- | --- |

| Title: | EE2-3.4: Enhanced subblock-based motion compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lei Zhao1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhaolei.0324@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

This proposal reports the performance of Test-3.4a, Test-3.4b, Test-3.4c and Test-3.4d for subblock-based motion compensation enhancement. It is reported that on top of ECM-10.0, simulation results of the proposed methods are as below:

Test 3.4a: TM-based subblock motion refinement

RA: -0.09% (Y), -0.06 % (U), -0.03 % (V), 101.0% (EncT), 100.9% (DecT)

LB: -0.03 % (Y),0.20 % (U), -0.02 % (V), 100.5% (EncT), 101.1% (DecT)

Test 3.4b: Interweaved affine prediction

RA: -0.02 % (Y), 0.08 % (U), -0.03 % (V), 100.6% (EncT), 96.9% (DecT)

LB: 0.01 % (Y), 0.01 % (U), 0.11 % (V), 100.5% (EncT), 93.3% (DecT)

Test 3.4c: RMVF candidate derivation with multiple CUs as input

RA: -0.02 % (Y), -0.01% (U), 0.02 % (V), 100.1% (EncT), 101.9% (DecT)

LB: -0.03 % (Y), 0.20 % (U), -0.07 % (V), 100.4% (EncT), 104.3% (DecT)

Test 3.4d: combination of Test 3.4a, Test 3.4b and Test 3.4c

RA: -0.14 % (Y), -0.02 % (U), -0.01 % (V), 101.4% (EncT), 100.7% (DecT)

LB: -0.05 % (Y), 0.05 % (U), -0.08 % (V), 100.2% (EncT), 98.7% (DecT)

- Introduction

At the 31th JVET meeting, Test 3.4 is established to investigate the enhanced subblock-based motion compensation method proposed in JVET-AE0117. In Test 3.4a, it is proposed to apply template matching (TM) to affine and SbTMVP mode. The control points of affine candidates and the motion shift of SbTMVP mode are refined using TM. In Test 3.4b, interweaved affine prediction is proposed which divides a coding block into subblocks with two different dividing patterns. In Test 3.4c, Regression-based Motion Vector Field (RMVF) candidate derivation is boosted by allowing the motion vector fields (MVF) from multiple CUs as input. Additionally, the combination of Test 3.4a, Test 3.4b and Test 3.4c is studied in Test 3.4d.

- Experimental tests

  - TM-based subblock motion refinement (Test 3.4a)

It is proposed to refine control point motion vectors (CPMVs) of uni-predicted affine merge candidates and the motion shift of SbTMVP candidates using TM. For a uni-predicted affine merge candidate, a same MV offset is assigned to all the CPMVs, and the TM cost of the affine candidate is calculated accordingly. The optimal CPMV offset with the minimum TM cost can be used to refine the corresponding affine candidate. For a SbTMVP candidate, the initial motion shift can be refined with TM, and then the refined motion shift will be utilized to derive subblock temporal motion information.

  - Interweaved affine prediction (Test 3.4b)

With the interweaved prediction, a coding block is divided into subblocks with two different dividing patterns. Then two auxiliary predictions are generated by Affine Motion Compensation (AMC) with the two dividing patterns. The final prediction is calculated as a weighted sum of the two auxiliary predictions.

First, a coding block is divided into subblocks with two different dividing patterns. The first dividing pattern (i.e., pattern 0) is the same as that in ECM, while the second dividing pattern also divides the coding block into 4×4 subblocks but with a 2×2 offset as pattern 1 shown in Fig. 1.

Second, two auxiliary predictions are generated by AMC with the two dividing patterns. The MV for each subblock in a dividing pattern is derived from CPMVs by the affine model.

The final prediction is calculated as a weighted sum of the two auxiliary predictions, formulated as:

| . | (1) |
| --- | --- |

As shown in Fig. 2, an auxiliary prediction sample located at the center of a subblock is associated with a weighting value 3, while an auxiliary prediction sample located at the boundary of a subblock is associated with a weighting value 1.

The subblocks associated with two dividing patterns are respectively refined by PROF unless the width or height is smaller than 4. Besides, subblock boundary deblocking/OBMC for affine mode is disabled.

Fig. 1. Interweaved prediction procedure.

Fig. 2. Weighting values in a subblock.

  - RMVF candidate derivation with multiple CUs (Test 3.4c)

It is proposed to derive additional RMVF affine candidates by feeding the MVF of multiple CUs as regression input. In particular, MVF of M (M=2 in the test) previously affine-coded CUs are simultaneously used to generate a RMVF candidate. The additional RMVF candidates are reordered together with other RMVF candidates through ARMC.

  - Test 3.4a + Test 3.4b + Test 3.4c (Test-3.4d)

In Test 3.4d, a combination of Test 3.4a, Test 3.4b and Test 3.4c is investigated.

- Experimental results

Four experiments are tested on top of ECM-10.0 under the common test conditions [1], which are listed as follows:

Test 3.4a: TM-based subblock motion refinement

Test 3.4b: Interweaved affine prediction

Test 3.4c: RMVF candidate derivation with multiple CUs as input

Test 3.4d: Test 3.4a + Test 3.4b + Test 3.4c

Table 1: Coding performance of Test 3.4a

|  | RA |  |  |  | LB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.01% | -0.02% | 100.9% | 100.8% |  |  |  |  |  |
| Class A2 | -0.17% | -0.14% | -0.09% | 100.9% | 101.7% |  |  |  |  |  |
| Class B | -0.07% | -0.01% | 0.02% | 100.8% | 100.6% | -0.03% | 0.30% | -0.32% | 100.4% | 101.7% |
| Class C | -0.08% | -0.11% | -0.04% | 101.3% | 100.8% | -0.03% | -0.11% | 0.05% | 100.2% | 100.9% |
| Class E |  |  |  |  |  | -0.04% | 0.43% | 0.36% | 101.2% | 100.6% |
| Overall | -0.09% | -0.06% | -0.03% | 101.0% | 100.9% | -0.03% | 0.20% | -0.02% | 100.5% | 101.1% |
| Class D | -0.04% | 0.00% | -0.25% | 100.8% | 102.2% | 0.03% | -0.06% | -0.26% | 100.2% | 102.7% |
| Class F | -0.13% | -0.18% | -0.15% | 100.8% | 100.5% | -0.11% | -0.26% | 0.26% | 100.9% | 100.8% |

Table 2: Coding performance of Test 3.4b

|  | RA |  |  |  | LB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | -0.02% | 100.2% | 95.7% |  |  |  |  |  |
| Class A2 | -0.04% | 0.01% | -0.06% | 100.3% | 92.6% |  |  |  |  |  |
| Class B | -0.02% | 0.13% | -0.07% | 100.5% | 97.6% | 0.03% | 0.30% | 0.07% | 100.6% | 90.3% |
| Class C | -0.03% | 0.10% | 0.05% | 101.3% | 100.2% | -0.02% | -0.43% | -0.01% | 100.6% | 96.3% |
| Class E |  |  |  |  |  | 0.01% | 0.11% | 0.33% | 100.0% | 94.2% |
| Overall | -0.02% | 0.08% | -0.03% | 100.6% | 96.9% | 0.01% | 0.01% | 0.11% | 100.5% | 93.3% |
| Class D | 0.04% | 0.21% | -0.06% | 101.0% | 100.2% | 0.19% | 0.13% | 0.06% | 100.0% | 95.0% |
| Class F | 0.01% | -0.03% | 0.03% | 100.6% | 100.4% | 0.06% | -0.05% | 1.13% | 100.2% | 97.7% |

Table 3: Coding performance of Test 3.4c

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.03% | 0.03% | 100.3% | 101.3% |  |  |  |  |  |
| Class A2 | -0.03% | 0.04% | -0.01% | 99.7% | 102.9% |  |  |  |  |  |
| Class B | -0.01% | 0.07% | 0.03% | 100.1% | 101.8% | -0.03% | 0.47% | -0.30% | 101.0% | 104.5% |
| Class C | -0.06% | -0.11% | 0.04% | 100.1% | 101.6% | -0.04% | -0.23% | -0.10% | 100.1% | 104.7% |
| Class E |  |  |  |  |  | -0.02% | 0.33% | 0.33% | 99.9% | 103.3% |
| Overall | -0.02% | -0.01% | 0.02% | 100.1% | 101.9% | -0.03% | 0.20% | -0.07% | 100.4% | 104.3% |
| Class D | -0.01% | 0.16% | 0.00% | 100.1% | 102.6% | 0.04% | -0.04% | -0.10% | 99.9% | 107.6% |
| Class F | -0.04% | 0.00% | 0.01% | 100.3% | 101.5% | 0.08% | -0.32% | 0.08% | 100.7% | 101.3% |

Table 4: Coding performance of Test 3.4d

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.06% | 0.05% | 101.3% | 99.1% |  |  |  |  |  |
| Class A2 | -0.26% | -0.13% | -0.17% | 100.7% | 97.7% |  |  |  |  |  |
| Class B | -0.14% | 0.06% | -0.01% | 101.3% | 101.6% | -0.05% | 0.25% | -0.28% | 100.4% | 96.7% |
| Class C | -0.11% | -0.02% | 0.05% | 102.2% | 103.3% | -0.08% | -0.30% | -0.04% | 100.6% | 102.1% |
| Class E |  |  |  |  |  | -0.03% | 0.19% | 0.21% | 99.2% | 97.5% |
| Overall | -0.14% | -0.02% | -0.01% | 101.4% | 100.7% | -0.05% | 0.05% | -0.08% | 100.2% | 98.7% |
| Class D | -0.06% | 0.12% | -0.24% | 101.7% | 104.2% | -0.06% | 0.07% | -0.08% | 99.8% | 106.6% |
| Class F | -0.18% | -0.21% | -0.24% | 101.2% | 102.4% | -0.10% | -0.55% | 0.28% | 101.3% | 99.4% |

- Conclusions

This proposal reports the performance of Test 3.4. Simulation results indicate promising coding gain in RA/LDB configurations with acceptable computational complexity. It is recommended to adopt this contribution in ECM.

- References

[1] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).