| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0165-v1 |
| --- | --- |

| Title: | Ahg12: On VTM-11ecm anchor low-delay test conditions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec, S. Puri, K. Naser, E. François | Tel: Email: | fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes to correct the VTM11-ecm test conditions, to align them with the ECM test conditions used in Ahg12.

In 31st JVET meeting, contribution JVET-AE0163 was adopted to significantly reduce the worst-case encoding time in low-delay configurations. This was achieved by reducing maximum MTT depth from 3 to 2 for inter slices with temporal depth greater or equal to 2 in class B.

However, this CTC modification was not ported to the test conditions of VTM-11ecm software, which is used as the anchor to evaluate ECM performances.

This contribution proposes to fix this CTC misalignment by using same encoder parameter modification for VTM-11ecm as for ECM.

# Introduction

During 31st JVET-meeting, contribution JVET-AE0163 [1] was adopted to significantly reduce the worst-case encoding time in low-delay configurations. This consisted in reducing the maximum MTT depth change from 3 to 2 for slices with temporal depth greater or equal to 2 in class B.

However, the porting of this CTC change to the VTM-11ecm10 anchor was not discussed. Therefore, the current test conditions of Ahg12 are misaligned between the VTM-11ecm10 anchor and ECM-10 with regards to the maximum MTT depth, in low-delay.

It is proposed to align the VTM-11ecm anchor and ECM low-delay test conditions. This is achieved by using the same value of following VTM-11ecm encoder parameter as for ECM:

- MaxMTTHierarchyDepthByTidOverrideByQP: 22 332222

It is to be noted that when similar worst-case coding time reduction was adopted in random access (JVET-AA0098 [2]), the CTC modification was adopted both for ECM and VTM-11ecm anchor.

# Impact on VTM-11ecm10 performances

The following Table 1 summarizes the impact of proposed CTC correction onto the performance of VTM-11ecm anchor.

Table 1: impact of proposed CTC correction on VTM-11ecm10 performances

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM11ecm9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.13% | 0.31% | 0.32% | 86.3% | 98.2% |
| Class C | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% |
| Class E | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% |
| Overall | 0.06% | 0.13% | 0.13% | 94.0% | 99.2% |
| Class D | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% |
| Class F | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% |
| Class TGM | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over VTM11ecm9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.11% | 0.34% | 0.43% | 84.9% | 103.7% |
| Class C | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% |
| Class E | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% |
| Overall | 0.05% | 0.14% | 0.18% | 93.4% | 101.5% |
| Class D | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% |
| Class F | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% |
| Class TGM | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% |

# Cited References

- JVET-AE0163, “HG12: Modified CTC proposal for low-delay configuration”, 	S. Puri, C. Bonnineau, F. Le Leannec, T. Poirier, E. Francois (InterDigital), 31st JVET meeting, Geneva, July 2023.

- JVET-AA0098, “AHG12: Encoder configuration proposal to reduce worst case encoding time”, F. Le Léannec, P. Andrivon, M. Radosavljević (Xiaomi), 27th JVET Meeting, by teleconference, July 2022

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).