| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0167-v1 |
| --- | --- |

| Title: | AHG9: On the Proposed Multiplane Image Information SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taoran Lu, Peng Yin, Sejin Oh, Sean McCarthy, Walt Husak, Gary J. Sullivan (Dolby) | Email: | {tlu, pyin, smcca}@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution provides an analysis and comparison of the capabilities and limitations of MIV (V3C) and the multiplane image information (MPII) SEI message proposed in JVET-AE0066. It is reported that the MIV specification provides flexibility to enable a variety of high-quality immersive video representations on devices that implement an MIV format decoder and one or more conventional 2D video decoders. In contrast, the proposed MPII SEI message is designed for the specific use case of distributing single-view MPI content to existing smartphones and other consumer devices equipped with a single conventional 2D decoder. It is also reported that the current MIV specification and TMIV reference software lack features required for the decoding of MPI content by devices equipped with only a single conventional 2D decoder. Temporal interleaved MPI representations are discussed, with commentary on relative bit rate overhead and implementation complexity of MIV and the proposed MPII SEI message. A key point is that a bitstream carrying an MPII SEI message can be decoded by an ordinary 2D video decoder, as contrasted with extraction of sub-bitstream NAL unit fragments from the MIV bitstream, followed by one or more ordinary 2D video decoders operating in parallel on the extracted data. It is suggested that an appropriate justification for applying the more general MIV approach is when the number of layers in an MIV representation is sufficiently large and the occupancy of each layer is sufficiently small to justify using an MIV atlas to reduce the amount of data processed through the video decoding processes.

This contribution also provides a software implementation of the proposed MPII SEI message for the VTM reference software.

# Introduction

During the meeting discussion of JVET-AE0066 [1], WG4 experts stated that the MPEG immersive video standard (MIV), specifically the MIV Extended Restricted Geometry profile, already supports MPI video distribution and questioned the need for an MPII SEI message. We therefore studied the MIV specification [2] and V3C specification [3] as well as the TMIV test model [4].

## Summary of MPII SEI message

As noted in JVET-AE0066, the proposed MPII SEI message is intended to enable virtual reality/immersive experiences on existing consumer devices using existing video codecs such as HEVC and VVC that were developed for conventional two-dimensional video. Using the SEI message approach, the bitstream is decoded by a conventional 2D video decoder and then the pictures that are output from the ordinary decoding process are reinterpreted as a packed representation of texture layers and opacity layers.

The syntax proposed in JVET-AE0066 is repeated in Table 1 for convenience. The MPII SEI message conveys information that can be used by a decoder to reconstruct an MPI picture from a decoded output picture containing packed MPI texture and opacity layers. The MPII SEI message supports side-by-side, top-and-bottom, and temporal interleaving packing arrangements. The MPII SEI message can also convey depth information corresponding to each MPI layer.

Table 1 – MPII SEI message syntax from JVET-AE0066

| multiplane_image_information( payloadSize ) { | Descriptor |
| --- | --- |
| mpii_num_layers_minus1 | ue(v) |
| mpii_layer_depth_equal_distance_flag | u(1) |
| if( mpii_layer_depth_equal_distance_flag ) { |  |
| depth_rep_info_element( ZNearSign, ZNearExp, ZNearMantissa, ZNearManLen ) |  |
| depth_rep_info_element( ZFarSign, ZFarExp, ZFarMantissa, ZFarManLen ) |  |
| } else |  |
| for( i = 0; i < = mpii_num_layer_minus1; i++ ) |  |
| depth_rep_info_element( ZSign[ i ], ZExp[ i ], ZMantissa[ i ], ZManLen[ i ] ) |  |
| mpii_texture_opacity_interleave flag | u(1) |
| if( mpii_texture_opacity_interleave flag = = 0 ) |  |
| mpii_texture_opacity_arrangement_flag /* 0: Top-and-Bottom, 1: Side-by-Side */ | u(1) |
| mpii_picture_num_layers_in_height_minus1 | ue(v) |
| } |  |

Appendix A contains a complete copy of the syntax and semantics from JVET-AE0066. No changes to the syntax and semantics of JVET-AE006 are proposed in this contribution.

# Findings

## Scopes of MIV Extended Restricted Geometry profile and MPII SEI message

A key finding is that the scopes of use of MIV Extended Restricted Geometry profile and the proposed MPII SEI message are distinct and complementary as indicated in Table 2. MIV can provide of a variety of high-quality 6 DoF immersive video representations. In contrast, the proposed MPII SEI message is designed for the specific use case of distributing low-overhead single-view 3DoF+ MPI content that can be decoded by a single decoder instance on existing consumer devices.

A key point is that a bitstream carrying an MPII SEI message can be decoded by an ordinary 2D video decoder, as contrasted with extraction of sub-bitstream NAL unit fragments from the MIV bitstream, followed by one or more ordinary 2D video decoders operating in parallel on the extracted data.

It is suggested that an appropriate justification for applying the more general MIV approach is when the number of layers in an MIV representation is sufficiently large and the occupancy of each layer is sufficiently small to justify using an MIV atlas to reduce the amount of data processed through the video decoding processes.

Table 2 – Comparison of the capabilities and limitations of the MIV Extended Restricted Geometry profile and the MPII SEI message

|  | MIV Extended Restricted Geometry profile | MPII SEI message |
| --- | --- | --- |
| Scope of use case | Single or multiple view MPI decodable by two conventional video decoders | Single view MPI, single bitstream decodable by one conventional decoder |
| Immersive experience capability | 6 DoF | 3 DoF+ |
| MPI content | Dense MPI layers; sparse objects | Small number of MPI layers and resolution; constrained by the profile/level definition of 2D decoders |

## Comparison of functionality, efficiency and deployability

This section summarizes a comparison of the functionality, efficiency, and ease of implementation of the MIV Extended Restricted Geometry profile and MPII SEI message. More details are contained in sections 2.3-2.6.

### Functionality gaps

- The MIV specification and TMIV software do not support delivery of MPI in a packed video format that can be decoded with a single conventional 2D video decoder. We have submitted a contribution ([5]) to WG4 that describes in more detail the issues in the current MIV specification and TMIV test model.

- The MPII SEI message supports temporal interleave packing to enable encoder optimization of the level/profile support required by the decoder. The MIV specification does not support temporal interleaved packing for texture and transparency. It only supports side-by-side or top-and-bottom packing in the spatial domain.

### Bit rate overhead and wrapping/unwrapping requirements

- The estimated bit rate overhead for MPI metadata signalling using MIV is about 7x greater than using the proposed MPII SEI message. The additional bit rate overhead is a result of the many MIV syntax elements that are present to enable immersive video use cases other than MPI.

- MIV (V3C) applies an additional “wrapper” (V3C unit) structure to contain conventional 2D video NALUs. The V3C unit wrapper adds extra bit overhead (32 bits for each V3C unit header).

### Compatibility with existing consumer devices

- MIV (V3C) applies an additional “wrapper” (V3C unit) structure to contain conventional 2D video NALUs. Thus, the bitstream cannot be decoded by an ordinary video decoder. A conventional 2D decoder would require normative MIV decoder capability to understand the V3C unit syntax and extract the conventional NALUs from the V3C units before being able to apply the conventional 2D decoding process.

- For MIV, the essential information needed to recover an MPI volumetric representation is carried in two different V3C units (V3C_VPS and V3C_AD). For the MPII SEI message, all essential information is carried in a single NALU.

### Packing efficiency

- TMIV software has been used to generate atlas video for the MPI content demonstrated in JVET-AE0066. No luma sample size reduction is observed compared to the MPII SEI message.

## MIV specification and test model issues to deliver MPI with single 2D conventional decoder

A WG4 contribution [5] has been provided to report issues found related to the MIV Extended Restricted Geometry profile in the MIV specification. Also, the TMIV reference software lacks support to generate a bitstream for MPI with frame packing, which is required to create a single bitstream decodable by a conventional 2D decoder.

## MIV lacks support for temporal interleaved packing

MIV/V3C supports frame packing by enabling the V3C packed video extension. Figure 2 illustrates how the packing information is carried in V3C_VPS. The packing information syntax contains a lot of elements, allowing flexible spatial domain packing of different attributes/geometry/occupancy with flexible positioning. However, there is no support for temporal interleaved packing. Also, such general flexibility provides no benefit for the specific MPI transmission use case and requires a large amount of bits compared with the 1-bit signalling to distinguish between side-by-side (SbS) and top-and-bottom (TaB) packing in the proposed MPII SEI message.

Figure 2. V3C signalling of frame packing to support SbS/TaB packing for MPI texture and transparency (in comparison, the MPII SEI message uses only a 1-bit flag to achieve the same functionality)

Temporal interleaved packing is a useful feature as it has half luma sample size compared with SbS/TaB packing and doubled frame rate. We analyzed the level requirement of a conventional (HEVC/VVC) decoder to support the decoding of MPI content (using 16 layers) captured by a single view reference camera with different packing options for the texture and transparency. The source camera features typical picture resolution from VGA (640x480) to Full HD (1920x1080) and captures at 30 frames per second. The results are shown in Figure 3. It shows that temporal interleaved packing substantially outperforms the spatial SbS/TaB packing for the capability to support the decoding of same resolution with lower decoder level requirements. The red color “Y” in the table indicating the cases that are supported by temporal interleaved packing only. For example, using spatial SbS/TaB packing, a level 6.0 decoder is needed to decode VGA resolution MPI, while level 5.1 decoder can decode the same MPI using temporal interleaved packing. For another example, Full-HD MPI cannot be decoded by level 6.2 decoder if spatial packing is used, but it can be decoded by level 6.1 decoder if temporal interleaved packing is used.

Figure 3. Level supporting capability of a conventional 2D decoder (HEVC/VVC) to decode a 16 layer MPI with typical image resolutions from VGA to FHD @30fps

## Bit rate overhead estimation for MPI metadata

We studied the bit rate overhead required to signal MPI metadata using MIV compared to using the proposed MPII SEI message.

The current version of TMIV cannot produce a single decoder MPI bitstream that could be used for direct comparison; thus we calculated bit rate overhead for one IRAP based on syntax tables for comparison. It is a rough estimation made with best effort, and without counting the bits for alignment and filling as defined in MIV/V3C specifications. Details of some key calculations can be found in Annex B. Note that for fair comparison, the layer depth values for both cases are assumed to be 32 bit for each layer (or patch in MIV), which is the maximum precision MIV can support.

For one typical example with 16 MPI layers, the overhead bits to signal the metadata to carry MPI information for MIV is about 7 times more than the overhead bits needed by the MPII SEI approach (3693 bits for MIV compared to 528 bits for MPII SEI message), as shown in Figure 4. The main reason is that MIV contains many syntax elements that support high level V3C unit encapsulation and general flexibility/functionality for V3C/MIV applications other than MPI.

Figure 4. Bit overhead estimation comparison for MIV single decoder and MPII SEI for one IRAP

## Example workflow of MPI coding using MIV vs. the MPII SEI approach

The MPI encoding workflow comparison between the MIV approach in TMIV reference software and the MPII SEI message approach is illustrated in Figure 5. The MIV approach is a multiple stage process that is significantly more complex than the MPII SEI message approach.

Figure 5. MPI encoding workflow of the MIV approach (in TMIV) vs. the SEI approach

The GuitarMan clip showed in JVET-AE0066 is used to test MPI workflow with TMIV [4]. The GuitarMan MPI is 768 × 384 × 16 layers × 30 frames. Using the MPII SEI approach in JVET-AE0066, the constituent picture resolution using a layer arrangement (2 row by 8 column) is [3072, 1536], as shown in Figure 6.

Figure 6 MPII SEI packed YUV,texture and transparency are packed with temporal interleaved packing. The resulting YUV is [3072, 1536] with 60 frames per second (doubled fps).

In order to use TMIV MPI encoder, which only takes the PCS format for MPI input, the raw MPI data (YUV of texture layers and transparency layers) has to be first converted to the PCS format by the TMIV MPIPCS tool, then this PCS file can be fed into the TMIV MpiEncoder with the cfg file (tmiv\config\test\miv_mpi\M_1_TMIV_encode.json) with proper modifications for I/O. Some observations are:

- There is a control parameter “framePacking”:false, but changing it to “true” does not make any difference in output. TMIV still produces two independent YUV outputs for texture and transparency to be compressed by two conventional 2D encoders, respectively.

- The TMIV MpiEncoder produced 16 patches automatically. The 16 patches each have the same size of one MPI layer ([768, 384]), but some rotations of the layers are observed in the output atlas YUV file.

- If we use the default value of "overrideAtlasFrameSizes": [[4096, 4096]], it produced the atlas YUV files of resolution [4096,4096] each, as shown in Figure 7. If we change the value to be equal to the constituent picture size in the corresponding MPII SEI approach [3072, 1536], it produced atlas YUV files very similar to the MPII SEI constituent pictures (but some patches/layers are rotated), as shown in Figure 8. We also tried to set a smaller resolution than this but then the TMIV_MpiEncoder dumped and failed to produce any results.

NOTE: The experimental results show that there is no reduction of luma sample size using the MIV approach compared with the packed video in MPII SEI for this kind of MPI content (with a small number of MPI layers that are not sparsely populated), thus will not gain any profile/level benefit by using the atlas patches.

Figure 7. TMIV MPI encoder outputed altases YUVs of texture and transparency. The atlas frame has default resolution [4096, 4096]

Figure 8. TMIV MPI encoder outputed altas YUVs of texture and transparency. Atlas frame size set to be [3072, 1536], the json file is the cfg file used to configure the TMIV MPI encoder. Setting a smaller size of atlas frames results in a runtime error for the TMIV MPI encoder.

# Conclusion

Based on the above analysis, it is concluded that the proposed MPII SEI message is preferable for distribution of single-view MPI content that is decodable by a single conventional 2D video decoder on existing smartphones and consumer devices. The current MIV specification is not compatible with this use case. Future amendments to the MIV specification might provide such capability, but significant bit rate overhead and decoder implementation complexity would remain.

It is thus proposed that the multiplane image information (MPII) SEI message be adopted into the next version of VSEI.

# References

- Taoran Lu, Peng Yin, Guan-Ming Su, Dae Yeol Lee, Tsung-wei Huang, Sean McCarthy, Walt Husak, and Gary J. Sullivan, “AHG9: Multiplane Image Information SEI”, JVET-AE0066, July 2023

- ISO/IEC 23090-12:2021(E)/AMD.1:2022, Information technology — Coded representation of immersive media — Part 12: MPEG Immersive video

- ISO/IEC 23090-5:2023(E), Information technology — Coded representation of immersive media — Part 5: Visual volumetric video-based coding (V3C) and video-based point cloud compression (V-PCC)

- TMIV software: https://gitlab.com/mpeg-i-visual/tmiv

- Taoran Lu, Sejin Oh, Peng Yin, Sean McCarthy, Walt Husak, Gary J. Sullivan, “[MIV] On MIV Extended Restricted Geometry Profile and MPI coding”, m65002, Oct. 2023

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex A
Multiplane image information SEI message semantics

The multiplane image information (MPII) SEI message specifies the MPI scene representation information that may be used for view synthesis.

When an MPII SEI message is present in any access unit of a CLVS, an MPII SEI message shall be present in the first access unit of the CLVS and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an MPII SEI message is output that follows the current picture in output order.

NOTE 1 – This SEI message can work together with the multiview acquisition information SEI message for view synthesis. The multiview acquisition information SEI message specifies the intrinsic and extrinsic parameters for current camera view. When multiple video bitstreams are available, the reconstructed novel views can be rendered from nearby multiview MPIs.

Use of this SEI message requires the definition of the following variables:

–	Cropped decoded output picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

–	a chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

–	a cropped decoded picture array decPicCurr0[ cIdx ][ x ][ y ], with cIdx = 0..(ChromaFormatIdc = = 0 ) ? 0 : 2, x = 0..(cIdx = = 0 ) ? CroppedWidth : CroppedWidth / SubWidthC − 1, y = 0..(cIdx = = 0 ) ? CroppedHeight : CroppedHeight / SubHeightC − 1.

–	In output order a temporally following cropped decoded picture array decPicCurr1[ cIdx ][ x ][ y ], with cIdx = 0..(ChromaFormatIdc = = 0 ) ? 0 : 2, x = 0..(cIdx = = 0 ) ? CroppedWidth : CroppedWidth / SubWidthC − 1, y = 0..(cIdx = = 0 ) ? CroppedHeight : CroppedHeight / SubHeightC − 1.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

mpii_num_layers_minus1 plus 1 specifies the number of texture and opacity layers for MPI representation.

mpii_layer_depth_equal_distance_flag equal to 1 indicates the equal distance is used to generate MPI layers and depth parameter for each layer Z[ i ] can be derive using nearest depth value ZNear and farthest depth value ZFar.

The depth value for i-th MPI layer Z[ i ] = i * ( ZFar − Znear ) / (mpi_num_layers_minus1 ) + ZNear (xx)

mpii_layer_depth_equal_distance_flag equal to 0 indicates that the depth information for each layer follows next in the SEI message.

The variables in the x column of Table xx are derived from the respective variables in the s, e, n and v columns of Table xx as follows:

–	If the value of e is in the range of 0 to 127, exclusive, x is set equal to ( − 1)s * 2( e − 31 ) * ( 1 + n ÷ 2v ).

–	Otherwise (e is equal to 0), x is set equal to ( − 1)s * 2−( 30 + v ) * n.

NOTE 2 – The above specification is similar to that found in IEC 60559:1989.

Table xx – Association between depth parameter variables and syntax elements

| x | s | e | n | v |
| --- | --- | --- | --- | --- |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| Z[ i ] | ZSign[ i ] | ZExp | ZMantissa | ZManLen |

NOTE 3 – In some applications, disparity is used instead of depth (the disparity value D and depth value Z relationship is D = 1/Z). For (xx), instead of deriving depth value, the disparity value for i-th MPI layer D[ i ] = i * ( DFar − Dnear ) / (mpi_num_layers_minus1 ) + DNear.

mpii_texture_opacity_interleave_flag equal to 1 indicates decoded output pictures correspond to temporally interleaved texture and opacity constituent pictures in output order as illustrated in Figure 5. mpii_texture_opacity_interleave_flag equal to 0 indicates decoded output pictures correspond to spatially packed texture and opacity constituent pictures as illustrated in Figures 3 and 4.

mpii_texture_opacity_arrangement_flag equal to 0 indicates decoded output pictures represent texture and opacity constituent pictures in a top-bottom packing arrangement as illustrated in Figure 3. mpii_texture_opacity_arrangement_flag equal to 1 indicates decoded output pictures represent texture and opacity constituent pictures in a side-by-side packing arrangement as illustrated in Figure 4.

For each specified picture packing arrangement scheme, there are two constituent pictures that are referred to as picture 0 and picture 1. When mpii_texture_opacity_interleave_flag is equal to 0, the constituent picture associated with the upper-left sample of the decoded picture is considered to be constituent picture 0 and the other constituent picture is considered to be constituent picture 1. When mpii_texture_opacity_interleave_flag is equal to 1, the first decoded picture in the current CLVS is constituent picture 0 and the next decoded picture in output order is constituent picture 1 and the display time of the constituent picture 0 should be delayed to coincide with the display time of constituent picture 1. The two constituent pictures form the spatially packed texture and opacity picture of a MPI, with picture 0 being associated with the spatially packed texture picture and picture 1 being associated with the spatially packed opacity picture.

mpii_picture_num_layers_in_height_minus1 plus 1 specifies the number of spatially packed layers in height for picture 0 and picture 1. The variable hLayers is set equal to mpii_picture_num_layers_in_height_minus1 + 1 and the variable wLayers is set equal to (mpii_num_layers_minus1 + 1) / hLayers.

Let variable fWidth and fHeight specify the width and height of picture 0 and picture 1, respectively, and are derived as follows:

–	If mpii_texture_opacity_interleave_flag is equal to 1, the following applies:

fWidth = CroppedWidth

fHeight = CroppedHeight

–	Otherwise (mpii_texture_opacity_interleave_flag is equal to 0)

–	If mpii_texture_opacity_arrangement_flag is equal to 0, the following applies:

fWidth = CroppedWidth , fHeight = CroppedHeight / 2

–	Otherwise (mpii_texture_opacity_arrangement_flag is equal to 1), the following applies:

fWidth = CroppedWidth / 2 , fHeight = CroppedHeight

Let variable cWidth = fWidth / subWidthC and variable cHeight = fHeight / subHeightC.

Let array picture0[ cIdx ][ x ][ y ] specify samples in picture 0 and array picture1[ cIdx ][ x ][ y ] specify samples in picture 1, with cIdx = 0..(ChromaFormatIdc = = 0 ) ? 0 : 2, x = 0..(cIdx = = 0 ) ? fWidth: cWidth − 1, y = 0..(cIdx = = 0 ) ? fHeight : cHeight − 1 and are derived as follows:

–	If mpii_texture_opacity_interleave_flag is equal to 1, the following applies:

picture0[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x ][ y ]

picture1[ cIdx ][ x ][ y ] = decPicCurr1[ cIdx ][ x ][ y ]

–	Otherwise (mpii_texture_opacity_interleave_flag is equal to 0)

–	Let variable cW = (cIdx = = 0 )? fWidth : cWidth

–	Let variable cH = (cIdx = = 0 )? fHeight : cHeight

–	If mpii_texture_opacity_arrangement_flag is equal to 0, the following applies:

picture0[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x ][ y ]

picture1[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x ][ y + cH ]

–	Otherwise (mpii_texture_opacity_arrangement_flag is equal to 1), the following applies:

picture0[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x ][ y ]

picture1[ cIdx ][ x ][ y ] = decPicCurr0[ cIdx ][ x + cW ][ y ]

Let variable layerWidth and layerHeight specify the width and height for decoded MPI layer, respectively. The variables are derived as follows:

layerWidth = fWidth/wLayers

layerHeight = fHeight/hLayers

The reconstruction of MPI process is described as follows:

The outputs of this process are:

–	a 4D mpi texture layer array recTextureLayer[ i ][ cIdx ][ w ][ h ] with i = 0..mpii_num_layers_minus1, cIdx = 0..(ChromaFormatIdc = = 0 ) ? 0 : 2, w = 0..(cIdx = = 0 ) ? layerWidth : layerWidth / SubWidthC − 1, and h = 0..(cIdx = = 0 ) ? layerHeight : layerHeight / SubHeightC − 1.

–	a 3D mpi opacity layer array recOpacityLayer[ i ][ w ][ h ] with i = 0..mpii_num_layers_minus1, x = 0..layerWidth − 1, and y = 0..layerHeight − 1.

The array recTextureLayer and array recOpacityLayer are derived as follows:

for( i = 0; i <= mpii_num_layers_minus1; i++ ) { 
	k = i % wLayers
	m = ( i − k ) / hLayers
	for ( cIdx = 0; cIdx < ChromaFormatIdc = = 0 ) ? 1 : 3; cIdx++ ) 
		for( h = 0; h < (cIdx = = 0 ) ? layerHeight : layerHeight / SubHeightC ; h++ ) 
			for( w = 0; w < (cIdx = = 0 ) ? layerWidth : layerWidth / SubWidthC ; w++ ) {
				u = k * (cIdx = = 0 ) ? layerWidth : layerWidth / SubWidthC + w
				v = m * (cIdx = = 0 ) ? layerHeight : layerHeight / SubHeightC + h
				recTextureLayer[ i ][ cIdx ][ w ][ h ] = picture0[ cIdx ][ u ][ v ]
			}
	for( h = 0; h < layerHeight; h++ ) 
		for( w = 0; w < layerWidth ; w++ ) 
			recOpacityLayer[ i ][ w ][ h ] = picture1[ 0 ][ k * layerWidth + w ][ m * layerHeight + h ]
}

# Annex B
Bit cost estimation information

- MIV syntax table bit calculations

There are 16 patch_data_unit() representing 16 MPI layers. Table below showed calculation for one.

- MPII SEI message syntax table calculations