| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0168-v2 |
| --- | --- |

| Title: | Non-EE2: Geometric partitioning mode with affine prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Zhipin Deng2, Li Zhang1 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | {zhangkai.video, zhipin.deng, lizhang.idm}@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This proposal presents a method of geometric partitioning mode (GPM) with affine prediction. A GPM partition can be predicted by affine motion compensation (AMC) or non-affine motion compensation (non-AMC), indicated by a flag. A GPM partition predicted by AMC can be blended with the other GPM partition predicted by AMC, non-AMC, or intra-prediction. Simulation results based on ECM-10.0 are reported as:

RA:{ -0.19%, -0.18%, -0.20%, 101.2%, 99.8%}.

# Introduction

The GPM mode in VVC splits a CU into two partitions separated by a straight splitting line, based on a pre-defined lookup table. The motion compensation for each GPM partition is performed to the whole CU for an easier hardware implementation. After obtaining two predictions, a sample-based weighted blending process is used to modify prediction values along the splitting line. The GPM mode is treated as a type of merge mode, and each partition of a GPM CU is merge predicted with its own merge candidate index, which is constrained to be uni-prediction coded.

In ECM, GPM has been further developed in several ways as: a GPM partition can be predicted by a merge MMVD candidate; one of the two GPM partitions can be predicted by intra-prediction; merge candidates of the two GPM partitions can be refined by template matching; the splitting ways can be reordered according to template costs; the blending weights in GPM can be adaptive; and bi-prediction is allowed to predict a GPM partition.

# Proposed method

In this contribution, GPM is further extended to enable affine motion compensation (AMC). Therefore, a GPM partition can be predicted by AMC inter-prediction, non-AMC inter-prediction or intra-prediction. In addition, a GPM partition predicted by AMC can be combined with the other GPM partition predicted by AMC, non-AMC, or intra-prediction.

When AMC is applied, a uni-prediction affine merge candidate list is constructed from the subblock-based merge candidate list after discarding sub-TMVP candidates, similar to the uni-prediction merge candidate list construction for GPM in VVC. AMC is performed for a GPM partition using the control point motion vectors (CPMVs) of a merge candidate in the uni-prediction affine merge candidate list.

A gpm_affine_flag is signaled for each GPM partition to indicate whether AMC is applied for the GPM partition. A merge candidate index for the GPM partition is signaled using different arithmetic context models depending on whether AMC or non-AMC is applied.

In the current implementation, AMC is not allowed for GPM-MMVD and GPM-TM.

# Experimental results

The proposed method is implemented on top of ECM-10.0 [1] and tested following the common test conditions [2]. The simulation results are summarized in Table 1.

Table 1: Coding performance over ECM-10.0

|  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.23% | -0.13% | 100.7% | 100.3% |  |  |  |  |  |
| Class A2 | -0.29% | -0.26% | -0.23% | 100.6% | 98.3% |  |  |  |  |  |
| Class B | -0.18% | -0.08% | -0.22% | 101.8% | 99.6% |  |  |  |  |  |
| Class C | -0.19% | -0.21% | -0.21% | 101.3% | 100.6% | -0.08% | -0.55% | -0.30% | 101.7% | 100.2% |
| Class E |  |  |  |  |  | -0.30% | 0.56% | 0.27% | 100.9% | 98.8% |
| Overall | -0.19% | -0.18% | -0.20% | 101.2% | 99.8% |  |  |  |  |  |
| Class D | -0.16% | -0.04% | -0.02% | 102.1% | 98.6% | -0.13% | -0.03% | 0.12% | 100.1% | 97.9% |
| Class F | -0.05% | -0.04% | 0.07% | 100.8% | 99.5% |  |  |  |  |  |

# Conclusions

The proposed method of GPM with affine prediction can achieve some BD-rate gains with minor runtime changes. It is recommended to study the proposed method in the next round of EE2.

# References

- ECM-10.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-10.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).