| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 32nd Meeting: Hannover, DE, 13–20 October 2023 | Document: JVET-AF0170-v3 |
| --- | --- |

| Title: | Non-EE2: CIIP with subblock-based motion compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lei Zhao1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhaolei.0324@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

In this contribution, CIIP with subblock-based motion compensation is presented, where the inter prediction signal of CIIP may be generated based on affine or sbTMVP motion compensation. It is reported that on top of ECM-10.0, simulation results of the proposed method are as below:

RA: -0.07 % (Y), -0.06 % (U), -0.07 % (V), 100.4% (EncT), 100.4% (DecT)

LB: -0.08 % (Y), -0.21 % (U), -0.28 % (V), 101.2% (EncT), 100.4% (DecT)

- Introduction

In ECM-10, the intra prediction signal of CIIP is generated with TIMD or PDPC (i.e., CIIP-PDPC). To generate the inter signal, a regular merge candidate or a TM-refined merge candidate (i.e., CIIP-TM) can be utilized . It is asserted that subblock-based inter merge candidate may benefit CIIP by providing a fine-granularity inter prediction signal.

- Proposed method

In this contribution, CIIP is improved by allowing subblock-based inter prediction. In particular, a subblock-based merge candidate may be used to generate the inter signal of CIIP, where the same subblock-based merge candidate list used by affine and sbTMVP is utilized.

When CIIP flag is true and CIIP-TM flag is false, a subbock-based CIIP flag is signalled. If subblock-based CIIP flag is true, an index indicating specific candidate in the subblock-based merge list is signalled, and TIMD is used to generate intra signal by default thus no CIIP-PDPC flag signalled any more.

- Experimental results

The proposed method has been implemented on top of the ECM-10.0 [1]. Results under RA/LDB configurations are presented in Table 1. The simulations were performed in accordance with the common test conditions [2]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the proposed method on ECM-10.0

|  | RA |  |  |  | LB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.23% | -0.03% | 102.5% | 100.3% |  |  |  |  |  |
| Class A2 | -0.03%* | 0.01% | -0.10% | 100.4% | 100.2% |  |  |  |  |  |
| Class B | -0.08% | 0.06% | -0.09% | 99.1% | 100.5% | -0.08% | 0.00% | -0.33% | 101.2% | 100.5% |
| Class C | -0.08% | -0.15% | -0.05% | 100.5% | 100.4% | -0.07% | -0.57% | -0.25% | 101.5% | 100.7% |
| Class E |  |  |  |  |  | -0.11% | -0.09% | -0.24% | 100.7% | 100.0% |
| Overall | -0.07% | -0.06% | -0.07% | 100.4% | 100.4% | -0.08% | -0.21% | -0.28% | 101.2% | 100.4% |
| Class D | -0.06% | 0.07% | -0.13% | 102.0% | 100.7% | -0.07% | 0.13% | -0.28% | 101.1% | 100.3% |
| Class F | -0.03% | -0.07% | -0.11% | 100.7% | 100.2% | 0.33% | 0.30% | 0.91% | 102.4% | 102.1% |

- Conclusions

CIIP with subblock-based motion compensation is presented in this contribution. Simulation results indicate 0.07%/0.08% coding gain in RA/LDB configurations with 100.4%/100.4% and 101.2%/100.4% complexity increase. It is recommended to include the proposed methods in the next round of EE.

- References

[1] ECM-10.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-10.0.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).