| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0173-v1 |
| --- | --- |

| Title: | EE2-2.4: Weighted edge enhancement filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tim Claßen, Mathias Wien | Tel: Email: | tim.classen@lfb.rwth-aachen.de mathias.wien@lfb.rwth-aachen.de |
| Source: | RWTH Aachen |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2 Test 4.2a and Test 4.2b. Those tests were motivated by the results proposed in JVET-AE0103. In the current ECM version, picture upscaling in RPR is dome by multi-phase interpolation filters. The upscaling is used for inter-prediction and to rescale the picture before it is written to the output stream. Test 4.2a modifies the inter-prediction such that a weighted edge enhancement filter is applied in the inter-prediction process. Test 4.2b modifies the software such that the edge enhancement filter is applied in the inter-prediction and to upscale the low-resolution pictures after upscaling. Unfortunately, there are some implementation issues that prevent the multi-pass slice encoding, which is required for Test 4.2a from working in the current ECM version. Deactivating REUSE_CU_RESULTS circumvents these problems but leads to a slightly different RD-performance. Due to time reasons, fixing the problem was not possible. Therefore, this fix was applied to all tests as well as the anchor. This approach was selected in order to isolate the effects of the proposed method.

# Introduction

# The VVC compression standard introduced Reference Picture Resampling (RPR) as a tool for adaptively changing the resolution within a Coded Layer Video Sequence. Unlike previous standards, the output resolution is not restricted to the resolution at which a given frame of the video sequence is coded. To achieve this functionality, VVC introduced a set of filters to facilitate switching between different resolutions.

# The filters used for resolution change in the VVC standard are currently fixed linear filters. These filters are well suited for a wide range of video content. However, the content within coded video sequences is locally correlated and depends on the specific video sequence. Furthermore, image upscaling presents an inherently non-linear problem. To account for that, we propose an adaptive locally weighted filter in JVET-AE0103. We make the assumption that optimizing a filter with the knowledge of errors caused by coding and the characteristics of the current frame improves the performance of the coding scheme.

# Current RPR implementation in ECM

ECM supports both the switching of coding resolution and, temporarily coding at lower resolution. Therefore, it necessitates both downsampling and upsampling procedures in order to switch resolution and to reference pictures of different resolution in inter-prediction. The current reference software in ECM employs a fixed upsampling procedure using interpolation filters for both of those applications. The upsampling is done by finite impulse response filters. Since the resolution of the output device remains unchanged, the output itself still needs to be displayed at a high resolution. This proposal addresses two application scenarios. The first application scenario makes use of enhancement filters only inside the coding loop, i.e. the filters are not used to improve upscaled low-resolution pictures for the output. Specifically, the proposal suggests changes to the upsampling procedure while keeping the downsampling procedure unaffected. As a result, the evaluation of these changes is most effectively conducted in the high-resolution domain.

# Problem Description

Pictures coded at lower resolution often lack high frequency information which is present in their high-resolution equivalents. Consequently, pictures which were coded at low resolution usually appear blurred after upsampling. Consequently, the prediction error is usually higher if low-resolution frames are referenced. The issue stems from the fact that the upsampling process cannot recover high-frequency information which was lost during downsampling. However, artifacts caused by Reference Picture Resampling (RPR) are most pronounced in areas that do not align with the underlying assumptions of the interpolation scheme (i.e. smoothness). In the reconstructed video generated by the VTM or the ECM, this is particularly evident in content containing high-frequency information, such as edges.

The current reference software employs a fixed set of downsampling and upsampling filters without utilizing any side information from the coded video or bitstream to enhance the quality of the upsampled picture or prediction signal. For example, the distribution of frequency components may differ depending on the coded video sequence or spatial location making the most optimal filter depend on the characteristics of the content. Current filtering methods, such as the adaptive loop filter, solve this problem by signaling a set of filters. The decision which filter is to be applied to a given picture partition is signaled and may additionally be inferred from the picture content. However, in case of upsampled image content, there are different types of errors compared to those addressed by in-loop filters. Typically, there blurring is introduced by the down- and upsampling processes. For that reason, applying sharpening, especially at the edges, can be beneficial.

# Proposed Methods

This proposal addresses limitations of the upsampling procedure in Reference Picture Resampling (RPR) and picture upsampling. The current implementation in the ECM reference software takes a frame or block as input and applies a set of multi-phase interpolation filters to interpolate subsample positions. An issue is that the problem of image upsampling is inherently ill-posed. Therefore, no filtering method can solve this problem for an arbitrary type of content. However, side information which is inferred from the coded video or encoded in the bitstream may be used to amend this problem. It is proposed to add another processing step after the upsampling; namely, an adaptive locally weighted filter.

The underlying assumption is that the optimal strength of the filter depends on the spatial location and can be inferred from the content. Since the filter is least-squared optimized based on the applied weighting and the upsampled picture, there is an interdependency. Therefore, the overall performance of the filter depends on the initial choice of weighting function. Consider, for example, a filter which is least-squares optimized (without weighting) such that the result is added to the filtered picture to obtain the output. After this optimization, certain pixels may have an optimal, too large, too small or incorrectly signed computed offset. Now, if it were possible to compute a local weighting that reduced the offset applied by the filter at regions where the computed offset is too high or has the wrong sign and increase the offset at regions where it is too small, the error would be decreased. Additionally, since the weighting is known beforehand, a better filter could be optimized. However, it is not practically feasible to find a weighting map function which is optimal for each type of content and at every location in the picture. Nonetheless, approximating the optimal weighting with a weighting map could already lead to improved results.

In the application of RPR upscaling, the content is usually blurred, especially in high-frequency structures such as edges. Linear filters usually exhibit ringing and overshoot artifacts, and the filtered edge frequently still has less steepness compared to the ground truth edge. Therefore, increasing the offset of the filter at the predicted location of the edge and decreasing it at the surrounding would improve the filter.

## Algorithmic description

An overview of the proposed method is depicted in Figure 2. The inputs are the decoded, upscaled picture or block and the side information which is encoded in an adaption parameter set in the bitstream. First, the filter parameters are decoded. This includes information regarding the weighting map function, filter shape, luma and chroma flags, and filter coefficients. Next, the weighting map function is applied to obtain the local weighting map. Then, the filter is applied to the upscaled picture or block to generate a filtered map. The filtered map is multiplied by the local weighting and added to the upsampled picture as illustrated in Figure 1.

Figure 1: Schematic diagram of weighted edge enhancement filter

A detailed description of the filter and its optimization can be found in JVET-AE0103. In this experiment, the filter is tested for two testing conditions.

# Test 4.2a

Test 4.2a evaluates the filter as enhancement filter for inter-prediction only. In this case, no enhancement of upscaled pictures for output is done. Moreover, the filter derivation differs a bit from JVET-AE0103. The reasons for this are given in the following. Let there be a video sequence which is encoded with RPR. The sequence will be encoded at high resolution for a certain number of pictures, then switch to a lower coding resolution and then switch to a higher resolution again. In JVET-AE0103 and Test 4.2b one filter for each low-resolution picture is RD-optimized and applied for the inter-predicted blocks from this picture. This approach is efficient in the scenario of Test 4.2b since the whole picture is upscaled and refined anyway. This is due to the fact that the filter is also used to improve the upscaled low-resolution output pictures. This is not the case in Test 4.2a. It has to be noted that not all low-resolution pictures may be used as reference for high-resolution pictures. Moreover, not all picture areas of an upscaled picture may be used as reference either. Consequently, it is not efficient to optimize filters for the whole picture as it is done in JVET-AE0103. Instead, the implementation of Test 4.2a optimizes the filter for the blocks which are estimated to be used by the high-resolution frame. An ad-hoc method has been implemented for this purpose: In order to find the referenced blocks, a trial encoding of the slice is done. Then, all referenced low-resolution blocks are stored alongside with the ground truth data of the motion compensated block in the current picture. The filter is optimized such that the error summed over all blocks is least-squares minimized. If the trial encoding indicates that the referencd low-resolution area is lower than a predefined threshold (2%), the optimization is skipped. Next, the filter is assigned to the high-resolution picture and the picture is encoded again. If the RD-costs of encoding the picture with the filter are lower than without, the filter is set. Otherwise, the picture is encoded without the filter. Below, the optimization algorithm is summarized:

- Encode the slice

- Test whether the references are from low-resolution pictures is at least 2% of the picture area

  - If true: Proceed with step 3

  - If false: Proceed with step 7

- RD-optimize the enhancement based on the inter-predicted blocks and their corresponding ground-truths

- Assign the filter as enhancement filter to the current slice and trial encode the slice

- Test whether the RD-performance with the filter is better than without the filter

  - If true: Proceed with step 7

  - If false: Proceed with step 6

- Remove the assigned filter and re-encode the slice without the filter

- End of slice compression

# Test 4.2b

Test 4.2b evaluates the filter as enhancement filter for inter-prediction and upscaled output picture enhancement. This test applies the optimization of JVET-AE0103 which differs from Test 4.2a. Here, one filter is optimized for each low-resolution picture. The distortion is given by the encoding costs of the picture and the filter. The distortion is the distortion after the filter is applied. A filter is chosen, if the RD-costs are lower with the filter being applied. This filter is then used to enhance any upscaled content that was generated from this picture. Consequently, the same filter is used, if the filter is output (i.e. written as upscaled picture) and if a block of this picture is referenced. The filtering function is the same as in test 4.2a.

# Experimental Results

The proposes method is implemented on top of ECM-10.0 and tested following the common testing conditions for RPR. The results for Test 4.2a are summarised in Table 1 and Table 2 and the results for Test 4.2b are summarised in Table 3 and Table 4. The implementation of Test 4.2a requires a multi-pass slice encoding. This is provided with the multi-pass adaptive QP encoding. Unfortunately, there are some implementation issues that prevent this from working in the current ECM version. Deactivating REUSE_CU_RESULTS circumvents these problems but leads to a slightly different RD-performance. Due to time reasons, fixing the problem was not possible. Therefore, this fix was applied to all tests as well as the anchor. This approach was selected in order to isolate the effects of the proposed method.

The results for experiment will be added as soon as the calculations are finished.

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Experiment 4.2a over ECM-10.0 (RPR 1.5x) |  |  |  |  |
|  | Y | U | V | EncT* | DecT* |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0% | 0% | 0% | 100% | 100% |
| Class C | 0% | 0% | 0% | 100% | 100% |
| Class E | 0% | 0% | 0% | 100% | 100% |
| Overall | 0% | 0% | 0% | 100% | 100% |
| Class D | 0% | 0% | 0% | 100% | 100% |
| Class F | 0% | 0% | 0% | 100% | 100% |
| Class TGM | 0% | 0% | 0% | 100% | 100% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Experiment 4.2a over ECM-10.0 (RPR 2.0x) |  |  |  |  |
|  | Y | U | V | EncT* | DecT* |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0% | 0% | 0% | 100% | 100% |
| Class C | 0% | 0% | 0% | 100% | 100% |
| Class E | 0% | 0% | 0% | 100% | 100% |
| Overall | 0% | 0% | 0% | 100% | 100% |
| Class D | 0% | 0% | 0% | 100% | 100% |
| Class F | 0% | 0% | 0% | 100% | 100% |
| Class TGM | 0% | 0% | 0% | 100% | 100% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Experiment 4.2b over ECM-10.0 (RPR 1.5x) |  |  |  |  |
|  | Y | U | V | EncT* | DecT* |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0% | 0% | 0% | 100% | 100% |
| Class C | 0% | 0% | 0% | 100% | 100% |
| Class E | 0% | 0% | 0% | 100% | 100% |
| Overall | 0% | 0% | 0% | 100% | 100% |
| Class D | 0% | 0% | 0% | 100% | 100% |
| Class F | 0% | 0% | 0% | 100% | 100% |
| Class TGM | 0% | 0% | 0% | 100% | 100% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Experiment 4.2b over ECM-10.0 (RPR 2.0x) |  |  |  |  |
|  | Y | U | V | EncT* | DecT* |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0% | 0% | 0% | 100% | 100% |
| Class C | 0% | 0% | 0% | 100% | 100% |
| Class E | 0% | 0% | 0% | 100% | 100% |
| Overall | 0% | 0% | 0% | 100% | 100% |
| Class D | 0% | 0% | 0% | 100% | 100% |
| Class F | 0% | 0% | 0% | 100% | 100% |
| Class TGM | 0% | 0% | 0% | 100% | 100% |

# Conclusions

This contribution presents a weighted edge enhancement filtering method in the context of reference picture resampling. The method is evaluated as enhancement filter for inter-predicted blocks of low-resolution pictures and in a scenario where the same filter is applied as enhancement filter for inter-predicted blocks of low-resolution pictures and as an enhancement filter for upscaled low-resolution pictures before their output. The results for experiment will be added as soon as the calculations are finished.

# Patent rights declaration(s)

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).