| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0177-v2 |
| --- | --- |

| Title: | AHG6: Memory reduction for ECM encoder |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu Vadim Seregin Marta Karczewicz Yukinobu Yasugi Tomohiro Ikai | Tel: Email: | nanh@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com yasugi.yukinobu@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Qualcomm Incorporated, Sharp Corporation |  |  |

_____________________________

# Abstract

To reduce the memory usage of ECM encoder, this contribution proposes to change the data type of adaptive loop filter covariance from double into float. On top of ECM-10.0 with common test condition, simulation results of the proposed method are reported as follows. For class A and B sequences, encoder memory usage is reduced by around 1 GiB.

|  | BD-rate Y | BD-rate U | BD-rate V | EncT | DecT | VmPeak |
| --- | --- | --- | --- | --- | --- | --- |
| All intra | 0.00% | 0.01% | -0.01% | 98.7% | 100.0% | 89.7% |
| Random access | 0.00% | 0.05% | -0.01% | 98.8% | 100.4% | 94.1% |
| Low delay B | -0.02% | 0.29% | -0.01% | 97.8% | 99.7% | 92.5% |

# Introduction

As the development of ECM, encoder requires more and more memory. When encoding a 4K sequence under random access common test condition, ECM-10.0 requires over 18GiB memory. For adaptive loop filter (ALF) encoder, to support the optimization of non-linearity and multiple classifiers for ALF, a multi-dimensional covariance matrix is allocated for each coding tree unit (CTU) in double data type. In addition, as the number of the coefficients in a filter increases, the memory of a covariance matrix increases. As such, the covariance matrices increase the memory usage of ALF encoder, especially for large resolution sequences with a large number of CTUs.

# Proposed method

To reduce the encoder memory burden caused by ALF covariance, as in JVET-Z0150, it is proposed to change its data type from double into float. By this method, the memory usage of ALF covariance could be reduced by half.

# Simulation results

The proposed method was implemented on top of ECM-10.0 and tested following common test conditions described in JVET-AE2017.

|  | All Intra Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Ref GiB | Test GiB |
| Class A1 | 0.00% | 0.05% | -0.03% | 98.0% | 100.3% | 88.5% | 8.6 | 7.6 |
| Class A2 | 0.00% | -0.01% | -0.02% | 98.5% | 100.5% | 88.5% | 8.6 | 7.6 |
| Class B | 0.00% | -0.01% | -0.02% | 99.6% | 101.6% | 85.7% | 6.9 | 5.9 |
| Class C | 0.00% | 0.00% | 0.00% | 98.9% | 98.1% | 95.4% | 4.5 | 4.3 |
| Class E | 0.00% | 0.01% | 0.01% | 97.6% | 98.9% | 91.5% | 5.2 | 4.8 |
| Overall | 0.00% | 0.01% | -0.01% | 98.7% | 100.0% | 89.7% | 6.5 | 5.8 |
| Class D | 0.00% | 0.00% | 0.00% | 100.5% | 100.1% | 98.5% | 4.1 | 4.0 |
| Class F | 0.00% | 0.00% | -0.01% | 99.9% | 101.1% | 91.4% | 5.7 | 5.2 |
|  | Random Access Main 10 |  |  |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Ref GiB | Test GiB |
| Class A1 | 0.03% | 0.08% | 0.09% | 98.8% | 100.3% | 95.3% | 18.2 | 17.3 |
| Class A2 | 0.05% | 0.00% | 0.05% | 99.2% | 101.1% | 95.0% | 18.8 | 17.8 |
| Class B | -0.03% | 0.13% | -0.11% | 98.1% | 100.0% | 90.8% | 9.4 | 8.5 |
| Class C | -0.02% | -0.04% | 0.01% | 99.4% | 100.6% | 96.6% | 5.1 | 4.9 |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.05% | -0.01% | 98.8% | 100.4% | 94.1% | 10.4 | 9.8 |
| Class D | 0.02% | 0.01% | -0.11% | 101.1% | 100.6% | 98.8% | 4.3 | 4.2 |
| Class F | -0.01% | 0.01% | 0.00% | 96.3% | 100.6% | 94.5% | 7.2 | 6.8 |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Ref GiB | Test GiB |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | 0.02% | 0.66% | 0.24% | 97.9% | 99.2% | 89.1% | 7.7 | 6.9 |
| Class C | -0.01% | 0.10% | -0.04% | 99.3% | 99.7% | 96.2% | 4.7 | 4.5 |
| Class E | -0.13% | -0.06% | -0.37% | 95.7% | 100.6% | 93.2% | 5.6 | 5.2 |
| Overall | -0.02% | 0.27% | -0.02% | 98.4% | 100.1% | 92.4% | 6.1 | 5.6 |
| Class D | 0.06% | 0.56% | 0.15% | 98.5% | 98.0% | 98.7% | 4.1 | 4.1 |
| Class F | 0.00% | 0.05% | -0.29% | 97.0% | 100.1% | 93.5% | 6.4 | 6.0 |

# Patent rights declaration(s)

Qualcomm Incorporated does not have any current or pending patent rights relating to the technology described in this contribution.

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).