| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0180-v1 |
| --- | --- |

| Title: | AHG11: HOP training adjustment to improve luma/chroma coding gain balance |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | D. Rusanovskyy, Y.Li, Marta Karczewicz (Qualcomm) | Email: | dmytror@qti.qualcomm.com yli30@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

A Unified Filter Architecture for the High-performance Operation Point (HOP) was defined at JVET-AD0380. Test results for stage 3 of HOP reported in JVET-AF00141 indicated a noticeable disbalance in coding gain for luma and chroma components. HOP bd-rate gain over the NNVC anchor is {-5.4%, -19.2%, -19.9%} in RA and {-3.3, -14.7, -15.8} in AI. Since HOP utilizes a single model for Luma and Chroma, the balance can be improved by adjusting the training strategy. It is proposed to increase training loss weight for the luma component from 6 to 12, similarly to the method utilized in EE1-0 LOP2 training. HOP with luma weight equal to 12 reportedly provides {-5.8%, -16.1%, -16.9%} and {-3.5%, -12.1%, -13.3%} bd-rate gain over the NNVC anchor in RA and AI configurations, respectively. Additional test was conducted with a weight for the luma component being equal to 24. Achieved results does not demonstrate favourable luma/chroma trade off. It is proposed to modify HOP training strategy by increasing luma training loss weight to 12.

# Introduction

A Unified Filter Architecture for the High-performance Operation Point (HOP) was defined at JVET-AD0380. Training strategy and intermediate results were presented in JVET-AE0191 and complete inference results for Stage3 HOP training are reported in JVET-AF0041.

It can be observed from the reported inference results, that all independently trained models resulted in a noticeable disbalance of the HOP coding gains over utilized anchors, either VTM or NNVC anchor, as shown in Tables 1 and 2 below.

Table 1 NNVC 6.0 vs NNVC 6.0 + HOP int16 (source JVET-AF0041)

|  | NNVC 4.0 vs NNVC-6.0rc VTM+HOP int16 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -2.58% | -15.45% | -18.02% | -5.41% | -17.63% | -22.03% |
| Class A2 | -3.12% | -15.30% | -13.68% | -5.71% | -19.22% | -19.10% |
| Class B | -2.88% | -14.39% | -14.82% | -5.06% | -19.76% | -18.79% |
| Class C | -3.63% | -12.62% | -15.24% | -5.52% | -19.47% | -20.33% |
| Class E | -4.52% | -16.74% | -18.11% |  |  |  |
| Overall | -3.31% | -14.72% | -15.81% | -5.38% | -19.15% | -19.91% |
| Class D | -2.98% | -13.49% | -16.04% | -5.84% | -19.48% | -19.97% |
| Class F | -2.82% | -12.02% | -12.45% | -3.74% | -13.98% | -13.51% |

Table 2 NNVC 4.0 vs NNVC 6.0rc VTM + HOP int16 (source JVET-AF0041)

|  | NNVC 4.0 vs NNVC-6.0rc VTM+HOP int16 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -7.20% | -18.27% | -22.13% | -10.52% | -20.62% | -25.85% |
| Class A2 | -7.09% | -20.09% | -18.85% | -10.79% | -23.75% | -22.69% |
| Class B | -6.98% | -19.22% | -19.78% | -9.65% | -25.88% | -24.33% |
| Class C | -8.08% | -18.41% | -21.09% | -10.49% | -27.44% | -27.39% |
| Class E | -10.45% | -20.65% | -21.76% |  |  |  |
| Overall | -7.86% | -19.27% | -20.64% | -10.27% | -24.82% | -25.12% |
| Class D | -7.92% | -17.77% | -21.44% | -11.88% | -26.75% | -27.45% |
| Class F | -5.27% | -15.78% | -15.91% | -5.81% | -18.45% | -17.28% |

Unified Filter Architecture has a single NN model for filtering of all color components and its features a joint processing of Luma and Chroma components in a single backbone. To adjust NN model performance between color components during the training, training loss weight parameters for luma and chroma components can be utilized. JVET-AE0191 sets component_loss_weightings = {6,2} for HOP training.

# Proposal

In this contribution HOP training strategy was adjusted to achieve better coding gains balance between luma and chroma components.

HOP training with component_loss_weightings = {12,2} was conducted, similarly to the training adjusted employed in LOP2 training (EE1-0).

component_loss_weightings = {12,2}

Additional test was conduction with following parameters:

component_loss_weightings = {24,2}

# Simulation results

Proposed training strategy was tested on the Stage 3 HOP training, following the guidance specified in JVET-AE0191. Inference test was conducted with NNVC6.0 reference SW, following the NNVC CTC.

Validation loss functions for the proposed training strategy, comparing to the HOP anchor validation loss L1 for Y and CbCr components are shown in Figure 1.

Figure 1. Validation loss comparison for HOP anchor and proposed method.

Summary of the inference test results, comparing to the NNVC anchor are shown in Table 3, complete results in CTC template are provide along this document.

Table 3. Proposed training strategy (luma weight =12) vs NNVC anchor.

|  | Proposed HOP (luma weight 12) vs NNVC Anchor |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -2.84% | -12.22% | -14.05% | -5.98% | -14.35% | -19.52% |
| Class A2 | -3.15% | -12.00% | -10.27% | -6.03% | -16.38% | -13.72% |
| Class B | -3.15% | -11.88% | -13.04% | -5.39% | -16.48% | -16.87% |
| Class C | -3.90% | -10.65% | -13.14% | -5.90% | -16.77% | -17.35% |
| Class E | -4.77% | -14.52% | -16.43% |  |  |  |
| Overall | -3.53% | -12.12% | -13.33% | -5.77% | -16.11% | -16.90% |
| Class D | -3.23% | -11.40% | -13.90% | -6.29% | -16.87% | -17.89% |

Summary of the inference test results comparing to the Anchor HOP training strategy are shown in Table 4.

Table 4. Proposed training strategy (luma weight =12) vs HOP anchor.

|  | Proposed HOP (luma weight 12) vs Anchor HOP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0.27% | 4.17% | 4.95% | -0.59% | 4.46% | 3.29% |
| Class A2 | -0.03% | 4.11% | 4.08% | -0.34% | 3.74% | 6.76% |
| Class B | -0.28% | 3.15% | 2.17% | -0.34% | 4.39% | 2.57% |
| Class C | -0.28% | 2.49% | 2.71% | -0.40% | 3.50% | 3.88% |
| Class E | -0.26% | 2.91% | 2.32% |  |  |  |
| Overall | -0.23% | 3.30% | 3.10% | -0.41% | 4.04% | 3.90% |
| Class D | -0.26% | 2.60% | 2.76% | -0.48% | 3.34% | 2.75% |

Simulation results for additional test, component_loss_weightings = {24,2} are shown in Table 5.

Table 5. Proposed training strategy (luma weight =24) vs HOP anchor.

|  | Proposed HOP (luma weight 24) vs Anchor HOP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | Random Access |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -0.18% | 10.67% | 10.78% | -0.49% | 11.74% | 11.65% |
| Class A2 | -0.18% | 8.68% | 8.09% | -0.49% | 11.19% | 12.50% |
| Class B | -0.45% | 8.01% | 7.23% | -0.64% | 12.07% | 9.93% |
| Class C | -0.45% | 7.43% | 8.51% | -0.65% | 11.65% | 12.42% |
| Class E | -0.46% | 7.80% | 9.13% |  |  |  |
| Overall | -0.36% | 8.40% | 8.56% | -0.58% | 11.72% | 11.45% |
| Class D | -0.39% | 8.81% | 9.09% | -0.57% | 12.97% | 11.70% |

# Conclusion

Test results for stage 3 training of HOP reported in JVET-AF00141 indicated a noticeable disbalance in coding gain for luma and chroma components. HOP bd-rate gain over the NNVC anchor is {-5.4%, -19.2%, -19.9%} in RA and {-3.3, -14.7, -15.8} in AI is reported. This contribution proposes a change to the HOP training strategy, namely by increasing training loss weight for the luma component by 2x, similarly to the training strategy adjustment utilized in EE1-0 LOP2 training. HOP with proposed training provides {-5.8%, -16.1%, -16.9%} and {-3.5%, -12.1%, -13.3%} bd-rate gain vs NNVC anchor in RA and AI configurations, respectively. It is proposed to modify HOP training by increasing luma training loss weight to 12, which would align training between HOP and LOP filters.

# References

- BoG report on NN-filter design unification, JVET-AD0380, E. Alshina, F. Galpin, April 2023

- AhG11: EE1-0 High Operation Point model, F. Galpin (InterDigital), S. Eadie, D. Rusanovskyy, Y. Li (Qualcomm), Y. Li, J. Li (ByteDance), L. Wang, R. Chang (Tencent), Z. Xie (Oppo), E. Alshina (Huawei), JVET-AE0191, 2023.

- AhG11: HOP full results, F. Galpin (InterDigital), D. Rusanovskyy, Y. Li (Qualcomm), Y. Li, J. Li (ByteDance), L. Wang, R. Chang (Tencent), Z. Xie (Oppo), E. Alshina (Huawei), JVET-AF0043, 2023.

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-Z2016, E. Alshina, R.-L. Liao, S. Liu, A. Segall.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).