| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0184-v1 |
| --- | --- |

| Title: | Non-EE2: Search range optimization for Intra TMP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gleb Verba, Zhi Zhang, Vadim Seregin, Marta Karczewicz | Tel: Email: | gverba@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to extend Intra TMP search area by enlarging the existed rectangular search area, adding horizontal and vertical oriented areas relative to the current block to the left and above of the rectangular search area, and adding areas around BV candidates. The experimental results on top of ECM-10.0 for SCC classes are reported as follows:

AI: Overall: {},
Class F: {-0.48%, -0.55%, -0.48%, 102.1%, 	109.4%}, 
Class TGM: {-0.53%, -0.52%, -0.43%, 102.8%, 115.1%}

RA: Overall: {}
Class F: {}, 
Class TGM: {}

# Introduction

In ECM-10.0, Intra TMP uses a rectangular search area with SearchRangeW and SearchRangeH defined as follows:

SearchRangeW=maxa⋅BlkW,minSR,

SearchRangeH=maxa⋅BlkH,minSR,
where a=5,  minSR=64.

JVET-AE0166 and JVET-AE0198 propose to extend the search area to match the IBC search area.

# Proposed method

This contribution proposes a method to extend Intra TMP search area as follows:

- The rectangular search area size is extended to a square with a side depending on a greater block dimension, with a restriction on the maximum size of the search area:
SearchRangeW=SearchRangeH=clipa⋅maxBlkW,BlkH,minSR;maxSR,
where parameter a is increased to 8 and maxSR=256.

- Horizontal and vertical oriented search areas are added beyond the existed search area relative to the current blocks. Those two search areas are defined as follows:

  - Horizontal area: -512;-SearchRangeW-1×-Ty;Ty

  - Vertical area: -Tx;Tx ×-256;-SearchRangeH-1,

where Tx=minblkW,8-1,Ty=minblkH,8-1.

- Regions around candidates derived using IBC merge process are added to the search area, dX and dY offsets not exceeding 32 in any direction are added to the BV candidates.

# Simulation results

The proposed method was implemented on top of ECM-10.0 and tested following common test conditions described in JVET-AE2017 for AI and RA configurations. Experimental results are summarized below:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.48% | -0.55% | -0.48% | 102.1% | 109.4% |
| Class TGM | -0.53% | -0.52% | -0.43% | 102.8% | 115.1% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).