| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0185-v2 |
| --- | --- |

| Title: | Non-EE2: Transform Coefficient Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pavel Nikitin, Marta Karczewicz, Muhammed Coban, Vadim Seregin | Tel: Email: | pnikitin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

# Abstract

This contribution proposes a change to the number of regular coded bins per coefficient in transform residual coefficient coding. It is reported that on top of ECM-10.0, the overall coding performance impact for {Y, U, V}  in AI, RA configurations.

AI {-0.09% Y, -0.04% U, -0.13% V, 100.7% EncT, 100.0% DecT, 99.9% VmPeak}

RA {-0.07% Y, -0.02% U, -0.04% V, 99.4% EncT, 98.5% DecT, 99.9% VmPeak}

# Introduction

In ECM and VVC residual coding, regular coded absolute coefficient levels are represented by sig_coeff_flag (sig), abs_level_gtx_flag[0] (gt1), par_level_flag (par), abs_level_gtx_flag[1] (gt3) syntax elements along with bypass coded abs_remainder (remLevel).  Associated coefficient level is reconstructed in ECM in the following way:

abs_coeff_level = sig + gt1 + par + 2*(gt3 + remLevel)

# Proposed method

This contribution proposes to change the number gtX flags coded in the regular mode of the arithmetic coding using adaptive context models before remainder coding. There are N+1 flags coded in the regular mode: sig_coeff_flag, gt1_flag, gt2_flag, gt3_flag, …, gtN_flag and parity_flag. Parity flag is coded after gtN_flag. In the experiments N was set to 8. Absolute coefficient level is reconstructed as follows:

abs_coeff_level = sig +gt1+ gt2 +gt3 +… + gtN + par + 2*remLevel,

The grouping of regular coded bins and by bypass coded bins is maintained.

# Simulation results

The proposed method was implemented on top of ECM-10.0 and tested following common test conditions for AI and RA configurations described in JVET-AE2017. Experimental results are summarized below:

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.10% | -0.17% | -0.17% | 100.2% | 99.5% | 99.8% |
| Class A2 | -0.13% | -0.02% | -0.06% | 100.7% | 100.1% | 99.9% |
| Class B | -0.11% | 0.05% | -0.15% | 100.7% | 100.6% | 99.9% |
| Class C | -0.10% | -0.14% | -0.06% | 100.9% | 99.5% | 100.0% |
| Class E | -0.03% | 0.05% | -0.23% | 101.0% | 100.1% | 100.0% |
| Overall | -0.09% | -0.04% | -0.13% | 100.7% | 100.0% | 99.9% |
| Class D | -0.08% | -0.08% | -0.09% | 100.3% | 100.1% | 100.0% |
| Class F | -0.04% | 0.01% | 0.07% | 99.2% | 100.4% | 100.1% |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.09% | -0.17% | -0.04% | 101.4% | 100.2% | 99.6% |
| Class A2 | -0.05% | -0.18% | -0.02% | 100.7% | 99.0% | 99.9% |
| Class B | -0.08% | 0.14% | 0.02% | 99.7% | 99.1% | 100.1% |
| Class C | -0.03% | 0.00% | -0.13% | 96.6% | 96.1% | 100.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.07% | -0.02% | -0.04% | 99.4% | 98.5% | 99.9% |
| Class D | -0.04% | 0.02% | -0.22% | 98.4% | 98.0% | 100.0% |
| Class F | -0.01% | 0.25% | 0.14% | 100.5% | 100.0% | 100.0% |

# 4 Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).