| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32th Meeting, Hanover, DE, 13–20 October 2023 | Document: JVET-AF0187-v2 |
| --- | --- |

| Title: | Compression of Gaming Contents |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zehui Lin Kangying Cai Yin Zhao Johannes Sauer Elena Alshina | Tel: Email: | caikangying@huawei.com |
| Source: | Huawei Technologies |  |  |

_____________________________

# Abstract

With the widely adoption of cloud gaming, efficient compression of gaming content becomes more and more important. The characteristics of gaming content are quite different from those of the TGM sequences. Performance on several gaming sequences for ECM, VTM and HM are tested. This contribution proposes to extend test condition by adding gaming contents and develop specific compression solution for gaming contents.

# Introduction

Cloud gaming is widely adopted by industry. Examples include Xbox cloud gaming, NVidia’s GeForce Now, Tencent cloud gaming and Genshin Impact Cloud from MIHOYO. Efficient compression of gaming sequences is one of the key factors for the success of cloud gaming. Nowadays, typical bitrates for high quality distribution of gaming sequences could go up to 8Mbps~20Mbps, which is much higher than natural video sequences. Therefore, it is necessary to improve the compression performance of gaming contents. Requirements for gaming sequence coding comprise

- Resolution: 8K, 4K, 1080p, 720p

- Frame rate: 120fps, 90fps, 60fps, 30fps

- Low latency/low complexity: encoding latency < 5ms, decoding latency < 5ms

- HDR (optional)

- 360 (for some games)

# Test dataset and compression performance

## Test dataset

It is lacking of gaming contents in current JVET test dataset. As show in Figure 1, typical gaming sequence is very different from the TGM class in JVET.

Moreover, as mentioned by JVET-Y0041 [1], gaming sequences usually accompanied by auxiliary information such as depth, albedo and motion vector, which comes from gaming engine and could be used to improve the compression efficiency. Those auxiliary information should also be provided with the corresponding test contents.

- (b)

Figure 1 (a) TGM class in JVET. (b) Typical gaming content: Genshin Impact CREDIT MIHOYO. Source: https://www.forbes.com/sites/davidthier/2020/10/05/genshin-impact-feels-destined-to-change-the-gaming-industry/?sh=5bfb2d4f68b0

## Compression performance of available codecs and evidence

Figure 3 shows the compression performance of HM, VTM and ECM over one gaming-like sequence rendered using the 3D asset “West Desert House” as illustrated by Figure 2. Although ECM could achieve significant gain over VTM and HM, its much longer coding time makes it unsuitable for cloud gaming scenario. One coding tool designed for gaming content, which takes advantage of those auxiliary information provided by game engine, has been developed and shows the similar gain with ECM over VTM, as shown in Figure 3 (VTM+GamCodingTool).

Figure 2 West Desert Town, https://www.unrealengine.com/marketplace/en-US/product/western-desert-town

Figure 3 Compression performance (low delay P mode) over one 1080p/60fps sequence rendered using UE v4.27 and the 3D asset “West Desert Town”.

# Recommendation

To meet the requirements from cloud gaming industry, it is also recommended to develop specific compression solutions for gaming content, preferably with minimal changes to VVC standard, targeting to extended functionality of VVC.

Given insufficient gaming content sequences in JVET test dataset, it is recommended to extend test condition by adding sufficient gaming sequences. At least one type of auxiliary information listed below, which could be provided by game engine, should also be provided along with the gaming contents

- Camera information (4x4 array, 32bit)

- Depth map (per-pixel, 32bit)

- Motion vector (per-pixel, 32bit)

Any other auxiliary information can be retrieved from the rendering pipeline is also welcomed.

# References

[1] G. Martin-Cocher, M.Badawi, T. Poirier, S. Puri, K. Nasser, “Proposed new class of sequences with depth and optical flow information”, JVET-Y0041, January 2022

# Patent rights declaration(s)

Huawei Technologies. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).