| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0190-v2 |
| --- | --- |

| Title: | EE2-Test4.1: Enabling template-based inter tools for the RPR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Hong-Jheng Jhu, C.-W. Kuo, Changyue Ma, Ning Yan, Wei Chen, Xianglin Wang | Email: | xiaoyuxiu@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of Exploration Experiment 2 Test-4.1 for enabling template-based inter tools for the RPR. There are two sub-tests in the EE test. In Test-4.1a, the template-based inter reordering tools, including ARMC, MMVD/affine MMVD reordering, template-based BCW derivation, reference picture list reordering and MVD sign and magnitude-suffix prediction, are enabled for scaled reference pictures in the RPR. Test-4.1b is on top of Test-4.1a, which additionally enables the LIC when any of reference pictures is in different resolution from the current picture. The simulation results are summarized as follows:

|  | Low-delay B PSNR1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | VmPeak |
| 4.1a 1.5x | -0.72% | -0.36% | -0.60% | 101.1% | 102.0% | 100.1% |
| 4.1b 1.5x | -0.94% | -0.63% | -0.85% | 102.2% | 102.9% | 100.1% |
| 4.1a 2.0x | -0.70% | -0.40% | -0.19% | 102.4% | 102.4% | 100.1% |
| 4.1b 2.0x | -0.79% | -0.70% | -0.81% | 102.9% | 103.0% | 100.0% |
|  | Low-delay B PSNR2 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| 4.1a 1.5x | -0.71% | -0.42% | -0.64% | 101.1% | 102.0% | 100.1% |
| 4.1b 1.5x | -0.92% | -0.69% | -0.93% | 102.2% | 102.9% | 100.1% |
| 4.1a 2.0x | -0.63% | -0.38% | -0.21% | 102.4% | 102.4% | 100.1% |
| 4.1b 2.0x | -0.69% | -0.71% | -0.82% | 102.9% | 103.0% | 100.0% |

Due to the fact that the contribution proposes to enable the existing coding modules in the ECM for the RPR, it is asserted that there is essentially no extra hardware cost to apply the proposed methods.

# Introduction

At JVET-AE meeting, JVET-AE0153 [1] was proposed to improve the efficiency of the RPR and was included in Exploration Experiment 2 (EE2) as Test-4.1 [2] for further evaluation. There are two sub-tests in the EE test. In Test-4.1a, the template-based inter reordering tools, including ARMC, MMVD/affine MMVD reordering, template-based BCW derivation, reference picture list reordering and MVD sign and magnitude-suffix prediction, are enabled for scaled reference pictures in the RPR. Test-4.1b is on top of Test-4.1b. Additionally, in Test-4.1b, the LIC is enabled when any of reference pictures is in different resolution to the current picture.

# Simulation results

The proposed scheme was implemented on top of ECM-10.0 [3] and tested using the common test conditions as defined in [4]. Table 1 and Table 2 show the RPR-1.5x results of Test-4.1a and Test-4.1b for the LDB configuration. Table 3 and Table 4 show the RPR-2.0x results of Test-4.1a and Test-4.1b for the LDB configuration.

Table 1 The RPR-1.5x results of Test-4.1a for the LDB

|  | Low-delay B PSNR1 |  |  |  |  |  | Low-delay B PSNR2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Y | U | V | EncT | DecT | VmPeak |
| Class B | -0.64% | 0.19% | -0.73% | 102.8% | 102.9% | 99.8% | -0.63% | 0.18% | -0.75% | 102.8% | 102.9% | 99.8% |
| Class C | -0.88% | -0.99% | -0.95% | 100.3% | 103.0% | 100.3% | -0.86% | -1.10% | -1.03% | 100.3% | 103.0% | 100.3% |
| Class E | -0.62% | -0.43% | 0.09% | 99.2% | 99.1% | 100.2% | -0.64% | -0.52% | 0.06% | 99.2% | 99.1% | 100.2% |
| Overall | -0.72% | -0.36% | -0.60% | 101.1% | 102.0% | 100.1% | -0.71% | -0.42% | -0.64% | 101.1% | 102.0% | 100.1% |
| Class D | -0.48% | 0.26% | 0.13% | 101.5% | 100.9% | 99.7% | -0.49% | 0.23% | 0.15% | 101.5% | 100.9% | 99.7% |
| Class F |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2 The RPR-1.5x results of Test-4.1b for the LDB

|  | Low-delay B PSNR1 |  |  |  |  |  | Low-delay B PSNR2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Y | U | V | EncT | DecT | VmPeak |
| Class B | -0.85% | -0.51% | -0.98% | 102.6% | 103.3% | 100.2% | -0.81% | -0.53% | -1.05% | 102.6% | 103.3% | 100.2% |
| Class C | -1.10% | -1.18% | -1.23% | 101.5% | 102.0% | 100.0% | -1.07% | -1.28% | -1.34% | 101.5% | 102.0% | 100.0% |
| Class E | -0.87% | -0.09% | -0.10% | 102.3% | 103.4% | 99.8% | -0.92% | -0.14% | -0.18% | 102.3% | 103.4% | 99.8% |
| Overall | -0.94% | -0.63% | -0.85% | 102.2% | 102.9% | 100.1% | -0.92% | -0.69% | -0.93% | 102.2% | 102.9% | 100.1% |
| Class D | -0.61% | 0.03% | -0.01% | 101.7% | 103.3% | 100.2% | -0.58% | 0.06% | -0.13% | 101.7% | 103.3% | 100.2% |
| Class F | -1.11% | -1.03% | -0.76% | 101.4% | 102.5% | 100.3% | -1.01% | -0.77% | -0.70% | 101.4% | 102.5% | 100.3% |

Table 3 The RPR-2.0x results of Test-4.1a for the LDB

|  |  | Low-delay B PSNR1 |  |  |  |  |  |  |  |  |  |  |  | Low-delay B PSNR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over ECM-10.0 |  |  |  |  |  |  |  |  |  |  |  | Over ECM-10.0 |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |  | VmPeak |  | Y |  | U |  | V | EncT |  | DecT |  | VmPeak |
| Class B |  | -0.62% |  | -0.19% |  | 0.06% |  | 102.8% |  | 102.4% |  | 100.1% |  | -0.58% |  | -0.15% |  | 0.06% | 102.8% |  | 102.4% |  | 100.1% |
| Class C |  | -0.87% |  | -0.74% |  | -0.58% |  | 102.1% |  | 103.0% |  | 100.2% |  | -0.76% |  | -0.57% |  | -0.47% | 102.1% |  | 103.0% |  | 100.2% |
| Class E |  | -0.59% |  | -0.28% |  | -0.08% |  | 102.3% |  | 101.7% |  | 100.0% |  | -0.53% |  | -0.53% |  | -0.31% | 102.3% |  | 101.7% |  | 100.0% |
| Overall | -0.70% | -0.40% | -0.19% | 102.4% | 102.4% | 100.1% | -0.63% | -0.38% | -0.21% | 102.4% | 102.4% | 100.1% |
| Class D |  | -0.57% |  | 0.01% |  | -0.04% |  | 101.0% |  | 101.7% |  | 99.8% |  | -0.40% |  | 0.06% |  | -0.02% | 101.0% |  | 101.7% |  | 99.8% |
| Class F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4 The RPR-2.0x results of Test-4.1b for the LDB

|  | Low-delay B PSNR1 |  |  |  |  |  | Low-delay B PSNR2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Y | U | V | EncT | DecT | VmPeak |
| Class B | -0.80% | -1.01% | -0.96% | 102.6% | 103.9% | 100.0% | -0.71% | -1.02% | -0.99% | 102.6% | 103.9% | 100.0% |
| Class C | -0.91% | -0.75% | -0.17% | 103.2% | 103.2% | 100.3% | -0.77% | -0.70% | -0.08% | 103.2% | 103.2% | 100.3% |
| Class E | -0.62% | -0.11% | -1.40% | 103.1% | 101.2% | 99.7% | -0.57% | -0.21% | -1.53% | 103.1% | 101.2% | 99.7% |
| Overall | -0.79% | -0.70% | -0.81% | 102.9% | 103.0% | 100.0% | -0.69% | -0.71% | -0.82% | 102.9% | 103.0% | 100.0% |
| Class D | -0.51% | 0.29% | -0.22% | 103.9% | 101.1% | 100.4% | -0.41% | 0.44% | 0.05% | 103.9% | 101.1% | 100.4% |
| Class F |  |  |  |  |  |  |  |  |  |  |  |  |

In the below, additional results are provided for Test-4.1a and Test-4.1b in the RA configuration.

Table 5 The RPR-1.5x results of Test-4.1a for the RA

|  | RA PSNR1 |  |  |  |  |  | RA PSNR2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Y | U | V | EncT | DecT | VmPeak |
| Class B | -1.20% | -0.94% | -1.03% | 100.9% | 102.0% | 100.2% | -1.13% | -0.96% | -1.04% | 100.9% | 102.0% | 100.2% |
| Class C | -1.10% | -1.27% | -1.03% | 101.1% | 101.8% | 99.8% | -0.98% | -1.21% | -0.95% | 101.1% | 101.8% | 99.8% |
| Overall | -1.16% | -1.09% | -1.03% | 101.00% | 101.9% | 100.0% | -1.07% | -1.07% | -1.00% | 101.00% | 101.9% | 100.0% |
| Class D | -0.95% | -0.51% | -0.96% | 101.1% | 102.0% | 99.9% | -0.68% | -0.50% | -0.91% | 101.1% | 102.0% | 99.9% |
| Class F | -0.99% | -0.74% | -0.47% | 102.1% | 101.7% | 99.9% | -0.79% | -0.75% | -0.41% | 102.1% | 101.7% | 99.9% |

Table 6 The RPR-1.5x results of Test-4.1b for the RA

|  | RA PSNR1 |  |  |  |  |  | RA PSNR2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Y | U | V | EncT | DecT | VmPeak |
| Class B | -1.57% | -1.04% | -1.05% | 102.1% | 103.0% | 99.8% | -1.44% | -1.05% | -1.09% | 102.1% | 103.0% | 99.8% |
| Class C | -1.32% | -1.41% | -1.10% | 102.2% | 102.9% | 100.1% | -1.13% | -1.31% | -0.99% | 102.2% | 102.9% | 100.1% |
| Overall | -1.46% | -1.20% | -1.07% | 102.2% | 102.3% | 99.9% | -1.30% | -1.17% | -1.04% | 102.2% | 102.3% | 99.9% |
| Class D | -1.11% | -1.10% | -1.04% | 102.2% | 102.1% | 99.7% | -0.79% | -1.10% | -0.95% | 102.2% | 102.1% | 99.7% |
| Class F | -1.34% | -1.10% | -1.10% | 103.1% | 104.2% | 100.4% | -1.00% | -1.07% | -1.05% | 103.1% | 104.2% | 100.4% |

Table 7 The RPR-2.0x results of Test-4.1a for the RA

|  | RA PSNR1 |  |  |  |  |  | RA PSNR2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Y | U | V | EncT | DecT | VmPeak |
| Class B | -1.25% | -0.97% | -1.08% | 100.4% | 102.0% | 100.5% | -1.01% | -0.80% | -0.89% | 100.4% | 102.0% | 100.5% |
| Class C | -1.08% | -1.33% | -1.32% | 101.1% | 102.1% | 100.3% | -0.83% | -1.16% | -1.18% | 101.1% | 102.1% | 100.3% |
| Overall | -1.17% | -1.13% | -1.19% | 100.7% | 102.1% | 100.5% | -0.93% | -0.96% | -1.02% | 100.7% | 102.1% | 100.5% |
| Class D | -1.01% | -0.85% | -0.90% | 99.6% | 103.0% | 99.9% | -0.60% | -0.70% | -0.69% | 99.6% | 103.0% | 99.9% |
| Class F | -0.95% | -0.85% | -0.80% | 101.0% | 104.4% | 100.5% | -0.72% | -0.48% | -0.43% | 101.0% | 104.4% | 100.5% |

Table 8 The RPR-2.0x results of Test-4.1b for the RA

|  | RA PSNR1 |  |  |  |  |  | RA PSNR2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | Y | U | V | EncT | DecT | VmPeak |
| Class B | -1.56% | -0.98% | -1.01% | 102.1% | 102.8% | 100.2% | -1.31% | -0.81% | -0.89% | 102.1% | 102.8% | 100.2% |
| Class C | -1.23% | -1.31% | -1.06% | 100.9% | 102.9% | 100.2% | -0.94% | -1.12% | -0.88% | 100.9% | 102.9% | 100.2% |
| Overall | -1.41% | -1.12% | -1.04% | 101.6% | 102.8% | 100.2% | -1.14% | -0.95% | -0.88% | 101.6% | 102.8% | 100.2% |
| Class D | -1.18% | -0.94% | -1.11% | 100.6% | 103.9% | 100.3% | -0.73% | -0.86% | -0.88% | 100.6% | 103.9% | 100.3% |
| Class F | -1.22% | -1.21% | -1.11% | 101.1% | 103.9% | 100.2% | -0.97% | -0.76% | -0.67% | 101.1% | 103.9% | 100.2% |

# Conclusion

The contribution presents the results of EE2-Test-4.1. Simulation results show that the tests provide promising coding performance improvements with reasonable enc/dec run-time impacts. Additionally, because the contribution proposes to enable the existing ECM tools in the RPR scenario, there is essentially no hardware cost to apply the proposed methods. It is recommended to adopt the proposed methods into ECM.

# Reference

- X. Xiu, H.-J. Jhu, C.-W. Kuo, C. Ma, N. Yan, W. Chen, X. Wang, “Non-EE2: Enabling template-based inter tools for scaled reference pictures in the RPR”, JVET-AE0153, July 2023.

- V. Seregin, J. Chen, L. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-AE2024, July 2023.

- ECM-10.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AE2017, July 2023.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).