| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0192-v2 |
| --- | --- |

| Title: | EE1-1.1.5: Combination test of EE1-1.1.1 and EE1-1.1.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Renjie Chang Liqiang Wang Xiaozhong Xu Shan Liu Dmytro Rusanovskyy Yun Li Marta Karczewicz | Email: | renjiechang@tencent.com liqiangwang@tencent.com xiaozhongxu@tencent.com shanl@tencent.com dmytror@qti.qualcomm.com yli30@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Tencent, Qualcomm |  |  |

_____________________________

- Abstract

This contribution reports the combination test results of EE1-1.1.1 and EE1-1.1.2. This contribution combines two optimizations to improve the complexity-performance trade-off of HOP network. Specifically, a new residual block with group convolution is proposed in EE1-1.1.1 to reduce the computational complexity without affecting the coding performance. In EE1-1.1.2, reduced ranking of the two-components decomposition is introduced and order of the decomposition is alternated every block in the backbone to mitigate the impact of the rank reduction on decomposition symmetry. Joint design was trained independently by Tencent and Qualcomm. Based on NNVC-6.0, the proposed method can achieve 18% kMAC/pixel reduction with only 0.3% luma loss for RA configuration. The test results are shown in order of RA and AI configurations as follows.

The propose method provide the following results, comparing to NNVC-6.0 HOP anchor (nnIntra ON):

RA: 0.32%	1.53%	   1.52%	EncT: 72%	DecT: 56%

AI : 0.14%	0.43%	   0.80%	EncT: 77%	DecT: 53%

Introduction

This contribution is a combination test of EE1-1.1.1 and EE1-1.1.2. The improved residual block with group convolution from EE1-1.1.1 and reduced ranking of the two-components decomposition and residual block type alternation from EE1-1.1.2 are integrated based on HOP network.

Proposed method

This combination test is firstly presented in Section 2.1. Then, the network information in both training stage and inference stage is provided in Section 2.2 and Section 2.3, respectively.

Network architecture

The overall network architecture of EE1-1.1.5 is shown in Fig. 1. Specifically, the C31 parameter of unified HOP filter is modified from 4*16 to 3*16. To alleviate introduced decomposition asymmetry with reduced rank being used in horizontal direction only, two types of the residue block in backbone are introduced. Besides, the ordinary convolutions in the first 3x1 and 1x3 convolution are replaced by the group convolution. g in convolution layers denotes group convolution and the group numbers are set as 2.

Fig. 1: Network architecture of EE1-1.1.5

Network information in training stage

All these models are trained based on training datasets and training scripts of HOP anchor model which are defined in JVET-AD0380. The network training details are shown as follows.

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Tesla A100 40GB |
|  | Framework: | Pytorch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 20 |
|  | Batch size: | 64 |
|  | Loss function: | L1, L2 |
|  | Training time (for 1 model): | ~11 days |
|  | Training data information: | DIV2K, TVD, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations | 1200 |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Learning rate update strategy |  |
|  | Optimizer: | ADAM |
|  | Preprocessing: | Data augmentation |
|  | Mini-batch selection process: |  |
|  | Training data update strategy: |  |
|  | Other information: |  |

Network information in inference stage

In these EE tests, SADL [2] is adopted as the inference library and all the models are quantized into int16 precision. The network inference details are shown as follows.

| - Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 1.2M |
|  | Total Number of Parameters (All Models) | 1.2M |
|  | Parameter Precision (Bits) | int16 |
|  | Memory Parameter (MB) | 2.4 MB |
|  | Multiply Accumulate (kMAC/pixel) | 392 (block-level) |
|  | Calculation Method | block basis |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144x144, 272x272×× |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Results

All the experiments are performed based on the common test condition in [3]. The test results of EE1-1.1.5 are shown as follows.

Table 1. RA Performance of EE1-1.1.5

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 HOP anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.06% | 0.72% | 0.61% | 70% | 55% |
| Class A2 | 0.41% | 2.04% | 5.16% | 72% | 56% |
| Class B | 0.26% | 1.51% | 0.25% | 73% | 58% |
| Class C | 0.51% | 1.77% | 1.07% | 70% | 54% |
| Class E |  |  |  |  |  |
| Overall | 0.32% | 1.53% | 1.52% | 72% | 56% |
| Class D | 0.48% | 0.34% | -0.35% | 73% | 56% |
| Class F | 0.22% | 0.71% | 0.36% |  |  |

Table 2. AI Performance of EE1-1.1.5

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 HOP anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.18% | 0.53% | 1.18% | 76% | 57% |
| Class A2 | 0.28% | 1.24% | 1.92% | 80% | 56% |
| Class B | 0.05% | 0.08% | 0.14% | 82% | 56% |
| Class C | 0.14% | 0.27% | 0.29% | 78% | 49% |
| Class E | 0.10% | 0.32% | 1.07% | 69% | 49% |
| Overall | 0.14% | 0.43% | 0.80% | 77% | 53% |
| Class D | 0.10% | 1.11% | 1.47% | 82% | 50% |
| Class F | -0.17% | -0.06% | 0.06% |  |  |

Conclusions

In this contribution, two optimizations from EE1-1.1.1 and EE1-1.1.5 are merged based on unified HOP network to further improve the trade-off of NNLF. Specifically, group convolution, reduced C31 parameter and switched decomposition order are integrated into HOP network. Experimental results demonstrate the effectiveness of proposed methods.

References

- E. Alshina, F. Galpin. BoG report on NN-filter design unification. JVET-AD0380, 30th Meeting, Antalya, TR, 21–28 April 2023.

- F. Galpin, T. Dumas, P. Bordes, E. François. AHG11: Small Ad-hoc Deep-Learning Library (SADL) update. JVET-AB0086, 28th Meeting, Mainz, DE, 20–28 October 2022.

- E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AE2016, 31st Meeting, Geneva, CH, 11–19 July 2023.

Patent rights declaration(s)

Tencent and Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).