| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0193-v2 |
| --- | --- |

| Title: | AHG11: Decoder complexity optimization for NNVC in-loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Renjie Chang Liqiang Wang Xiaozhong Xu Shan Liu | Email: | renjiechang@tencent.com liqiangwang@tencent.com xiaozhongxu@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, an encoder-only coding tool is proposed to optimize the decoder complexity of NNVC in-loop filter. In the proposed method, the filter gain of each CTU is first calculated if the block-level NNLF on/off is enabled. Then, NNLF will be disabled for the CTUs with small filter gains.

Based on NNVC-6.0, the test results are shown in order of RA and LDB as follows.

Compared with NNVC-6.0 HOP anchor (NnIntra + HOP):

RA:  0.05%	0.03%	0.01%					EncT: 98%	DecT: 91%

LDB: 0.13%	-0.72%	-1.19%					EncT: 99%	DecT: 86%

Compared with NNVC-6.0 LOP anchor (NnIntra + LOP):

RA:  0.06%	0.01%	  0.04%		EncT: 100%	DecT: 87%

LDB: 0.09%	-0.71%	  -1.28%		EncT: 100%	DecT: 81%

Introduction

In NNVC-6.0, the block-level on/off is used for NNLF including HOP filter [1] and LOP filter. When the block-level NNLF on/off is enabled, NNLF will be enabled for only partial CTUs in one frame. Since different CTUs filtered by NNLF usually have different filter gains, inspired by [2], an encoder-only coding tool for decoder complexity optimization is proposed to skip CTUs with small filter gains.

Proposed method

Although the processing time of NNLF is almost the same on each CTU, the filter gain of each CTU is very different. Fig. 1 shows the gain distribution of each CTU in one frame (The filter gain gradually increases from left to right). The filter gain is measured using Euclidean distance and calculated as follows.

DCTU=(INNLF-IORG)2-(IREC-IORG)2

Specifically, INNLF denotes the filter result of NNLF, IREC denotes the reconstruction result and IORG denotes the original CTU without compression. DCTU denotes the filter gain of current CTU. DCTU<0 means some filter gains can be achieved by using NNLF. The larger the absolute value of DCTU, the greater the filter gain. Based on Fig. 1, it can be observed that the filter gain on many CTUs is relatively small.

Fig. 1 The gain distribution of each CTU in one frame

Therefore, to optimize the decoder complexity, an encoder-only tool is proposed in this contribution to adaptively disable NNLF at the CTU level according to filter gain. The algorithm process is show as follows.

- Check the block-level NNLF on/off is enabled.

- Calculate the filter gain of each CTU: DCTU=(INNLF-IORG)2-(IREC-IORG)2.

- Sort the CTUs with DCTU<0 from small to large filter gains.

- Calculate the gain ratio of each CTU: RCTU=DCTU/DCTUMAX, where DCTUMAX= -(IREC-IORG)2.

- Compare all RCTU with r. If RCTU< r, disable NNLF for current CTU. Specifically, r is a predefined threshold.

Inference stage

The proposed method is used for both HOP filter and LOP filter. The inference information is identical to NNVC-6.0 HOP filter and LOP filter.

| - Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
| Mandatory | HW environment: |  |  |
|  | GPU Type | CPU only |  |
|  | Framework: | SADL |  |
|  | Number of GPUs per Task | 0 |  |
|  | Number of Parameters (Each Model) | HOP: 1.45 M/model LOP: |  |
|  | Total Number of Parameters (All Models) | HOP: 1.45 M/model LOP: |  |
|  | Parameter Precision (Bits) | 16 |  |
|  | Memory Parameter (MB) | HOP: 2.8 MB/model LOP: 130 K/model |  |
|  | Multiply Accumulate (kMAC/pixel) | HOP: 477, LOP: 17.0 |  |
|  | Calculation Method | block basis |  |
| Optional |  |  |  |
|  | Total Conv. Layers |  |  |
|  | Total FC Layers | 0 |  |
|  | Total Memory (MB) |  |  |
|  | Batch size: | 1 |  |
|  | Patch size | 144x144, 272x272 |  |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage (Total) |  |  |
|  | Peak Memory Usage (per Model) |  |  |
|  | Border handling |  |  |
|  | Other information: |  |  |

Results

All the experiments are performed based on the common test conditions in [3]. The proposed method is implemented on top of NNVC-6.0 unified HOP and LOP filter. Specifically, the used unified LOP filter mode is from Qualcomm in JVET-AE-EE1-0 [4]. The results are shown as follows.

Compared with NNVC-6.0 HOP anchor (NNIntra + HOP)

Table 1 RA Performance of proposed method

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0_NNIntra+HOP |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.02% | 0.04% | 0.06% | 98% | 92% |
| Class A2 | 0.08% | 0.12% | 0.18% | 98% | 85% |
| Class B | 0.07% | -0.01% | -0.09% | 98% | 89% |
| Class C | 0.02% | -0.01% | -0.03% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.03% | 0.01% | 98% | 91% |
| Class D | 0.00% | -0.01% | -0.01% | 98% | 99% |
| Class F | -0.02% | 0.08% | 0.00% | 98% | 94% |

Table 2 LDB Performance of proposed method

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0_NNIntra+HOP |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.13% | -0.32% | -0.92% | 99% | 89% |
| Class C | -0.01% | -0.63% | -0.44% | 98% | 95% |
| Class E | 0.31% | -1.50% | -2.65% | 98% | 71% |
| Overall | 0.13% | -0.72% | -1.19% | 99% | 86% |
| Class D | 0.01% | -0.74% | -0.27% | 98% | 98% |
| Class F | -0.08% | -0.30% | -0.26% | 98% | 87% |

Compared with NNVC-6.0 LOP2 anchor (NNIntra + LOP2)

Table 3 RA Performance of proposed method

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0_NNIntra+LOP2 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.04% | 0.12% | 0.04% | 100% | 88% |
| Class A2 | 0.15% | 0.14% | 0.17% | 100% | 79% |
| Class B | 0.08% | -0.11% | 0.02% | 100% | 85% |
| Class C | -0.01% | -0.03% | -0.04% | 100% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.01% | 0.04% | 100% | 87% |
| Class D | 0.02% | -0.05% | 0.00% | 100% | 97% |
| Class F | -0.01% | 0.03% | 0.09% | 100% | 92% |

Table 4 LDB Performance of proposed method

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0_NNIntra+LOP2 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | -0.40% | -0.74% | 100% | 86% |
| Class C | -0.11% | -0.43% | -0.62% | 100% | 94% |
| Class E | 0.37% | -1.57% | -3.06% | 100% | 60% |
| Overall | 0.09% | -0.71% | -1.28% | 100% | 81% |
| Class D | 0.08% | -1.01% | -0.35% | 100% | 97% |
| Class F | -0.28% | -0.23% | -0.04% | 100% | 91% |

Conclusions

In this contribution, an encoder-only coding tool is designed to reduce the decoder complexity by disabling NNLF on some CTUs with small filter gains. Experimental results demonstrate the effectiveness of the proposed method.

References

- E. Alshina, F. Galpin. BoG report on NN-filter design unification. JVET-AD0380, 30th Meeting, Antalya, TR, 21–28 April 2023.

- H. Zhu, X. Xu, S. Liu. Decoder complexity optimization based on deep learning loop filtering tool. AVS-M5388, June 2020.

- E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AE2016, 31st Meeting, Geneva, CH, 11–19 July 2023.

- D. Rusanovskyy, Y. Li, T. Shao, P. Yin, J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya, J. Li, Y. Li, C. Lin, K. Zhang, L. Zhang, R. Chang, L. Wang, X. Xu, S. Liu. AhG11/EE1: Status of the joint EE1-0 (LOP.2) training. JVET-AF0043, 32nd Meeting, Hannover, DE, 13–20 October 2023.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).