| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0197_v1 |
| --- | --- |

| Title: | EE2-5.2: Luma Residual Taps in Chroma-ALF and CCALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Zhipin Deng1, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com zhipin.deng@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

In the Adaptive Loop Filter (ALF) design of ECM-10.0, the luma residual values are stored and used in the Luma-ALF. In EE2-5.2a, luma residual values are further introduced into Chroma-ALF. In EE2-5.2b, luma residual values are introduced to both Chroma-ALF and CCALF.

On top of ECM-10.0, simulation results of the proposed method are reported as below:

EE2-5.2a

AI:  0.00%, -0.01%, -0.01%, 100.0%, 100.1%;

RA: 0.01%, -0.10%, -0.24%, 99.9%, 99.8%;

LB: -0.04%, -0.10%, -0.32%, 99.5%, 99.7%

EE2-5.2b

AI:  0.00%, 0.07%, 0.05%, 100.0%, 99.7%;

RA: 0.01%, -0.31%, -0.47%, 99.8%, 99.9%;

LB: 0.01%, -2.03%, -1.91%, 99.8%, 100.0%

Introduction

In ALF of ECM-10.0, a Chroma-ALF filter uses a diamond 9x9 filter shape with 20 taps in total, as shown in figure 2-1. All these taps take the spatial chroma reconstruction as input.

Figure 2-1 The Currently Used Filter Shape of Chroma ALF.

In CCALF of ECM-10.0, a CCALF filter uses a cross-liked filter shape with 24 taps in total, as shown in figure 2-2. All these taps take the spatial luma reconstruction as input.

Figure 2-2 The Currently Used Filter Shape of CCALF.

Proposed Method

The proposed methods are shown in Figure 3-1 and Figure 3-2. All the spatial based taps in Chroma-ALF and CCALF are kept unchanged.

For Chroma-ALF, only one luma residual tap is added. It takes the down-sampled luma residual as input.

For CCALF, five luma residual taps in a cross 3x3 shape are added. The extended taps take the co-located and neighbouring luma residual values as input.

Figure 3-1 The Newly Introduced Filter Shape for Chroma ALF.

Figure 3-2 The Newly Introduced Filter Shape for CCALF.

In test 5.2a, only luma-residual based tap in Chroma-ALF is tested. In test 5.2b, the luma-residual based taps in Chroma-ALF and CCALF is tested.

Simulation Results

The proposed method was implemented on top of the ECM-10.0 [1]. Simulations were performed following the common test conditions [2]. More detailed results could be found in the attached excel sheets. The results of 5.2A and 5.2B are shown in Table 1 and Table 2, respectively.

Table 1 Results of 5.2a

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | -0.05% | 98.2% | 98.5% | 0.04% | -0.46% | -0.82% | 99.8% | 99.6% |  |  |  |  |  |
| Class A2 | 0.00% | -0.15% | 0.04% | 101.0% | 100.4% | 0.02% | -0.17% | -0.05 | 99.5% | 99.0% |  |  |  |  |  |
| Class B | 0.00% | -0.02% | -0.11% | 99.6% | 100.2% | -0.01% | 0.15% | -0.31% | 100.4% | 100.4% | 0.02% | 0.09% | -0.50% | 99.8% | 100.0% |
| Class C | 0.00% | 0.05% | 0.05% | 100.8% | 100.1% | 0.00% | -0.10% | 0.12% | 99.7% | 99.9% | -0.12% | -0.65% | -0.07% | 99.7% | 99.9% |
| Class E | 0.00% | 0.05% | 0.08% | 100.7% | 100.9% |  |  |  |  |  | -0.05% | 0.31% | -0.36% | 98.7% | 99.1% |
| Overall | 0.00% | -0.01% | -0.01% | 100.0% | 100.1% | 0.01% | -0.10% | -0.24% | 99.9% | 99.8% | -0.04% | -0.10% | -0.32% | 99.5% | 99.7% |
| Class D | 0.00% | 0.05% | 0.12% | 100.3% | 95.4% | 0.01% | 0.04% | 0.26% | 99.8% | 100.1% | -0.01% | 0.37% | -0.64% | 100.5% | 103.4% |
| Class F | 0.00% | 0.08% | 0.01% | 100.3% | 100.6% | 0.03% | -0.04% | -0.05% | 100.1% | 99.5% | 0.23% | 0.38% | 0.33% | 100.4% | 98.6% |

Table 2 Results of 5.2b

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | -0.04% | 98.8% | 99.1% | 0.06% | -0.62% | -1.20% | 99.4% | 100.0% |  |  |  |  |  |
| Class A2 | 0.01% | -0.14% | 0.03% | 100.2% | 100.5% | 0.04% | -0.46% | -0.74% | 99.4% | 99.2% |  |  |  |  |  |
| Class B | 0.01% | 0.04% | -0.07% | 99.6% | 101.0% | 0.01% | -0.32% | -0.46% | 100.4% | 99.9% | 0.06% | -2.23% | -2.04% | 98.9% | 99.5% |
| Class C | 0.00% | 0.18% | 0.14% | 100.7% | 98.2% | -0.03% | 0.04% | 0.24% | 99.7% | 100.4% | -0.07% | -1.45% | -0.89% | 100.5% | 100.1% |
| Class E | 0.00% | 0.23% | 0.26% | 100.9% | 99.6% |  |  |  |  |  | 0.03% | -2.45% | -3.04% | 100.4% | 100.6% |
| Overall | 0.00% | 0.07% | 0.05% | 100.0% | 99.7% | 0.01% | -0.31% | -0.47% | 99.8% | 99.9% | 0.01% | -2.03% | -1.91% | 99.8% | 100.0% |
| Class D | -0.01% | 0.27% | 0.39% | 100.1% | 97.0% | 0.02% | 0.34% | 0.40% | 99.7% | 99.7% | 0.01% | -1.83% | -0.53% | 100.1% | 101.8% |
| Class F | 0.00% | 0.14% | 0.08% | 100.5% | 101.6% | 0.03% | 0.06% | 0.04% | 99.8% | 99.1% | 0.24% | -1.35% | -0.70% | 101.0% | 105.1% |

Conclusions

The EE2-5.2 simply extend the usage of luma residuals into Chroma-ALF and CCALF. The proposed methods can achieve promising chroma coding performance with negligible encoding and decoding time increase. It is recommended to adopt EE2-5.2 into next version of ECM.

Reference

- ECM-10.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-10.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AE2017, CH, July 2023.

Patent Rights Declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).