| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0205-v1 |
| --- | --- |

| Title: | EE1-1.2.4: An improved inference design of NN-based loop-filters at high operation point |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junru Li, Yue Li, Chaoyi Lin, Kai Zhang, Li Zhang | Tel: Email: | lijunru@bytedance.com yue.li@bytedance.com linchaoyi.cy@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

- Abstract

This proposal reports the experimental results of EE1-1.2.4. It is proposed to enhance NN-based filter inference at the high operation point (HOP) by padding extended samples of inference block with zero value at picture boundary. Compared with NNVC-6.0 anchor and NNVC-6.0 HOP anchor, the BD-rate changes of HOP filter for {Y, Cb, Cr} components are shown as below reportedly:

AI: -3.34%, -14.82%, -15.90%, RA: -5.42%, -19.22%, -19.99% (On top of NNVC-6.0 anchor)

AI: -0.03%, -0.13%, -0.12%, RA: -0.04%, -0.10%, -0.11% (On top of NNVC-6.0 HOP anchor)

Introduction

The high operation point (HOP) filter is adopted in NNVC common software base (NNVC-6.0) [1] and EE1-0 is established to design a unified low operation point (LOP) filter. In JVET-AE0238 [2], a unified interface design is proposed for HOP and LOP.

Proposed method

This proposal introduces an improved inference scheme into the implementation of the HOP filter which is kept same as the LOP filter in EE1-0 such that the LOP filter could share a unified inference design as the HOP filter. In particular, the extended samples of inference block are padded with zero value at picture boundary.

Experimental results

The proposed method is tested according to the common test conditions defined in [3]. The test results on top of NNVC-6.0 anchor are shown in Table 1 and Table 2. Under AI configurations, the proposed method can bring 3.34%, 14.82%, and 15.90% BD-rate reductions on average for Y, Cb, and Cr, respectively. Under RA configurations, the proposed method can bring 5.42%, 19.22%, and 19.99% BD-rate reductions on average for Y, Cb, and Cr, respectively.

Table 1. Performance of the proposed method (AI) on top of NNVC-6.0 anchor

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.60% | -15.52% | -18.12% | 128% | 575% |
| Class A2 | -3.14% | -15.35% | -13.62% | 114% | 565% |
| Class B | -2.90% | -14.49% | -14.91% | 113% | 541% |
| Class C | -3.69% | -12.74% | -15.42% | 105% | 487% |
| Class E | -4.56% | -16.87% | -18.23% | 116% | 557% |
| Overall | -3.34% | -14.82% | -15.90% | 114% | 540% |
| Class D | -3.07% | -13.87% | -16.44% | 103% | 441% |
| Class F | -2.85% | -12.09% | -12.59% | 108% | 497% |

Table 2. Performance of the proposed method (RA) on top of NNVC-6.0 anchor

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -5.47% | -17.66% | -22.04% | 136% | 602% |
| Class A2 | -5.75% | -19.19% | -19.09% | 133% | 625% |
| Class B | -5.09% | -19.86% | -18.92% | 136% | 572% |
| Class C | -5.54% | -19.63% | -20.48% | 128% | 573% |
| Class E |  |  |  |  |  |
| Overall | -5.42% | -19.22% | -19.99% | 133% | 588% |
| Class D | -5.94% | -19.38% | -19.85% | 127% | 535% |
| Class F | -3.77% | -14.09% | -13.75% | 154% | 183% |

Compared with NNVC-6.0 anchor, the BD-rate changes of the proposed method are -0.03%, -0.13%, and -0.12% for Y, Cb, and Cr, respectively under AI configuration. And -0.04%, -0.10%, and -0.11% BD-rate changes for Y, Cb, and Cr can be observed under RA configuration, respectively. The experimental results are shown in Table 3 and Table 4.

Table 3. Performance of the proposed method (AI) on top of NNVC-6.0 HOP anchor

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.10% | -0.13% | 101% | 98% |
| Class A2 | -0.02% | -0.06% | 0.06% | 101% | 98% |
| Class B | -0.02% | -0.13% | -0.11% | 99% | 98% |
| Class C | -0.06% | -0.17% | -0.25% | 100% | 104% |
| Class E | -0.05% | -0.18% | -0.16% | 100% | 100% |
| Overall | -0.03% | -0.13% | -0.12% | 100% | 100% |
| Class D | -0.09% | -0.47% | -0.52% | 98% | 105% |
| Class F | -0.03% | -0.09% | -0.16% | 101% | 98% |

Table 4. Performance of the proposed method (RA) on top of NNVC-6.0 HOP anchor

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.01% | 0.00% | 99% | 97% |
| Class A2 | -0.04% | 0.04% | 0.02% | 99% | 97% |
| Class B | -0.03% | -0.14% | -0.16% | 101% | 99% |
| Class C | -0.02% | -0.20% | -0.21% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.10% | -0.11% | 100% | 99% |
| Class D | -0.11% | 0.11% | 0.12% | 103% | 106% |
| Class F | -0.03% | -0.14% | -0.29% | 101% | 100% |

Conclusions

This contribution presents an improved inference for HOP filter. It is recommended to adopt the proposed method into EE1.

References

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/NNVC-6.0

- J. Li, Y. Li, C. Lin, K. Zhang, L. Zhang, AHG11: A unified design of NN-based loop-filters at low and high operation points, JVET-AE0238, July. 2023.

- E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AE2016, July. 2023.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).