| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0206-v1 |
| --- | --- |

| Title: | AHG11: Complexity reduction of NN-based loop-filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junru Li, Yue Li, Chaoyi Lin, Kai Zhang, Li Zhang | Tel: Email: | lijunru@bytedance.com yue.li@bytedance.com linchaoyi.cy@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

- Abstract

This contribution proposes a unified filter architecture, with reported complexity of  kmac/pixel, which is even lower than 17.0 kmax/pixel of the lower operation point (LOP) in NNVC. The training and inference are kept same as the existing NN-based filters in NNVC.

Compared with NNVC-6.0 VTM anchor the BD-rate changes are shown as below reportedly:

AI: %, %, %, RA: %, %, % (On top of NNVC-6.0 VTM anchor)

Introduction

The high operation point (HOP) filter with complexity of 477 kmac/pixel and low operation point (LOP) filter with complexity of 17.0 kmac/pixel are adopted in NNVC common software base (NNVC-6.0) [1]. This contribution introduces a unified filter architecture with  kmac/pixel for NNVC.

Proposed method

The proposed network architecture is designed based on the basic structure of LOP. The number of feature map and residual block are modified accordingly to reduce the network complexity.

Experimental results

The proposed method is tested according to the common test conditions defined in [2]. The test results on top of NNVC-6.0 VTM anchor are shown in Table 1 and Table 2. Under AI configurations, the proposed method can bring %, %, and % BD-rate reductions on average for Y, Cb, and Cr, respectively. Under RA configurations, the proposed method can bring %, %, and % BD-rate reductions on average for Y, Cb, and Cr, respectively.

Table 1. Performance of the proposed method (AI) on top of NNVC-6.0 VTM anchor

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -6.25% | -9.74% | -9.74% |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -6.24% | -8.53% | -9.82% |  |  |
| Class F |  |  |  |  |  |

Table 2. Performance of the proposed method (RA) on top of NNVC-6.0 VTM anchor

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Conclusions

This contribution presents a unified filter architecture for NNVC. It is recommended to further study the proposed method into EE1.

References

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/NNVC-6.0

- E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AE2016, July. 2023.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).