| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0209-v1 |
| --- | --- |

| Title: | AHG13: Frequency domain Film Grain Objective Metrics |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuewei Meng, Wenhao Zhang, Scott Labrozzi | Author(s) or Contact(s): | xuewei.meng@disney.com wenhao.zhang2@disney.com scott.labrozzi@disney.com |
| Source: | Disney Streaming |  |  |

_____________________________

# Abstract

In this proposal, we propose a subband based film grain assessment method (SFGA), which shows promising results. SFGA, designed as a full-reference film grain quality assessment tool, initially transforms both reference and test images from the spatial domain to the frequency domain. It then selects several sub-bands within the frequency domain, compares their sub-band noise power spectra, and subsequently combines the sub-band results. The PLCC and SROCC between the proposed metric and MOS are both larger than 0.75, outperforming existing metrics like NSS (Natural Scene Statistics), FFT (Fast Fourier Transform), PSNR, and SSIM. This metric not only enhances our understanding of film grain technology, but also provides a valuable tool for fine-tuning film grain parameters in the modeling and synthesis process.

# Introduction

In the entertainment industry, film grain[1][2][3] is widely present in TV and motion pictures. Although digital cameras do not produce film grain, film grain is often added to the captured videos for creative intent. When encoding these TV or motion picture videos, preserving film grain under limited bandwidth is a challenging task. To address this issue, film grain synthesis methods are proposed in video coding. A typical film grain framework can be described as shown in Figure 1.

Figure 1. A typical framework of film grain synthesis.

Although film grain synthesis has been regarded as one of the most important features in state-of-the-art video coding standards, such as H.266/VVC, and AV1. There are no widely used objective metrics to evaluate the quality of the synthesized grainy video, which highly limits the development of film grain related technologies.

In this proposal, we propose a subband based film grain assessment method (SFGA).

# Film Grain Objective Metric

## Framework

Here is the framework of the proposed SFGA. For reference image and test image, we first convert them from space domain to frequency domain. And select several sub-bands in the frequency domain, FR0 to FRn and FT0 to FTn. Then calculate the metric scores based on these bands. And finally fuse these scores to derive the final output.

Figure 2. Film grain objective metric framework.

## Space domain to frequency domain conversion

- Step 1: Steerable Pyramid

- Step 2: Fast Fourier Transform (FFT)

For the space to frequency domain conversion, we first conduct steerable pyramid on the noise image and then conduct the FFT on the output of steerable pyramid. The steerable pyramid is a multi-scale representation that is translation-invariant, but that also includes representation of orientation. The basic functions are oriented (steerable) filters, localized in space and frequency. The steerable pyramid can help us to derive several sub-bands for the metric calculation.

Here is an example of steerable pyramid bandpass filter with four directions in the space domain.

Figure 3. An example of steerable pyramid bandpass filter (four directions) in space domain.

And if we take the following as the input, the output of the steerable pyramid is like Figure 4. There are 4 bands and 4 directions. And the number of band and direction of steerable pyramid can be adjusted based on the requirement.

Figure 4. Examples of steerable pyramid input and output.

In this proposal, we use 4 bands and 6 directions. And choose the high frequency band (corresponding to height00 in Figure 4) as the input of the FFT.

## Metrics calculation

- Step 1: 2D FFT to 1D FFT

- Step 2: Noise power spectrum (NPS)

- Step 3: Jensen Shannon Divergence (JSD) between NPS of reference image and test image, which is corresponding to (Score0, …, Scoren)

For the metric calculation and score fusion, we first convert the results of 2D FFT to 1D. And calculate the noise power spectrum for each band and direction. Then evaluate the similarity of the NPS using JS divergence.

Figure 5. Framework of metric calculation.

## Score fusion

Score= Score0+ Score1+ ,,, + Scorenn+1

Average the JSD of different bands and directions. (See the equation below.)

# Subjective Viewing

To evaluate the effectiveness of objective metrics, subjective viewing is a necessary and important part.

## Clean image capture

The clean images are captured by Sony- RX1RM2 with ISO 100.

Figure 6. Clean images.

## Add film grain (Ground-truth/reference images)

The film grain is added by Davinci Resolve 18.5 (preset 8mm250D, 8mm500T, 16mm, 16mm250D, 35mm).

- Preset “8mm 250D”.       (b) Preset “8mm 500T”.     (c) Preset “16mm”.

(d) Preset “16mm 250D”.    (e) Preset “35mm”.

Figure 7. Film grain parameters in Davinci Resolve 18.5.

## Film grain synthesis by AR and frequency model (test images)

The test grainy images are generated by frequency and AR models.

Figure 8. Generate test images by AR and frequency model.

## Subjective viewing settings

- Dark room viewing environment.

- Equipment used: MacBook Pro.

- Adjust the screen brightness to the highest.

- 5 expert viewers and preferably familiar with film grain.

- The viewing distance is not fixed for all viewers and the viewer can get as close as possible to the display.

- Viewers can look at test images for as long time as they want.

- All the YUV files are displayed by YUView with x1~1.5 zoom-in and side-by-side mode.

- Before the subjective viewing, we train the

- The score is shown below,

| Score | Similarity |
| --- | --- |
| 1 | Totally different |
| 2 | Different |
| 3 | Similar |
| 4 | Very similar |
| 5 | Undistinguishable/Same |

# Performance of Objective Metrics

We use PLCC and SROCC for the evaluation of the proposed objective metric. Other than the proposed method, we also evaluate the NSS, FFT, PSNR and SSIM. Based on the results, the proposed method shows the best performance.

## Metric1: Natural Scene Statistics

Spatial domain NSS features [4] are extracted from local image patches that effectively capture the essential low-order statistics of natural images. The conventional spatial NSS model that we use begins by pre-processing the image by local mean removal and divisive normalization,

INSSi,j=Ii,j-μ(i,j)σi,j+1,

where i∈1,2,…,M, j∈{1,2,…,N} are indices in the spatial domain. M and N represent the width and height of image, respectively. Ii,j is the pixel value of the sample with location i,j. μ(i,j) which represents the local mean is calculated by,

μi,j=k=-KKl=-LLwk,lIi+k,j+l,

where w={wk,l|k=-K,…,K;l=-L,…,L} is a 2D circularly-symmetric Gaussian weighting function sampled out to 3 standard deviations (K=L=3) and rescaled to unit volume. σi,j which represents local contrast is derived by,

σi,j=k=-KKl=-LLwk,l[Ii+k,j+l- μi,j]2.

The product of diagonally adjacent coefficients captures local spatial correlations,

IPMSCNi,j= INSSi,jINSSi+1,j+1.

There are two methods to compare the statistics of source video and film grain synthesized video.

- The empirical distribution of IPMSCNi,j.

- The Jensen Shannon divergence computed on the empirical distributions of IPMSCNi,j, which denoted by JSDNSS. A smaller JSDNSSvalue indicates that the two distributions are more similar.

## Metric2: Fast Fourier Transform

The first step of this method is FFT. Then convert the 2D FFT results to 1D. And derive the Noise Power Spectrum (NPS). And then calculate the JS Divergence. The result of JS Divergence is the output of this metric.

Figure 9. Framework of FFT based metric.

## Results

The PLCC and SROCC between objective metric results and MOS is shown below. We can see that the PLCC and SROCC of the proposed SFGA shows the best result (> 0.75).

Figure 10. Results of PSNR, SSIM and the proposed SFGA.

# Conclusion

In this proposal, a subband based film grain assessment method was introduced, which shows promising results. This metric is designed as a comprehensive full-reference tool for assessing film grain quality. The PLCC and SROCC of the proposed metric and MOS is larger than 0.75, outperforming existing metrics like NSS, FFT, PSNR, and SSIM. It not only enhances our understanding of film grain technology but also provides a valuable tool for fine-tuning film grain parameters in the modeling and synthesis process.

# References

[1] X. Meng, W. Zhang, S. Labrozzi. AHG13: On Film Grain Synthesis Subjective Evaluation. JVET-AC0199, 29th JVET meeting, teleconference, Jan. 2023.

[2] X. Meng, W. Zhang, A. Norkin, J. Sole, A. Duenas. “On Film Grain Synthesis Subjective Evaluation and Further Work”. JVET-AD0238, 30th JVET meeting, Antalya, Apr. 2023.

[3] X. Meng, W. Zhang, S. Labrozzi. “AHG13: On Film Grain Objective Metrics”. JVET-AD0241, 30th JVET meeting, Antalya, Apr. 2023.

[4] Mittal, A., Soundararajan, R., & Bovik, A. C. (2012). Making a “completely blind” image quality analyzer. IEEE Signal processing letters, 20(3), 209-212.

# Patent rights declaration(s)

Disney Streaming Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).