| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0224-v3 |
| --- | --- |

| Title: | Crosscheck of JVET-AF0075 (Evaluation results of LCEVC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Christian Lehmann | Email: | firstname.lastname@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI, Einsteinufer 37, 10587 Berlin, Germany |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results of the LCEVC evaluation results reported in JVET-AF0075. The results have been reproduced and can be confirmed. Furthermore, an informal subjective evaluation has been performed and the examples giving in JVET-AF0075 can be confirmed as well.

# Test conditions and settings

The crosschecks include 6 UHD sequences from classes A1 und A2 from JVET Common Testing Conditions (CTC) by using random access (RA) configurations and default encoder configuration.
Following codecs have been tested as anchors:

- HM-16.25 reference software for H.265/HEVC ( base config: encoder_randomaccess_main10.cfg)

- VTM-19.0 reference software for H.266/VVC (base config: encoder_randomaccess_vtm.cfg)

The LCEVC reference software LTM-5.4.1 was used from:

https://mpeg.expert/software/MPEG/LCEVC/LTM/-/tree/LTM_5_4_1

by compiling the software based on the given documentation (README.md and SoftwareManual_LCEVCTestModelLTM.pdf). As the tag LTM_5_4_1 includes older versions of the base codecs, the binaries of the anchor (HM-16.25 respectively VTM-19.0) were used by copying the binaries and encoder configuration into the documented path “external_codecs/HM” respectively “external_codecs/VTM”.

For all encodings, decodings and PSNR calculation an AMD server (AMD EPXC 7502P) running Ubuntu 20.04 (gcc 10.5.0) was used.

# Test results

## Objective test results

Table 1: VTM-19.0 (1080p base layer) + LTM-5.4.1 (2160p enhancement layer) vs. 
 VTM-19.0 (2160p single layer) as anchor

| Sequence | Over VTM 19.0 (single layer) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Random Access |  |  |  |
|  | PSNR Y | PSNR Cb | PSNR Cr | PSNR YCbCr |
| Tango2 | -0.11% | -2.30% | 9.29% | 1.24% |
| FoodMarket4 | 6.03% | 4.99% | 5.31% | 5.87% |
| Campfire | 32.66% | 156.97% | 59.76% | 52.80% |
| CatRobot | 49.23% | 49.91% | 80.22% | 53.49% |
| DaylightRoad2 | 86.81% | 17.06% | 48.41% | 67.43% |
| ParkRunning3 | 14.95% | 370.64% | 220.60% | 33.05% |
| Overall | 31.59% | 99.55% | 70.60% | 35.65% |

Table 2: HM-16.25 (1080p base layer) + LTM-5.4.1 (2160p enhancement layer) vs. 
HM-16.25 (2160p single layer) as anchor

| Sequence | Over HM 16.25 (single layer) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Random Access |  |  |  |
|  | PSNR Y | PSNR Cb | PSNR Cr | PSNR YCbCr |
| Tango2 | -8.04% | 4.76% | 7.81% | -3.82% |
| FoodMarket4 | -5.75% | 7.83% | 5.74% | -2.93% |
| Campfire | 6.99% | 153.47% | 19.89% | 24.39% |
| CatRobot | 17.72% | 34.20% | 55.32% | 23.48% |
| DaylightRoad2 | 42.87% | 12.06% | 41.83% | 39.64% |
| ParkRunning3 | -6.45% | 255.76% | 177.94% | 8.57% |
| Overall | 7.89% | 78.01% | 51.42% | 14.89% |

## Informal viewing

### DaylightRoad – License Plate

Cropped Window: offset (x:y) 1700:1200, size 150x80

- DaylightRoad original, frame 55

- VVC (VTM19.0) RA cfg @QP27, 8042.70 kbps, frame 55

- LCEVC (LTM5.4.1) + VVC, 8570.70 kbps, frame 55

- HEVC (HM16.25) RA cfg @QP27, 10328.04 kbps, frame 55

- LCEVC (LTM5.4.1) + HEVC, 9795.04 kbps, frame 55

| DaylightRoad, Original frame 55, pos(x,y) 1700:1200, cropped size 150x80 |  |
| --- | --- |
| VVC (VTM 19.0), QP27 @ 8042.70kbps | LCEVC + VVC (LTM5.4.1+VTM19.0) @8570.70kbps |
| HEVC (HM 16.25), QP27@ 10328.04kbps | LCEVC + HEVC (LTM5.4.1+HM16.25) @ 9795.04 kbps |

### DaylightRoad – Traffic Signs

Cropped Window: offset (x:y) 2280:1040, size 150x80

- DaylightRoad original, frame 55

- VVC (VTM19.0) RA cfg @QP27, 8042.70 kbps, frame 55

- LCEVC (LTM5.4.1) + VVC, 8570.70 kbps, frame 55

- HEVC (HM16.25) RA cfg @QP27, 10328.04 kbps, frame 55

- LCEVC (LTM5.4.1) + HEVC, 9795.04 kbps, frame 55

| DaylightRoad, Original frame 55, position offset(x:y) 2280:1040, cropped size 150x80 |  |
| --- | --- |
| VVC (VTM 19.0), QP27 @ 8042.70kbps | LCEVC + VVC (LTM5.4.1+VTM19.0) @8570.70kbps |
| HEVC (HM 16.25), QP27@ 10328.04kbps | LCEVC + HEVC (LTM5.4.1+HM16.25) @ 9795.04 kbps |

### ParkRunning

Cropped Window: offset (x:y) 634:535, size 240x360

- ParkRunning original, frame 217

- VVC (VTM19.0) RA cfg @QP37, 7732.45 kbps, frame 217

- LCEVC (LTM5.4.1) + VVC 6929.14 kbps, frame 217

- HEVC (HM16.25) RA cfg @QP37, 9772.12 kbps, frame 217

- LCEVC (LTM5.4.1) + HEVC, 9310.53 kbps, frame 217

| ParkRunning, Original frame 217, position offset(x:y) 634:535, cropped size 240x360 |  |
| --- | --- |
| VVC (VTM 19.0), QP37 @ 7732.45kbps | LCEVC + VVC (LTM5.4.1+VTM19.0) @6929.14kbps |
| HEVC (HM 16.25), QP27@ 9772.12kbps | LCEVC + HEVC (LTM5.4.1+HM16.25) @ 9310.53 kbps |

### CatRobot –  Fabric

Cropped Window: offset (x:y) 2000:1200, size 150x80

- CatRobot original, frame 47

- VVC (VTM19.0) RA cfg @QP32, 3987.29 kbps, frame 47

- LCEVC (LTM5.4.1) + VVC, 4119.10 kbps, frame 47

- HEVC (HM16.25) RA cfg @QP32, 5428.68 kbps, frame 47

- LCEVC (LTM5.4.1) + HEVC, 5511.26 kbps, frame 47

| CatRobot, Original frame 47, position offset(x:y) 2000:1200, cropped size 150x80 |  |
| --- | --- |
| VVC (VTM 19.0), QP32 @ 3987.29kbps | LCEVC + VVC (LTM5.4.1+VTM19.0) @4119.10kbps |
| HEVC (HM 16.25), QP32@ 5428.68kbps | LCEVC + HEVC (LTM5.4.1+HM16.25) @ 5511.26 kbps |

### CatRobot –  Book

Cropped Window: offset (x:y) 2000:1200, size 150x80

- CatRobot original, frame 47

- VVC (VTM19.0) RA cfg @QP32, 3987.29 kbps, frame 47

- LCEVC (LTM5.4.1) + VVC, 4119.10 kbps, frame 47

- HEVC (HM16.25) RA cfg @QP32, 5428.68 kbps, frame 47

- LCEVC (LTM5.4.1) + HEVC, 5511.26 kbps, frame 47

| CatRobot, Original frame 47, position offset(x:y) 2000:1200, cropped size 150x80 |  |
| --- | --- |
| VVC (VTM 19.0), QP32 @ 3987.29kbps | LCEVC + VVC (LTM5.4.1+VTM19.0) @4119.10kbps |
| HEVC (HM 16.25), QP32@ 5428.68kbps | LCEVC + HEVC (LTM5.4.1+HM16.25) @ 5511.26 kbps |

# Appendix

## Encoder configuration

### LTM 5.4.1 + VTM 19.0

The used encoder command for VTM configuration is:

ModelEncoder --input_file={input_file} --format={format} --fps={fps} --width={width} --height={height} --intra_period={intra_period} --limit={limit} --output_file=str.bin --base_encoder=vvc --encapsulation=nal --cq_step_width_loq_0={cq_step_width_loq_0}

cq_step_width_loq_0 = [2303, 2370, 2438, 2505, 2573, 2640, 2708, 2775, 2843, 2911, 2978, 3046, 3113, 3181, 3248, 3316, 3384, 3451, 3519, 3586, 3654]

cq_step_width_loq_0 is equal to SW2 when applying the formula

VVC base codec

𝑄𝑃VVC = ROUNDDOWN ⌊0.0148∗𝑆𝑊2−22.07⌋

If 𝑄𝑃VVC ≥ 41, then 𝑆𝑊1= 3600 else 𝑆𝑊1= 32767

### LTM 5.4.1 + HM 16.25

The used encoder command for HM configuration is:

ModelEncoder --input_file={input_file} --format={format} --fps={fps} --width={width} --height={height} --intra_period={intra_period} --limit={limit} --output_file=str.lvc --base_encoder=hevc --encapsulation=nal --cq_step_width_loq_0={cq_step_width_loq_0} > en.log

cq_step_width_loq_0 = [2281, 2349, 2416, 2484, 2551, 2619, 2686, 2754, 2822, 2889, 2957, 3024, 3092, 3159, 3227, 3294, 3362, 3430, 3497, 3565, 3632]

cq_step_width_loq_0 is equal to SW2 when applying the formula

HEVC base codec

𝑄𝑃HEVC = ROUNDDOWN ⌊0.0148∗𝑆𝑊2−21.751⌋

If 𝑄𝑃HEVC ≥ 39, then 𝑆𝑊1= 3600 else 𝑆𝑊1= 32767

## Observation when forcing to enhance Luma and Chroma planes

When calling the help menu of the used LTM version 5.4.1 it can be assumed that per default only luma planes are processed when encoding the enhancement layer, since the parameter ‘--num_processed_planes’ default is 1.

Abstract from the LTM 5.4.2 help text:

Global Configuration options:

--num_image_planes arg             Number of planes in input sequence (default: 3)

--num_processed_planes arg    Number of planes for which enhancement shall be added (1 or 3) (default: 1)

The QP range which is defined as cq_step_width_loq_0 and documented in 3.1 Encoder configuration is corresponding to a QP range of the base codec from QP 12-32.

After running the whole QP range it can be observed that the parameter ‘num_processed_planes’ is QP sensitive and adapted internally depending of the used value for SW2 (cq_step_width_loq_0).

For both VTM and HM configuration the ‘num_processed_planes’ parameter behavior is:

If 𝑄𝑃(BaseCodec) ≥ 27, then num_processed_plane = 1 else num_processed_plane = 3

That means for higher bitrates luma and chroma planes are processed and for lower bitrates only luma is processed in the enhancement layer.

After running a new simulation by forcing the enhancement layer to process all planes by setting ‘num_processed_plane = 3’ it can be observed that only the highest QP value has an impact to the results.

For the HM results only the rate point that matches the QP=27 of the HM anchor will slightly change the results.

Table 3 shows the results when forcing LTM to enhance all planes for HM configuration compared to the default, QP-dependent behaviour.

Table 3: HM-16.25 (1080p base layer) + LTM-5.4.1 (2160p enhancement layer) +num_processed_planes=3 vs. 
HM-16.25 (1080p base layer) + LTM-5.4.1 (2160p enhancement layer) as anchor

| Sequence | Over HM-16.25 (1080p base layer) + LTM-5.4.1 (2160p enhancement layer) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Random Access |  |  |  |
|  | PSNR Y | PSNR Cb | PSNR Cr | PSNR YCbCr |
| Tango2 | 0.11% | 0.08% | 0.07% | 0.10% |
| FoodMarket4 | 0.08% | 0.06% | 0.06% | 0.08% |
| Campfire | 0.00% | 0.00% | 0.00% | 0.00% |
| CatRobot | 0.18% | 0.15% | 0.11% | 0.17% |
| DaylightRoad2 | 0.11% | 0.09% | 0.09% | 0.11% |
| ParkRunning3 | 1.16% | -0.30% | 0.25% | 1.01% |
| Overall | 0.27% | 0.01% | 0.10% | 0.24% |