| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0250-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AF0134 (AHG12: AMVP with SbTMVP mode) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Fangjun Pu | Tel: Email: | fpu@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution is a crosscheck report of JVET-AF0134 on improvement of AMVP with SbTMVP mode. In JVET-AF0134, given a CU coded in AMVP with SbTMVP mode, the CU is predicted in a similar way as that of SbTMVP in merge mode except that the motion shift is signaled in the bitstream instead of being derived from neighboring blocks. It is confirmed that the implementation matches the proposal and crosschecking results for random access and low delay B configurations match with the proponent’s results.

# Introduction

The AMVP with SbTMVP mode was firstly proposed in JVET-AC0103 and was studied in EE2. The simulation result on top of ECM-8.0 was reported in JVET-AD0152, and the overall coding performance and runtime for {Y, U, V, EncT, DecT} was {-0.06%, -0.03%, -0.04%, 102%, 100%} for RA and {-0.22%, 0.06%, 0.04%, 102%, 100%} for LB. It was commented that the results did not provide attractive benefit in terms of compression, compared to the increased encoding runtime.

To improve the coding efficiency and reduce the encoding complexity of the AMVP with SbTMVP mode, the following changes are made:

- For low delay pictures, one CU-level flag is signaled to indicated which one of the two collocated pictures is used to derive the motion. For non-low delay pictures, the collocated picture is fixed to the same collocated picture used in VVC.

- The reference pictures of subblocks are fixed to the one with reference picture index equal to 0.

- The number of MVD is always set to 16 which comprises 4 motion directions and 4 motion magnitudes (Figure 1).

- When picture resolution is larger than or equal to 1280x720 luma samples, the AMVR is enabled for the proposed mode. Two MVD resolutions are supported including 4-pel and 64-pel. When 4-pel MVD resolution is selected, the motion magnitudes are {4, 8, 12, 16}-pel. On the other hand, when 64-pel MVD resolution is selected, the motion magnitudes are {64, 128, 192, 256}-pel.

Figure 1. 16 MVD candidates for all pictures

# Crosscheck results

The methods are implemented on top of ECM-10.0 and are tested under CTC. This crosscheck report tests with random access and low delay B configurations for JVET-AF0134. Results are show in Table 1 and 2.

Table 3. Test results over ECM-10.0 for random access configuration

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class B | -0.08% | 0.06% | -0.02% | 101.0% | 99.5% | 100.1% |
| Class C | -0.17% | -0.18% | 0.03% | 101.2% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class D | -0.17% | 0.03% | -0.22% | 101.1% | 99.7% | 100.3% |
| Class F | -0.08% | -0.09% | -0.07% | 100.7% | 100.2% | 100.1% |

Table 2. Test results over ECM-10.0 for low delay B configuration

|  | Low delay B Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class C | -0.21% | -0.31% | 0.08% | 102.3% | 100.1% | 100.0% |
| Class E | -0.93% | -0.79% | -0.79% | 103.5% | 99.9% | 100.0% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class D | -0.42% | -0.60% | -0.52% | 102.6% | 98.8% | 100.6% |
| Class F | -0.13% | 0.29% | 0.31% | 102.0% | 98.9% | 100.2% |

# Patent rights declaration(s)

Dolby Laboratories, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.