| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0254 |
| --- | --- |

| Title: | Cross-check of JVET-AF0085 (EE1-1.2.1: on residual adjustments for NNLF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | T. Dumas | Author(s) or Contact(s): | thierry.dumas@interdigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-check of the contribution: "EE1-1.2.1: on residual adjustments for NNLF” described in JVET-AF0085. In JVET-AF0085, a frame-level residual offset can be used to modify the output of the HOP in-loop filter. The residual is adjusted by decreasing the absolute value of each of its coefficients by a small value. This small value belongs to {1, 2}.

The cross-checked rate-distortion results and encoding/decoding runtimes of JVET-AF0085 are reported. In AI, RA, and LDB configurations, the cross-checked rate-distortion results match exactly those presented in JVET-AF0085.

# Introduction

JVET-AF0085 [2] suggests a frame-level residual offset adjusting the output of the HOP in-loop filter. The residual is adjusted by decreasing the absolute value of each of its coefficients by a small value. This small value belongs to {1, 2}. The applications of the residual adjustment for Y, U, and V components are signaled via three distinct frame-level flags. For a given color component, if its associated frame-level flag is true, another frame level flag signals which offset value must be used.

On top of NNVC-6.0, the neural network-based intra prediction being activated (--NnIntraPred=1), LOP being deactivated and HOP being activated (--NnlfOption=4), JVET-AF0085 reports

- -0.02%, -0.05%, -0.04% of mean BD-rate gains, for 100% and 99% of encoding and decoding runtimes respectively.

# Cross-check report

The source code has been run under AHG11 CTCs [1]. The experimental results are depicted in Figure 1, Figure 2, and Figure 3.

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0, --NnIntraPred=1, --NnlfOption=4 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.04% | -0.03% | 0.18% | -0.02% | 0.45% | 0.68% | 102% | 101% |
| Class A2 | -0.01% | -0.07% | 0.00% | -0.01% | 0.33% | -0.06% | 102% | 100% |
| Class B | -0.01% | -0.19% | -0.29% | 0.03% | -0.08% | -0.16% | 102% | 100% |
| Class C | -0.03% | -0.17% | -0.09% | -0.11% | -0.25% | 0.30% | 102% | 102% |
| Class E | -0.01% | 0.33% | 0.18% | 0.00% | 0.41% | 0.25% | 102% | 102% |
| Overall | -0.02% | -0.05% | -0.04% | -0.02% | 0.12% | 0.17% | 102% | 101% |
| Class D | -0.04% | -0.02% | -0.01% | -0.05% | -0.25% | -0.15% | 103% | 104% |
| Class F | -0.04% | -0.06% | 0.25% | 0.13% | -0.09% | 1.02% | 105% | 104% |

Figure 1: mean BD-rate gains of JVET-AF0085 on top of VTM, the neural network-based intra prediction mode being activated (--NnIntraPred=1), LOP being deactivated and HOP being activated (--NnlfOption=4) in AI configuration.

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0, --NnIntraPred=1, --NnlfOption=4 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.06% | -0.09% | -0.25% | 0.05% | -0.29% | 0.18% | 102% | 101% |
| Class A2 | -0.03% | -0.06% | -0.02% | -0.08% | -0.06% | -0.09% | 101% | 100% |
| Class B | -0.01% | -0.27% | -0.90% | -0.06% | -0.23% | -0.94% | 101% | 99% |
| Class C | 0.00% | -0.19% | -0.58% | 0.05% | -0.18% | -0.62% | 102% | 103% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.02% | -0.17% | -0.51% | -0.01% | -0.19% | -0.46% | 102% | 101% |
| Class D | 0.13% | -0.73% | -1.01% | 0.17% | -0.50% | -0.75% | 102% | 102% |
| Class F | -0.07% | -0.20% | -0.36% | 0.07% | 0.12% | -0.15% | 101% | 106% |

Figure 2: mean BD-rate gains of JVET-AF0085 on top of VTM, the neural network-based intra prediction mode being activated (--NnIntraPred=1), LOP being deactivated and HOP being activated (--NnlfOption=4) in RA configuration.

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0, --NnIntraPred=1, --NnlfOption=4 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -0.14% | -1.73% | -4.75% | -0.11% | -3.18% | -5.25% | 103% | 102% |
| Class C | -0.16% | -0.80% | -2.44% | -0.31% | -2.01% | -3.47% | 105% | 101% |
| Class E | -0.66% | -3.39% | -8.27% | -1.20% | -4.25% | -10.68% | 102% | 115% |
| Overall | -0.28% | -1.83% | -4.86% | -0.45% | -3.06% | -6.01% | 103% | 105% |
| Class D | -0.01% | -2.93% | -5.52% | -0.11% | -4.79% | -8.10% | 104% | 103% |
| Class F | -0.16% | -0.68% | -1.17% | -0.05% | -1.32% | -0.81% | 102% | 106% |

Figure 3: mean BD-rate gains of JVET-AF0085 on top of VTM, the neural network-based intra prediction mode being activated (--NnIntraPred=1), LOP being deactivated and HOP being activated (--NnlfOption=4) in LDB configuration.

# References

- E. Alshina, R.-L. Liao, S. Liu, A. Segall. Common Test Conditions and evaluation procedures for neural network-based video coding technology. JVET-AE2016, 31st JVET meeting, Geneva, 11-19 July 2023.

[2] Z. Dai, Y. Yu, H. Yu, D. Wang. EE1-1.2.1: on residual adjustments for NNLF. JVET-AF0085, 32nd JVET meeting, Hannover, 13-20 October 2023.

# Patent rights declaration(s)

Interdigital does not have any current or pending patent rights relating to the technology described in this contribution.