| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0280 |
| --- | --- |

| Title: | EE2-related: complete results for EE2-1.1b in JVET-AF0249 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | T. Dumas R. Utida F. Le Léannec | Author(s) or Contact(s): | thierry.dumas@interdigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

During the 31st JVET meeting, Geneva, 11-19 July 2023, the tests of ECM with various maximum MTT depth increments were requested, c.f. EE2-1.1b in JVET-AE2024. This will provide a fair comparison for EE2-1.1a evaluating ECM with the non-square QT, c.f. JVET-AF0108. JVET-AF0249 reports the cross-check of JVET-AF0108. Besides, JVET-AF0249 gives the final mean BD-rate versus encoding runtime curve resulting from the runs of ECM with different maximum MTT depth increments. This contribution reports all the results used to construct this curve.

It is reported that, in RA configuration, over all classes, at equivalent encoding runtimes, ECM including the non-square QT yields -0.04% of mean BD-rate gains in luma with respect to ECM with the maximum MTT depth increased by 1 on I-slices and the maximum MTT depth increased by 1 for the first two TIDs.

# Introduction

During the 31st JVET meeting, Geneva, 11-19 July 2023, it was requested to run ECM with different maximum MTT depth increments, c.f. EE2-1.1b in JVET-AE2024 [1]. EE2-1.1a, corresponding to ECM with the non-square QT, c.f. JVET-AF0108 [2], can be then compared to the trade-offs between mean BD-rate gains versus encoding runtime of ECM with different maximum MTT depth increments. JVET-AF0249 [3] reports the cross-check of JVET-AF0108. JVET-AF0249 [3] also presents the final mean BD-rate versus encoding runtime curve of ECM with changes in maximum MTT depth. This contribution shows all the results leading to this final curve.

# Experiments on ECM with maximum MTT depth increments

In this section, each ECM-10-based code is run under the JVET CTC in RA configuration.

## Increasing by 1 the maximum MTT depth on I-slices only

For ECM-10.0, in the configuration files of Classes A, B, C, and D, “MaxMTTHierarchyDepthI”, “MaxMTTHierarchyDepthISliceL”, and “MaxMTTHierarchyDepthISliceC” are increased by 1. This experiment is denoted [1] and called “max MTT depth I-slice + 1”. The results are presented in Table 1.

Table 1: mean BD-rate gains of [1] “max MTT depth I-slice + 1” on top of ECM-10.0.

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.01% | -0.19% | -0.17% | 109.1% | 100.5% |  |
| Class A2 | -0.05% | -0.61% | -0.35% | 109.4% | 100.9% |  |
| Class B | -0.15% | -0.39% | -0.48% | 112.1% | 100.7% |  |
| Class C | -0.13% | 0.04% | -0.30% | 110.8% | 100.9% |  |
| Class E |  |  |  |  |  |  |
| Overall | -0.10% | -0.28% | -0.34% | 110.6% | 100.8% |  |
| Class D | -0.12% | -0.38% | -0.46% | 109.0% | 100.1% |  |
| Class F | 0.00% | 0.00% | 0.00% | 101.0% | 101.4% |  |

## Increasing by 1 the maximum MTT depth for the first two TIDs

For ECM-10.0, in the configuration files of Classes A, B, C, and D, the first two values of “MaxMTTHierarchyDepthByTid” are increased by 1. This experiment is denoted [2] and called “max MTT depth first 2 TIDs + 1”. The results are displayed in Table 2.

Table 2: mean BD-rate gains of [2] “max MTT depth first 2 TIDs + 1” on top of ECM-10.0.

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.20% | -0.22% | -0.21% | 115.0% | 100.7% |  |
| Class A2 | -0.26% | -0.43% | -0.45% | 118.3% | 100.9% |  |
| Class B | -0.23% | -0.15% | -0.22% | 116.0% | 99.9% |  |
| Class C | -0.15% | -0.13% | -0.29% | 111.7% | 103.1% |  |
| Class E |  |  |  |  |  |  |
| Overall | -0.21% | -0.21% | -0.28% | 115.1% | 101.1% |  |
| Class D | -0.04% | 0.01% | -0.37% | 111.5% | 100.8% |  |
| Class F | 0.00% | 0.00% | 0.00% | 100.2% | 100.4% |  |

## Increasing by 1 the maximum MTT depth for the first three TIDs

For ECM-10.0, in the configuration files of Classes A, B, C, and D, the first three values of “MaxMTTHierarchyDepthByTid” are increased by 1. This experiment is denoted [3] and called “max MTT depth first 3 TIDs + 1”. The results are presented in Table 3.

Table 3: mean BD-rate gains of [3] “max MTT depth first 3 TIDs + 1” on top of ECM-10.0.

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.29% | -0.34% | -0.40% | 129.3% | 100.2% |  |
| Class A2 | -0.43% | -0.70% | -0.69% | 132.8% | 100.6% |  |
| Class B | -0.33% | -0.31% | -0.42% | 129.9% | 99.6% |  |
| Class C | -0.26% | -0.38% | -0.55% | 123.1% | 103.1% |  |
| Class E |  |  |  |  |  |  |
| Overall | -0.32% | -0.41% | -0.51% | 128.5% | 100.9% |  |
| Class D | -0.20% | -0.27% | -0.59% | 120.7% | 100.6% |  |
| Class F | 0.00% | 0.00% | 0.00% | 100.7% | 100.0% |  |

## Increasing by 1 the maximum MTT depth on I-slices only and increasing by 1 the maximum MTT depth for the first two TIDs

For ECM-10.0, in the configuration files of Classes A, B, C, and D, “MaxMTTHierarchyDepthI”, “MaxMTTHierarchyDepthISliceL”, and “MaxMTTHierarchyDepthISliceC” are increased by 1. Moreover, the first two values of “MaxMTTHierarchyDepthByTid” are increased by 1. This experiment is denoted [1] + [2]. The results are depicted in Table 4.

Table 4: mean BD-rate gains of [1] + [2] on top of ECM-10.0.

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.24% | -0.53% | -0.40% | 120.9% | 100.1% |  |
| Class A2 | -0.35% | -1.12% | -0.90% | 122.8% | 100.0% |  |
| Class B | -0.34% | -0.79% | -0.80% | 123.4% | 100.0% |  |
| Class C | -0.26% | -0.37% | -0.63% | 117.4% | 104.5% |  |
| Class E |  |  |  |  |  |  |
| Overall | -0.30% | -0.69% | -0.69% | 121.2% | 101.2% |  |
| Class D | -0.19% | -0.65% | -0.42% | 117.5% | 100.5% |  |
| Class F | 0.00% | 0.00% | 0.00% | 100.5% | 100.8% |  |

## Increasing by 1 the maximum MTT depth on I-slices only and increasing by 1 the maximum MTT depth for the first three TIDs

For ECM-10.0, in the configuration files of Classes A, B, C, and D, “MaxMTTHierarchyDepthI”, “MaxMTTHierarchyDepthISliceL”, and “MaxMTTHierarchyDepthISliceC” are increased by 1. Moreover, the first three values of “MaxMTTHierarchyDepthByTid” are increased by 1. This experiment is denoted [1] + [3]. The results are shown in Table 5.

Table 5: mean BD-rate gains of [1] + [3] on top of ECM-10.0.

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.37% | -0.67% | -0.53% | 140.3% | 102.0% |  |
| Class A2 | -0.51% | -1.35% | -1.06% | 134.5% | 99.2% |  |
| Class B | -0.45% | -1.00% | -1.05% | 138.7% | 100.0% |  |
| Class C | -0.38% | -0.60% | -0.87% | 130.7% | 101.7% |  |
| Class E |  |  |  |  |  |  |
| Overall | -0.43% | -0.90% | -0.90% | 136.0% | 100.7% |  |
| Class D | -0.33% | -0.96% | -0.64% | 134.2% | 100.5% |  |
| Class F | 0.00% | 0.00% | 0.00% | 100.8% | 101.8% |  |

## Mean BD-rate versus encoding runtime tradeoffs

### Per-class

#### Class C

The results presented in Sections 2.1 to 2.5, for Class C only, are gathered in Figure 1.

- [3] (Ericsson) corresponds to the same experiment as [3] but cross-checked by Ericsson.

- [2] and [1] (Ofinno) is the same experiment as [2] and [1] but cross-checked by Ofinno.

- “non-square QT” (LGE) corresponds to ECM with the non-square QT, c.f. JVET-AF0108.

Figure 1: mean BD-rate versus encoding runtime, averaging over Class C.

#### Class B

The results presented in Sections 2.1 to 2.5, for Class B only, are gathered in Figure 2.

Figure 2: mean BD-rate versus encoding runtime, averaging over Class B.

#### Class A1

The results presented in Sections 2.1 to 2.5, for Class A1 only, are gathered in Figure 3.

Figure 3: mean BD-rate versus encoding runtime, averaging over Class A1.

#### Class A2

The results presented in Sections 2.1 to 2.5, for Class A2 only, are gathered in Figure 4.

Figure 4: mean BD-rate versus encoding runtime, averaging over Class A2.

### All classes

The results presented in Sections 2.1 to 2.5, for all classes, are gathered in Figure 5.

Figure 5: mean BD-rate versus encoding runtime, averaging over all classes.

# Conclusion

This contribution reports the complementary results of EE2-1.1b, required by JVET-AE2024 [1]. ECM with various changes in maximum MTT depths have been run under the JVET CTC.

In RA configuration, over all classes, at equivalent encoding runtimes, it is reported that ECM including the non-square QT yields -0.04% of mean BD-rate gains in luma with respect to ECM with the maximum MTT depth increased by 1 on I-slices and the maximum MTT depth increased by 1 for the first two TIDs.

# References

- V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang. Exploration Experiment on Enhanced Compression beyond VVC capability (EE2). JVET-AE2024, 31st JVEt meeting, Geneva, 11-19 July 2023.

- Y. Ahn, J. Nam, N. park, J. Lim, S. Kim. EE2-1.1: non-square quadtree partitioning. JVET-AF0108, 32nd JVET meeting, Hannover, 13-20 October 2023.

- R. Utida, F. Le Léannec, T. Dumas. Crosscheck of JVET-AF0108 (EE2-1.1: Non-square quadtree partitioning). JVET-AF0249, 32nd JVET meeting, Hannover, 13-20 October 2023.

# Patent rights declaration(s)

Interdigital does not have any current or pending patent rights relating to the technology described in this contribution.