| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0285-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AF0102 (EE1-1.1.2: Complexity-performance trade off of decomposition) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check report |  |  |
| Author(s) or Contact(s): | Yue Li | Email: | yue.li@bytedance.com |
| Source: | Bytedance |  |  |

_____________________________

- Abstract

This contribution reports the cross-check results for EE1-1.1.2 (JVET-AF0102). In sub-test EE1-1.1.2.a, the ranking of the two-components decomposition in residue block of the backbone, C31 parameter, is reduced from 64 to 48. In sub-test EE1-1.1.2.b, order of the decomposition (horizontal-vertical or vertical-horizontal) is alternated every block in the backbone to mitigate the impact of the rank reduction on decomposition symmetry.

Inference testing has been finished. The cross-checker was able to reproduce inference bd-rate results for both sub-tests. The encoder/decoder run-time reported by the cross-checker is obviously longer than the numbers reported by proponents. It is noted that this run-time increase seems to be consistent with other EE1 tests proposed by cross-checker.

Training have been finished for both sub-tests. Observed behaviour of validation loss functions follows the expected patterns for both EE1-1.1.2.a and EE1-1.1.2.b.The cross-checker has launched inference tests using the reproduced models. This document will be updated later after results are available.

Introduction

In proposed method tested in EE1-1.1.2.a, the ranking of the two-components decomposition in residue block of the backbone, C31 parameter, is reduced from 64 to 48.

To alleviate introduced decomposition asymmetry with reduced rank being used in horizontal direction only, in sub-test EE1-1.1.2.b, the order of the decomposition (horizontal/vertical) is being switched every residue block in the backbone. Two types of the residue block in backbone introduced. In type 1 (T1), residue bock implements 3x1 convolution with C31=48, followed by 1x3 convolution while in type 2 (T2), residue bock first performs 1x3 convolution with C31=48, followed by the 3x1 convolution. Network architecture is shown in Fig. 1

Fig. 1. Proposed model in EE1-1.1.2.b with switched order decompositions (Type 1 and Type2) and reduced rank, C31=48.

The simulation results reported in JVET-AF0102 is shown in Table 1 and Table 2.

Table 1. Summary of the inference test results, sub-test EE1-1.1.2.a vs NNVC6.0 HOP anchor, nnIntra ON.

|  | Proposed a vs NNVC6.0 HOP anchor |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  |  |  | Random Access |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.12% | 1.07% | 0.55% | 87% | 76% | 0.17% | 3.42% | 0.03% | 83% | 78% |
| Class A2 | 0.20% | 1.76% | 2.03% | 89% | 78% | 0.32% | 3.40% | 5.67% | 84% | 77% |
| Class B | 0.01% | 1.14% | 0.89% | 91% | 77% | 0.20% | 2.45% | -0.67% | 85% | 77% |
| Class C | 0.09% | 0.60% | 0.71% | 85% | 67% | 0.26% | 1.25% | 0.40% | 81% | 73% |
| Class E | 0.19% | 1.37% | 0.97% | 78% | 68% |  |  |  |  |  |
| Overall | 0.11% | 1.15% | 1.00% | 86% | 73% | 0.24% | 2.51% | 1.02% | 83% | 76% |
| Class D | 0.03% | 1.09% | 1.13% | 88% | 67% | 0.16% | 1.08% | -0.40% | 87% | 76% |

Table 2. Summary of the inference test results, sub-test EE1-1.1.2.b vs NNVC anchor, nnIntra ON.

|  | Proposed a vs NNVC6.0 HOP anchor |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  |  |  | Random Access |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.16% | 0.74% | -0.20% | 84% | 69% | -0.11% | 1.11% | -0.72% | 80% | 70% |
| Class A2 | 0.16% | 0.53% | 1.63% | 88% | 71% | 0.22% | 1.83% | 4.73% | 85% | 72% |
| Class B | 0.04% | -0.31% | 0.05% | 88% | 70% | 0.11% | 0.68% | -1.24% | 85% | 73% |
| Class C | 0.11% | -0.39% | 0.25% | 82% | 60% | 0.22% | 0.75% | 0.00% | 78% | 67% |
| Class E | 0.11% | 0.86% | 0.78% | 73% | 61% |  |  |  |  |  |
| Overall | 0.11% | 0.18% | 0.44% | 83% | 66% | 0.12% | 1.02% | 0.39% | 82% | 71% |
| Class D | 0.06% | 0.66% | 1.16% | 85% | 61% | 0.11% | -0.67% | -1.18% | 81% | 68% |

Cross-check of the Inference

Inference testing of models shard by the proponents has been finished and results are reported in Table 3 and Table 4.

Table 3. Summary of the inference test results from crosschecker, sub-test EE1-1.1.2.a vs NNVC6.0 HOP anchor, nnIntra ON.

|  | Proposed a vs NNVC6.0 HOP anchor |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  |  |  | Random Access |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.12% | 1.07% | 0.55% | 116% | 164% | 0.17% | 3.42% | 0.03% | 119% | 163% |
| Class A2 | 0.20% | 1.76% | 2.03% | 109% | 164% | 0.32% | 3.40% | 5.67% | 119% | 168% |
| Class B | 0.01% | 1.14% | 0.89% | 107% | 160% | 0.20% | 2.45% | -0.67% | 120% | 157% |
| Class C | 0.09% | 0.60% | 0.71% | 105% | 161% | 0.26% | 1.25% | 0.40% | 117% | 161% |
| Class E | 0.19% | 1.37% | 0.97% | 111% | 163% |  |  |  |  |  |
| Overall | 0.11% | 1.15% | 1.00% | 109% | 162% | 0.24% | 2.51% | 1.02% | 119% | 162% |
| Class D | 0.03% | 1.09% | 1.13% | 106% | 151% | 0.16% | 1.08% | -0.40% | 116% | 161% |

Table 3. Summary of the inference test results from crosschecker, sub-test EE1-1.1.2.b vs NNVC anchor, nnIntra ON.

|  | Proposed a vs NNVC6.0 HOP anchor |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  |  |  | Random Access |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.16% | 0.74% | -0.20% | 122% | 187% | -0.11% | 1.11% | -0.72% | 127% | 185% |
| Class A2 | 0.16% | 0.53% | 1.63% | 114% | 189% | 0.22% | 1.83% | 4.73% | 127% | 191% |
| Class B | 0.04% | -0.31% | 0.05% | 110% | 184% | 0.11% | 0.68% | -1.24% | 128% | 180% |
| Class C | 0.11% | -0.39% | 0.25% | 109% | 189% | 0.22% | 0.75% | 0.00% | 123% | 180% |
| Class E | 0.11% | 0.86% | 0.78% | 117% | 183% |  |  |  |  |  |
| Overall | 0.11% | 0.18% | 0.44% | 113% | 186% | 0.12% | 1.02% | 0.39% | 126% | 183% |
| Class D | 0.06% | 0.66% | 1.16% | 106% | 178% | 0.11% | -0.67% | -1.18% | 122% | 178% |

Cross-checker was able to reproduce inference bd-rate results. The encoder/decoder run-time reported by cross-checker is obviously longer than the numbers reported by proponents. It is noted that this run-time increase seems to be consistent with other EE1 tests proposed by cross-checker.

Cross-check of the training

Training of the EE1-1.1.2.a model and EE1-1.1.2.b model shared by the proponent has been finished. Inference tests using the reproduced models from crosschecker are in progress.

Observations from validation losses in the reproduced training meet expectation: EE1-1.1.2a's validation loss is slightly larger than HOP, while EE1-1.1.2b's validation loss is smaller than EE1-1.1.2a and almost on par with HOP loss.