INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 4

MPEG VIDEO CODING

ISO/IEC JTC 1/SC 29/WG 4 m 65556

October 2023, Hannover (DE)

Title: 	Considerations on LCEVC performances in relation to input document JVET-AF0075/m65560

Source: 	Simone Ferrara, Lorenzo Ciccarelli, Guendalina Cobianchi, Guido Meardi

Executive summary

This document is produced to provide constructive feedback on the methodology and the conclusions of input document JVET-AF0075 [1]  and a summary of the expected performance of MPEG-5 Low Complexity Enhancement Video Coding (LCEVC) on objective metrics vs. formal MOS, according to tens of extensive cross-checked evaluations performed over the course of LCEVC’s collaborative development phase as well as during its Verification Tests.

Most of the hypotheses and results presented in JVET-AF0075 had already been evaluated, expanded, discussed at length, and resolved during the development and verification of LCEVC, so this document aims to provide useful reminders for experts that may have been less involved in the standardization of LCEVC. Over the course of 11 MPEG meetings (MPEG 124 to MPEG 134), there have been over 30 documents evaluating the performances of LCEVC in terms of PSNR, VMAF, and formal subjective assessments, including LCEVC’s Verification Tests (m54455[10]) and several evaluations of LCEVC using the same sequences as in [1] (such as N19160 [9], m53796 [6], m52997 [3] and m53523 [7]). A good summary of LCEVC tests preceding Verification Tests can be found in N19571 [5].

All the above test results, including those from Verification Tests, are aligned with results in JVET-AF0075 when it comes to PSNR. In previous tests, VMAF results complemented PSNR on objective metrics, as requested by the CTC (following evidence presented in m51438 [8] which proved the lack of correlation between PSNR and subjective performance and a higher correlation between VMAF and formal subjective performances) and, more importantly, ITU-R BT.500 formal MOS results consistently disprove some of the conclusions in JVET-AF0075 with regards to the correlation of objective metrics, and PSNR in particular, with visual quality. In this document we report a selection of previous conclusions.

Furthermore, the tests in JVET-AF0075 have been configured using the LTM at bitrates that are not consistent with the LCEVC CTC, according to which LCEVC encodings should have been “designed in order to achieve bitrates compatible with mass market distribution and quality levels amenable to subjective testing” as outlined in m52997 [3]. Testing very high data rates (e.g., up to 100 Mbps) using LTM default configurations originally aimed at operating points below 20 Mbps may lead to questionable conclusions, which therefore pertain to the specific settings and implementation of the current LTM, rather than to the LCEVC format per se. For instance, due to the intended range of LTM’s default configurations, the LCEVC quantization stepwidths used at very high bitrate were much higher (like almost an order of magnitude higher, as shown by configurations used in past tests at very high bitrates highlighted in N19571 et al.) than it is necessary for said very high levels of quality, consequently producing suboptimal results. Furthermore, LTM default configurations were calibrated for VTM 11 and should be adapted to the latest VTM.

In addition to LTM tests, this document presents the results obtained with commercial implementations of LCEVC (within ffmpeg) using two different base codecs – namely HEVC (in its x265 implementation) and VVC (in its beta MainConcept implementation) on the same set of contents used in JVET-AF0075.  Results are presented in terms of a wide range of objective metrics (including PSNR, VMAF, MS-SSIM and VMAF_NEG) with some sample screenshots to support the metrics. Once again, results are in line with previous tests, showing a very different behaviour (from very good to very bad) depending on the metric and on luma vs. chroma. It is thus very risky to draw conclusions by focusing on a single metric, in particular if that metric is PSNR, which for LCEVC consistently demonstrated lack of correlation with ground-truth formal MOS results, for reasons that were largely identified and clarified during the LCEVC standardization process.

This document also provides data about the possible luma and chroma settings to be used with LCEVC. These examples demonstrate the effects of different luma-chroma configurations on both visual quality and BD-rate gains for the sequences tested.  Additionally, the document confirms the potential validity of luma-based metrics like VMAF for LCEVC evaluations, consistent with other video coding schemes and contrary to the conclusions made in JVET-AF0075. LCEVC can indeed encode also chroma residuals (up to lossless if needed), and the default luma-only configuration is just a minor design choice aimed at optimising the visual quality impact of the LCEVC enhancement.  In any case, even when LCEVC residuals are applied to chroma planes with smaller quantization step-width than luma plane, the negative impact on the BD-rate of luma metrics, such as VMAF, is very small (< 2 percentage point) and does not materially affect the results.

Finally, this document also provides a reminder of the importance of the Low Complexity requirements of LCEVC, which provide uniquely impactful benefits in many of its real-world use cases.

# Reminder of previous conclusions relative to LCEVC Test Results on Objective Metrics vs. ITU BT.500 Formal MOS

Since the beginning of the development of LCEVC - in fact, even when preparing the Call for Proposal - there have been numerous studies and investigations conducted around LCEVC performances, particularly focusing on performances in terms of objective metrics such as PSNR and whether they would correlate with actual visual quality of the sequences.

Over the course of 11 MPEG meetings (MPEG 124 to MPEG 134), there have been over 30 documents specifically focusing on the evaluation of performances of LCEVC in terms of PSNR, VMAF, and formal MOS subjective assessments. This collaborative effort involved 14 contributing companies/universities and many video experts who have reviewed the evidence provided, discussed it, cross-checked the results and reached conclusions in the various MPEG meetings where those documents were produced.

That body of work shows that the majority of findings in JVET-AF0075 had been previously assessed, extensively discussed, and addressed during LCEVC's development and verification, even using some of the same test sequences.

LCEVC evaluations consistently demonstrated a) relatively poor BD-rate benefits (or even loss) on PSNR-Y, b) BD-rate benefits on VMAF, and c) much larger benefits on ground-truth formal MOS. VMAF was consistently found to be the objective metric best correlated with formal subjective assessments, although also VMAF consistently understated LCEVC benefits vs. formal MOS assessments.  We will refer to a selection of those evaluations as a reminder of some of the key findings and decisions taken, relevant to the points raised in [1].

## Examples of LCEVC evaluations performed before and as part of the Verification Tests

Contribution m54455 [10] by Università Politecnica delle Marche (UNIVPM) provided an in-depth study and analysis of the performance of LCEVC enhancing, among other codecs, HEVC and VVC. The study focused on six 4K/UHD sequences, of which three were also included in JVET-AF0075.

A summary of the results are shown below, including both PSNR-Y and VMAF BD-rate calculated on an extended bitrate range (QP 42 to 22 for the HM and VTM anchors).

The study concluded that:

“From the experimental results obtained with the 6 sequences under test, it is already clear that the choice of SW2 with different sequences and different bitrates has an impact on both the PSNR and VMAF performance, that can be optimized with an optimized choice of SW2 depending on the type of sequence and the bitrate”,

So, this study highlighted that the configuration of LCEVC – which follows a different logic than that of a single-layer codec – is critical in making a fair assessment and comparison of the performances when compared with single-layer formats. Incidentally, similar conclusions seem to have been reached in recent studies on multi-layer VVC  JVET-AB0045-v2 [17],[17]

This study clarified that the main goal of the experiments was performing visual assessments (i.e., objective metrics measurement was functional to subjective assessments), and experiments were conducted to determine the best configurations and bitrate ranges for this purpose.

The discussions resulted in output document N00029 (w19827), which included the procedure used to find the best configuration parameters for LCEVC testing and their corresponding bitrate range limitations.

The performances of LCEVC using LTM 3.0 were further tested in N19160 [8] by V-Nova and ATEME, where cross-checked results for LTM 3.0 were reported. Importantly, this study provided subjective assessment evaluations across the CTC sequences, measured in terms of formal MOS.  The study showed very limited correlation of MOS with objective metrics, and in particular inverse correlation with PSNR-Y. Below we report the table for LCEVC-enhanced HM.

Table 1 – BD-rate PSNR, VMAF and MOS for LCEVC-enhanced HM

The N19160 results show, again, poor performances in terms of PSNR, however, the VMAF performances and, more importantly, the ground-truth ITU-R BT.500 formal subjective performances show significant improvement provided by LCEVC over the HM anchors. N19160 also provides an important clarification on how to interpret result for layered coding structures such as LCEVC was presented, namely:

“LCEVC requires to compare a traditional coding structure with a “layered” coding structure leveraging non-overlapping transform sizes as well as scaling operations, which is known to decrease the reliability of objective metrics […]. Comparing different video codecs with simple objective metrics implicitly assumes that the tested codecs treat pixels in a similar way: this is not the case for LCEVC, and not all pixels are created equal.

In fact, LCEVC does not necessarily perform badly with metrics.  Simply, objective metric BD-rates often diverge materially across different metrics (e.g., PSNR vs. VMAF), and subjective assessments often diverge from conclusions that can be drawn by just looking at metrics.  For all tests conducted so far by the proponents and/or the BOG, formal subjective assessments double-checked with expert viewing have showed materially higher BD-rate benefits than it was suggested by objective metrics.”

LTM 4.0 was tested enhancing HM in m52997 [3], with consistent results to previous tests.  The table below, for example, showed the performances of the LTM 4.0 using the configurations proposed in [3] reporting PSNR, VMAF and formal subjective MOS assessments.

Once again, in the above graphs it is evident that formal subjective evaluations show some correlation with VMAF and limited or even inverse correlation with PSNR.

Afterwards, several dry-run visual quality assessments for the Verification Test were conducted in MPEG 132 and MPEG 133, including a formal subjective visual quality assessment in MPEG 133 which reported an early indication of the expected results for the verification test – see the report m56225 [12] by the Test Chair. Further analyses were provided in m56013[13].

Finally, the verification test report was provided in N0076 (m20173) [15] in April 2021. The metrics (PSNR and VMAF) and the MD5 checksum used in the verification tests were reported (and cross-checked) in m56330[16], with the following BD-rates results for HM 16.2 and VTM 11:

| HM vs LTM - UHD | PSNR | VMAF | MOS |  | VTM11 vs LTM – UHD | PSNR | VMAF | MOS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LupoPuppet | -23.90% | -27.12% | -31.14% |  | Marathon2 | 4.10% | -9.38% | -16.86% |
| CatRobot | 0.03% | -14.47% | -41.88% |  | MountainBay2 | 4.57% | -6.84% | -16.90% |
| DrivingPoV_Logo | 30.50% | -9.45% | -26.01% |  | DrivingPoV_Logo | 65.92% | 4.84% | -10.33% |
| Boxe_Logo | 16.63% | -8.00% | -24.44% |  | Boxe_Logo | 50.98% | 10.63% | -18.55% |
| Average Class-A | 5.81% | -14.76% | -30.87% |  | Average Class-A | 31.39% | -0.19% | -15.66% |

In summary, we conclude that the results presented in JVET-AF0075 were already reviewed and discussed within MPEG in combination with other performance metrics, and those results should be considered with caution as they only provide a very partial view for evaluation of LCEVC.

## Operating points and bitrate ranges to be used for evaluations

In JVET-AF0075 [1], the authors provide an evaluation of LCEVC in terms of PSNR using the JVET Random Access (RA) Common Testing Conditions (CTC).

We note that the JVET CTC extends to a range of bitrates which is well beyond the “bandwidths and operating conditions relevant to mass market distribution” which were in the Requirement for LCEVC as outlined in N18098 [4] (which specifies that the compression efficiency targets were to be reached “at bandwidths and operating conditions relevant to mass market distribution”).

It is for this reason that the CTC for LCEVC, including configurations of LTM and anchors, was focused around a different range of bitrates, typically below 15 Mbps for UHD sequences – with the HM anchors ranging from QP 26-27 to QP 39-40, depending on the sequence – see also N18988 [11]. For the Verification Tests the QP values chosen for the anchors ranged between QP 27 and QP 42.

If we use the PSNR and bitrate values reported in [1], and limit the lowest QP to QP 27 – thus excluding QP 22 which generates bitrates over 100 Mbps as they are not within “bandwidths and operating conditions relevant to mass market distribution” – the results are different and more aligned with previous tests reported above (e.g., [3]).

Table 2 - Comparison of results of HEVC + LCEVC vs HEVC as anchor, on the left limiting the range to the LCEVC CTC range, on the right using the range proposed in JVET-AF0075

Table 3 - Comparison of results of VVC + LCEVC vs VVC as anchor, on the left limiting the range to the LCEVC CTC range, on the right using the range proposed in JVET-AF0075

The above results were already known and have already been discussed in previous MPEG meetings. In addition to the N19160 [9] output document discussed [9]above, other documents, such as m53796 [6] (a study done by Ateme and Mediakind, using LTM 3.0 and HM 16.20) and m53523 [7] (one of a couple of studies done by Zhejiang University using LTM 4.0 and HM 16.20) show performances both in terms of PSNR and VMAF consistent with the above.

## Integration of LCEVC in TMIV and evaluation with VVC in MIV

The documents m60330 and m60740 [29] by V-Nova and Philips summarize the results of the exploration experiments using LCEVC in combination with VVC as mezzanine encoder in TMIV [30], for MPEG Immersive Video (MIV). The experiment proposed to test the coding efficiency of MIV views using the LCEVC and VVC Test Models to compress the material before being encoded by the MIV framework. In this experiment the aim was to use LCEVC-enhanced VVC to compress the atlases generated by the MIV encoder to evaluate the coding efficiency and the encoding runtime speed-up provided by LCEVC-enhanced VVC compared to native VVC. The results are shown in the table below. It can be observed that LCEVC can bring an average savings of between 30% and 40% for VMAF (with peaks of 60%) and around 5% for SSIM (with peaks of over 9%). However, the test shows a loss of about 16% in PSNR.  Once again, objective metrics results were consistent with all previous evaluations, so the team proceeded with subjective evaluations to assess ground truth.

| Sequence |  | High-BR BD rate Y-PSNR | Low-BR BD rate Y-PSNR | High-BR BD rate IV-PSNR | Low-BR BD rate IV-PSNR |  | High-BR BD rate VMAF | Low-BR BD rate VMAF | High-BR BD rate SSIM | Low-BR BD rate SSIM | High-BR BD rate MS-SSIM | Low-BR BD rate MS-SSIM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Museum | B | 10.0% | -3.7% | 10.1% | 4.2% |  | -5.9% | -3.4% | 0.9% | -3.9% | 16.6% | 6.0% |
| Fan | O | 13.4% | 6.8% | 8.1% | 3.5% |  | -49.6% | -46.3% | -4.8% | -3.0% | -1.0% | -1.2% |
| Painter | D | 16.1% | 0.7% | -1.5% | -4.9% |  | -49.6% | -47.8% | -7.5% | -9.1% | -1.2% | -6.9% |
| Frog | E | 12.0% | 5.2% | -11.6% | -6.3% |  | -60.2% | -52.0% | -7.3% | -4.5% | -1.9% | -1.9% |
| Chess | N | 7.9% | 11.9% | 9.9% | 1.6% |  | -10.2% | -2.5% | -9.3% | -5.3% | 20.9% | 6.0% |
| Fencing | L | 36.0% | 3.5% | -2.3% | -11.4% |  | -55.9% | -46.6% | -4.6% | -8.5% | 2.2% | -4.6% |
| Average |  | 15.9% | 4.1% | 2.1% | -2.2% |  | -38.6% | -33.1% | -5.4% | -5.7% | 5.9% | -0.4% |

After having confirmed the benefits of LCEVC by visually inspecting the sequences, a formal subjective evaluation of the pose traces was conducted to verify if the visual quality improvements could be observed also by naïve viewers. For each sequence 3 pose-traces per rate were evaluated according to the MIV CTC, using 16 naïve viewers.

The average MOS scores for each sequence, rate and pose-traces are reported in table below, comparing the VVC anchor and LCEVC-enhanced VVC as target (i.e., with negative BD-rate values meaning that LCEVC produces a benefit).

LCEVC enhancing VVC brought an average savings between 4% and 20% across sequence on the same pose-traces and between 0% and 20% for on the same sequence but across all the pose-traces.  These visual quality results were also complemented by the processing speed benefits offered by LCEVC (despite it being used at higher bit depth than native VVC and without spatial scaling), which are particularly important considering the high resolutions required by MIV.

| Sequence |  | P01 | P02 | P03 | Average |
| --- | --- | --- | --- | --- | --- |
| Museum | B | -0.8% | -28.7% | 3.5% | -8.7% |
| Fan | O | -22.7% | -23.4% | -14.6% | -20.2% |
| Painter | D | -26.4% | 0.5% | 15.5% | -3.5% |
| Frog | E | -16.9% | -2.6% | -20.6% | -13.4% |
| Chess | N | -9.9% | 32.8% | -22.9% | 0.0% |
| Fencing | L | -39.5% | -4.9% | -13.7% | -19.3% |
| Average |  | -19.4% | -4.4% | -8.8% |  |

## LTM configurations

In JVET-AF0075 [1], the authors used a configuration for LTM corresponding to the one used in the LCEVC Verification Test.

Being an enhancement video coding technology, LCEVC configuration is typically dependent on the configuration as well as the specific implementation of the video coding technology it enhances.

This was for example shown in document m52997 [3], where a study of different configurations for LTM 4.0 was provided. That study showed that by simply changing a single parameter in LTM 4.0 the amount of bitrate used in LCEVC changes significantly and with it also the performances.

As explained in Section 1.2 above, the configurations used for the Verification Tests were defined for a range of bitrates consistent with the Requirements, i.e. “bitrates compatible with mass market distribution and quality levels amenable to subjective testing”. This can be seen, for example, by noting that for stepwidths in sub-layer 2 (SW2) with a value below 1,500, the QPs of the base codec should be negative!

In addition, we note that in [1] the versions of HM and VTM differ from those used in the Verification Tests. For example, in the Verification Test VTM 11.0 was used, whereas in [1] VTM 19.0 was used. The change of version for the base codec would have an impact on the configuration of LTM, therefore the configurations used in the Verification Tests are likely not to be applicable with different versions of base codecs.

As such, the results presented in JVET-AF0075 should be considered with caution, since the configurations used for both LCEVC and the base codecs are i) suboptimal for the higher bitrate operating points and ii) most likely suboptimal also at the lower bitrate operating points. Having said that, the direction of objective metric results in JVET-AF0075 is broadly in line with past LCEVC evaluations and thus already studied and addressed at length during the LCEVC standardization phase.

# Assessment with commercially available implementations

## Testing conditions and settings used

For LCEVC we have used the LCEVC implementation provided by V-Nova whereas for the enhanced codecs we have used the open-source implementation of HEVC (namely, x265 [21]) and the MainConcept implementation of VVC [22]. Tests were performed for 4K content (Class A test sequences) as presented in JVET Random Access (RA) Common Testing Conditions (CTC). The encoded streams are made available, with link provided in the Appendix.

For this test, in line with JVET-AF0075, the following six 4K sequences were encoded.

Table 4  - Test sequences and sequence settings

| Class | Sequence name | Frame count | Frame rate | Bit depth |
| --- | --- | --- | --- | --- |
| A1 | Tango2 | 294 | 60 | 10 |
| A1 | FoodMarket4 | 300 | 60 | 10 |
| A1 | Campfire | 300 | 30 | 10 |
| A2 | CatRobot | 300 | 60 | 10 |
| A2 | DaylightRoad2 | 300 | 60 | 10 |
| A2 | ParkRunning3 | 300 | 50 | 10 |

The following LCEVC software enhancing HEVC and VVC implementations were used:

- Ffmpeg (version 6.0) build of LCEVC production code, release 3.9.4 (hash f467a7620), available for testing upon registration at https://download.v-nova.com/

- HEVC: Libx265 (version 3.5), as anchor and corresponding base layer encoder, used in combination with LCEVC;

- VVC: MainConcept Beta VVC implementation, as anchor and corresponding base layer encoder, used in combination with LCEVC.

All encodings have been performed with CBR rate control for all tests at bitrates consistent to those of JVET-AF0075, as illustrated in the table below.

Table 5 - Test rates for HEVC and VVC tests

|  | HEVC |  |  |  | VVC |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence name | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 1 | Rate 2 | Rate 3 | Rate 4 | Rate 5 |
| Tango2 | 2,509 | 4,415 | 8,051 | 18,491 | 1,115 | 1,956 | 3,505 | 6,692 | 15,783 |
| FoodMarket4 | 2,583 | 4,785 | 8,861 | 17,888 | 1,090 | 2,026 | 3,782 | 7,245 | 14,300 |
| Campfire | 5,533 | 10,002 | 16,951 | 52,226 | 1,645 | 3,495 | 6,594 | 12,167 | 46,144 |
| CatRobot | 2,987 | 5,429 | 10,125 | 21,410 | 1,165 | 2,135 | 3,988 | 7,837 | 16,567 |
| DaylightRoad2 | 2,793 | 5,312 | 10,329 | 24,855 | 1,082 | 2,038 | 3,939 | 8,043 | 20,764 |
| ParkRunning3 | 9,773 | 21,874 | 46,082 | 112,320 | 3,289 | 7,733 | 17,376 | 39,186 | 97,288 |

More details on command lines and generated encodes can be found in the Appendix 1.

## Experimental results - Objective metrics

The following tests are performed, encoding LCEVC in different configurations, including the default configuration – in which the LCEVC enhancement is limited to the luma plane – and an alternative configuration, activated by with command encoding_plane_mode=yuv, in which the LCEVC enhancement is applied to both luma and chroma planes, with equal quantization stepwidth for both.  Additionally, for the sake of stress-testing chroma residual application, we tested a “chroma enhanced” configuration, achieved via the command encoding_plane_mode=yuv;chroma_sw_multiplier=45, which assigns more weight to chroma residuals, specifically applying a chroma quantization stepwith that is 30% lower than the luma one.

For HEVC vs. LCEVC-enhanced HEVC we performed the following tests:

- Test1. HEVC: x265 (single layer encoding of 4K content);

- Test2. LCEVC x265 default configuration, with LCEVC enhancement on luma only: x265 (1080p base encoder) + LCEVC (4K enhancement layer);

- Test3. LCEVC x265, with LCEVC enhancement on luma and chroma: x265 (1080p base encoder) + LCEVC (4K enhancement layer, adding encoding_plane_mode =yuv);

- Test4. LCEVC x265, with LCEVC “Chroma enhanced” (LCEVC enhancement on luma and chroma, with chroma step-width reduced by 30% vs. the luma step-width): x265 (1080p base encoder) + LCEVC (4K enhancement layer, adding encoding_plane_mode=yuv;chroma_sw_multiplier=45). 
Note that the same settings can be enabled in the LTM (as they are part of LCEVC tools) by using   
--num_processed_planes=3 and --chroma_step_width_multiplier=45.

For VVC vs LCEVC-enhanced VVC:

- Test5. VVC: MainConcept VVC (single layer encoding of 4K content);

- Test6. LCEVC VVC default configuration, with LCEVC enhancement on luma only: MainConcept VVC (1080p base encoder) + LCEVC (4K enhancement layer);

In the tests, a comprehensive set of metrics are computed and used to calculate BD-rates:

- PSNR_Y (computed for luma component), PSNR_U and PSNR_V (computed for two chroma components);

- SSIM_Y (computed for luma component), SSIM_U and SSIM_V (computed for two chroma components);

- MS-SSIM_Y (computed for luma component), MS-SSIM_U and MS-SSIM_V (computed for two chroma components);

- Libvmaf (version 2.3.1) metrics [23][23] (marked with the suffix [VMAF]), computed for luma component:

  - [VMAF] SSIM - Structural Similarity Index Method

  - [VMAF] MS-SSIM - Multi Scale Structural Similarity Index Method

  - [VMAF] VMAF - Video Multimethod Assessment Fusion

  - [VMAF] VMAF-NEG - Video Multimethod Assessment Fusion No Enhancement Gain

BD-rates are illustrated in Table 6, Table 7, Table 8 and Table 9.

Table 6 - BD-rate (%) results of Test2 (x265 + LCEVC default luma-only) vs Test1 (x265) as anchor.

| Sequence | Y, U, V metrics |  |  |  |  |  |  |  |  | Y-only VMAF metrics |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PSNR |  |  | SSIM |  |  | MS-SSIM |  |  | [VMAF] SSIM | [VMAF]MS-SSIM | [VMAF]VMAF | [VMAF] VMAF_NEG |
|  | PSNR_Y | PSNR_U | PSNR_V | SSIM_Y | SSIM_U | SSIM_V | MS-SSIM_Y | MS-SSIM_U | MS-SSIM_V |  |  |  |  |
| ParkRunning3 | -9.40 | 146.40 | 90.99 | -4.99 | 131.03 | 89.44 | -8.10 | 30.51 | 21.60 | -11.13 | -8.27 | -45.78 | -34.31 |
| CatRobot | -1.89 | 1.88 | 12.49 | -1.61 | 5.59 | 10.31 | -12.20 | -5.63 | -3.65 | -16.15 | -11.76 | -33.17 | -22.08 |
| Tango2 | -33.40 | -45.11 | -36.66 | -38.83 | -65.72 | -55.19 | -34.54 | -45.39 | -38.07 | -30.73 | -34.80 | -39.72 | -34.85 |
| FoodMarket4 | -25.35 | -25.19 | -26.73 | -24.13 | -22.79 | -27.05 | -27.38 | -19.54 | -22.10 | -26.92 | -27.23 | -39.54 | -34.57 |
| DaylightRoad2 | -1.42 | -18.20 | -4.36 | -3.89 | -23.73 | -0.10 | -18.93 | -20.16 | -12.90 | -20.11 | -18.38 | -32.92 | -23.36 |
| Campfire | -19.41 | 54.18 | -5.14 | -19.84 | 37.08 | -12.92 | -33.19 | -19.89 | -26.23 | -36.74 | -32.83 | -49.33 | -38.74 |
| Overall | -15.15 | 18.99 | 5.10 | -15.55 | 10.24 | 0.75 | -22.39 | -13.35 | -13.56 | -23.63 | -22.21 | -40.08 | -31.32 |

Table 7 - BD-rate (%) results of Test3 (x265 + LCEVC, luma+chroma) vs Test1 (x265) as anchor

| Sequence | Y, U, V metrics |  |  |  |  |  |  |  |  | Y-only VMAF metrics |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PSNR |  |  | SSIM |  |  | MS-SSIM |  |  | [VMAF] SSIM | [VMAF]MS-SSIM | [VMAF]VMAF | [VMAF] VMAF_NEG |
|  | PSNR_Y | PSNR_U | PSNR_V | SSIM_Y | SSIM_U | SSIM_V | MS-SSIM_Y | MS-SSIM_U | MS-SSIM_V |  |  |  |  |
| ParkRunning3 | -5.30 | 120.16 | 84.07 | -1.83 | 115.49 | 82.81 | -5.27 | 30.85 | 22.53 | -8.80 | -5.47 | -44.31 | -31.78 |
| CatRobot | -1.37 | 1.81 | 11.99 | -1.15 | 5.63 | 10.17 | -11.79 | -5.64 | -3.75 | -15.76 | -11.35 | -32.84 | -21.68 |
| Tango2 | -33.34 | -45.33 | -37.45 | -38.83 | -65.82 | -55.60 | -34.51 | -45.58 | -38.64 | -30.71 | -34.78 | -39.68 | -34.80 |
| FoodMarket4 | -25.23 | -25.26 | -26.86 | -24.00 | -22.80 | -27.07 | -27.22 | -19.52 | -22.11 | -26.74 | -27.08 | -39.38 | -34.40 |
| DaylightRoad2 | -1.13 | -18.51 | -5.00 | -3.64 | -23.89 | -0.54 | -18.77 | -20.43 | -13.23 | -19.96 | -18.20 | -32.73 | -23.10 |
| Campfire | -16.72 | 58.55 | -13.62 | -17.52 | 32.53 | -16.30 | -31.78 | -19.87 | -26.44 | -35.63 | -31.41 | -48.11 | -36.75 |
| Overall | -13.85 | 15.24 | 2.19 | -14.49 | 6.86 | -1.09 | -21.56 | -13.37 | -13.61 | -22.93 | -21.38 | -39.51 | -30.42 |

Table 8  - BD-rate (%) results of Test4 (x265 + LCEVC, chroma-enhanced) vs Test1 (x265) as anchor.

| Sequence | Y, U, V metrics |  |  |  |  |  |  |  |  | Y-only VMAF metrics |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PSNR |  |  | SSIM |  |  | MS-SSIM |  |  | [VMAF] SSIM | [VMAF]MS-SSIM | [VMAF]VMAF | [VMAF] VMAF_NEG |
|  | PSNR_Y | PSNR_U | PSNR_V | SSIM_Y | SSIM_U | SSIM_V | MS-SSIM_Y | MS-SSIM_U | MS-SSIM_V |  |  |  |  |
| ParkRunning3 | -3.01 | 105.88 | 76.95 | 0.28 | 107.40 | 78.98 | -2.98 | 30.41 | 22.47 | -6.57 | -3.29 | -42.70 | -30.03 |
| CatRobot | 0.08 | 1.50 | 11.06 | 0.11 | 5.49 | 9.76 | -10.72 | -5.94 | -4.17 | -14.76 | -10.28 | -31.89 | -20.54 |
| Tango2 | -32.29 | -45.14 | -38.43 | -37.95 | -65.70 | -56.04 | -33.57 | -45.40 | -39.36 | -29.76 | -33.84 | -38.81 | -33.81 |
| FoodMarket4 | -24.53 | -25.19 | -26.93 | -23.31 | -22.50 | -26.80 | -26.51 | -19.27 | -21.93 | -26.00 | -26.37 | -38.75 | -33.74 |
| DaylightRoad2 | 0.11 | -19.37 | -6.28 | -2.54 | -24.34 | -1.59 | -17.95 | -21.32 | -14.19 | -19.17 | -17.35 | -31.77 | -21.98 |
| Campfire | -13.09 | 56.88 | -16.35 | -14.41 | 29.04 | -17.58 | -29.61 | -19.05 | -26.47 | -33.68 | -29.19 | -46.04 | -33.91 |
| Overall | -12.12 | 12.43 | 0.00 | -12.97 | 4.90 | -2.21 | -20.22 | -13.43 | -13.94 | -21.66 | -20.06 | -38.33 | -29.00 |

Table 9 - BD-rate (%) results of Test6 (MC VVC + LCEVC default luma-only) vs Test5 (MC VVC) as anchor.

| Sequence | Y, U, V metrics |  |  |  |  |  |  |  |  | Y-only VMAF metrics |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PSNR |  |  | SSIM |  |  | MS-SSIM |  |  | [VMAF] SSIM | [VMAF]MS-SSIM | [VMAF]VMAF | [VMAF] VMAF_NEG |
|  | PSNR_Y | PSNR_U | PSNR_V | SSIM_Y | SSIM_U | SSIM_V | MS-SSIM_Y | MS-SSIM_U | MS-SSIM_V |  |  |  |  |
| -6.85 | -5.9 | 111.54 | 77.80 | -1.47 | 105.29 | 82.81 | -5.62 | 33.24 | 25.95 | -6.85 | -5.9 | -22.85 | -15.87 |
| 0.22 | 3.66 | 30.88 | 38.45 | 13.06 | 46.05 | 44.60 | 3.61 | 20.28 | 20.09 | 0.22 | 3.66 | -7.64 | 1.08 |
| -4.46 | -3.54 | 4.03 | 6.50 | -1.61 | 10.32 | 7.74 | -3.42 | 2.39 | 1.58 | -4.46 | -3.54 | -12.79 | -7.80 |
| -4.07 | -2.46 | 4.58 | 3.35 | 0.91 | 16.71 | 13.46 | -2.44 | 5.81 | 3.96 | -4.07 | -2.46 | -11.82 | -8.04 |
| -4.01 | 2.83 | 23.14 | 35.98 | 21.18 | 40.56 | 57.49 | 2.32 | 18.23 | 22.65 | -4.01 | 2.83 | -3.54 | 3.82 |
| -15.78 | -12.77 | 31.34 | 13.96 | -1.11 | 31.54 | 16.93 | -13.14 | -7.44 | -6.25 | -15.78 | -12.77 | -14.70 | -7.18 |
| -5.82 | -3.03 | 34.25 | 29.34 | 5.16 | 41.74 | 37.17 | -3.12 | 12.09 | 11.33 | -5.82 | -3.03 | -12.22 | -5.66 |

The BD-rates above, performed with a commercial LCEVC implementation, are consistent with results provided in previous LCEVC tests. They show a very different behaviour (from very bad to very good) depending on the metric, as well as for luma vs. chroma. It is hence very risky to draw conclusions by focusing on a single metric, in particular if that metric is PSNR in its Y-, U- and V- components, for which all past LCEVC tests consistently demonstrated lack of correlation with ITU-R BT.500 formal subjective MOS, which establish the ground truth for visual quality assessment.

Moreover, by comparing BD-rates across the three planes between the “luma-only” LCEVC HEVC tests (Table 3) and the “luma + chroma” LCEVC HEVC tests (Tables 4), we show that U- and V-metrics improve when the chroma component is added to LCEVC. As an example, the overall BD-rate gain on PSNR_U and PSNR_V improves by 3.8 p.p. and 2.9 p.p. respectively. Conversely, the opposite impact is registered on Y-metrics, as an example PSNR_Y BD-rate is reduced by 1.3 p.p. (from -15.1% to -13.8%), VMAF by 0.6 p.p. (from -40.1% to -39.5%), VMAF-NEG by 0.9 p.p. (from -31.3% to -30.4%). Regardless of the configuration chosen for luma vs. chroma, the size of the impact on Y BD-rates is relatively small and not such to materially affect the results.

Similarly, when the “enhanced chroma” configuration is used, BD-rates in U- and V-planes are further improved, while on Y-plane BD-rates are reduced. Once again, even in this relatively extreme case, BD-rate in VMAF is just reduced to -38.3% from the original -40.1%, maintaining the overall significance of the result.

We conclude that the behaviour and the trade-offs of LCEVC in the context of luma vs. chroma is analogous to that of any video coding scheme (especially in consideration of the fact that the majority of chroma bitrate is inevitably included in the base layer), and thus the use of Y-only metrics like VMAF is as justified as it is for any other video coding scheme.

## Subjective results

Objective scores often provide conflicting information between luma and chroma and among different objective metrics. ParkRunning is an obvious example, with chroma metrics (e.g., PSNR and SSIM in U- and V-planes) pointing to a clear superiority of Test1 (x265) over Test2 (LCEVC x265), while luma metrics suggesting an opposite insight, with Test2 winning over Test1.

Using a YUV player (i.e., Vooya YUV player [24]) capable of displaying Y- U- and V-planes independently, we report visual observations on chroma-only (e.g., U-plane) and on full YUV visualization. In the first two chroma-only screenshots, displaying ParkRunning Rate 3 on U-plane (example at frame 158), Test1 shows a visibly sharper, more detailed image compared to Test2, consistently to metrics.

U-plane: Test1. x265, 46.5 Mbps

PSNR_U: 33.7 dB

U-plane: Test2. LCEVC x265, 38.7 Kbps

PSNR_U: 31.0 dB

The second pair of screenshots displays the full YUV image. When combining luma with chroma, the quality loss in U-plane is not perceptible and the overall picture quality seems to be more aligned to what suggested by Y-only metrics.

YUV: Test1. x265, 46.5 Mbps

PSNR_Y: 37.1 dB, VMAF: 92.6

YUV: Test2. LCEVC x265, 38.7 Mbps

PSNR_Y: 36.6 dB, VMAF: 95.9

Below screenshot examples for VVC, showing a sample of Test 5 (VVC) vs. Test 6 (LCEVC + VVC) on Campfire.

Chroma metrics, in the example below PSNR_U, suggest Test5 (VVC) superiority over Test6 (LCEVC+VVC), in fact more details are perceivable in the first Test5 screenshot, combined though with more blocking artifacts.

YUV: Test5. VVC, 6.75 Mbps

PSNR_U: 34.1 dB

(frame 59)

YUV: Test6. LCEVC VVC, 6.52 Mbps

PSNR_U: 33.1 dB

(frame 59)

The following screenshots represent the combination of all three planes.

YUV: Test5. VVC, 6.75 Mbps

PSNR_Y: 35.4 dB, VMAF: 85.5

(frame 59)

YUV: Test6. LCEVC VVC, 6.52 Mbps

PSNR_Y: 35.31 dB, VMAF: 86.6

(frame 59)

Test6 screenshot, despite slightly softer on selected details (i.e., it’s possible to crop a moving object that may look softer in frame-by-frame comparison), shows less visible motion artefacts and/or blocking/banding, specifically on the flag and in the smoke close to the fire. In motion, this is making the LCEVC-enhanced VVC video more consistent and overall more pleasant to watch than the native VVC one, but it’s hardly reflected in frame-based metrics, PSNR in particular, and/or side-by-side frame-based comparisons.

LCEVC, as a multi-layer format, is not unique in is its lack of correlation between PSNR and subjective performance, demonstrated over and over during the LCEVC standardization process. The issue is known in literature [18] [19] [20] and a recent JVET input document JVET-AD0288-v2 [25] observed with regards to multilayer VVC  that ‘Remarkably, the relation of PSNR-over-rate curves diverges significantly from the relation of the corresponding MOS-over-rate curves'.  We were thus surprised to read some of the conclusions about PSNR in JVET-AF0075 [1].

After all, PSNR is just the frame-based logarithm of the ratio between the squared maximum pixel value and Mean Squared Error.  When performing visual quality assessment of LCEVC vs. single layer anchors, it is always recommended to perform subjective observations; ITU-R BT.500 formal MOS is the ‘ground truth’ to draw conclusions, while assessment approaches based on PSNR and/or chroma metrics do not appear reliable methods to drive conclusions.

## Further experiments with LCEVC chroma enhancement

The following screenshots of LCEVC-enhanced videos demonstrate the impact on subjective quality of applying LCEVC residuals exclusively on the luma plane, or on sharing the residual budget between luma and chroma. This experiment compares sample LCEVC encodes from Test2 (LCEVC on luma only) vs. Test 4 (LCEVC on luma and chroma, over-weighting the chroma component).

In this example, we focus on a detail where Test4 allocates a significant amount of residuals to the U-plane, investing bitrate in a detailed reconstruction of the plant in Chroma, while Test2 allocates zero residuals in U (and V), this is shown in the screenshots of first line of table 10. However, observing the subjective impact of such a chroma investment in the full YUV visualization, no visual gain is perceivable.

Table 10 – LCEVC x265, detail of ParkRunning (frame 157) @ 46082 kbps

| Test2. (LCEVC on luma-only) | Test4. (LCEVC on luma and chroma, with enhanced chroma) | Vooya difference between Test2. and Test4. |
| --- | --- | --- |
| U-plane only |  |  |
| Test2. (LCEVC on luma-only) | Test4. (LCEVC on luma and chroma, with enhanced chroma) | Vooya difference between Test2. and Test4. |
| Full YUV planes |  |  |

Obviously, when a portion of the LCEVC residuals is allocated to chroma, less bitrate is available for luma, resulting in a slight loss of luma quality vs. a scenario in which 100% of the residuals is on luma, but as we have shown before that difference is small and does not materially change luma BD-rates.

In summary, LCEVC can indeed encode also chroma residuals (up to lossless if needed), and the default luma-only configuration is just a minor design choice aimed at optimising the visual quality impact of the LCEVC enhancement.

# The Low Complexity of LCEVC

“Low Complexity” was a fundamental requirement of the LCEVC standard (as per LCEVC requirements and LCEVC CTC), and – alongside compression efficiency – it has been evaluated and discussed at length during the development and verification of LCEVC. The initial deployments and commercial use cases of LCEVC are confirming the importance of that requirement, as the low complexity design of the standard translates into a number of practical benefits for real-world implementations.

For real-world applications, compression efficiency is in trade-off with compute complexity, so that a given codec format does not enjoy its theoretically maximum compression efficiency all the time, but is represented by a line in the compression-compute trade-off, wherein faster encoding presets produce lower compression efficiency for the sake of meeting real-time and/or computing power constraints. Time and cost to deploy a given codec format also depend on the feasibility to decode on existing devices with sustainable power consumption.

Consequently, evaluations of the benefits of enhancing with LCEVC should include considerations on both compression efficiency and compute complexity metrics – as low complexity is a key LCEVC requirement impacting its real-world use cases.

LCEVC’s Low Complexity has been verified in encoding and in decoding by measuring the encoding time of the LCEVC-enhanced version of several base codecs vis-à-vis a single layer anchor of the respective codec using both LTM and commercial LCEVC implementations. We report below a selection of results from those assessments, along with the references.

## Low Complexity in Encoding

Document m47215 [26] provides a description of LCEVC in response to the Call for Proposal [27]

As shown, the encoding time of LCEVC is roughly 70% smaller of the encoding time required by the respective AVC/ HEVC anchor for both HD and UHD resolutions.

Table 10 - AVC 8-bit, Encoding time reductions between LCEVC (P11) and Anchor

| HD Sequence | LCEVC vs. Anchor Encoding time reduction (%) |  | UHD sequrnce | LCEVC vs. Anchor Encoding time reduction (%) |
| --- | --- | --- | --- | --- |
| Ritual Dance | -69.9% |  | Campfire | -72.1% |
| Basketball Drive | -70.9% |  | Food Market | -71.0% |
| Cactus | -68.9% |  | Park Running | -70.5% |
| Average | -70.0% |  | Average | -70.9% |

Table 11  - HEVC 10-bit, Encoding time reductions between LCEVC (P12) and Anchor

| UHD Sequence | LCEVC vs. Anchor Encoding time reduction (%) |
| --- | --- |
| Campfire | -68.0% |
| Food Market | -72.5% |
| Park Running | -70.1% |
| Average | -70.6% |

Document m52297 [3] reports the test results performed on LTM 4.0 software and provides further performance results on LCEVC enhancing AVC and HEVC. Encoding times and LCEVC complexity reductions are consistent to previous tests and confirm satisfaction of complexity requirements set forth in N18098.

Table 12  - AVC and HEVC, Encoding time reductions between LCEVC and Anchor

| Sequences | Base & Anchor codec | LCEVC vs. Anchor Encoding time reduction (%) |
| --- | --- | --- |
| Class B | AVC | -48.52% |
| Class A | AVC | -67.01% |
| Class A | HEVC | -65.56% |

Input contribution report m56859 [28] by Università Politecnica delle Marche (UNIVPM), uses a set of test sequences of LCEVC Verification Tests and reports processing time data for LCEVC in comparison to VVC, also pointing out the difference in encoding time between the VVC base layer and the LCEVC enhancement.

Table 13 – Encoding time (in seconds) for VVC and LCEVC. Encoding time reduction between LCEVC and VVC Anchor (in %). LCEVC enhancement as a % of total LCEVC encoding time

| Sequence | VVC Full (Anchor) | VVC Base | LCEVC Enh. | LCEVC (VVC) | LCEVC (VVC) vs. VVC | LCEVC Enh. vs. LCEVC (VVC) |
| --- | --- | --- | --- | --- | --- | --- |
| Traffic | 90.364 | 39.049 | 0.761 | 39.811 | -55.94% | 1.91% |
| Starcraft | 131.402 | 51.027 | 0.822 | 51.848 | -60.54% | 1.59% |
| AVG 1080 | 110.883 | 45.038 | 0.792 | 45.830 | -58.67% | 1.73 % |
| DrivingPov | 347.269 | 124.731 | 2.966 | 127.697 | -63.23% | 2.32% |
| Boxe | 305.102 | 133.955 | 2.894 | 136.848 | -55.15% | 2.11% |
| AVG 2160 | 326.186 | 129.343 | 2.930 | 132.273 | -59.45% | 2.22 % |

The results show that the timing required when encoding LCEVC using a VVC Base is roughly 60% smaller than when encoding VVC Full resolution. The results further show that the processing time for the LCEVC Enhancement accounts for just ~2% of the overall {VVC Base + LCEVC Enhancement} time.

Finally, documents m60330 and m60740 [29] by V-Nova and Philips summarize the results of the exploration experiments using LCEVC in combination with VVC as mezzanine encoder in TMIV [30] The table below reports the end-to-end timing analysis. In this exploration experiment, LCEVC achieves average savings on encoding times of 23% (over 30% in some sequences. The TMIV encoder generates 4 surfaces, 2 for the depth maps and 2 for the texture. It is important to note that, in this specific exploration experiment LCEVC (in combination with VVC) was configured to encode the 2 depth map at higher bit depth and using LCEVC without applying any down scaling,

Table 14 - Encoding time reductions between LCEVC and VVC Anchor

| Sequence |  | Encoding time reduction (%) |
| --- | --- | --- |
| Museum | B | -9.0% |
| Fan | O | -21.7% |
| Painter | D | -18.0% |
| Frog | E | -26.9% |
| Chess | N | -32.1% |
| Fencing | L | -30.2% |
| Average MIV |  | -23.0% |

In addition to tests performed with the LTM, during the LCEVC standardization process several tests have also been performed to validate LCEVC Low Complexity using commercial software implementations, enhancing several base codecs, including AVC, HEVC and AV1.

m51438 [8] presents a performance characterization of LCEVC’s encoding low complexity over AVC and HEVC in their open source x264 and x265 implementations over a range of complexity presets, a high number of sequences and several evaluation methodologies. The results are summarised in the chart below.

Consistently with tests performed using Test Models, LCEVC was 44% to 59% faster in encoding than x264 and 54% to 62% faster than x265, with higher gains on slower encoding presets.

The SPIE 2022 report “Enhancing SVT-AV1 with LCEVC to improve quality-cycles trade-offs and enhance sustainability of VOD transcoding” [32] by Intel, Meta and V-Nova evaluated LCEVC’s compression efficiency vs cycles trade-offs within a convex-hull framework using the Dynamic Optimizer (DO) algorithm developed in [33] for VOD applications. The comprehensive tests were performed on the open source ElFuente dataset comparing LCEVC-enhanced SVT-AV1, x264 and x265 to the same native codecs used alone, to evaluate the trade-offs between compression efficiency and encoding time using two DO approaches [34].

The paper concluded that “with fast-encoding parameter selection using the discrete convex hull methodology [34], LCEVC improves the quality-cycles trade-offs for all the tested codecs and across the full complexity range. In the case of SVT-AV1, LCEVC yields a ~40% reduction in computations while achieving the same quality levels according to VMAF_NEG”.

Finally, a recent presentation at Mile High Video conference 2023 applied a similar methodology as [32] to measure the compression efficiency vs complexity trade-off of LCEVC-enhanced VVC vs. a full resolution VVC anchor in its MainConcept implementation [22].

The figure below shows the BD-rates vs encoding complexity trade-off on UHD contents from the Netflix OpenContent dataset [31]. The BD-rate was calculated on the convex hull across 2160p, 1440p and 1080p resolutions, encoding time was computed in line with the methodology of [32] with encodings performed on a server based on AMD EPYC 7542 processor.

Irrespective of BD-rates, consistent with VVC results in Table 6, the encoding time figures on the horizontal axis show a reduction in encode time between 67% and 69% for LCEVC vs VVC.

## Low Complexity in Decoding

Document m47215 [26] also provides results in terms of Decoding time.  In particular, AVC-based LCEVC (submission P11 in the document) required over 90% less of the decoding time needed by the AVC anchor, whereas HEVC-based LCEVC (submission P12) required about 34% less of the decoding time needed by the HEVC anchor.

Table 15 - AVC 8-bit, Decoding time reductions between LCEVC (P11) and Anchor

| HD Sequence | LCEVC vs. Anchor Decoding time reduction (%) |  | UHD sequrnce | LCEVC vs. Anchor Decoding time reduction (%) |
| --- | --- | --- | --- | --- |
| Ritual Dance | -93.5% |  | Campfire | -93.6% |
| Basketball Drive | -93.1% |  | Food Market | -89.0% |
| Cactus | -92.2% |  | Park Running | -91.8% |
| Average | -93.0% |  | Average | -91.5% |

Table 16 - HEVC 10-bit, Decoding time reductions between LCEVC (P12) and Anchor

| HD Sequence | LCEVC vs. Anchor Decoding time reduction (%) |
| --- | --- |
| Ritual Dance | -34.0% |
| Basketball Drive | -32.9% |
| Cactus | -34.3% |
| Average | -33.7% |

Similarly, m52297 [3] reports LTM 4.0 tests on decoder as well, results are reported in the table below.

Table 17 - AVC and HEVC, Decoding time reductions between LCEVC and Anchor

| Sequences | Base & Anchor codec | LCEVC vs. Anchor Decoding time reduction (%) |
| --- | --- | --- |
| Class B | AVC | -3.3% |
| Class A | AVC | -18.1% |
| Class A | HEVC | -35.8% |

In addition documents m60330 and m60740 [29] by V-Nova and Philips summarize the results of the exploration experiments using LCEVC in combination with VVC as mezzanine encoder in TMIV [30]. This document shows an average savings of 12% on decoding times can be achieved (over 16% in some sequences), despite the increase in bit depth and the use of LCEVC without spatial scaling.

Table 18 - Decoding time reductions between LCEVC and VVC Anchor

| Sequence |  | Decoding & Rendering time reduction (%) |
| --- | --- | --- |
| Museum | B | -16.6% |
| Fan | O | -14.1% |
| Painter | D | -4.9% |
| Frog | E | -13.9% |
| Chess | N | -11.3% |
| Fencing | L | -11.6% |
| Average MIV |  | -12.1% |

With regards to decoding, the above-mentioned SPIE 2022 report [32] also provided figures on LCEVC performance in a real-life scenario of mobile device video playback. Specifically, the paper addressed the challenges of AV1 software decoding, in terms of playback capabilities and battery consumption on a large spectrum of mobile phones that today, and for the foreseeable future, won’t be able to benefit from hardware-accelerated decoding for next-generation codecs such as AV1.

Experimental tests showed that LCEVC “enlarges the set of mobile devices capable of playing HD as well as high-frame-rate content encoded with AV1 and extends mobile battery life by up to 50% with respect to state-of-the-art AV1 software decoding”.

Battery drain over one-hour playback of AV1 vs. LCEVC +AV1 on Android mobile phones

The low complexity benefits of LCEVC are extremely relevant to commercial use cases, since they are key to enable rapid deployment, cost savings, power efficiencies and sustainability benefits.  Recent examples include the web delivery pilots performed by Globo TV and others after inclusion of efficient HTML5 LCEVC scripted decoding into Google’s Shaka and THEO web players, the scalable web delivery over DASH showcased by Unified Streaming on dash.js, the very small size of the LCEVC RTL silicon IP cores commercialized by Allegro DTV, the showcase of high-efficiency LCEVC-enhanced cloud gaming and cloud XR low-latency delivery by NVIDIA on existing GPUs at IBC 2023,  and the recent announcement of RealTek support for LCEVC not just on its upcoming Set-Top-Box SoCs but even on the existing ones.

# Conclusions

This document is produced to provide constructive feedback on the methodology and the conclusions of input document JVET-AF0075 [1]  and a summary of the expected performance of MPEG-5 Low Complexity Enhancement Video Coding (LCEVC) on objective metrics vs. formal MOS, according to extensive cross-checked evaluations performed over the course of LCEVC’s collaborative development phase as well as during its Verification Tests.

Most of the hypotheses and results presented in JVET-AF0075 had already been evaluated, expanded, discussed at length, and resolved, so this document aims to provide useful reminders for experts that may have been less involved in the standardization of LCEVC. Over the course of 11 MPEG meetings (MPEG 124 to MPEG 134), there have been over 30 documents with the involvement of over 16 companies and universities evaluating the performances of LCEVC in terms of PSNR, VMAF, and formal subjective assessments, including LCEVC’s Verification Tests (m54455[10]) and several evaluations of LCEVC using the same sequences as in [1] (such as N19160[9], m53796 [6], m52997 [3] and m53523 [7]). A good summary of LCEVC tests preceding Verification Tests can be found in N19571 [5].

Despite suboptimal LCEVC configurations used in JVET-AF0075 (which used LTM default configurations outside of their applicable bitrate range), PSNR results are broadly aligned with all the above test results, including those from Verification Tests. Following evidence presented in m51438 [8], which proved the lack of correlation between PSNR and subjective performance and a higher correlation between VMAF and formal subjective performances, the LCEVC CTC complemented PSNR with VMAF results, while maintaining ITU-R BT.500 formal MOS evaluations as ground truth. Importantly, formal MOS results consistently confirmed the lack of correlation of PSNR with visual quality when assessing LCEVC performance. In this document we report a selection of previous conclusions in contrast with statements in JVET-AF0075.

The document also includes additional information to address considerations in JVET-AF0075 with regards to the balance of LCEVC bitrate allocation among luma and chroma planes.  Example encodings with LCEVC settings that allocate more quality to chroma planes demonstrate that, contrary to JVET-AF0075 statements, different allocation strategies vs. the current LTM luma-only defaults provide marginal differences in luma BD-rates. Even when LCEVC residuals are applied to chroma planes with smaller quantization step-width than luma plane, the negative impact on the BD-rate of luma metrics, such as VMAF, is very small (< 2 percentage point) and does not materially affect the results.

Finally, this document also provides a reminder of the importance of the Low Complexity requirements of LCEVC, which provide unique benefits in many of its real-world use cases.

# APPENDIX

The encoding and decoding command lines used for tests with ffmpeg production code in Section 2 are illustrated in the tables below.

Encoding command lines:

| Test | Codec | Encoding command lines (example with 2160p60 contents) |
| --- | --- | --- |
| 1 | x265 | ffmpeg -y -f rawvideo -pix_fmt yuv420p10le -s 3840x2160 -r 60 -i source.yuv -g 128 -b:v {bitrate} -c:v libx265 -preset medium -r 60 -s 3840x2160 -x265-params vbv-bufsize={2x bitrate}:vbv-maxrate={bitrate}:vbv-minrate={bitrate} output.ts |
| 2 | LCEVC x265 (luma only) | ffmpeg -y -f rawvideo -pix_fmt yuv420p10le -s 3840x2160 -r 60 -i source.yuv -g 128 -b:v {bitrate} -c:v lcevc_hevc -base_encoder x265 -r 60 -s 3840x2160 -eil_params "preset=medium;lcevc_tune=vmaf_neg” output.ts |
| 3 | LCEVC x265 (luma + chroma) | ffmpeg -y -f rawvideo -pix_fmt yuv420p10le -s 3840x2160 -r 60 -i source.yuv -g 128 -b:v {bitrate} -c:v lcevc_hevc -base_encoder x265 -r 60 -s 3840x2160 -eil_params "preset=medium;lcevc_tune=vmaf_neg;encoding_plane_mode=yuv” output.ts |
| 4 | LCEVC x265 (luma + enhanced chroma) | ffmpeg -y -f rawvideo -pix_fmt yuv420p10le -s 3840x2160 -r 60 -i source.yuv -g 128 -b:v {bitrate} -c:v lcevc_hevc -base_encoder x265 -r 60 -s 3840x2160 -eil_params "preset=medium;lcevc_tune=vmaf_neg;encoding_plane_mode=yuv;chroma_sw_multiplier=45” output.ts |
| 5 | MainConcept VVC | ffmpeg -y -f rawvideo -pix_fmt yuv420p10le -s 3840x2160 -r 60 -i source.yuv -g 128 -b:v {bitrate} -c:v lcevc_vvc -base_encoder mainconcept_vvc -r 60 -s 3840x2160 -eil_params "encoding_mode=native;scaling_mode_level0=0D;scaling_mode_level1=0D” output.ts |
| 6 | LCEVC MainConcept VVC (luma only) | ffmpeg -y -f rawvideo -pix_fmt yuv420p10le -s 3840x2160 -r 60 -i source.yuv -g 128 -b:v {bitrate} -c:v lcevc_vvc -base_encoder mainconcept_vvc -r 60 -s 3840x2160 -eil_params "lcevc_tune=vmaf_neg” output.ts |

Decoding command lines to generate YUV reconstruction of the LCEVC streams:

| Test | Codec | Decoding command line |
| --- | --- | --- |
| 2, 3, 4 | LCEVC HEVC | ffmpeg -vcodec lcevc_hevc -i output.ts -pix_fmt yuv420p10le output.yuv |
| 6 | LCEVC VVC | ffmpeg -vcodec lcevc_vvc -i output.ts -pix_fmt yuv420p10le output.yuv |

The full results can be found in the attached excel:

<Include Link>

The resulting encoded streams can be found here:

<Include Link>

The experiments in Paragraph 2 with open source base layer can be replicated by downloading, upon registration, LCEVC’s ffmpeg encoding and decoding libraries from https://download.v-nova.com/

# 6  References

- “Evaluation results of Low Complexity Enhancement Video Codec (LCEVC) with HM and VTM on 4K content”, JVET-AF0075, Hannover, October 2023.

- “Common Test Conditions of Low Complexity Enhancement Video Coding”, ISO/IEC JTC 1/SC 29/WG 11 N18457, Geneva, CH – March 2019.

- “[LCEVC] - LTM 4.0 Test and Update on Performance Results” ISO/IEC JTC 1/SC 29/WG4 m52997, Alpbach, AT – April 2020.

- “Requirements for Low Complexity Video Coding Enhancements” ISO/IEC JTC 1/SC 29/WG 11 N18098, Macau, CN – October 2018.

- “Summary of LCEVC tests conducted to date” ISO/IEC JTC 1/SC 29/WG 11 N19571, Online, July 2020.

- “[MPEG-5 part 2] LCEVC LTM 3 performance analysis” ISO/IEC JTC 1/SC 29/WG 11 m53796, Alpbach, AT – April 2020.

- “[LCEVC]Test and analysis of LTM4.0” ISO/IEC JTC 1/SC 29/WG 11 m53523, Alpbach, AT – April 2020.

- “LCEVC - Performance Characterization over a range of Enhanced Codecs” ISO/IEC JTC 1/SC 29/WG 11 m51438, Geneva, CH – October 2020

- “Test Results of LTM 3.0” ISO/IEC JTC 1/SC 29/WG 11 N19160, Brussels, BE – January 2020.

- “[LCEVC] Performance assessment of LCEVC in conjunction with HEVC, EVC, VVC” ISO/IEC JTC 1/SC 29/WG 11 m54455, Online, July 2020.

- “[LCEVC] – Common Test Condition of Low Complexity Enhancement Video Coding”, ISO/IEC JTC 1/SC 29/WG 11 N18988, Brussels, BE – January 2020.

- “Draft Report on LCEVC Verification Test” ISO/IEC JTC 1/SC 29/AG 5 m56225 – Online, January 2021.

- “Draft Report on MPEG-5 Part 2 LCEVC Verification Tests Phase 1 Results” ISO/IEC JTC 1/SC 29/WG 4 m56013 – Online, January 2021.

- “Verification Test Plan for Low Complexity Enhancement Video” ISO/IEC JTC 1/SC 29/WG 4 N00029 (m19827) – Online, October 2020.

- “Verification Test Report on the Compression Performance of Low Complexity Enhancement Video Coding” ISO/IEC JTC 1/SC 29/WG 4 N0029 (m20173) – Online, April 2021

- “[LCEVC] Verification Test – Overview of bitrate, PSNR, VMAF, and MD5 checksums” ISO/IEC JTC 1/SC 29/WG 4 m56330 – Online, April 2021

- “AHG10: study of VVC spatial scalability performance” ISO/IEC JTC 1/SC 29/WG5 28th Meeting, Mainz, DE, 20–28 October 2022

- Z. Wang and A. Bovik, (2009) “Mean Squared Error: Love it or leave it?”, IEEE Signal Processing Magazine, p98-117, 2009,

- W. Lin, D. Li, and P. Xue, (2003) “Discriminative analysis of pixel difference towards picture quality prediction,” in Proceedings 2003 International Conference on Image Processing (Cat. No. 03CH37429), vol. 3. IEEE, 2003, pp. III–193.

- “On learning based video quality metrics”, ISO/IEC JTC 1/SC 29/AG 5 m56636, April 2021

- x265, open source HEVC software encoder, https://www.videolan.org/developers/x265.html

- MainConcept SDK, VVC/H.266. https://www.mainconcept.com/vvc

- Libvmaf, https://github.com/Netflix/vmaf/tree/master/libvmaf

- Vooya: raw Video Sequence Player: https://www.offminor.de/

- “Results of expert viewing for the spatial scalability category of the VVC multilayer VT”, JVET, 31st Meeting, Geneva, CH, 11–19 July 2023

- “Description of video coding technology proposal by V-Nova for Low Complexity Video Coding Enhancements” ISO/IEC JTC 1/SC 29/WG11 Geneva, SW, March 2019

- “Call for Proposals for Low Complexity Video Coding Enhancements” ISO/IEC JTC1/SC29/WG11 N17944, Macao, CN, Oct. 2018.

- “[LCEVC] Performance – Timing data” ISO/IEC JTC1/SC29/WG11 N17944, Online,  April 2021.

- “Result of exploration experiment using LCEVC in MIV” ISO/IEC JTC 1/SC 29/WG5 28th Meeting, Mainz, DE, 20–28 October 2022

- A. Dziembowski, Gwangsoon Lee (Eds.), Test Model 16 for MPEG immersive video, ISO/IEC JTC 1/SC 29/WG 04 N 0347, April 2023, Antalya.

- Netflix OpenContent dataset: http://download.opencontent.netflix.com.s3.amazonaws.com/

- G. Cobianchi, G. Meardi, S. Poularakis, A. Walisiewicz, O. Abdelkafi, F. Ben Amara, F. Kossentini, C. Stejerean, H. Tmar “Enhancing SVT-AV1 with LCEVC to improve quality-cycles trade-offs and enhance sustainability of VOD transcoding”, Proc. SPIE 12226, Applications of Digital Image Processing XLV, 122260S (3 October 2022); doi: 10.1117/12.2633882

- I. Katsavounidis, "The NETFLIX tech blog: Dynamic optimizer - A perceptual video encoding optimization framework," https://netflixtechblog.com/dynamic-optimizer-a-perceptual-video-encoding-optimization-frameworke19f1e3a277f (2018)

- Ping-Hao Wu, Ioannis Katsavounidis, Zhijun Lei, David Ronca, Hassene Tmar, Omran Abdelkafi, Colton Cheung, Foued Ben Amara, Faouzi Kossentini, "Towards much better SVT-AV1 quality-cycles tradeoffs for VOD applications," Proc. SPIE  1842, Applications of Digital Image Processing XLIV, 118420T (1 August 2021); doi: 10.1117/12.2595598