| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF02023-v3 |
| --- | --- |

| Title: | Exploration Experiments on Neural Network-based Video Coding (EE1) |  |  |
| --- | --- | --- | --- |
| Status: | Output document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina, R. Chang, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, and Z. Xie | Tel: Email: | elena.alshina@huawei.com renjiechang@tencent.com franck.galpin@interdigital.com yue.li@bytedance.com dmytror@qti.qualcomm.com maria.santamaria_gomez@nokia.com jacob.strom@ericsson.com xiezhihuang@oppo.com |
| Source: | EE Coordinators |  |  |

_____________________________

# Abstract

This document summarizes Exploration Experiment 1 (EE1) tests to be performed between the JVET-AF and JVET-AH meetings to evaluate Neural Network-based Video Coding (NNVC) technologies, analyze their performances and complexity aspects.

# Introduction

This round of EE1 tests will include:

- EE1-1: HOP

  - EE1-1.0: HOP re-training and luma/chroma balance changes (JVET-AF0155, JVET-AF0180), possible tuning from JVET-AF0150, JVET-AF0296

  - EE1-1.1: EE1-0 with architecture change variant 1 (JVET-AF0102, JVET-AF0103, JVET-AF0182), as defined in JVET-AF0307.

  - EE1-1.2: EE1-0 with architecture change variant 2 (JVET-AF0102, JVET-AF0103, JVET-AF0153), as defined in JVET-AF0307.

  - EE1-1.3: Comparison test for HOP single model and two models (JVET-AF0183)

  - EE1-1.4: Study joint inference design of JVET-AF0154 (rotation) and JVET-AF0086 (flipping)

  - EE1-1.5: HOP In-loop filter with transformer blocks (JVET-AF0158)

- EE1-2: LOP

  - EE1-2.1: LOP/HOP fast training (JVET-AF0043, fast Stage III training)

  - EE1-2.2: LOP Content adaptive (JVET-AF0056)

  - EE1-2.3: Further complexity reduction of LOP (JVET-AF0071)

- EE1-3: inter prediction

  - EE1-3.1: Deep Reference Frame Generation for Inter Prediction Enhancement (JVET-AF0208)

- EE1-4: super-resolution

  - EE1-4.1: Unified CNN super resolution for resampling-based video coding (JVET-AF0143)

All tests in EE1 to use NNVC-7.0 as code base and to follow NNVC CTC, unless it is explicitly specified by the test description. The anchor for EE1 test is the default configuration of NNVC-7.0 as defined by AhG11/AhG14 (NN-intra and low complexity NN-filter enabled by default) in JVET-AE2016. Anchor performance and reference point for HOP NN-filters will be provided by AhG14.

All HOP tests should be done using stage 3 HOP official dataset. LOP tests uses official LOP stage 3 dataset, except EE1-2.1 which uses official HOP stage 3 dataset.

LOP-related tests will report results vs NNVC-7.0 Anchor (LOP.2 filter and NN-Intra are enabled by default). HOP-related tests to report results in comparison to HOP Anchor (HOP.1 filter and NN-Intra enabled), unless alternative anchor is specified in the test description.

Proponents of EE1 should follow the prescribed timeline, and any delay compared to timeline for code release should be advertised and agreed on the reflector. Code releases after timeline milestones without notice would be reverted on the repository.

# Overview

Tests planned in this EE1 are summarized in Table 1.

Table 1 Overview of EE1 tests – In gray, modified aspect

| Test EE1 | Topic | Architecture | Training | Usage in a codec | Tester/‌Trainer | Cross-check | Cross-checker |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.0 | HOP training | HOP.1 | HOP new scripts, HOP dataset | NNVC 7.0 HOP usage | InterDigital/ Qualcomm | Training and inference | InterDigital/ Qualcomm + ??? |  |
| 1.1 | HOP model | HOP changed | HOP new scripts, HOP dataset | NNVC 7.0 HOP usage | Bytedance/ Qualcomm/‌Tencent | Training and inference | Bytedance/ Qualcomm/‌Tencent |  |
| 1.2 | HOP model | HOP changed | HOP new scripts, HOP dataset | NNVC 7.0 HOP usage | Tencent/ Qualcomm/‌Bytedance | Training and inference | Tencent/ Qualcomm/‌Bytedance |  |
| 1.3 | HOP single/dual | HOP.1 and separate models, HOP.2 and separate models | HOP training | NNVC 7.0 usage (same usage for both) | Bytedance, InterDigital, Qualcomm, Tencent | Training and inference |  |  |
| 1.4 | HOP/LOP usage | HOP.1 | No retraining | New usage | OPPO/ Tencent | Inference |  |  |
| 1.5 | HOP model | HOP transformers | HOP training | NNVC 7.0 HOP usage | Qualcomm | Training and inference |  |  |
| 2.1 | LOP/HOP training | LOP.2/HOP.2 | LOP/HOP training, HOP/LOP datasets | NNVC 7.0 LOP/HOP usage | Qualcomm | Training and inference |  |  |
| 2.2 | LOP model | LOP.2 with fine-tuned parameters | Fine tuning | NNVC 7.0 LOP usage | Nokia | Finetuning and inference | Ittiam + OPPO |  |
| 2.3 | LOP model | LOP.2 changed | LOP.2 training, LOP.2 dataset | NNVC 7.0 LOP usage | Dolby/ Ittiam | Training and inference | Qualcomm |  |
| 3.1 | Inter prediction | JVET-AF0208 | JVET-AF0208 | JVET-AF0208 | Wuhan Univ./ Tencent | Training and inference | OPPO |  |
| 4.1 | Super resolution | LOP.2/HOP.1 | JVET-AF0143 | JVET-AF0143 | Bytedance | Training and inference | HUST |

Table 1 Overview of EE1 tests – In gray, modified aspect

# EE1 Description

## EE1-1 HOP related

In this group of tests, various aspects of HOP ILF training and inference are tested.

### EE1-1.0 HOP re-training and luma/chroma balance changes

In this test, original HOP.1 architecture is used.

Training strategy: original HOP stageIII dataset is used. New training strategy is tested:

- Loss function YUV weighting is changed from 6:1:1 to 12:1:1, no changes to learning rate.

- Possibly batch size and learning rate are changed if they bring gains over default hyper-parameters.

Inference: the NNVC 7.0 software is used for inference, with a QP chroma offset equal to 1 (for both Cb and Cr components). Final model will be integerized using the default quantization script for HOP. Best model and training strategy will be used as anchor for further EE1 tests.

Crosscheck: Interdigital and Qualcomm will independently train/crosscheck each other for the training and inference part.

Model and test results expected to be ready by Nov. 15, 2023.

### EE1-1.1 EE1-0 with architecture change variant 1

This joint training test was defined in BoG, JVET-AF0307.

In this test, the HOP filter architecture of NNVC6.0 is modified to include the following changes:

- Alternation of Backbone Blocks Types and channel reduction (C31=48) as specified JVET-AF0102 (EE1-1.1.2.b).

- Applying 1x1 CONV for IPB and BS input, as specified in JVET-AF0103.

- Applying group convolution in BBB, location and size as in JVET-AF0182 (EE1-1.1.4b).

- Complexity-performance optimization targeting achieving complexity of <477kMAC/pixel will be achieved by changes to the number of BBBs, and others to be define by November 20, 2023. These changes and to be aligned with respective changes in test EE1-1.1 (Section 2.1.1).

The specific network design is shown as follows. Parameters will can be adjusted to keep complexity not higher than 477 kMAC/pxl.

Target complexity: HOP.1.

Training strategy: original HOP stage III dataset and training strategy of HOP with changes concluded from EE1-1.0 are used.

Inference: the NNVC 7.0 software is used for inference, with additional changes to inference concluded in EE1-1.0, e.g. QP chroma offset equal to 1 (for both Cb and Cr components).

### EE1-1.2 EE1-0 with architecture change variant 2

This joint training test was defined in BoG, JVET-AF0307.

In this test, the HOP filter architecture of NNVC6.0 is modified to include the following changes:

- Alternation of Backbone Blocks Types and channel reduction (C31=48) as specified JVET-AF0102 (EE1-1.1.2.b).

- Applying 1x1 CONV for IPB and BS input, as specified in JVET-AF0103.

- Applying group convolution in BBB, location and size as in JVET-AF0153 (EE1-1.1.1-t1).

- Complexity-performance optimization targeting achieving complexity of <477kMAC/pixel will be achieved by changes to the number of BBBs, and others to be define by November 20, 2023. These changes and to be aligned with respective changes in test EE1-1.1 (Section 2.1.1).

The specific network design is shown as follows. The kMAC/pxl is about 471.

Target complexity: HOP.1.

Training strategy: original HOP stage3 dataset and training strategy of HOP with changes concluded from EE1-1.0 are used.

Inference: the NNVC 7.0 software is used for inference, with additional changes to inference concluded in EE1-1.0, e.g. QP chroma offset equal to 1 (for both Cb and Cr components).

Crosscheck: Both training and inference crosscheck are planned, crosschecker TBA.

### EE1-1.3 comparison test for HOP single model and two models

This test is to compare HOP single model and separate models.

EE1-1.3.1: no retraining, compare HOP NNVC-6.0 single model and EE1-1.1.4.c two models, inference aligned with HOP

EE1-1.3.2: retrain separate models, with architectures designed according to HOP.2 (best of EE1-1.0, EE1-1.1, EE1-1.2), inference aligned with HOP

### EE1-1.4 Study joint design of rotation and flipping

This test studies the joint design of rotation and flipping of input/output of the NNLF network to further improve the coding performance of current NNLF. Both the operations of rotation and flipping are shown in the below figures, respectively.

Rotation of inputs/output

Flipping of inputs/output

In the current design of LOP and HOP, there are several input information fed into the network, including reconstructed samples, prediction samples, boundary strength, IPB information and QP information. It is proposed to rotate or flip those inputs then inject them into the network. After inference of network, the output samples are rotated/flipped back to restore the original order of filtered samples. It is allowed to rotate left or right, and flip horizontally or vertically in the proposed method.

The joint design of rotation and flipping will be studied in this EE to get a better trade-off between encoding time and coding performance.

Training strategy: no training is needed.

Inference: the NNVC 7.0 software is used for inference. Inference results will be reported with HOP and LOP filters.

Crosscheck: Only inference crosscheck is planned, crosschecker TBA.

### EE1-1.5 HOP In-loop filter with transformer blocks

This test is based on JVET-AF0158 where transformer was introduced to the backbone block of HOP architecture.

Filter architecture: HOP-based.

Target complexity: HOP.

Training strategy: HOP-based.

Proponent: Qualcomm.

Cross-checker:

## EE1-2 LOP related

### EE1-2.1: LOP fast Stage III training

This test studies feasibility of LOP/HOP training datasets unification. An alternative training strategy for LOP and HOP development. This test is based on JVET-AF0043, section 7.4.

Subtest 2.1.1: LOP.2 architecture to be trained with HOP Stage III dataset.

Subtest 2.1.2: HOP.2 architecture to be trained with LOP Stage III dataset.

Training strategy: LOP based training in 2.1.1 and HOP based training in 2.1.2.

Inference: Unified HOP/LOP inference.

Proponent: Qualcomm.

### EE1-2.2: LOP Content adaptive

This test studies a content-adaptive technique for the LOP filter, where some parameters are finetuned on the test sequence at encoder side. The finetuned parameters are compressed-decompressed by MPEG-NNR. The NN update is signaled to the decoder within an Adaptation Parameter Set (APS). The following items will be studied:

•	Fine-tuning and signaling of NN updates for each intra-frame period.

•	Application to LOP2.

Also, as part of this test, the finetuning and the inference will be crosschecked.

Proponent: Nokia

Cross-checkers: Ittiam and OPPO.

### EE1-2.3: Further complexity reduction on the joint LOP.2

New backbone block architecture for unified LOP is proposed in the figure above. Based on the architecture of the adopted LOP-2.3 in JVET-AF0043, the order of the layers in backbone blocks is adjusted and hence the last two 1x1 convolutional layers could be fused into one single 1x1 convolution for further complexity reduction as well as reduction in the number of convolution layers in LOP that could reduce the peak decoder memory consumption during inference. This test studies the coding efficiency of the new LOP architecture as well as the trade-off between BD-Rate and complexity. Complexity metrics such as kilo mac per pixel, number of layers, decoder run time and peak decoder memory consumption will be studied in EE. Excepted computational complexity 16.56 kMAC/pxl (vs 17.05 kMAC/pxl for NNVC-7.0-LOP).

Training strategy: Fast Stage 3 training will be conducted following the training strategy of unified LOP described in JVET-AF0043 using the stage 3 training dataset used for the adopted LOP.2 architecture. Some training parameters such as batch size, learning rate may be adapted for better performance including balancing of luma-chroma RD performance.

Cross-check: Inference and training cross-checks are planned, cross-checker TBA.

## EE1-3 inter prediction

### EE1-3.1: Deep Reference Frame Generation for Inter Prediction Enhancement

Network architecture: The above figure illustrates the network architecture of the inter frame generation network, which is based on the model described in JVET-AF0208.

Training strategy: The new model will be trained using compressed BVI and TVD datasets or other datasets that can be used for NNVC standardization. Based on the JVET-AF0208, adjustments will be made to the loss function and some training parameters, including batch size and learning rate, to enhance performance.

Training to be conducted: Wuhan University and Tencent.

Cross-check: Inference and training cross-checks are planned.

Cross-checker: OPPO.

Note: This test aims to adjust the NN-Inter training strategies to follow NNVC standardization and enhance model performance. This test will analyze the compression performance of this method with LOP or HOP.

## EE1-4 Super-resolution

### EE1-4.1: Unified CNN super resolution for resampling-based video coding

This test studies the unified LOP SR and HOP SR architecture which is aligned to LOP and HOP in-loop filters. The following items will be studied:

- EE1-4.1.1.a: Study the LOP SR architecture based on the latest LOP in-loop filter as well as the trade-off between BD-Rate and complexity. The number of layers, the number of channels, and training parameters (learning rate, batch size, etc.) will be studied.

- EE1-4.1.1.b: Study the HOP SR architecture based on the latest HOP in-loop filter as well as the trade-off between BD-Rate and complexity. The number of layers, the number of channels, architecture (alternation of 3x1 and 1x3 CONVs, etc.), and training parameters (learning rate, batch size, etc.) will be studied.

- EE1-4.1.2.a: Studies the combination test of LOP SR and CNN in-loop filters. Train the SR models with CNNLF turned on will be studied.

- EE1-4.1.2.b: Studies the combination test of HOP SR and CNN in-loop filters. Train the SR models with CNNLF turned on will be studied.

Cross-check: Inference and training cross-checks are planned.

Cross-checker: HUST

The unified HOP SR architecture.

The unified LOP SR architecture.

# Timeline

T1 - 2 weeks after JVET-AF meeting (3 Nov 2023): EE description (JVET-AF2023) finalized and uploaded. NNVC-7.0 software is available, including updated scripts.

T2a – ~4 weeks after JVET-AF meeting (15 Nov 2023): EE1-1.0 models and results are made available. Hyper-parameters are fixed.

T2 - 4 weeks after JVET-AF meeting (16 Nov 2023): Results discussed during teleconference. Training of EE1-1.1/EE1-1.2 started.

T3 – 4 weeks before T6 (19-Dec-2023): EE1 with training crosscheck software is fixed.

T4 - 2 weeks before T6 (02-Jan-2024): EE1 with inference crosscheck software is fixed.

T5 – 3 days before T6 (13-Jan-2024): Cross-checkers report status to EE1 coordinators (sending e-mail).

T6 –16-Jan-2024: EE1 summary is uploaded as input contribution.

# SW location

https://vcgit.hhi.fraunhofer.de/jvet-af-ee1/VVCSoftware_VTM/-/tree/EE1-X.X

# References

[1]	E. Alshina, R.-L. Liao, S. Liu, A. Segall  Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AE2016.