| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET- AG0015-v1 |
| --- | --- |

| Title: | JVET AHG report: Gaming content compression (AHG15) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Saurabh Puri, InterDigital Johannes Sauer, Huawei Roman Chernyak, Tencent Alberto Duenas, WBD Limin Wang, Nokia | Email: | Saurabh.Puri@InterDigital.com johannes.sauer@huawei.com chernyak@global.tencent.com alberto.duenas@wbd.com limin.2.wang@nokia.com |
| Source: | AHG15 on gaming content compression |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AHG15 on gaming content compression that have taken place between the 32nd and 33rd JVET meetings. Two teleconferences were held on 18th December 2023 and on 27th December 2023 respectively. The first teleconference [1] mainly focused on testing conditions and candidate sequences for gaming content compression. In the second call 5s segments of the candidate sequences were reviewed. It was decided to encode all of them to gather some first data which should help for experts to choose appropriate sequences for a gaming content test set. Encoding of the selected segments has started for both VTM-11ecm11 and ECM-11.0. The results will be added to this report when available. It is expected that only VTM-11ecm11 results will be available at the beginning of the meeting.

# Introduction

AHG15 on gaming content compression was established at the 32nd JVET meeting. Mandates of this ad hoc group are:

- Identify gaming content application scenarios and their requirements for codec operation.

- Identify and characterize required types of content; solicit contributions, collect, and make a variety of gaming content available, in coordination with AHG4 and AG 5.

- Propose test conditions appropriate for gaming applications.

- Evaluate JVET test models (such as ECM, VTM, NNVC, etc.) under the proposed test conditions.

- Investigate possibilities to enhance compression capability for gaming content.

# Testing conditions for gaming content

This section describes simulations’ setup that is planned to be used within AhG15.

The simulations will be performed prior to the January meeting.

Anchor: VTM-11ecm11 (https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/VTM-11.0ecm11.0)

Test: ECM-11.0 (https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-11.0)

Configurations: AI/LDB/LDP

Testing conditions: JVET CTC with QP set [27, 32, 37, 42]

Content: 5s; TBD

Extra parameters: with and without ClassF cfg file

# Gaming content sequence survey

A plenitude of candidate sequences for gaming content compression were identified by the group. The sequences along with their source are listed in the following table: In addition to the rendered view (YUV) for some sequence auxiliary information is available (depth, optical flow, etc.).

| Sequence name | Provided by | Resolution, FPS, color space, bit depth | Availability | Auxiliary information available |
| --- | --- | --- | --- | --- |
| Level1 | InterDigital | 1920x1080, 60p, 420, 10bits | JVET ftp | yes |
| Darktree | InterDigital | 1920x1080, 60p, 420, 10bits | JVET ftp | yes |
| ArenaOfValor | Tencent | 1920x1080, 60p, 420, 8bits | JVET ftp | no |
| ARPG | Huawei | 1920x1080, 60p, 10s (600 Frames), YUV420(8 bits) | to be provided | yes |
| DesertTown1 | Huawei | 1920x1080, 60p, 10s (600 Frames), YUV420(8 bits) | to be provided | yes |
| DesertTown2 | Huawei | 1920x1080, 60p, 10s (600 Frames), YUV420(8 bits) | to be provided | yes |
| Sun_Temple1 | Huawei | 1920x1080, 60p, 10s (600 Frames), YUV420(8 bits) | to be provided | yes |
| Sun_Temple2 | Huawei | 1920x1080, 60p, 10s (600 Frames), YUV420(8 bits) | to be provided | yes |
| CSGO | Twitch/SA4 | 1920x1080, 60p, 10s (600 Frames), YUV420(8 bits) | h264 uncompressed / ReferenceSequences | to be checked |
| DOTA2 | Twitch | 1920x1080, 60p, 10s (600 Frames), YUV420(8 bits) | h264 uncompressed | to be checked |
| EuroTruckSimulator2 | Twitch | 1920x1080, 60p, 10s (600 Frames), YUV420(8 bits) | h264 uncompressed | to be checked |
| Fallout4 | Twitch | 1920x1080, 60p, 10s (600 Frames), YUV420(8 bits) | h264 uncompressed | to be checked |
| GTAV | Twitch | 1920x1080, 60p, 10s (600 Frames), YUV420(8 bits) | h264 uncompressed | to be checked |
| Hearthstone | Twitch | 1920x1080, 60p, 10s (600 Frames), YUV420(8 bits) | h264 uncompressed | to be checked |
| Minecraft | Twitch/SA4 | 1920x1080, 60p, 10s (600 Frames), YUV420(8 bits) | h264 uncompressed / ReferenceSequences | to be checked |
| Rust | Twitch | 1920x1080, 60p, 10s (600 Frames), YUV420(8 bits) | h264 uncompressed | to be checked |
| Starcraft | Twitch/SA4 | 1920x1080, 60p, 10s (600 Frames), YUV420(8 bits) | h264 uncompressed / ReferenceSequences | to be checked |
| Witcher3 | Twitch | 1920x1080, 60p, 10s (600 Frames), YUV420(8 bits) | h264 uncompressed | to be checked |
| Baolei-Man | Tencent/SA4 | 1920x1080, 60p, 10s (600 Frames), YUV420(8 bits) | ReferenceSequences | to be checked |
| Baolei-Balloon 4K | Tencent/SA4 | 4096x2160, 60p, 10s (600 Frames), YUV420(8 bits) | ReferenceSequences | to be checked |
| Baolei-Yard 4K | Tencent/SA4 | 4096x2160, 60p, 10s (600 Frames), YUV420(8 bits) | ReferenceSequences | to be checked |
| Baolei-Woman | Tencent/SA4 | 1920x1080, 60p, 10s (600 Frames), YUV420(8 bits) | ReferenceSequences | to be checked |
| Jianling-Temple | Tencent/SA4 | 1920x1080, 60p, 10s (600 Frames), YUV420(8 bits) | ReferenceSequences | to be checked |
| Jianling-Beach | Tencent/SA4 | 1920x1080, 60p, 10s (600 Frames), YUV420(8 bits) | ReferenceSequences | to be checked |
| Heroes of the Storm part 1 | Kingston/SA4 | 1920x1080, 30p, 30s (900 Frames), YUV420(8 bits) | ReferenceSequences | to be checked |
| Project CARS | Kingston/SA4 | 1920x1080, 30p, 30s (900 Frames), YUV420(8 bits) | ReferenceSequences | to be checked |
| WoW part 2 | Kingston/SA4 | 1920x1080, 30p, 30s (900 Frames), YUV420(8 bits) | ReferenceSequences | to be checked |

# Auxiliary information for gaming sequences

Various additional information may be available from the gaming engine. This can include:

| Type | Format | Used in |
| --- | --- | --- |
| depth map | per-pixel, 14 bit | JVET-Y0041 |
| optical flow | quarter-pel | JVET-Y0041 |
| camera information | 4x4 array, 32bit | JVET-AF0187 |
| depth map | per-pixel, 32bit | JVET-AF0187 |
| motion vector | per-pixel, 32bit | JVET-AF0187 |
| camera information | 4x4 array, 32bit | DesertTown/ Sun_Temple when released |
| depth map | per-pixel, 32bit (R32) | DesertTown/ Sun_Temple when released |
| motion vector | per-pixel, 2x32bit (R32G32) | DesertTown/ Sun_Temple when released |
| albedo texture | per-pixel, 4x8bit (R8G8B8A8) | DesertTown/ Sun_Temple when released |
| normal map | per-pixel, 3x32bit (R32G32B32) | DesertTown/ Sun_Temple when released |

# Sequence segments to be simulated

Of each sequence a segment of 5 seconds was selected for encoding for January meeting. This data should help identifying the most interesting sequences for a new class of gaming content sequences.

The table below lists the selected segments for each sequence. Note that for some sequence more than one potentially interesting segment was identified.

| Sequence name | Frame count | Frame rate | Bit depth | Start frame | End frame |
| --- | --- | --- | --- | --- | --- |
| Level1 | 600 | 60 | 10 | 0 | 299 |
| Darktree | 600 | 60 | 10 | 300 | 599 |
| ArenaOfValor | 600 | 60 | 8 | 0 | 299 |
| ARPG | 600 | 60 | 8 | 100 | 399 |
| DesertTown1 | 600 | 60 | 8 | 200 | 499 |
| DesertTown2 | 600 | 60 | 8 | 150 | 449 |
| Sun_Temple1 | 600 | 60 | 8 | 40 | 339 |
| Sun_Temple2 | 600 | 60 | 8 | 50 | 349 |
| CSGO | 3600 | 60 | 8 | 1100 | 1399 |
| DOTA2 | 3600 | 60 | 8 | 1734 | 2033 |
| EuroTruckSimulator2 | 3600 | 60 | 8 | 1000 | 1299 |
| Fallout4 | 3600 | 60 | 8 | 600 | 899 |
| GTAV | 3600 | 60 | 8 | 300 | 599 |
| Hearthstone | 3600 | 60 | 8 | 3099 | 3398 |
| Minecraft | 3600 | 60 | 8 | 600 | 899 |
| Rust | 3600 | 60 | 8 | 3000 | 3299 |
| Starcraft | 3600 | 60 | 8 | 2900 | 3199 |
| Witcher3 | 3600 | 60 | 8 | 1300 | 1599 |
| Baolei-Man | 600 | 60 | 8 | 300 | 599 |
| Baolei-Balloon 4K | 600 | 60 | 8 | 300 | 599 |
| Baolei-Yard 4K | 600 | 60 | 8 | 60 | 359 |
| Baolei-Woman | 600 | 60 | 8 | 120 | 419 |
| Jianling-Temple | 600 | 60 | 8 | 0 | 299 |
| Jianling-Beach | 600 | 60 | 8 | 0 | 299 |
| Heroes of the Storm part 1 | 300 | 30 | 8 | 150 | 299 |
| Project CARS | 300 | 30 | 8 | 0 | 149 |
| WoW part 2 | 300 | 30 | 8 | 150 | 299 |

# Coding performance results

Encoding ongoing. Will be added when available.

# Recommendations

The AHG recommends to:

- Continue to evaluate candidate sequences for gaming content compression

- Continue to refine testing conditions (use classF yes/no, potentially adjust QPs)

- Identify if/how auxiliary information can be used for coding of gaming content

- Review input documents on gaming content compression

# References

- S. Puri, J. Sauer, R. Chernyak, A. Duenas, L. Wang, “AHG15: Report on AhG meeting on gaming content compression”, JVET-AG0047, Dec. 2023.

# Annex A. Test sequence checksums

For each test sequence included in the common test conditions, the MD5 hash is listed below. The list can be copied into a text file and hashes can be checked using the md5sum tool:

md5sum -c <filename>

| Sequence name | md5sum |
| --- | --- |
| Level1 | 314468a1ce25a2132d442ee3bda5991a |
| Darktree | e105da995a5d2880ef7797ccd17330d2 |
| ArenaOfValor | 26ca111ff83ecf4e9c9e3de3dc7d1fe4 |
| ARPG | d8fe44ed8e2c1417261b0b2e075a66ec |
| DesertTown1 | 9369d1aeae138aecfb15e7ab15defe32 |
| DesertTown2 | 6fbb5f9ef998cb0618f2c9d5ded83a48 |
| Sun_Temple1 | be30f44c1a2a8d03dab1c423af2ae327 |
| Sun_Temple2 | a417325a4e768f2eceeac54091d78f7b |
| CSGO | 1477f59223506131735ffde6deb9e12a |
| DOTA2 | 0afecf2b882a33f905e682998ea0e6d5 |
| EuroTruckSimulator2 | 1c26ed7104bff313b1197488623a9b20 |
| Fallout4 | 09980ed191d285449d591fb0cf3dbe43 |
| GTAV | fb2e6baa7d01b2c2df31efd9662165c7 |
| Hearthstone | de95a45bda1c39f35bfe83878a63ae52 |
| Minecraft | b6a4d0c30d3d06094dd2efa00d38bd40 |
| Rust | 02fb09c502e91b0f339bcd1680c484f9 |
| Starcraft | 7efa9cdbc0c6c88ec658d4baa2c1fb20 |
| Witcher3 | 89a54257307fb961a8dcf753801df8e2 |
| Baolei-Man | 6b41e581136cef2a9bd92259237736bf |
| Baolei-Balloon 4K | e537665c18e32bbaf8e5e9d63e18dd2c |
| Baolei-Yard 4K | 9ee5cf01da50eaf5a286e6128684d481 |
| Baolei-Woman | cd279102ed3c0f30647871d92a098eed |
| Jianling-Temple | 408e5ed2c441a2fe71a6653615c9489d |
| Jianling-Beach | 06bacef74bc091ba6d670bd730adb547 |
| Heroes of the Storm part 1 | 2d9ba463d2bda0948b648eb6bd057318 |
| Project CARS | c3872a5371508667d89a5b4b3f31eabb |
| WoW part 2 | 07acbb319bb4c46e077fc296159b4131 |

# Annex B. Disscussion items during conference calls

## Testing conditions

### Should all tools be enabled?

One expert recommended to keep all tools enabled as in default configuration.

Decision: Keep all tools. Additionally run test with classF config.

### Used QPs

One expert commented that QP22 will almost never be used for practical streaming applications of gaming content, as it requires too much bitrate. For example rate with QP22 crosses default profile/level threshold.

Question: What is typical kbps at which gaming content is streamed?

Decision:  Do not use QP22 in first round of experiment, due to the increase of simulation time this would entail. In particular for 4K sequences.

Question: Do regular sequences also cross profile/level threshold? A: No. But need to check which sequence had this problem.

### Resolution of content

Question: is gaming content actually streamed in 4K resolution? Or only lower resolutions are used? Comment: 4K low delay might take a long time, especially for QP22. 
Comment: 4K for low delay with VTM previously took 167h w/o classF cfg.

Comment:  Showing results for 4K may be helpful for investigation whether new standard is required

## Available gaming content

One expert commented that there is more gaming content available from Twitch/3GPP. Need to check license of those. Probably can be used, as some of them were also used by Twitch / 3GPP SA4 S5 in the past. Currently no auxiliary data for these. Could try ask creators if it can be provided.
It was commented that JVET-U0043 proposed Minecraft, CSGO, and StarCraft of this set. License unclear:
“These video game recordings are placed in the public domain. The recordings may contain copyrighted work which can only be used in compliance with fair use.”

Update: The 3GPP SA4 S5 gaming sequences are available at: 
Index of /129021/dash/WAVE/3GPP/5GVideo/ReferenceSequences (dash-large-files.akamaized.net)

The 3GPP SA4 S5 gaming class consisted of 13 gaming sequences that are listed in the 3GPP SA4 TR 26.955 document under Table C.2.1-1 on page 221. The information on the licenses for each of these sequences are also available in this document under Annex C.