| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0064-v2 |
| --- | --- |

| Title: | AHG12: On Context modeling in Chroma Coefficient Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luhang Xu, Haoping Yu, Yue Yu, Dong Wang (OPPO) | Tel: Email: | xuluhang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution proposes to use the same and consistent spatial neighbors to derive context model index and Rice parameter for the chroma blocks not coded by LFNST or NSPT. The proposed method is implemented on the ECM-11.0 software.

The simulation results of the proposed method under CTC are shown as follows,

RA: -0.00% (Y), -0.02% (U), -0.04% (V)

The simulation results of the proposed method when DualITree=0 are shown as follows,

AI: 0.01%(Y), -0.22%(U), -0.18%(V)

Introduction

In JVET-AE0102 [1], a different derivation method for context model index and Rice parameter was proposed for the coefficients in the LFNST and NSPT coded transform blocks. In particular, the pattern of 5 spatial neighbor positions shown on the right side of Figure 1, was proposed to be used in deriving the context models for sig_flag, gtx_flag, par_flag and the Rice parameters for abs_remainder, dec_abs_level, when the block is coded with LFNST and NSPT transforms. For regular primary transform blocks, the pattern on the left of Figure remains used without any change in the derivation of context models and the Rice parameters.

Figure 1. Coefficients used to model the context in ECM, left pattern for modeling non-LFNST/NSPT, right pattern for modeling LFNST/NSPT

The selection of above pattern is dependent on the number of cu.lfnstIdx in the ECM-11.0 reference software. In inter slices, LFNST/NSPT is not applied to chroma blocks when cu.lfnstIdx is nonzero. However, the right pattern is still used for modeling the context for the chroma blocks in inter slices.

Proposed method

It is proposed to use the left pattern for deriving context modeling index and the Rice parameter when the chroma blocks are not coded with LFNST/NSPT. This proposal will make the context modeling and Rice parameter derivation processes consistent among all the non-directional transform coded blocks.

Simulation results

The proposed method is implemented on top of the ECM-11.0 software [2], and tested based on the common test condition [3]. The test results are summarized in Table 1.

Table-1. The proposed method over ECM-11.0 (RA)

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over 20231122-ECM11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | -0.03% | 0.00% | 102.6% | 98.4% | 99.8% | 100.2% |
| Class A2 | -0.01% | 0.07% | -0.02% | 103.3% | 101.7% | 100.1% | 100.1% |
| Class B | 0.02% | -0.07% | -0.05% | 104.0% | 101.1% | 100.0% | 100.0% |
| Class C | -0.01% | -0.02% | -0.07% | 104.2% | 100.0% | 100.1% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.04% | 103.7% | 100.4% | 100.0% | 100.0% |
| Class D | -0.01% | -0.03% | 0.02% | 104.7% | 97.5% | 99.9% | 100.0% |
| Class F | -0.01% | 0.01% | -0.04% | 105.1% | 96.3% | 99.6% | 100.0% |

*note the test results above are not with accurate encoding/decoding time.

The proposed method is also tested when DualITree is set to 0. In this condition, Dualtree partitioning is disabled for I slice.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over DualITree=0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.01% | -0.23% | -0.19% | 100.4% | 99.4% | 100.0% | 100.0% |
| Class A2 | 0.01% | -0.14% | -0.14% | 99.7% | 99.7% | 99.6% | 99.9% |
| Class B | 0.00% | -0.26% | -0.19% | 100.4% | 100.1% | 100.0% | 100.0% |
| Class C | 0.01% | -0.38% | -0.26% | 100.4% | 98.9% | 100.0% | 100.0% |
| Class E | 0.00% | -0.05% | -0.11% | 99.8% | 101.7% | 100.0% | 100.0% |
| Overall | 0.01% | -0.22% | -0.18% | 100.2% | 99.9% | 99.9% | 100.0% |
| Class D | -0.02% | -0.36% | -0.13% | 99.7% | 97.2% | 99.9% | 100.0% |
| Class F | -0.01% | -0.11% | 0.03% | 101.4% | 100.1% | 100.0% | 100.0% |

Conclusions

This contribution proposes to use the same and consistent spatial neighbors to derive context model index and Rice parameter for the chroma blocks not coded with LFNST or NSPT. It is suggested to adopt the proposed method in the next version of ECM reference software.

References

- P. Nikitin, M. Coban, C. Coban, B. Ray, V. Seregin, M. Karczewicz, “EE2-4.3: Context modeling for transform coefficients for LFNST/NSPT”, JVET-AE0102, July, 2023.

- ECM-11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-11.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AD2017, April, 2023.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).