| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0068-v2 |
| --- | --- |

| Title: | EE2-3.2 related: On CABAC bin budget |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Gan Yue Yu Haoping Yu | Tel: Email: | v-jonathan.gan@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution proposes two aspects related to the implementation of JVET-AG0100 (EE2-3.2) with CABAC bin budget management. The first aspect proposes to change the CABAC bin check to match the increased number of flags proposed by JVET-AG0100 and brings minor loss of {0.00%, 0.04%, 0.01%} in AI relative to EE2-3.2b. The second aspect proposes to apply the same CABAC bin budget used by EE2-3.2 to TSRC and shows minor loss of {0.00%, 0.03%, 0.02%} on CTC and gain of {-0.45%, -0.49%, -0.54%} on TGM class in AI relative to EE2-3.2b.

# Introduction

In the regular residual coding (RRC) process of ECM-11.0 [1], absolute coefficient levels are represented by sig_coeff_flag (sig), abs_level_gtx_flag[0] (gt1), par_level_flag (par), abs_level_gtx_flag[1] (gt3) syntax elements along with bypass coded abs_remainder (remLevel). The four flags are context coded and usage of context coding is controlled by a CABAC bin budget set at 1.75 bins per coefficient. The design of this budget is intended to provide some guarantees on the worst-case throughput required by context coding. At each coefficient, the use of context coding is first gated by a check of whether the remaining CABAC bin budget is ≥ 4. If the bin budget is exhausted, then the remaining coefficients must be bypass coded by dec_abs_level.

JVET-AG0100 [2] modifies the RRC process by increasing the number of gtX flags to 7, moving the parity flag in coding order to after the gt7_flag, and coding the parity flag as a bypass bin. Overall, the number of context coded flags that could potentially be signalled per coefficient is increased to 8. Additionally, the bin budget is increased to 2.5 bins per coefficient.

# Aspect 1

Although JVET-AG0100 increases the number of context coded flags per coefficient to 8, the bin budget check still checks for ≥ 4. The impact of this is that each transform block can exceed the bin budget by up to 4 bins. This contribution asserts that this is undesirable as it implies the bin budget is not proportional to the number of coefficients, but instead has a worst case for the smallest TB size.

Aspect 1 of this contribution proposes to correct the bin budget check to ≥ 8. That is, the condition gating the first scan pass is modified:

| for( n = firstPosMode0; n >= 0 && remBinsPass1 >= 8; n− − ) { |  |
| --- | --- |
| /* signal sig_coeff_flag, abs_level_gtx_flag[ j ] and par_level_flag */ |  |
| } |  |

Aspect 1 was implemented on top of EE2-3.2b. Results are shown in Table 1 and 2. Runtimes are not accurate as the anchor was obtained from JVET-AG0100.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.23% | -0.24% | -0.22% | 101.8% | 102.5% | 100.7% | 99.8% |
| Class A2 | -0.22% | -0.12% | -0.10% | 101.8% | 102.5% | 102.3% | 99.9% |
| Class B | -0.18% | -0.12% | -0.26% | 101.8% | 102.1% | 100.7% | 100.0% |
| Class C | -0.24% | -0.22% | -0.21% | 102.2% | 101.1% | 100.3% | 99.9% |
| Class E | -0.17% | -0.02% | -0.19% | 102.3% | 101.7% | 100.0% | 100.0% |
| Overall | -0.21% | -0.15% | -0.20% | 102.0% | 101.9% | 100.8% | 99.9% |
| Class D | -0.23% | -0.32% | -0.28% | 103.5% | 101.4% | 100.0% | 100.0% |
| Class F | -0.08% | -0.10% | 0.20% | 101.5% | 101.2% | 101.4% | 100.0% |
| Class TGM | -0.05% | -0.04% | -0.12% | 101.7% | 99.5% | 101.3% | 99.9% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.20% | -0.20% | -0.09% | 105.6% | 103.1% | 100.1% | 100.0% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | -0.15% | 0.02% | -0.08% | 105.9% | 103.0% | 100.0% | 99.9% |
| Class C | -0.13% | -0.18% | -0.32% | 105.5% | 101.2% | 100.2% | 99.9% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0.14% | -0.32% | -0.36% | 105.8% | 97.5% | 100.1% | 100.0% |
| Class F | -0.13% | -0.17% | -0.16% | 106.3% | 98.9% | 100.0% | 99.8% |
| Class TGM | 0.00% | -0.07% | 0.08% | 107.0% | 98.6% | 100.6% | 100.3% |

Table 1: Aspect 1 results over ECM-11.0

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over EE2-3.2b |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.01% | 0.01% | 109.9% | 105.5% | 100.4% | 99.9% |
| Class A2 | 0.00% | 0.02% | 0.02% | 109.5% | 107.2% | 102.3% | 100.0% |
| Class B | 0.00% | -0.02% | -0.04% | 108.3% | 105.5% | 100.7% | 99.8% |
| Class C | -0.01% | 0.15% | 0.05% | 106.8% | 108.2% | 100.3% | 99.8% |
| Class E | 0.01% | 0.01% | 0.02% | 107.3% | 105.5% | 99.9% | 99.8% |
| Overall | 0.00% | 0.04% | 0.01% | 108.2% | 106.4% | 100.7% | 99.8% |
| Class D | 0.03% | -0.03% | -0.02% | 109.2% | 112.4% | 99.9% | 99.9% |
| Class F | 0.01% | 0.04% | 0.11% | 113.6% | 130.4% | 101.6% | 100.1% |
| Class TGM | -0.04% | -0.03% | -0.07% | 113.1% | 130.7% | 101.2% | 99.9% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over EE2-3.2b |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | 0.01% | -0.02% | 109.5% | 107.3% | 100.0% | 99.8% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | 0.00% | -0.02% | 0.00% | 108.6% | 106.0% | 100.1% | 99.9% |
| Class C | 0.01% | -0.12% | -0.08% | 106.9% | 106.2% | 100.0% | 99.9% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0.02% | 0.04% | -0.08% | 107.0% | 106.5% | 100.1% | 99.8% |
| Class F | 0.00% | -0.08% | -0.05% | 113.1% | 118.4% | 100.0% | 99.5% |
| Class TGM | -0.01% | -0.05% | 0.02% | 123.3% | 138.4% | 100.6% | 100.8% |

Table 2: Aspect 1 results over EE2-3.2b

# Aspect 2

JVET-AG0100 increases the bin budget for RRC to 2.5 bins per coefficient, but the bin budget for TSRC is still 1.75 bins per coefficient.

Aspect 2 of this contribution proposes to increase the bin budget of TSRC to 2.5 bins per coefficient, matching the increased bin budget for RRC introduced by JVET-AG0100. The worst-case context coding throughput is asserted to be unaffected by this change.

Aspect 2 was implemented on top of EE2-3.2b. Results are shown in Tables 3 and 4. Runtimes are not accurate.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.22% | -0.31% | -0.21% | 94.3% | 97.6% | 101.1% | 100.0% |
| Class A2 | -0.21% | -0.13% | -0.10% | 93.9% | 96.6% | 100.0% | 99.9% |
| Class B | -0.19% | -0.15% | -0.25% | 94.4% | 97.2% | 100.0% | 100.0% |
| Class C | -0.24% | -0.21% | -0.24% | 94.3% | 97.3% | 100.0% | 100.0% |
| Class E | -0.18% | 0.02% | -0.11% | 94.4% | 96.9% | 100.0% | 100.0% |
| Overall | -0.21% | -0.16% | -0.19% | 94.3% | 97.2% | 100.2% | 100.0% |
| Class D | -0.28% | -0.32% | -0.11% | 94.3% | 98.2% | 100.1% | 100.0% |
| Class F | -0.24% | -0.35% | 0.10% | 94.8% | 97.1% | 100.1% | 100.0% |
| Class TGM | -0.47% | -0.51% | -0.60% | 102.6% | 102.2% | 100.0% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class C | -0.16% | -0.24% | -0.11% | 101.3% | 99.9% | 99.1% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0.15% | -0.33% | -0.46% | 100.9% | 97.2% | 99.8% | 100.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class TGM | -0.36% | -0.51% | -0.43% | 98.8% | 95.6% | 100.3% | 100.2% |

Table 3: Aspect 2 results over ECM-11.0

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over EE2-3.2b |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.01% | -0.05% | 0.02% | 101.8% | 100.4% | 100.8% | 100.0% |
| Class A2 | 0.00% | 0.01% | 0.02% | 100.9% | 101.1% | 100.0% | 100.0% |
| Class B | 0.00% | -0.05% | -0.03% | 100.5% | 100.5% | 100.0% | 99.8% |
| Class C | -0.02% | 0.16% | 0.02% | 98.6% | 104.2% | 99.9% | 99.8% |
| Class E | 0.00% | 0.06% | 0.10% | 99.1% | 100.6% | 99.9% | 99.9% |
| Overall | 0.00% | 0.03% | 0.02% | 100.1% | 101.4% | 100.1% | 99.9% |
| Class D | -0.02% | -0.03% | 0.14% | 99.5% | 108.9% | 100.0% | 99.9% |
| Class F | -0.15% | -0.21% | 0.01% | 106.1% | 125.2% | 100.3% | 100.1% |
| Class TGM | -0.45% | -0.49% | -0.54% | 114.1% | 134.2% | 100.0% | 100.1% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over EE2-3.2b |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class C | -0.02% | -0.18% | 0.12% | 102.6% | 104.8% | 98.9% | 99.9% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0.03% | 0.03% | -0.18% | 102.1% | 106.1% | 99.9% | 99.9% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class TGM | -0.36% | -0.49% | -0.49% | 113.9% | 134.2% | 100.3% | 100.8% |

Table 4: Aspect 2 results over EE2-3.2b

# Supplementary results (informative)

As both JVET-AG0100 and aspect 2 of this contribution introduce increased CABAC bin budget, this contribution also investigates whether such an increase brings benefit, and how the trade-off is compared with the existing RRC in ECM-11.

Tests were performed varying the CABAC bin budget over the range {1.25, 1.50, 1.75, 2.00, 2.25, 2.50} bins per sample. In all cases the bin budget is adjusted for both RRC and TSRC. Due to limited compute resources, the tests were only performed over AI configuration.

Full results are included in the folder “Supplementary results” of this contribution. Tables 5 and 6 summarise the results for varying the bin budget on ECM-11.0 and EE2-3.2b respectively.

| Budget | Overall |  |  | Class F |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| 1.25 | 0.13% | 0.12% | 0.09% | 0.46% | 0.37% | 0.54% |
| 1.50 | 0.02% | 0.07% | 0.06% | 0.20% | 0.24% | 0.39% |
| 1.75 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| 2.00 | 0.00% | -0.04% | -0.02% | -0.07% | -0.20% | 0.06% |
| 2.25 | -0.01% | -0.04% | 0.01% | -0.14% | -0.15% | 0.20% |
| 2.50 | -0.01% | -0.03% | -0.02% | -0.18% | -0.27% | -0.13% |

Table 5: ECM-11.0 at different CABAC bin budgets (both RRC and TSRC)

| Budget | Overall |  |  | Class F |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| 1.25 | -0.06% | -0.03% | -0.07% | 0.37% | 0.39% | 0.63% |
| 1.50 | -0.16% | -0.10% | -0.15% | 0.11% | 0.12% | 0.29% |
| 1.75 | -0.19% | -0.13% | -0.16% | -0.05% | -0.08% | 0.26% |
| 2.00 | -0.20% | -0.16% | -0.20% | -0.19% | -0.19% | -0.11% |
| 2.25 | -0.21% | -0.20% | -0.19% | -0.21% | -0.29% | -0.07% |
| 2.50 | -0.21% | -0.16% | -0.19% | -0.24% | -0.35% | 0.10% |

Table 6: EE2-3.2b at different CABAC bin budgets (both RRC and TSRC)

# References

[1] https://vcgit.hhi.fraunhofer.de/ecm/ECM

[2] 	P. Nikitin, M. Coban, M. Karczewicz, P. Garus, V. Seregin, “EE2-3.2: Transform coefficient coding”, JVET-AG0100, Meeting by teleconference, 17-26 January 2024.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).