| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0071-v0 |
| --- | --- |

| Title: | Informal Subjective Evaluation of Low Complexity Enhancement Video Codec (LCEVC) with VVC on SDR UHD (4K) Content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Olena Chubach, Hong-Hui Chen, Ching-Yeh Chen, Tzu-Der Chuang, Yi-Wen Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Email: | {olena.chubach, hh.chen, chingyeh.chen, peter.chuang, yiwen-yw.chen, cw.hsu, yuwen.huang, shawmin.lei} mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution presents additional objective and subjective evaluation results of Low Complexity Enhancement Video Codec (LCEVC) using its reference implementation (LTM-5.4.1) when combined with VVC (VTM-19.0) as base layer codec. Tests were performed for 4K content, following JVET Random Access (RA) Common Testing Conditions (CTC). An informal viewing session was performed to evaluate visual quality of LCEVC when combined with full resolution or up-scaled single layer VVC, which showed that no clear visual benefit can be observed for LCEVC when used with VVC at various bitrates of an extended bitrate range. Moreover, at lower bitrates, using simple downscaling and upscaling of the coded results can easily provide visual quality similar to or better than LCEVC.

Introduction

The formal assessment of LCEVC [1 - 3] has been completed in April 2021 at the 134th MPEG meeting. The official report on LCEVC verification tests was published at MPEG website as a WG04 document, N0076 [4] (it should be noted that confidence intervals (CIs), present in [4], have been omitted in [3]). This report, among others, contains LCEVC evaluation results with SDR UHD (4K) content coded with following codecs:

•	HEVC (HM-16.20) with bitrates 845 kbps to 12.6 Mbps

•	VVC (VTM-11) with bitrates 227 kbps to 9.4 Mbps.

A detailed information for SDR UHD tests with VVC reported in [4] is presented in Table I.

Table I. Table of results for SDR UHD sequences

| Sequence | full-VTM |  |  |  | half-VTM |  |  |  | LTM+VTM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP | bitrate | MOS | CI | QP | bitrate | MOS | CI | QP | bitrate | MOS | CI |
| Marathon | 30 | 9256.35 | 7.97 | 0.25 | 26 | 7580.79 | 7.04 | 0.30 | 26 | 7599.06 | 7.97 | 0.27 |
|  | 34 | 5362.65 | 6.63 | 0.33 | 29 | 4762.11 | 5.58 | 0.22 | 29 | 4770.36 | 6.64 | 0.30 |
|  | 38 | 3100.83 | 4.83 | 0.36 | 32 | 2997.54 | 4.38 | 0.38 | 32 | 3005.76 | 5.42 | 0.28 |
|  | 42 | 1777.83 | 3.02 | 0.38 | 36 | 1673.64 | 2.25 | 0.30 | 36 | 1688.61 | 3.61 | 0.31 |
| MountainBay2 | 34 | 1672.14 | 6.63 | 0.26 | 29 | 1498.41 | 6.26 | 0.29 | 29 | 1505.88 | 6.79 | 0.42 |
|  | 38 | 860.58 | 5.61 | 0.35 | 33 | 777.09 | 5.05 | 0.27 | 33 | 786.03 | 5.75 | 0.27 |
|  | 42 | 442.47 | 3.71 | 0.34 | 37 | 408.30 | 3.49 | 0.29 | 37 | 419.37 | 4.21 | 0.24 |
|  | 46 | 226.91 | 1.79 | 0.37 | 40 | 248.70 | 2.03 | 0.22 | 40 | 260.34 | 2.50 | 0.26 |
| DrivingPOVLogo | 28 | 9413.40 | 8.66 | 0.30 | 24 | 6850.98 | 7.49 | 0.29 | 24 | 7675.50 | 8.79 | 0.26 |
|  | 32 | 5073.78 | 7.78 | 0.24 | 27 | 4279.14 | 6.13 | 0.29 | 27 | 4915.26 | 7.91 | 0.29 |
|  | 38 | 2218.44 | 5.88 | 0.28 | 33 | 1712.58 | 4.63 | 0.28 | 33 | 2078.94 | 5.97 | 0.21 |
|  | 42 | 1311.42 | 4.57 | 0.28 | 37 | 993.90 | 3.66 | 0.26 | 37 | 1209.36 | 4.63 | 0.31 |
| BoxeLogo | 30 | 1996.44 | 7.29 | 0.35 | 27 | 1286.34 | 5.08 | 0.40 | 27 | 1603.74 | 7.67 | 0.46 |
|  | 34 | 1326.42 | 6.28 | 0.34 | 30 | 917.16 | 4.38 | 0.29 | 30 | 1172.64 | 6.42 | 0.22 |
|  | 38 | 908.58 | 4.91 | 0.24 | 32 | 742.80 | 3.88 | 0.25 | 32 | 939.84 | 5.67 | 0.27 |
|  | 42 | 632.04 | 3.92 | 0.39 | 37 | 455.46 | 2.62 | 0.24 | 37 | 586.32 | 4.34 | 0.26 |

As it can be seen from Table I, two sequences (MountainBay2 and BoxeLogo) out of total four SDR UHD sequences, used for verification test of LCEVC with VVC, were encoded with bitrates ranging between 226 kbps and 2 Mbps. For the other two sequences (Marathon and DrivingPOVLogo), half of the test points are below 3 Mbps, while maximum bitrate is below 10 Mbps.

In the N0076 report, it is revealed that “Numerical analysis of the average benefit of LCEVC and its statistical significance compared to the corresponding full resolution […] VVC codec was more difficult to interpret, due to several test points having overlapping confidence intervals, as such the MOS BD-rate savings are not reported.” Meaning that visual benefits of LCEVC with VVC are unclear at the tested bitrate range.

In case of UHD content, the common bitrates are between 5 and 20 Mbps, depending on the content. Bitrates below 2 Mbps are of worse quality. Showing original resolution at such low (and lower) bitrates is irrational. In the cases of very low bitrate requirements, a much better solution is to use a down-sampled HD (or lower resolution) version of UHD content. This allows to keep higher quality of the coded content, while preserving the low bitrate constraints.

A more recent evaluation of LCEVC with HEVC (and VVC) on SDR UHD content coded at extended bitrates, was performed by MediaTek (JVET-AF0075, [5]) and crosschecked by Broadcom [6], HHI [7], and Qualcomm [8]. For this evaluation, tested bitrate range was extended for SDR UHD (4K) content coded with HEVC and VVC. Results of this evaluation identified the following: although LCEVC may show some benefits on certain types of content, coded at very low bitrates, there are bitrate limitations, after which LCEVC may not provide benefit anymore. For example, LCEVC did not show any clear benefit at higher bitrates, while resulting in significant losses in chroma components in terms of PSNR-BD-rate for all sequences coded with both, HEVC and VVC. Such significant chroma losses were obviously identifiable during the subjective evaluation of the compressed frames.

Testing Conditions and Settings Used for LCEVC Evaluation

In the current test, 4K sequences in JVET CTC with RA configuration [9] are encoded.

Following codec is used as anchor and corresponding base layer encoder, when used in combination with LCEVC:

•	H.266/VVC: VTM-19.0 reference software [10]

For LCEVC, LTM-5.4.1 reference software implementation was obtained from the MPEG Git (obtaining it requires registration) [11].

LTM-5.4.1 was compiled according to the instructions from the software manual provided with the LTM software package. For running LTM with HM and VTM, it is recommended to check the LTM software manual and [5].

Following tests are performed:

Test1. H.266/VVC: VTM-19.0 (single layer encoding of 4K content with QP=22, 27, 32, 37)

Test2. H.266/VVC + LCEVC: VTM-19.0 (1080p base encoder) + LTM-5.4.1 (4K enhancement layer encoder)

Test2 is run for base layer QPs between 12 and 32 for VVC. After that, the closest rates of Test2 (base layer + enhancement layer) are chosen for the corresponding rates of Test1.

After running Test1 and Test2, it was noticed that two sequences – Campfire and ParkRunning3 – have high bitrates when coded with the lowest QP values. For VVC (VTM):

- Campfire (QP22: 46.14 Mbps)

- ParkRunning3 (QP22: 97.29 Mbps; QP27: 39.19 Mbps)

As an additional exercise, these QP points of Test1 and Test2 are replaced such that maximum bitrates of all tested sequences do not exceed ~20 Mbps:

- Test3. H.266/VVC: for Campfire replace QP22 with QP25 (17.72 Mbps); for ParkRunning3 replace QP22 and QP27 with QP35 (11.00 Mbps) and QP40 (4.66 Mbps)

- Test4. H.266/VVC + LCEVC: find the closest match to the corresponding new QP points of Test3.

Objective Results

In the current tests, PSNR_Y (PSNR computed for luma component), PSNR_Cb, and PSNR_Cr (PSNR computed for two chroma components) are used as the quality metric to calculate BD-rates.

Combined PSNR_YCbCr is obtained as follows:

PSNRYCbCr  = 0.75*PSNRY  + 0.125*PSNRCb  + 0.125*PSNRCr

Simulation results of Test2 (VVC + LCEVC) vs Test1 (VVC) presented in Table II. Positive values mean that test has a larger file size than the anchor, under the same PSNR value. More detailed results of LCEVC with VVC (and HEVC) can be found in [5].

Table II. Simulation results of Test2 (VVC + LCEVC, VTM-19.0 + LTM-5.4.1) vs Test1 (VVC, VTM-19.0) as anchor

| Sequence | Random Access, Over VTM-19.0 |  |  |  |
| --- | --- | --- | --- | --- |
|  | PSNR_Y | PSNR_Cb | PSNR_Cr | PSNR_YCbCr |
| Tango2 | -0.11% | -2.30% | 9.29% | 1.24% |
| FoodMarket4 | 6.03% | 4.99% | 5.31% | 5.87% |
| Campfire | 32.66% | 156.97% | 59.76% | 52.80% |
| CatRobot | 49.23% | 49.91% | 80.22% | 53.49% |
| DaylightRoad2 | 86.81% | 17.06% | 48.41% | 67.43% |
| ParkRunning3 | 14.95% | 370.64% | 220.60% | 33.05% |
| Overall | 31.59% | 99.55% | 70.60% | 35.65% |

Results of Test4 vs Test3, which are considering new QP points for Campfire and ParkRunning3, provided in Table III. The PSNR_YCbCr loss is 26.99%, which is still considered to be significant.

Table III. Simulation results of Test4 (VVC + LCEVC, VTM-19.0 + LTM-5.4.1) vs Test3 (VVC, VTM-19.0) as anchor

| Sequence | Random Access, Over VTM-19.0 |  |  |  |
| --- | --- | --- | --- | --- |
|  | PSNR_Y | PSNR_Cb | PSNR_Cr | PSND_YCbCr |
| Tango2 | -0.11% | -2.30% | 9.29% | 1.24% |
| FoodMarket4 | 6.03% | 4.99% | 5.31% | 5.87% |
| Campfire | 12.11% | 107.66% | 41.39% | 28.03% |
| CatRobot | 49.23% | 49.91% | 80.22% | 53.49% |
| DaylightRoad2 | 86.81% | 17.06% | 48.41% | 67.43% |
| ParkRunning3 | -3.06% | 154.88% | 49.57% | 5.88% |
| Overall | 25.17% | 55.37% | 39.03% | 26.99% |

As an additional exercise, Video Multimethod Assessment Fusion (VMAF) BD-rates are computed for sequences from Test4 (VVC + LCEVC, VTM-19.0 + LTM-5.4.1) vs Test5 (VVC, VTM-19.0) as anchor. Results are reported in Table IV as VMAF. Positive value means that the LCEVC has a worse VMAF score than single-layer VVC under the same bitrate.

Table IV. VMAF BD-rate computed for sequences from Test4 (VVC + LCEVC, VTM-19.0 + LTM-5.4.1) vs Test3 (VVC, VTM-19.0) as anchor

| Random Access, Over VTM-19.0 |  |  |  |
| --- | --- | --- | --- |
| Sequence | VMAF | Sequence | VMAF |
| Tango2 | 39.25% | CatRobot | 92.31% |
| FoodMarket4 | 20.18% | DaylightRoad2 | 85.72% |
| Campfire | 18.26% | ParkRunning3 | 3.43% |
| Overall | 43.19% |  |  |

It is important to mention that in its current design, VMAF uses only luma features, and does not use or consider any chroma features. Considering the imbalance between luma and chroma components in LCEVC, authors of this contribution still discourage using VMAF for evaluating LCEVC.

Subjective Evaluation Results

Test Setup and Evaluation Procedure

Screenshots demonstrating a subjective quality of the video frames encoded with different base encoders (with and without LCEVC) at closest matching rates are available in [5].

A visual assessment of 4K content was performed at MediaTek in Hsinchu (Taiwan). The goal was to compare visual quality of sequences coded with LCEVC when used with VVC at various bitrates, extending the bitrate range used in [4].

51 people participated, aging between 24 and 34 years old, 28 male, 23 female. Each session had three people viewing the content, which resulted in total 17 viewing sessions.

Three out of four Test Points (TPs) of each sequence from Test3 and corresponding TPs from Test4 were used for the viewing. Sequences and respective bitrates used for the viewing are reported in Table V.

In cases of very low bitrates, streaming UHD (4K) content may introduce significant artifacts and becomes unreasonable. In particular, for the lowest bitrates (TP3) it is suggested to replace the 4K sequence with 1080p version of the same sequence coded by VVC at matching bitrate (TP4). For TP4, sequences at UHD resolution (3840×2160), are downscaled to 1920×1080 using the TAppDownConvertStatic tool from the SHM repository [12]. The following example command lines are used for UHD BT.709-color content (chroma location 0):

TAppDownConvertStatic 3840 2160 <input_UHD.yuv> 1920 1080 <output_HD.yuv> 10 10

The downscaled sequences are coded with VTM-19.0, and then upscaled using the VVC upscaling filters. The upscaled videos are then compared with the LCEVC encodings of TP3.

The subjective test was carried out according to the recommendations in [13]. Model of the TV used for viewing: Samsung 65” QLED TV (QA65Q70TAW). The distance between the TV and the viewers was set at 1.5*H, where H is the height of the TV screen.

Two coded sequences (using VTM-19.0 and LTM-4.5.1+VTM-19.0) at matching bitrates are demonstrated to the viewers. The viewers compared the visual quality of the UHD or upscaled version encoded with VTM-19.0 to the visual quality of the LTM-5.4.1+VTM-19.0 version at matching rates. Original uncompressed content is not used in this evaluation. The assignment of sequences and tests is randomized and not revealed to the test subjects.

Test subjects are asked to evaluate the quality of two reconstructed videos relative to each other, according to the viewers’ personal preference. The test subjects are asked to provide relative score of the visual quality of reconstructed video sequences presented in each test cell as follows:

- “A much better than B”

- “A better than B”

- “A same as B”

- “B better than A”

- “B much better than A”

Table V. Sequences and corresponding TP bitrates used for the informal viewing VTM-19.0 / LTM-5.4.1+VTM-19.0

| Sequence | Bitrate, kbps |  |  |  |
| --- | --- | --- | --- | --- |
|  | VTM-19/0 / LTM-5.4.1 + VTM-19.0 |  |  |  |
|  | TP1 | TP2 | TP3 | TP4 |
| Tango2 | 15782.42/14037.77 | 6691.40/6253.37 | 1955.98/1859.26 | 1858.15/1859.26 |
| FoodMarket4 | 14299.10/13048.51 | 7244.36/7572.08 | 2025.59/1935.47 | 1932.38/1935.47 |
| Campfire | 12166.24/12206.41 | 6593.15/6101.85 | 3494.37/3321.34 | 3221.33/3321.34 |
| CatRobot | 16566.97/15159.76 | 7836.80/8323.48 | 2134.32/2091.01 | 2073.31/2091.01 |
| DaylightRoad2 | 20763.04/17120.03 | 8042.71/8570.71 | 2037.04/1997.71 | 1972.72/1997.71 |
| ParkRunning3 | 17375.88/16913.40 | 10999.65/10021.34 | 4659.57/4204.92 | 4177.13/4204.92 |

Test Results

The MOS and the 95% confidence intervals were calculated for each TP of each sequence. The votes of participants voting the two original sequences being very different are not regarded in the final evaluation; 8 single votes are excluded as obvious outliers.

A resulting MOS value of zero indicates that VTM-19.0 and LTM-5.4.1+VTM-19.0 were rated to have the same visual quality, a positive number indicates a visual benefit for LTM-5.4.1+VTM-19.0, a negative number indicates the visual benefit of VTM-19.0. MOS values and the corresponding confidence intervals (CI) resulting from the evaluation are reported in Fig. 2.

Among 18 TPs of comparing LCEVC with full resolution single layer VVC, only two TPs with the lower bitrates of Campfire show that LCEVC has some visual benefit (confidence interval is above zero); 8 TPs show that VVC has a better visual quality, and the remaining 10 TPs, that the visual quality of VVC and LCEVC is similar (confidence intervals overlap with zero). When the bitrate goes higher, the viewers tend to prefer VVC over LCEVC more. It can be concluded that no clear visual benefit of LCEVC can be observed at the tested bitrate ranges. Also, among 6 TPs of comparing LCEVC with upsampled quarter-resolution single layer VVC, only one TP for FoodMarket4 shows that some visual benefit can be obtained by LCEVC, while for the other 5 TPs, simple upscaling performs at least as good or better than LCEVC (it should be noted that FoodMarket4 has a scene cut). Hence, no additional visual benefit is demonstrated by LCEVC combined with VVC, when coding 4K content at matching bitrates.

| a |  | b |  |
| --- | --- | --- | --- |
| c |  | d |  |
| e |  | f |  |

Fig. 1. Viewing results of VTM-19.0 vs LTM-5.4.1+VTM-19, where a positive number indicates a visual benefit for LTM-5.4.1+VTM-19.0, a negative number indicates the visual benefit of VTM-19.0, and zero means the visual quality is the same. In case of TP4, sequences were upscaled to the original resolution and compared with corresponding LCEVC.

Conclusions

This contribution presents evaluation results of LCEVC reference implementation in combination with VVC reference encoder implementation. Overall, no clear visual benefit was observed for LCEVC when used with VVC at various bitrates of an extended bitrate range, in both subjective evaluation and objective evaluation. Moreover, at lower bitrates, using simple downscaling and upscaling of the coded results can easily provide visual quality similar to or better than LCEVC.

Considering the demonstrated objective losses on the extended bitrate ranges, and no clear subjective benefit of LCEVC when combined with VVC, it is proposed to perform additional formal evaluation of LCEVC for UHD content coded at extended bitrate ranges, which are more reasonable for UHD content, and are not covered by LCEVC verification tests, N0076 [4].

References

ISO/IEC 23094-2—Information Technology—General Video Coding—Part 2: Low Complexity Enhancement Video Coding, Standard ISO/IEC 23094-2:2021, Nov. 2021.

ISO/IEC 23094-3—Information Technology—General Video Coding—Part 3: Conformance and Reference Software for Low Complexity Enhancement Video Coding, Standard ISO/IEC 23094-3:2021, 2022.

S. Battista et al., "Overview of the Low Complexity Enhancement Video Coding (LCEVC) Standard," in IEEE Transactions on Circuits and Systems for Video Technology, vol. 32, no. 11, pp. 7983-7995, Nov. 2022, doi: 10.1109/TCSVT.2022.3182793.

Verification Test Report on the Compression Performance of Low Complexity Enhancement Video Coding, Standard ISO/IEC WG04 N0076, May 2021.

“Evaluation results of Low Complexity Enhancement Video Codec (LCEVC) with HM and VTM on 4K content”, JVET-AF0075, Hannover, October 2023.

“Crosscheck of JVET-AF0075 (Evaluation results of Low Complexity Enhancement Video Codec with HM and VTM on 4K content)”, JVET-AF0228, Hannover, October 2023.

“Crosscheck of JVET-AF0075 (Evaluation results of LCEVC)”, JVET-AF0224, Hannover, October 2023.

“Crosscheck of JVET-AF0075 (Evaluation results of Low Complexity Enhancement Video Codec (LCEVC) with HM and VTM on 4K content)”, JVET-AF0277, Hannover, October 2023.

F. Bossen, X. Li, V. Seregin, K. Sharman, K. Sühring “VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video”, JVET-AB2010, Dec. 2022.

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.

https://mpeg.expert/software/MPEG/LCEVC/LTM/-/tree/LTM_5_4_1

https://vcgit.hhi.fraunhofer.de/jvet/SHM/-/tree/SHM-12.4?ref_type=tags

ITU-R BT.500-15—Methodologies for the Subjective Assessment of the Quality of Television Images,” document Rec. ITU-R BT.500:2023, May. 2023

Patent rights declaration

MediaTek Inc. does not have any current or pending patent rights relating to the technology described in this contribution.