| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0075-v2 |
| --- | --- |

| Title: | AHG12: Adaptive MRL Fusion |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Saverio Blasi Jani Lainema | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes to modify MRL Fusion in ECM. The MRL indexes used to obtain the additional MRL Fusion predictors are selected depending on the current MRL index. Also, the fusion weights are determined adaptively for each block using a template-based search. The impact on coding efficiency and runtimes of the proposed adaptive MRL Fusion over ECM-11.0 are reportedly {for Y, U, V, EncT, DecT, EncVmPeak, DecVmPeak}:

AI {-0.07%, -0.05%, -0.02%, 100.6%, 101.4%, 100.5%, 100.9%}

RA {%, %, %, %, %, %, %}

# Introduction

ECM includes Multiple Reference Line (MRL) Fusion. When performing intra-prediction with a given directional intra-prediction mode, the predictor obtained with the current MRL index i is fused with one additional predictor obtained using the same intra prediction mode with MRL index i + 1. In ECM, fixed weights set to 3/4  and 1/4 are used for the main predictor and the additional predictor, respectively.

# Proposed Method

An Adaptive MRL Fusion process is proposed. The MRL Fusion process is modified so that two additional predictors P1 and P2 are computed, rather than only one. When the current MRL index i = 0, then MRL indexes 1 and 2 are used to compute the two additional predictors. Conversely, when the current MRL index is i > 0, then the MRL indexes i + 1 and i – 1 are used to compute the two additional predictors.

The final prediction is then obtained as a weighted average of the two additional predictors P1 and P2 and the main predictor Pmain:

P = Wmain x Pmain + W1 x P1 + W2 x P2

If the current MRL index is i > 0, then fixed weights set to W1 = 2/8, W2 = 2/8 and Wmain = 4/8 are used.

Instead, if the current MRL index is 0, then a template-based process is used to determine the weights. A template is considered, formed of one line of reconstructed samples located directly above the current block and one column of reconstructed samples located directly on the left of the current block, as illustrated in Figure 1. Then, the samples in the template are predicted with the current intra-prediction mode, using two different reference lines. A first prediction is computed using MRL index 1, resulting in template cost C1. A second prediction is also computed using MRL index 2, resulting in template cost C2, as shown in the figure.

Each template cost is then compared with a set of fixed thresholds as follows:

- if Cx ≤ Tmin then Wx = 3/8

- else if Cx ≤ Tmax then Wx = 2/8

- else Wx = 1/8

for x = 1, 2. The thresholds Tmin and Tmax are set to:

- Tmin = 2 × S  <<  ( BD – 8 )

- Tmax = 6 × S  <<  ( BD – 8 )

where S is the total area of the template, namely width + height of the current block, and BD is the input bit-depth.

The remaining MRL Fusion process is unaltered with respect to ECM. As in ECM, the MRL Fusion process is only used on blocks with area larger or equal than 32 samples. Also, MRL Fusion is only used for non-integer slope directional modes.

Figure 1- template-based weight derivation for Adaptive MRL Fusion

# Experimental results

The tables below summarize the coding efficiency and runtime impact of the proposed method in ECM 11.0 using the JVET common test conditions.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.07% | -0.05% | -0.03% | 101.0% | 100.6% | 100.7% | 100.9% |
| Class A2 | -0.08% | 0.02% | 0.03% | 100.4% | 99.8% | 100.7% | 101.1% |
| Class B | -0.06% | -0.06% | -0.05% | 100.9% | 100.5% | 99.8% | 101.0% |
| Class C | -0.04% | -0.07% | -0.11% | 99.7% | 101.0% | 100.8% | 100.9% |
| Class E | -0.09% | -0.07% | 0.08% | 101.0% | 101.9% | 100.6% | 100.7% |
| Overall | -0.07% | -0.05% | -0.02% | 100.6% | 100.8% | 100.5% | 100.9% |
| Class D | -0.07% | -0.13% | -0.15% | 101.2% | 99.8% | 100.9% | 100.7% |
| Class F | -0.02% | -0.24% | -0.01% | 100.0% | 101.1% | 100.7% | 101.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | 0.00% | -0.05% | 100.3% | #NUM! | 100.4% | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | -0.01% | -0.10% | -0.05% | 100.7% | #NUM! | 100.3% | #NUM! |
| Class C | 0.00% | -0.16% | 0.01% | 100.2% | #NUM! | 99.8% | #NUM! |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0.04% | 0.09% | -0.56% | 100.8% | #NUM! | 100.6% | #NUM! |
| Class F | -0.01% | -0.09% | -0.10% | 100.3% | #NUM! | 101.1% | #NUM! |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).