| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0085-v2 |
| --- | --- |

| Title: | [AHG8] Continuation of study on different VTM versions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Christopher Hollmann | Tel: Email: | christopher.hollmann@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In this contribution the study originally presented in JVET-AE0096 is reported to be continued. Results are made available for VTM-21, VTM-22, and VTM-23. The performance of VTM-21 and VTM-22 is reported to be identical to VTM-20, whereas VTM-23 provides an additional benefit in machine vision task performance compared to VTM-20 for random access and low delay.

Results for ECM-11 are also reported. On SFU-HW, using ECM-11 is reported to give a BD-rate result of around −14% for random access and −10% for both low delay and all intra configurations. On TVD, ECM-11 is reported to give a BD-rate result of around −15% for random access, −7% for low delay, and −16% for all intra configurations.

In version 2 of this contribution results for ECM-11 for TVD in AI configuration were added.

# Introduction

At the JVET meeting in October 2023 it was decided to utilize the most recent VTM version as anchor for the study on optimizations for encoders and receiving systems for machine analysis of coded video content. At that point VTM-22 was available and the templates in the CTC document [1] include that version.

In this contribution we compare these anchor results to the originally used VTM-12 anchor, which we previously did with the VTM versions 13.0 to 20.0 in JVET-AE0096 [2]. As VTM-23 was released in November 2023, we include it in this contribution as well.

For further information, we also tested ECM-11 in the same manner to see how large of a difference to VTM there is in this specific application.

# VTM Results

Tests were performed according to CTC [1] and compared to VTM-12.0. It was noted that VTM-21.0 and VTM-22.0 in their default configuration generate the same bitstreams as VTM-20.0, thus results are identical. Detailed results can be found in Table 1 – Table 6 and a summary in Table 7. Underlined results indicate usage of the Pareto front, i.e., not all data points are used to generate monotonic RD-curves.

Note that in version 2 of this contribution a bug in the VTM-13 results for TVD was noticed and fixed.

Table 1. Results for SFU-HW sequence-wise in random access configuration

| Sequence | VTM13 | VTM14/15 | VTM16 | VTM17/18 | VTM19 | VTM20/21/22 | VTM23 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic | 2.62% | −0.66% | −2.91% | −20.36% | −20.36% | −23.43% | −23.43% |
| ParkScene | 14.28% | 14.51% | 17.66% | 3.72% | 3.72% | 21.79% | 21.99% |
| Cactus | 4.59% | 7.71% | −11.36% | 8.04% | 8.04% | −1.71% | −3.66% |
| BasketballDrive | 6.63% | 4.86% | −2.33% | 3.05% | 3.05% | 0.31% | −2.14% |
| BQTerrace | −4.37% | −6.04% | −5.42% | −7.16% | −7.16% | −6.82% | −5.57% |
| BasketballDrill | −0.76% | −6.68% | −3.78% | −8.81% | −8.81% | −2.21% | −2.89% |
| BQMall | −0.05% | 9.62% | 9.81% | −1.46% | −0.98% | 5.00% | 0.10% |
| PartyScene | 4.42% | −11.37% | 2.38% | −5.10% | −5.10% | −7.10% | −12.88% |
| RaceHorsesC | 14.47% | 7.49% | 10.11% | −1.23% | −1.23% | −5.07% | −5.94% |
| BasketballPass | −5.64% | 0.19% | 5.25% | −2.77% | −2.77% | −7.35% | 0.77% |
| BQSquare | −0.38% | −0.07% | −6.45% | −2.78% | −2.78% | −0.58% | 1.02% |
| BlowingBubbles | −8.46% | 3.23% | −6.07% | −1.62% | −1.62% | −4.40% | −6.43% |
| RaceHorsesD | −3.78% | 3.76% | 0.63% | −10.39% | −10.39% | −8.51% | −13.37% |
| Average | 1.81% | 2.04% | 0.58% | −3.61% | −3.57% | −3.08% | −4.03% |
| Y-PSNR | −0.31% | −0.50% | −0.37% | −0.88% | −0.88% | −0.94% | −0.91% |

Table 2. Results for TVD in random access configuration

| Sequence | VTM13 | VTM14/15 | VTM16 | VTM17/18 | VTM19 | VTM20/21/22 | VTM23 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TVD-01_01 | 3.23% | 4.54% | −6.13% | 3.55% | 3.50% | 4.44% | −3.45% |
| TVD-01_02 | −10.82% | −9.03% | 6.31% | 16.60% | 16.70% | −3.73% | −3.80% |
| TVD-01_03 | −7.12% | −3.53% | −21.47% | −10.43% | −10.43% | −8.20% | −8.23% |
| TVD-02_01 | 43.11% | 21.39% | 21.46% | −2.51% | −2.52% | 10.63% | 12.87% |
| TVD-03_01 | 4.06% | −4.31% | 5.00% | 1.11% | 1.10% | 0.65% | 0.90% |
| TVD-03_02 | −16.43% | −17.54% | −11.49% | −14.93% | −12.99% | −8.79% | −16.45% |
| TVD-03_03 | 7.66% | 3.39% | 0.62% | −8.31% | −6.97% | −5.80% | −0.43% |
| Average | 3.38% | −0.73% | −0.81% | −2.13% | −1.66% | −1.54% | −2.65% |
| Y-PSNR | −0.16% | −0.14% | −0.71% | −1.67% | −1.68% | −1.71% | −1.70% |

Table 3. Results for SFU-HW sequence-wise in low delay configuration

| Sequence | VTM13/14 | VTM15 | VTM16 | VTM17/18 | VTM19 | VTM20/21/22 | VTM23 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic | 0.00% | −2.95% | −18.39% | −0.77% | −0.77% | −2.53% | −2.53% |
| ParkScene | −0.12% | −14.48% | −5.26% | −10.88% | −10.88% | −4.77% | −1.29% |
| Cactus | 0.00% | 2.01% | −7.86% | 8.82% | 8.82% | 5.87% | 6.20% |
| BasketballDrive | 0.00% | 4.71% | 6.68% | 9.81% | 9.81% | 4.95% | 2.57% |
| BQTerrace | 0.00% | −8.69% | −0.06% | −3.14% | −3.14% | −7.12% | −6.66% |
| BasketballDrill | 0.00% | −0.75% | 0.78% | −0.18% | −0.36% | 1.72% | −5.99% |
| BQMall | 0.12% | −6.51% | −1.91% | −6.31% | −5.97% | −3.13% | −9.30% |
| PartyScene | 0.00% | −7.96% | −4.39% | −6.54% | −6.54% | −7.71% | −17.56% |
| RaceHorsesC | 0.00% | 7.98% | 7.72% | 4.06% | 4.06% | −2.84% | −4.60% |
| BasketballPass | −0.13% | −14.38% | 0.36% | −2.18% | −2.18% | −7.91% | −5.97% |
| BQSquare | 0.00% | 6.61% | 4.29% | −13.43% | −13.43% | −8.87% | 10.50% |
| BlowingBubbles | 0.00% | −4.07% | 3.51% | 8.79% | 8.79% | −1.88% | 3.01% |
| RaceHorsesD | 0.00% | 0.80% | −0.16% | −4.24% | −4.24% | −5.67% | −7.64% |
| Average | −0.01% | −2.90% | −1.13% | −1.25% | −1.23% | −3.07% | −3.17% |
| Y-PSNR | 0.00% | −0.24% | −0.12% | −0.92% | −0.93% | −1.02% | −1.00% |

Table 4. Results for TVD in low delay configuration

| Sequence | VTM13/14 | VTM15 | VTM16 | VTM17/18 | VTM19 | VTM20/21/22 | VTM23 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TVD-01_01 | 1.64% | 18.93% | 7.98% | −11.36% | −10.54% | −7.63% | 5.74% |
| TVD-01_02 | 0.51% | −10.20% | −19.01% | −21.73% | −21.73% | −15.55% | −9.44% |
| TVD-01_03 | −1.22% | 14.34% | 1.40% | −20.86% | −16.87% | −6.27% | −8.88% |
| TVD-02_01 | −10.45% | −13.64% | −10.98% | −35.46% | −35.46% | −23.50% | −23.50% |
| TVD-03_01 | 0.21% | 2.05% | −2.00% | −6.08% | −6.36% | −2.95% | −6.56% |
| TVD-03_02 | 1.52% | −41.30% | −4.36% | −1.91% | 5.86% | −9.41% | −14.47% |
| TVD-03_03 | −1.35% | 4.36% | 9.16% | −8.15% | −6.20% | −5.18% | −18.62% |
| Average | −1.30% | −3.64% | −2.54% | −15.08% | −13.04% | −10.07% | −10.82% |
| Y-PSNR | 0.00% | −0.26% | −0.93% | −4.58% | −4.57% | −4.65% | −4.65% |

Table 5. Results for SFU-HW sequence-wise in all intra configuration

| Sequence | VTM13/14/15 | VTM16/17/18/19 | VTM20/21/22/23 |
| --- | --- | --- | --- |
| Traffic | 0.00% | −5.92% | −7.69% |
| ParkScene | 0.00% | 0.15% | 9.24% |
| Cactus | 0.00% | 5.76% | 7.04% |
| BasketballDrive | 0.00% | 1.17% | 1.57% |
| BQTerrace | 0.00% | 2.39% | 3.79% |
| BasketballDrill | 0.00% | −0.95% | 2.60% |
| BQMall | 0.00% | 1.69% | 4.02% |
| PartyScene | 0.00% | 4.85% | 4.00% |
| RaceHorsesC | 0.00% | 5.38% | 7.42% |
| BasketballPass | 0.00% | −4.01% | −2.17% |
| BQSquare | 0.00% | −1.64% | −0.87% |
| BlowingBubbles | 0.00% | −1.43% | 0.01% |
| RaceHorsesD | 0.00% | 6.05% | 6.86% |
| Average | 0.00% | 1.04% | 2.76% |
| Y-PSNR | 0.00% | −0.22% | −0.21% |

Table 6. Results for TVD in all intra configuration

| Sequence | VTM13/14/15 | VTM16/17/18 | VTM19 | VTM20/21/22/23 |
| --- | --- | --- | --- | --- |
| TVD-01_01 | 0,03% | 1.39% | 1.23% | −2.84% |
| TVD-01_02 | 0,10% | −4.52% | −4.52% | −3.10% |
| TVD-01_03 | 0,11% | −6.20% | −6.21% | −11.33% |
| TVD-02_01 | −0,24% | 15.20% | 15.20% | 11.86% |
| TVD-03_01 | −0,30% | 0.97% | 0.97% | 0.30% |
| TVD-03_02 | −0,48% | −2.85% | −2.85% | −0.15% |
| TVD-03_03 | 0,25% | −4.80% | −4.80% | −8.19% |
| Average | −0,08% | −0.12% | −0.14% | −1.92% |
| Y-PSNR | 0.00% | −0.93% | −0.93% | −0.93% |

Table 7. Summarized results for all configurations

| Dataset / Configuration | VTM13 | VTM14 | VTM15 | VTM16 | VTM17/18 | VTM19 | VTM20/21/22 | VTM23 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SFU-HW / RA | 1.81% | 2.04% | 2.04% | 0.58% | −3.61% | −3.57% | −3.08% | −4.03% |
| SFU-HW / LD | −0.01% | −0.01% | −2.90% | −1.13% | −1.25% | −1.23% | −3.07% | −3.17% |
| SFU-HW / AI | 0.00% | 0.00% | 0.00% | 1.04% | 1.04% | 1.04% | 2.76% | 2.76% |
| TVD / RA | 3.38% | −0.73% | −2.12% | −0.81% | −2.13% | −1.66% | −1.54% | −2.65% |
| TVD / LD | −1.30% | −1.30% | −3.64% | −2.54% | −15.08% | −13.04% | −10.07% | −10.82% |
| TVD / AI | −0.08% | −0.08% | −0.08% | −0.12% | −0.12% | −0.14% | −1.92% | −1.92% |

# Additional results

Besides VTM we also tested ECM-11, which was released in November. Results compared to VTM-12.0 can be found in Table 8 for the SFU-HW dataset and in Table 9 for TVD.

It was noted that for Low Delay, the ECM encoder crashed when using a QP of 55 or higher. This affects the last test point for Traffic, BasketballDrive, BQTerrace, and Kimono, and the last two test points for Cactus. BD-mAP is determined for these sequences using the Pareto front, which excludes the points where no results are produced. The Y-PSNR average is determined using five or four rate points, respectively, for both anchor and test, i.e., anchor and test use the same number of rate points.

Table 8. Results for SFU-HW sequence-wise for ECM-11.0

| Sequence | Random access | Low Delay | All Intra |
| --- | --- | --- | --- |
| Traffic | −13.90% | −0.77% | −15.32% |
| ParkScene | 40.44% | 8.37% | 9.44% |
| Cactus | −33.06% | −9.53% | −23.64% |
| BasketballDrive | −19.35% | −8.33% | −7.43% |
| BQTerrace | −21.49% | −19.30% | −17.92% |
| BasketballDrill | −24.57% | −17.43% | −14.64% |
| BQMall | −16.77% | −11.41% | −11.74% |
| PartyScene | −16.27% | −22.04% | −14.78% |
| RaceHorsesC | 3.09% | 14.28% | −1.53% |
| BasketballPass | −23.05% | −13.57% | −8.83% |
| BQSquare | −19.12% | −28.74% | −20.12% |
| BlowingBubbles | −16.42% | −15.59% | −1.93% |
| RaceHorsesD | −22.21% | −9.44% | 2.02% |
| Average | −14.05% | −10.27% | −9.73% |
| Y-PSNR | −19.22% | −16.22% | −12.52% |

Table 9. Results for TVD for ECM-11.0

| Sequence | Random access | Low Delay | All Intra |
| --- | --- | --- | --- |
| TVD-01_01 | −15.68% | −14.98% | −20.07% |
| TVD-01_02 | −23.28% | −41.87% | −30.11% |
| TVD-01_03 | −36.78% | −29.49% | −28.38% |
| TVD-02_01 | −18.14% | −29.29% | −2.87% |
| TVD-03_01 | 6.59% | −2.13% | −5.42% |
| TVD-03_02 | −11.47% | 23.81% | −4.96% |
| TVD-03_03 | −5.82% | 42.29% | −17.24% |
| Average | −14.94% | −7.38% | −15.58% |
| Y-PSNR | −12.27% | −16.12% | −14.42% |

# Patent rights declaration(s)

Ericsson does not have any current or pending patent rights relating to the technology described in this contribution.

# References

| [1] | S. Liu and C. Hollmann, "Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content," JVET-AF2031, Hannover, DE, October 2023. |
| --- | --- |
| [2] | C. Hollmann and J. Ström, "[AHG8] Study on using different VTM versions," JVET-AE0096, Geneva, CH, July 2023. |